| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0116r1 |
| --- | --- |

| Title: | 2D compatible frame packing stereo 3D video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaofeng Yang, Peiyu Yue, Yuanyuan Zhang Nanjing R&D Center, 101 Software Avenue, Yuhuatai District, Nanjing, P.R.China 200012 | Tel: Email: | +86 25 56620730 yx.yangxiaofeng@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

Frame compatible stereo 3D video was supported by current draft with frame packing arrangement SEI (FPA_SEI). In cases where video stream is displayed only on 2D display due to display set restriction or user preference, half of each decoded picture in the stream is discarded before output. This is a big waste for decoder’s computing and buffer resources.

This contribution proposes a solution with following steps:

1) the bistream is encoded in such a way that there is a region in the coded pictures which can be independently decoded;

2) signal the decoder an independent decodable region and optionally the corresponding profile and level in this stream;

3) decoder for 2D display extracts the sub-bitstream of the region according to the signaling, then decodes the sub-bitstream.

# Introduction

The current draft supports frame compatible stereo 3D coding by frame packing arrangement SEI (FPA_SEI), which is defined as the same as SEI defined in AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10). In the 9th and 10th meeting, JCTVC-I0072, JCTVC-J0070 and JCTVC-I0056 suggested adding explicit cropping information to extract output image region instead of frame packing type only, such as to provide more flexibility and functionality. By setting the cropping window on the output view image position of side-by-side (or top-bottom) coding bitstream, standard cropping process can directly be substituted. One issue raised is that when video stream is displayed only on 2D display half of every decoded picture in the stream is discarded before output, which is a big waste for decoder’s computing and buffer resources.

Another issue is that, if only half of the pictures in a stream need to be decoded, the profile and level of the stream may reduce to that of the decoded region; this expands the scope of decoding capabilities for decoders conforming to lower levels or profiles when decoding a frame compatible stereo 3D video stream.

# Proposal

In order to solve the above 2 issues, this contribution proposes to add a message in the stream to inform the decoder that a independent decodable region exists in this stream. The independent decodable region satisfies the following restrictions:

- For a give combination of frame packing type and flipping flag (both are syntax from FPA_SEI), region position keeps the same along the entire bitstream.

- Inner-region PUs reference to inner-region PUs of the frame that corresponds to the same  frame packing type and the same flipping flag

- Cross region in-loop filtering is disabled

Information along with the independent decodable region includes:

- Region position

- Cropping information for display (optional)

- Profile and level of the region (optional)

Two merits are achieved by this solution when decoder output to 2D display only:

- Computing and buffer resources are saved for decoders

- The profile and level of the bitstream may be reduced if only half of the pictures in the stream are to be decoded.  In such a way, decoders in some low-end 2D devices would be possible to process the bitstream.

## 2.1 	Independent decodable region SEI message syntax and semantics

Define one or more tiles as the independent decodable region, encapsulate information in a new SEI, it is tentatively named as indec_rgn_sei, this SEI message is sent to decoder once when a picture is of different frame packing type or flipping flag than the proceeding one.

Decoder knows the existence of an independent decodable region in the stream by indec_rgn_sei, and extracts and decodes the region alone before output to 2D display.

Syntax and Semantics

| INDEC_RGN_SEI(payloadSize){ | Descriptor |
| --- | --- |
| tile_num | ue(v) |
| for (i=0;i<tile_num;i++){ |  |
| tile_ids[i] | ue(v) |
| } |  |
| cropping_enable_flag | u(1) |
| new_profile level_flag | u(1) |
| if (cropping_enable_flag) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| if (new_profile_level_flag) { |  |
| profile_and_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |

tile_num the number of tiles the indepent decodable region includes

tile_ids[i] tile id in of the tiles in the region

cropping_enable_flag equals to 1 indicates cropping is required before output

new_profile_level_flag equals to 1 indicates the independent decodable region conforming to a new profile or level than the bitstream

pic_crop_left_offset specifies the offset in pixel of the left border of the output area to the left border of the region

pic_crop_right_offset specifies the offset in pixel of the right border of the output area to the left border of the region

pic_crop_top_offset specifies the offset in pixel of the top border of the output area to the top border of the region

pic_crop_bottom_offset specifies the offset in pixel of the bottom border of the output area to the top border of the region

profile_and_level( 1, sps_max_sub_layers_minus1 ) contains the profile and level information for correctly decoding the independent decodable region.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).