| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0117 |
| --- | --- |

| Title: | Parameter ID change information SEI for bitstream splicing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaofeng Yang, Peiyu Yue, Yuanyuan Zhang Nanjing R&D Center, 101 Software Avenue, Yuhuatai District, Nanjing, P.R.China 200012 | Tel: Email: | +86 25 56620730 yx.yangxiaofeng@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

During splicing, two bitstreams may refer to a few parameter sets with the same ID for each type of parameter sets but with different content. This causes fatal error when decoding the second bitstream with a parameter set of the first stream if parameter sets are transmitted out-of-band. This contribution proposes to send to the receiver a message containing flags indicating which parameter set is changed and optionally their information about the new IDs, with which the decoder automatically changes the old IDs to the new IDs before decoding. This solution supports a lightweight splicer without entropy encoding and decoding capability, besides, it requires minor revision to the current HEVC draft specification.

# Introduction

Out-of-band parameter transmission is a typical reliable solution when parameters changes infrequently in a bitstream. When splicing, parameter sets of the first bitstream may have the same IDs as those of the second bitstream, since parameter sets are uniquely identified by their IDs, pictures of the second bitstream who refer to the parameter sets of the second bitstream will now be redirected to that of the first bistream, which causes fatal decoding error.

Detailed introduction of parameter ID problems caused by splicing is stated in JCTVC-J0108 of the 10th meeting. It also proposes a constant length coding solution for parameter IDs along with a group of syntax changes and additional restrictions on the current HEVC draft. It changes the parameter set ID of the second bitstream into a new ID and changes old ID into the new ID in every packet wherever the parameter set of the second bitstream is referenced.

# Proposal

This contribution proposes to define a new SEI message for splicing; this SEI message is inserted into the bitstream when splicing to inform the decoder which parameter set ID of the following packets is changed along with information about the new ID. The parameter set of the second bitstream is transmitted out-of-band. When the decoder receives the new SEI message, it changes the parameter set ID of the following packets in decoding order into the new ID before decoding, decode picture with the new parameter set ID and output the decoded picture.

By this way, a light splicer only needs to transmit the new parameter set out-of-band and insert a SEI message into the bitstream to inform the decoder the parameter ID change. Compared with the JCTVC-J0108 solution, higher encoding efficiency is ensured and only a minor revision needs to be applied on the current HEVC draft specification.

# Syntax and Semantics

The new SEI is tentatively named as parameter_set_id_change_info.

| parameter_set_id_change_info (PayloadSize) { | Descriptor |
| --- | --- |
| aps_id_change_flag | u(1) |
| pps_id_change_flag | u(1) |
| If (aps_id_change_flag) |  |
| aps_id_diff | u(5) |
| If (pps_id_change_flag) |  |
| pps_id_diff | u(8) |
| } |  |

aps_id_change_flag  specifies whether value of aps needs to change before decoding, if it equals to 1, an offset of the changed aps value to the original value is followed.

pps_id_change_flag  specifies whether value of pps needs to change before decoding, if it equals to 1, an offset of the changed pps value to the original value is followed.

aps_id_diff  offset of the new aps id from the original aps id, if the original aps id of a packet is aps_id, the new aps id is yielded with aps_id+ aps_id_diff.

pps_id_diff offset of the new pps id from the original pps id, if the original pps id of a packet is pps_id, the new pps id is yielded with aps_id+ pps_id_diff

The new aps id and new pps id refers to the aps and pps of the second bistream which is transmitted out-of-band.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).