| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0118 |
| --- | --- |

| Title: | Encoding complexity reduction by removal of some unnecessary CU segmentations and partition types based on spatial and temporal correlation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaohai He Guoyun Zhong Yuan Li Linbo Qing Di Wu | Tel: Email: | 86 028 85462766 nic5602@scu.edu.cn |
| Source: | Sichuan University, China |  |  |

_____________________________

# Abstract

In this document, an encoding complexity reduction method is presented by removal of some unnecessary CU segmentations and partition types based on spatial and temporal correlation. The temporal correlation between the corresponding-located (co-located) 64×64 pixels size block and the current 64×64 pixels size block is used to skip the unnecessary partition modes for current CU. The spatial correlation between the co-located block and its surrounding CUs is used to skip the unnecessary deeper depths of CU segmentation. This contribution brings 25%-28% total coding time saving for class B and E.

# Introduction

In HEVC, the picture is divided into slices. Each slice is composed of lines of largest coding units (LCUs). The LCU size is 64×64 pixels and its notion is generally similar to that of a macroblock in H.264/AVC. Fig.1 shows an example of splitting a 64×64 LCU into various sizes of CUs. A LCU is defined at the first depth level (CUDepth=0) and it can be split into four identical sub-CUs with 32×32 pixels size. These four sub-CUs are at the second depth level (CUDepth=1) and each one can be split into four identical sub-CUs with 16×16 pixels size. These four sub-CUs are said to belong to the third depth level (CUDepth=2) and each one can still be divided into four sub-CUs with 8×8 pixels size. These four sub-CUs are at the fourth and maximum split depth level (CUDepth=3) [1], which means that the CUs of 8×8 pixels size are the smallest CU in size.

Fig.1 hierarchical quadtree structure of CU in HEVC

Along with CU, HEVC introduces a basic prediction unit (PU) for carrying the information related to the prediction process. The PU is defined only for the leaf node CU which is not split into deeper depth of sub-CUs. The size of the PU is less than or equal to that of the CU. It is not restricted to be square in shape, in order to facilitate partitioning which matches the boundaries of objects within the picture. The PU splitting depends on the prediction mode. Fig.2 shows the eight PU partition modes for a CU with a size of 2N×2N in inter prediction mode. Each CU may contain one or more PUs depending on PU partition mode. CU with PART_2Nx2N contains a PU. CU with PART_N×N contains 4 PUs. The other prediction mode contains 2 PUs, respectively. For CU with 64×64 pixels size, only partition modes PART_2N×2N, PART_2N×N and PART_N×2N are adopted in order to maintain lower computational complexity. This strategy depends on the assumption that the objects in the region of the CU have the same motion direction in case of the CU with 64×64 pixels size is the leaf node one. Since the objects have similar motion directions, the three abovementioned partition modes are enough for inter prediction. For CUs with sizes of 32×32 and 16×16 pixels, there are seven partition modes except for PART_N×N mode [2]. The reason why PART_N×N is removed in CUs with sizes larger than 8×8 pixels is that the size of PU in PART_N×N mode in upper CU depth level is equal to that of CU in lower CU depth. They have the same meaning in inter prediction and this partition is redundant for the CUs with the three sizes. All the partition modes are remained in CU with 8×8 pixels size [3].

Fig.2    Eight partition modes

Even though this CU segmentation and partition method exhibits the best possible coding efficiency, its computational complexity is tremendous which can be seen from the RD cost calculation abovementioned.

To reduce the computational complexity, several fast inter mode decision algorithms have been proposed for HEVC. A SKIP mode was added into the mode decision process in [4], thus skipping the sub-tree computations and achieving a saving of approximately 40% in coding time. In [5], the other PU modes except for 2N×2N is skipped when the cbf is zero(0), which means all the AC coefficients are zeros after quantization of the CU. This algorithm reduces 60% coding time.

# Proposed algorithm

There is a high degree of similarity between current CU and its co-located block, the segmentation mode of the former dependent on that of the latter. It can be assumed that if the objects in the co-located LCU and its surrounding CUs are static or have the same motion, the deeper depth of CU segmentation can be skipped except for current depth of CU. To validate our hypothesis, a condition is set up, under which the deeper depth CU is skipped as shown in Fig.3. In fig.3, the co-located block and its surrounding CUs have 64×64 pixels sizes. In this case, it is regarded as possible that the objects in the co-located are static or have the same motion vector because the surrounding CUs have the largest size. It can also be imaged that the objects in the surrounding CUs are also static or have the same motion vector. Considering the motion in time domain, some of the objects in the current CU may have move from the surrounding CUs in the previous. All the objects in current CU are still static or have the same motion vector because all the objects in the co-located block and its surrounding CUs are considered to be static and have the similar motion vectors. Therefore, the method skipping the deeper depth CUs is designed as follows. If the co-located CU and its four surrounding CUs are 64×64 pixels size and that the partition modes of the surrounding CUs are PART_2N×2N, only 64×64 pixels size CU depth level is evaluated and the deeper depths of CU segmentation are skipped. The reason why only the size of 64×64 pixels are defined is that the CU sizes less than 64×64 pixels (e.g. 32×32, 16×16 and 8×8 pixels) mean that the objects are not static and have various motion vectors. In the consideration of that there are only three partition modes for 64×64 pixels size CU, all the three partition modes are evaluated in this case.

Fig.3  condition of skipping the deeper depths of CUs

# Experiments

## Test conditions

The common test conditions [6] are used.

## Simulation results

Table1 presents the simulation results.

Table 1 Coding efficiency of the contribution compared to HM8.0

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | -0.6% | -0.5% | 0.3% | -0.5% | -0.8% |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E | 0.9% | -0.3% | 0.1% | 0.7% | 0.1% | -0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.6% | -0.4% | -0.1% | 0.6% | -0.2% | -0.5% |
|  | 0.6% | -0.5% | -0.2% | 0.5% | -0.2% | -0.4% |
| Enc Time[%] | 75% |  |  | 72% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.5% | -0.7% | 0.2% | -0.4% | -0.2% |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E | 0.6% | -0.6% | -0.7% | 0.6% | -0.1% | -0.4% |
| Class F |  |  |  |  |  |  |
| Overall | 0.4% | -0.5% | -0.7% | 0.4% | -0.2% | -0.3% |
|  | 0.4% | -0.5% | -0.7% | 0.4% | -0.2% | -0.3% |
| Enc Time[%] | 74% |  |  | 74% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusion

It has be shown how to skip some unnecessary CU segmentations and partition types based on spatial and temporal correlation. The simulation results have shown that this method can achieve 25%-28% total coding time saving for class B and E than HM8.0.

# References

[1] G. Correa, P. Assuncao, L.Agostini, and L. A. D. S. Cruz, “Complexity control of high efficiency video encoders for power-constrained devices,” IEEE Trans. Consum. Electron., vol.57, no. 4, pp. 1866–1874, Nov. 2011.

[2] I.-K. Kim, K. McCann, K. Sugimoto, B. Bross, W.-J. Han, “High efficiency video coding (HEVC) test model 7 (HM 7) encoder description,” JCTVC-I1002, JCTVC of ISO/IEC and ITU-T. Geneva, Switzerland, Apr. 2012.

[3] W.-J. Han, J. Min, I. K. Kim, E. Alshina, A. Alshin, T. Lee, J. Chen, V. Seregin, S. Lee, Y. M. Hong, M.-S. Cheon, N. Shlyakhov, K. M. Can, T. Davies, and J.-H. Park, “Improved video compression efficiency through flexible unit representation and corresponding extension of coding tools,” IEEE Trans. Circuits Syst. Video Technol., vol. 20, no. 12, pp. 1709–1720, Dec. 2010.

[4] K. Choi, S.-H. Park, and E.S. Jang, Coding tree pruning based CU early termination, JCTVC-F092, JCTVC of ISO/IEC and ITU-T. Torino, Italy, Jul. 2011.

[5] R. H. Gweon Y.-L. Lee, Early termination of CU encoding to reduce HEVC complexity, JCTVC-F045, JCTVC of ISO/IEC and ITU-T. Torino, Italy, Jul. 2011.

[6] JCTVC-J1100, “HM 8 Common Test Conditions and Software Reference Configurations,” Stockholm, SE, 11–20 July 2012.

# Patent rights declaration(s)

Sichuan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).