| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0120r1 |
| --- | --- |

| Title: | AHG9: High-level syntax clean-ups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes some high-level syntax clean-ups, related to NAL unit header, SPS syntax, PPS and slice header syntaxes, end of sequence RBSP semantics, VPS and SPS activation, output order and POC, restrictions of SPS ID and PPS ID values in the Main profile, and SEI messages.

Revision 1 of this contribution provides spec text changes for the proposal in subsection 1.5 and items 2 to 2 in subsection 1.8.

# Proposed clean-ups

## NAL unit header clean-ups

- Add (to the end of the subclause for NAL unit header semantics) a restriction to say that the NAL unit header of a dependent slice shall be identical to that of its parent slice, as follows:

It is a requirement of bitstream conformance that, when a VCL NAL unit has dependent_slice_flag equal to 1, the nal_unit_header( ) syntax structure shall be identical to the nal_unit_header( ) syntax structure of the previous VCL NAL unit, in decoding order, that has dependent_slice_flag equal to 0.

- Make the reserved NAL unit types 21..24 also as VCL NAL units, such that there first cluster of reserved NAL unit types (15..24) are all reserved VCL NAL units, and the second cluster of reserved NAL unit types (33.47) are all reserved non-VCL NAL units. This had actually been the intent at the first place to make the reserved NAL unit types as two clusters before the Stockholm meeting, at which it was agreed to make NAL unit types 15..20 reserved VCL NAL unit types, but without knowing the HRD problem pointed out at the Stockholm meeting by Miska Hannuskela in JCTVC-J0074.

## SPS syntax

- Move sps_temporal_id_nesting_flag to the beginning to replace reserved_zero_bit, which was there only for the purpose of byte alignment for the profile information in the SPS.

- The lists_modification_present_flag is present in the SPS when restricted_ref_pic_lists_flag is equal to 1. The semantics of the two flags are as follows:

restricted_ref_pic_lists_flag equal to 1 indicates that all P and B slices (if present) that belong to the same picture have an identical reference picture list 0, and that all B slices (if present) that belong to the same picture have an identical reference picture list 1.

lists_modification_present_flag equal to 0 specifies that the syntax structure ref_pic_list_modification( ) is not present in the slice header. lists_modification_present_flag equal to 1 specifies that the syntax structure ref_pic_list_modification( ) is present in the slice header. When not present, the value of lists_modification_present_flag is inferred to be equal to 1.

Though they are somewhat related, but the two conditions are still basically orthogonal, as the four possible combinations of 00, 10, 10, and 11 are all meaningful.

Therefore, it is proposed to either remove lists_modification_present_flag or make the persence of restricted_ref_pic_lists_flag independent of restricted_ref_pic_lists_flag.

- Change tiles_fixed_structure_flag to 2-bit tiles_fixed_structure_idc, such that whether all PPSs referred in the coded video sequence have tiles_enabled_flag equal to 0 is signalled in addition to whether they have the same tiles structure.

- It is proposed to add a flag to VUI to indicate whether POC value differences are proportional to presentation time differences, and if the flag is equal to 1, add the number of ticks minus 1 that corresponds to any POC difference equal to 1.

With this information, decoders or other entities may be able to figure out the true presentation time difference between pictures for purposes like verifying system timing, better error concealment, and so on.

## PPS and slice header syntaxes

- It is proposed to discuss whether to allow pic_output_flag equal to 0 for all pictures in a coded video sequence or a bitstream.

- Include a flag in the PPS to control the presence of num_ref_idx_active_override_flag in the slice header, such that when not needed, num_ref_idx_active_override_flag does not have to be signalled in each slice header. This not only saves one bit for each slice header in those cases without the need of the overriding, which is actually typical, but also provides a convenient way to signal that all slices in a picture actually use the same number of entries in each reference picture list.

- The pic_temporal_mv_enabled_flag is signalled for B or P slices, not for I slices. The semantics of pic_temporal_mv_enabled_flag includes the following sentence:

When both slice_temporal_mvp_enable_flag and TemporalId are equal to 0, all coded pictures that follow the current picture in decoding order shall not use temporal motion vectors from any picture that precedes the current picture in decoding order.

As can be seen, the flag not just specifies an inter prediction behavior of the current picture itself, but also possibly for future pictures. In particular, it specifies the refreshing or flushing of motion information. Therefore, it also applies to I slices, and should typically be associated with RAP pictures. However, the current syntax does not allow specifying such refreshing or flushing behavior for intra pictures, including CRA and BLA pictures. If the encoder finds that disabling temporal motion prediction is desirable starting from an intra picture, it cannot specifies it immediately, but has to wait for the next non-intra picture with TemporalId equal to 0.

Therefore, it is proposed to include pic_temporal_mv_enabled_flag in the slice header also for I slices, under the same condition as for B or P slices.

- It is proposed to remove the text for entropy slices.

## End of sequence RBSP semantics

The semantics of end of sequence RBSP semantics should be changed as follows:

When the next subsequent NAL unit in decoding order is not an end of bitstream NAL unit, tThe end of sequence RBSP specifies that the next subsequent access unit in the bitstream in decoding order (if any) is an IDR or BLA access unit. The syntax content of the SODB and RBSP for the end of sequence RBSP are empty.

## VPS and SPS activation

HRD parameters may come from the VPS. However, the buffering period SEI message contains an SPS ID but does not contain a VPS ID. When the HRD initializes from an access unit that contains a buffering period SEI message and the chosen HRD parameters for the HRD operation are from the VPS, parsing of syntax elements in the buffering period SEI message would depend on information in the VPS, not in the SPS. In this case, parsing of the buffering period SEI message would require firstly parsing the SPS, find the VPS ID, then parse the VPS, and finally parse the buffering period SEI message itself.

The active parameter sets SEI message can include both the active VPS ID and the active SPS ID. It would be more straightforward to use the active parameter sets SEI message to activate both the VPS and the SPS, instead of relying on the buffering period SEI message and the indirect activation.

It is therefore proposed to remove the SPS ID from the buffering period SEI message, and specify VPS and SPS activation through the parameter sets active SEI message, which is actually more lightweight than the picture buffering SEI message as it only contains the IDs.

The proposed syntax and semantics changes are as follows:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| seq_parameter_set_id | ue(v) |
| ... |  |
| } |  |

| active_parameter_sets( payloadSize ) { | Descriptor |
| --- | --- |
| active_vps_id | u(4) |
| num_sps_ids_minus1 | ue(v) |
| for( i = 0; i <= num_sps_ids_minus1; i++ ) |  |
| active_sps_id[ i ] | ue(v) |
| active_sps_id_present_flag | u(1) |
| if( active_sps_id_present_flag ) |  |
| active_seq_param_set_id | ue(v) |
| active_param_set_sei_extension_flag | u(1) |
| } |  |

num_sps_ids_minus1 plus 1 specifies the number of sequence parameter sets (some of which may also be referred to as active layer sequence parameter sets) that are active for the VCL NAL units of the access unit associated with the SEI message. The value of num_sps_ids_minus1 in the range of 0 to 31, inclusive.

active_sps_id[ i ] specifies the identifier of the i-th sequence parameter set that is active for the VCL NAL units of the access unit associated with the SEI message. The value of active_sps_id[ i ] shall be in the range of 0 to 31, inclusive.

The proposed active parameter sets SEI message syntax enables ignalling of multiple SPS IDs. This way, the same syntax can be applied to HEVC extensions for multiview, 3DV and/or scalable video coding extensions without any change. In a bitstream of a base HEVC spec profile, only one bit is needed to code num_sps_ids_minus1 (as the value is equal to 0).

## Output order and POC

### Definitions of output order and picture order count

The current definition of output order is as follows:

output order: The order in which the decoded pictures are output from the decoded picture buffer in case the decoded pictures are to be output from the decoded picture buffer. The output order of a picture is specified by the picture order count value, regardless of whether the picture is to be output.

The second sentence has caused confusions, particularly when people read restrictions like the following:

- When a picture has PicOutputFlag equal to 1 and precedes a RAP picture in decoding order, it shall also precede the RAP picture in output order.

- Any picture that has PicOutputFlag equal to 1 that precedes a CRA or BLA picture in decoding order shall precede any DLP picture associated with the CRA or BLA picture in output order.

Also, the second is somewhat conflicting with the first sentence in the definition.

It is therefore proposed to remove the second sentence in the definition of output order, which will also solve the multi-sentence editorial problem for the definition.

Accordingly, the definition of picture order count is proposed to be changed as follows:

picture order count: A variable that is associated with each picture that indicates the position of the associated picture in output order relative to the output order positions of the another pictures in the same coded video sequence when both the associated picture and the other picture have PicOutputFlag equal to 1.

### Output order restrictions

Currently, the following two output order restrictions on are included in subclause 7.4.1.2:

- When a picture has PicOutputFlag equal to 1 and precedes a RAP picture in decoding order, it shall also precede the RAP picture in output order.

- Any picture that has PicOutputFlag equal to 1 that precedes a CRA or BLA picture in decoding order shall precede any DLP picture associated with the CRA or BLA picture in output order.

The second restriction listed should be changed to cover IDR pictures too, as IDR pictures can also have DLP pictures, and desire to make the presentation continuous when random accessing with DLP pictures being also displayed is same for all types of RAP pictures. Therefore, the restriction should be changed to the following:

- Any picture that has PicOutputFlag equal to 1 that precedes a RAP CRA or BLA picture in decoding order shall precede any DLP picture associated with the RAP CRA or BLA picture in output order.

For editorial improvement, the above-listed two restrictions should be combined, as follows (by adding the highlighted to the first restriction):

- When a picture has PicOutputFlag equal to 1 and precedes a RAP picture in decoding order, it shall also precede the RAP picture in output order, and it shall precede any DLP picture associated with the RAP picture in output order.

### POC restrictions

Sublcuase 7.4.8.1 includes the following restriction:

The value of DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb + pic_order_cnt_lsb − PocLsbLt[ i ] shall be in the range of 1 to 224 − 1, inclusive.

This was added in reponse to the following adoption item at the 9th JCT-VC meeting:

A limit on the value of the signalled POC difference relative to an LTRP needs to be specified. Specify that the total POC difference of a long-term ref picture relative to the current picture shall be less than 224−1.

However, the current restriction only takes care of LTRPs for which DeltaPocMSBCycleLt[ i ] is derived. Therefore, it is proposed to implement the adoption in a better way, by adding the following restriction as one of the RPS restrictions in subclause 8.3.2:

- There shall no entry in RefPicSetLtCurr for which the difference between the picture order count value of the current picture and the picture order count value of the entry is equal to or greater than 224.

The existing restriction in sublcuase 7.4.8.1 may still be kept there as a restriction to the value of the syntax element delta_poc_msb_cycle_lt[ i ], though with the new restriction being proposed, it becomes redundant.

## Restrictions of SPS ID and PPS ID values in the Main profile

In the current Main profile definition, SPS ID is restricted to be less than or equal to 15, and PPS ID is restricted to be less than or equal to 63. In the semantics, these two values are 31 and 255, respectively.

We understand that the intention for the further restrictions in the profile definition is save some memory for storage of the worse-case SPS and PPS tables at decoders. However, assume that the smaller numbers are not too restrictive; it would be good in any case to have consistent parameter set ID value ranges among different profiles. Otherwise transcoding from a bitstream using a parameter set ID greater than the upper limit, e.g. of a potential future profile, would usually require complete re-encoding for "merging" of the parameter sets.

According to the syntaxes, the worse-case SPS and PPS sizes in HEVC should be in similar magnitudes, respectively, as in AVC High profile. Note that in AVC the two values are also 31 and 255, respectively in semantics, with no further restriction in profile definitions.

Therefore, unless there is a clear conclusion that the values 31 and 255 for SPS and PPS ID values are too expensive to support by envisioned HEVC decoders to be put into the market, the restrictions in the profile definition should be removed. If indeed it is too expensive, the profile restrictions should also be removed, but in this case the values in the semantics should be changed to be smaller.

## SEI messages

- In the end of subclause 7.4.1.4.2 (Order of access units and association to coded video sequences) is the following paragraph:

It is a requirement of bitstream conformance that, when an SEI NAL unit contains data that pertain to more than one access unit (for example, when the SEI NAL unit has a coded video sequence as its scope), it shall be contained in the first access unit to which it applies. [Ed. (GJS): Is this needed?]

To answer the editing question added in the end of the paragraph, we also think this constraint is not needed. In any case, this constraint should not be interpreted as that the SEI NAL unit cannot be repeated later on for example for error resiilence purposes, for which the word "shall" should be "shall and shall only". However, it may be interpreted wrongly that way by many readers. In addition, the presence of each SEI message is anyway clearly specified in the semantics of the SEI message itself. Lastly, this constraint does not really fit the subject implied by the title of this subclause.

It is therefore proposed to remove the paragraph.

- Add the support of suffix SEI NAL units using their own NAL unit type, and specify the decoded picture harsh SEI message as a suffix SEI message. Suffix SEI messages are associated with certain preceding NAL units and shall not precede the first VCL NAL unit in the access unit, while normal (or prefix) SEI messages are associated with certain succeeding NAL units and shall not follow the last VCL NAL unit in the access unit.

- Include an extension flag to the end of all types of SEI messages, such that they all become extensible, with the cost of at most one bit (in 7/8 of all cases, zero bit cost).

- Currently, most types of SEI messages are required to precede the first VCL NAL unit in the AU, with the following language:

When an SEI NAL unit that contains a XXX SEI message and has nuh_reserved_zero_6bits equal to 0 is present, the SEI NAL unit shall precede, in decoding order, the first VCL NAL unit in the access unit.

This restriction disallows those SEI messages to be repeated inside the AU containing the SEI message, after the first VCL NAL unit and before the last NAL unit, in decoding order. However, when an AU includes multiple slices, and they are transported in more than one packet in an error-prone environment, it may be beneficial to include a copy of an SEI message in each packet, to enable recover information conveyed in the SEI message in case other packets of the same AU are lost.

For those SEI messages that apply to multiple AUs, enabling (not mandting) them to be repeated in the applicable AUs is useful.

Furthermore, for these SEI messages that apply to multiple AUs, which are basically sequence-level, it should be restricted that they can only be present in AUs with TemporalId equal to 0, similarly as requiring TemporalId to be 0 for all RAP pictures.

Therefore, we have checked against all the SEI messaegs currently supported in HEV, and propose the restrictions regarding their presence and content in the table below.

The terms scene period and post-filter period are defined as follows (similarly as the current definition of buffering period).

A scene period includes and only includes all AUs from an AU containing a scene information SEI message to the next AU, in decoding order, containing a scene information SEI message or the end of the coded video sequence, whichever is earlier, in decoding order.

A post-filter period includes and only includes all AUs from an AU containing a post-filter hint SEI message to the next AU, in decoding order, containing a post-filter hint SEI message or the end of the coded video sequence, whichever is earlier, in decoding order.

SEI message (payloadType); Application scope; May be present; Multi-instance restriction. Buffering period (0); The buffering period containing the SEI message; In any AU with TemporalId equal to 0; All instances within a buffering period shall be identical. Picture timing (1); The AU containing the SEI message; In any AU with any value of TemporalId; All instances within an AU shall be identical. Pan-scan rectangle (2); Specified by syntax; In any of the applicable AUs with TemporalId equal to 0; All instances within the specified application scope shall be identical. Filler payload (3); The AU containing the SEI message; In any AU with any value of TemporalId; No restriction. User data registered (4); Unspecified; In any AU with any value of TemporalId; No restriction. User data unregistered (5); Unspecified; In any AU with any value of TemporalId; No restriction. Recovery point (6); The AU containing the SEI message; In any AU with TemporalId equal to 0; All instances within an AU shall be identical. Scene information (9); The scene period containing the SEI message; In any AU with TemporalId equal to 0; All instances within an AU shall be identical. Full-frame snapshot (15); The AU containing the SEI message; In any AU with any value of TemporalId; All instances within an AU shall be identical. Progressive refinement segment start (16); The AU containing the SEI message; In any AU with any value of TemporalId; All instances within an AU shall be identical. Progressive refinement segment end (17); The AU containing the SEI message; In any AU with any value of TemporalId; All instances within an AU shall be identical. Film grain characteristics (19); Specified by syntax; In any of the applicable AUs with TemporalId equal to 0; All instances within the application scope shall be identical. Deblocking filter display preference (21); Specified by syntax; In any of the applicable AUs with TemporalId equal to 0; All instances within the application scope shall be identical. Post-filter hint (22); The post-filter period containing the SEI message; In any AU with TemporalId equal to 0; All instances within an AU shall be identical. Tone mapping information (23); Specified by syntax; In any of the applicable AUs with TemporalId equal to 0; All instances within the application scope shall be identical. Frame packing arrangement (45); Specified by syntax; In any of the applicable AUs with TemporalId equal to 0; All instances within the specified application scope shall be identical. Display orientation (47); Specified by syntax; In any of the applicable AUs with TemporalId equal to 0; All instances within the specified application scope shall be identical. Structure of pictures description (128); The SOP containing the AU that contains the SEI message; In any AU with TemporalId equal to 0; All instances within an SOP shall be identical. Field indication (129); The AU containing the SEI message; In any AU with any value of TemporalId; All instances within an AU shall be identical. Decoded picture hash (130); The AU containing the SEI message; In any AU with any value of TemporalId; All instances within an AU shall be identical. Active parameter sets (131); The coded video sequence containing the SEI message; In any AU with TemporalId equal to 0; All instances within a coded video sequence shall be identical. Sub-picture timing (132); The decoding unit containing the SEI message; In any AU with any value of TemporalId; All instances within a decoding unit shall be identical.

# Specification text changes for some of the proposed items

The following spec text changes are proposed, in relative to JCTVC-K0030v4 (additions are highlighted in yellow, and removals are strikethroughs with red fonts).

## Text changes for the proposal in subsection 1.5 (VPS and SPS activation)

### Order of video, sequence, and picture parameter set RBSPs and their activation

...

- A sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more SEI NAL units containing a buffering period an active parameter sets SEI message. Each sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one sequence parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular sequence parameter set RBSP results in the deactivation of the previously-active sequence parameter set RBSP (if any).

- When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period an active parameter sets SEI message (using that value of seq_parameter_set_id), it is activated. This sequence parameter set RBSP is called the active sequence parameter set RBSP until it is deactivated by the activation of another sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0, unless the sequence parameter set is provided through external means. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 1 – Because an IDR or BLA access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by a buffering period an active parameter sets SEI message when the buffering period active parameter sets SEI message is part of an IDR or BLA access unit.

- Any sequence parameter set NAL unit containing the value of seq_parameter_set_id for the active sequence parameter set RBSP for a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period an active parameter sets SEI message (when present) of another coded video sequence.

- A video parameter set RBSP includes parameters that can be referred to by one or more sequence parameter set RBSPs or one or more SEI NAL units containing an active parameter sets SEI message. Each video parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one video parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular video parameter set RBSP results in the deactivation of the previously-active video parameter set RBSP (if any).

- When a video parameter set RBSP (with a particular value of video_parameter_set_id) is not already active and it is referred to by activation of a sequence parameter set RBSP (using that value of video_parameter_set_id) or is referred to by an SEI NAL unit containing an active parameter sets SEI message (using that value of seq_parameter_set_id), it is activated. This video parameter set RBSP is called the active video parameter set RBSP until it is deactivated by the activation of another video parameter set RBSP. A video parameter set RBSP, with that particular value of video_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0, unless the video parameter set is provided through external means. An activated video parameter set RBSP shall remain active for the entire coded video sequence.

- Any video parameter set NAL unit containing the value of video_parameter_set_id for the active video parameter set RBSP for a coded video sequence shall have the same content as that of the active video parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit, and the first sequence parameter set NAL unit, and the first SEI NAL unit containing an active parameter sets SEI message (when present) of another coded video sequence.

- ...

### Syntax and semantics for the active parameter sets SEI message

| active_parameter_sets( payloadSize ) { | Descriptor |
| --- | --- |
| active_vps_id | u(4) |
| num_sps_ids_minus1 | ue(v) |
| for( i = 0; i <= num_sps_ids_minus1; i++ ) |  |
| active_sps_id[ i ] | ue(v) |
| active_sps_id_present_flag | u(1) |
| if( active_sps_id_present_flag ) |  |
| active_seq_param_set_id | ue(v) |
| active_param_set_sei_extension_flag | u(1) |
| } |  |

The active parameter sets SEI message provides indicates which video parameter set is active for the VCL NAL units of the access unit associated with the SEI message. The SEI message may also provide information on which sequence parameter set is active for the VCL NAL units of the access unit associated with the SEI message, and possibly other information.

When an SEI NAL unit that contains an active parameter sets SEI message and has nuh_reserved_zero_6bits equal to 0 is present, the SEI NAL unit shall precede, in decoding order, the first VCL NAL unit in the access unit.

active_vps_id identifies the video parameter set that is active for the VCL NAL units of the access unit associated with the SEI message. The value of active_vps_id shall be in the range of 0 to 15, inclusive.

num_sps_ids_minus1 plus 1 specifies the number of sequence parameter sets that are active for the VCL NAL units of the access unit associated with the active parameter sets SEI message. The value of num_sps_ids_minus1 shall be in the range of 0 to 31, inclusive. In bitstreams conforming to this Specification, num_sps_ids_minus1 shall be equal to 0. Although the value of num_sps_ids_minus1 is required to be equal to 0 in this version of this Specification, decoders shall allow other values to appear in the num_sps_ids_minus1 syntax.

active_sps_id[ i ] specifies the identifier of the i-th sequence parameter set that is active for the VCL NAL units of the access unit associated with the SEI message. The value of active_sps_id[ i ] shall be in the range of 0 to 31, inclusive.

active_sps_id_present_flag equal to 1 specifies that active_seq_param_set_id is present in the active parameter sets SEI message. active_sps_id_present_flag equal to 0 specifies that active_seq_param_set_id is not present in the active parameter sets SEI message.

active_seq_param_set_id indicates the identifier of the sequence parameter set that is active for the VCL NAL units of the access unit associated with the SEI message. The value of active_seq_param_set_id shall be in the range of 0 to 31, inclusive.

active_param_set_sei_extension_flag equal to 0 indicates that no additional data follows within the active parameter sets SEI message. It is a requirement of bitstream conformance that the value of active_param_set_sei_extension_flag shall be equal to 0. The value 1 for active_param_set_sei_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of active_param_set_sei_extension_flag in an active parameter sets SEI message and shall ignore all data that follows within an active parameter sets SEI message after the value 1 for active_param_set_sei_extension_flag.

## Text changes for items 2 to 4 in subsection 1.8

See the attachment of this document.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).