| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0121 |
| --- | --- |

| Title: | AHG9: NAL unit header design for support of multi-standard extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This is a follow-up proposal of JCTVC-I0355 and JCTVC-J0113, for support an HEVC extension where the base layer/view is AVC and the enhancement layers/views are HEVC. In both of these proposals, NAL unit header syntax changes were proposed so that distinguishing of AVC and HEVC NAL units by checking the NAL unit header becomes possible. Both the approaches in JCTVC-I0355 and JCTVC-J0113 require a bit to be reserved at the HEVC NAL unit header.

In this proposal, it is proposed that the order of syntax elements to be changed to support the multi-standard functionality, which is a "should" requirement for the scalable enhancement of HEVC and was also supported in 3DV discussions.

# Introduction

In general, the following application scenario has been discussed and supporting techniques are being developed.

Base layer/view: AVC

Enhancement layer/view: HEVC scalability or 3D extension

Target application scenarios:

- The bitstream of the base layer/view is AVC compatible and hence can be decoded by AVC decoders. The entire bitstream, including both the base layer/view and the enhancement layers/views can be decoded by decoders conforming to a scalable or 3DV extension profile of HEVC that enables such multi-standard support.

Inter-layer/view prediction:

- Within one access unit, an enhancement layer/view may use the reconstructed based layer/view for inter-layer/view prediction (multi-loop decoding).

# Proposal

## Syntax

Texts not highlighted are the same as in the latest HEVC draft, and those highlighted in yellow are additions, and those strikethroughs in red fonts are removals, compared to the latest HEVC draft.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_reserved_zero_6bits | u(6) |
| nal_unit_type | u(6) |
| nuh_reserved_zero_6bits | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

## Semantics

No change, except for changing of the order of the semantics.

## Constraint in multi-standard codec

The value of ( ( nuh_reserved_zero_6bits  >>  3 ) & 0x01 ) shall be equal to 1. Note that for all profiles which do not enable multi-standard support, the above constraint is not needed. The constraint disallows the following values for nuh_reserved_zero_6bits:

•	8 through 15, inclusive

•	24 through 31, inclusive

•	40 through 47, inclusive

•	56 through 63, inclusive

Note that in typical multi-standard applications, only 2~4 layers will be needed. Therefore, the above constraint doesn’t impact practical applications. In the HEVC base spec, layer_id (nuh_reserved_zero_6bits) is required to be equal to 0. In AVC, excluding its scalable or multiview extension, the first bit of nal_unit_type is always equal to 0, i.e., the value of nal_unit_type is always less than 16 (except for coded slice of an auxiliary coded picture that have nal_unit_type equal to 19, which is not widely used).

# Comparison of AVC NAL unit header and the proposed HEVC NAL unit header in multi-standard extensions

| HEVC |  | AVC |  |
| --- | --- | --- | --- |
| forbidden_zero_bit | f(1) | forbidden_zero_bit | f(1) |
| layer_id_msb_2bits | u(2) | nal_ref_idc | u(2) |
| 1 (layer_id_msb_3rd_bit) | f(1) | nal_unit_type | u(5) |
| layer_id_lsb_3bits | u(3) |  |  |
| nal_unit_type | u(6) |  |  |
| nuh_temporal_id_plus1 | u(3) |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).