| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0122 |
| --- | --- |

| Title: | AHG9: On CRA and BLA pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes the use of an external-means-determined flag, UseAltCpbParamsFlag, to specify which set of initial CPB removal delay and offset parameters is used, for BLA pictures with nal_unit_type equal to BLA_W_LP and CRA pictures. The proposed flag is also used to specify which of the BLA NAL unit types is used when a CRA picture is indicated to be handled as a BLA picture.

# Background

## Handling of a CRA picture as a BLA picture

The latest HEVC draft includes the following text in its subclause 8.1 for handling of a CRA picture as a BLA picture:

When the current picture is a CRA picture, the following applies.

–	If some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value, HandleCraAsBlaFlag is set to the value provided by the external means.

–	Otherwise, the value of HandleCraAsBlaFlag is set to 0.

When HandleCraAsBlaFlag is equal to 1, the following applies during the parsing and decoding processes for each coded slice NAL unit.

–	The value of nal_unit_type is set to BLA_W_LP.

–	The value of no_output_of_prior_pics_flag is set to 1.

A CRA picture has nal_unit_type equal to CRA_NUT, and it may have associated TFD pictures and DLP pictures.

## Selection of HRD parameters for RAP pictures

HRD parameters can be provided in the bitstream in the hrd_parameters( ) syntax structure (which may be present in VPS and/or the SPS), the buffering period SEI message, and the picture timing SEI message. The buffering period SEI message mainly includes initial CPB removal delays and initial CPB removal delay offsets. Two sets of such information can be provided, referred to as the default set, signalled by the syntax elements initial_cpb_removal_delay[ ] and initial_cpb_removal_delay_offset[ ], and the alternative set, signalled by the syntax elements initial_alt_cpb_removal_delay[ ] and initial_alt_cpb_removal_delay_offset[ ].

When sub_pic_cpb_params_present_flag is equal to 0, and rap_cpb_params_present_flag is equal to 1, the alternative set is used when the HRD is initialized at a BLA picture that does not have associated TFD pictures in the bitstream, i.e. that has nal_unit_type equal to BLA_W_DLP or BLA_N_LP. In this case, if the default set was used, the CPB might overflow. When the HRD is not initialized with a BLA picture that has nal_unit_type equal to BLA_W_DLP or BLA_N_LP, the default set is used.

The above behavior is specified in the following text in subclause C.2.1 of the latest HEVC draft:

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set as follows.

–	If either of the following conditions is true:, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set to the values of the corresponding initial_alt_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ], respectively, of the associated buffering period SEI message:

–	Access unit 0 is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_DLP or BLA_N_LP, and the value of rap_cpb_params_present_flag of the associated buffering period SEI message is equal to 1;

–	SubPicCpbFlag is equal to 1.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set to the values of the corresponding initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ], respectively, of the associated buffering period SEI message.

As can be seen from above, the selection of which set of initial CPB removal delay and offset is partly based on the nal_unit_type of a BLA picture.

# Problems

The first problem is associated with the selection of HRD parameters for RAP pictures. Assuming that a CRA picture had associated TFD pictures, and the associated TFD pictures are discarded by a streaming server or an intermediate network element. To enable selection of the appropriate set of initial CPB removal delay and offset (which is the alternative set in this case), the streaming server or the intermediate network element must change the CRA picture to a BLA picture in the bitstream before sending the bits to the decoder. However, many streaming server or the intermediate network element are not capable of doing this. In such situations, the selection of the appropriate set of initial CPB removal delay and offset cannot be successful.

The above problem also applies for BLA pictures with nal_unit_type equal to BLA_W_LP.

The second problem is associated with the process for handling of a CRA picture as a BLA picture. When a CRA picture had associated TFD pictures, and the associated TFD pictures are discarded, an external means may be sent to the decoder to specify to handle the CRA picture as a BLA picture. In this case, the nal_unit_type of the picture would be set to BLA_W_LP, as currently specified. However, setting the nal_unit_type to BLA_W_LP would result the use of the default set of initial CPB removal delay and offset, while the alternative set should be used instead. Consequently the CPB may overflow.

# Proposal

This document proposes the use of an external-means-determined flag, UseAltCpbParamsFlag, to specify which of the default set and the alternative set of the initial CPB removal delay and offset parameters is used, for BLA pictures with nal_unit_type equal to BLA_W_LP and CRA pictures.

When a CRA picture is indicated to be handled as a BLA picture, UseAltCpbParamsFlag also specifies which of the BLA NAL unit types is used.

The proposed spec text changes are provided in the next section.

# Specification text changes

Firstly, the text in subclause 8.1 and quoted above in section 1.1 of this document are changed as follows (additions are highlighted in yellow, and removals are strikethroughs with red fonts).

When the current picture is a BLA picture that has nal_unit_type equal to BLA_W_LP or is a CRA picture, the following applies.

–	If some external means not specified in this Specification is available to set the variable UseAltCpbParamsFlag to a value, UseAltCpbParamsFlag is set to the value provided by the external means.

–	Otherwise, the value of UseAltCpbParamsFlag is set to 0.

When the current picture is a CRA picture, the following applies.

–	If some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value, HandleCraAsBlaFlag is set to the value provided by the external means.

–	Otherwise, the value of HandleCraAsBlaFlag is set to 0.

When the current picture is a CRA picture and when HandleCraAsBlaFlag is equal to 1, the following applies during the parsing and decoding processes for each coded slice NAL unit.

–	If UseAltCpbParamsFlag is equal to 0, the The value of nal_unit_type is set to BLA_W_LP. Otherwise, the value of nal_unit_type is set to BLA_W_DLP.

–	The value of no_output_of_prior_pics_flag is set to 1.

Secondly, the text in subclause C.2.1 and quoted above in section 1.1 of this document are changed as follows (additions are highlighted in yellow, and removals are strikethroughs with red fonts).

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set as follows.

–	If one either of the following conditions is true:, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set to the values of the corresponding initial_alt_cpb_removal_delay[ SchedSelIdx ] and initial_alt_cpb_removal_delay_offset[ SchedSelIdx ], respectively, of the associated buffering period SEI message:

–	Access unit 0 is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_DLP or BLA_N_LP, and the value of rap_cpb_params_present_flag of the associated buffering period SEI message is equal to 1;

–	Access unit 0 is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_LP or is a CRA access unit, UseAltCpbParamsFlag is equal to 1, and the value of rap_cpb_params_present_flag of the associated buffering period SEI message is equal to 1;

–	SubPicCpbFlag is equal to 1.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set to the values of the corresponding initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ], respectively, of the associated buffering period SEI message.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).