| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0123 |
| --- | --- |

| Title: | AHG9: Reference picture set clean-ups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes the following clean-up changes related to reference picture set signalling and derivation:

- Addition of restrictions such that each LTRP signalled (explicitly or indexed) in the slice header shall be a distinct reference picture

- Addition of a restriction that an LTRP entry shall not be directly signalled in any slice header when an equivalent LTRP entry is included in the SPS

- Addition of a restriction that disallows duplicate LTRP entries signalled in the SPS

- Addition of a restriction that disallows duplicate short-term RPS candidates in the SPS and a short-term RPS pattern in the SPS being repeatedly (i.e. explicitly) signalled in the slice header

- Removal of the restriction that POC LSBs for LTRPs are signalled in a non-increasing order

- Not counting STRPs in determination of sending MSB cycle for LTRPs

# 1	Introduction

According to the latest HEVC draft, the signalling of long-term reference pictures (LTRPs) involves signalling the LSBs in the SPS or the slice header, and signalling the MSB bits, if necessary, in the slice header. The syntax elements in the SPS and the slice header relevant to the signalling are listed below:

Table 1: Syntax elements related to LTRPs in SPS

| … |  |
| --- | --- |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

Table 2: Syntax elements related to LTRPs in slice header

| … |  |
| --- | --- |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |

For simplicity, we define a few terms used in this document. An LTRP entry in the SPS is defined as a tuple consisting of lt_ref_pic_poc_lsb_sps[ i ] and used_by_curr_pic_lt_sps_flag[ i ]. Similarly, an LTRP entry in the slice header is defined as either the LSB and the flag signalled by poc_lsb_lt[ i ] and used_by_curr_pic_lt_flag[ i ], or the LSB and the flag corresponding to the entry in the SPS indicated by lt_idx_sps[ i ]. When an LTRP entry or LTRP is signalled in a slice header using lt_idx_sps[ i ], the LTRP entry or LTRP is said to be indexed in the slice header. When an LTRP entry or LTRP is signalled in a slice header using poc_lsb_lt[ i ] and used_by_curr_pic_lt_flag[ i ], the LTRP entry or LTRP is said to be explicitly signalled in the slice header.

The current syntax and the semantics have the following shortcomings:

- There is no restriction in the semantics that disallows the same LTRP entry to be signalled (explicitly or indexed) more than once in one slice header. Moreover, the same long-term reference picture may be explicitly signalled or indexed in the slice header both with used_by_curr_pic_flag as zero and one; in other words, the current syntax and semantics allow for an LTRP to be signalled in both the RPS subsets RefPicSetLtCurr and RefPicSetLtFoll. Consequently, what is said in note 5 in subclause 8.3.2, i.e. a picture cannot be included in more than one of the five reference picture set lists, is not ensured.

- LTRP entries may be signalled in the SPS and indexed in the slice header. However, the current semantics permit any LTRP entry signalled in the SPS to be also explicitly signalled in the slice header. Such flexibility is unnecessary and should be disallowed.

- There is no restriction in the semantics for the same LTRP entry being signalled more than once in the SPS.

- It is possible to repeat the same short-term RPS in the list of short-term RPSs, which should be disallowed. It is also possible that a short-term RPS explicitly signalled in the slice header is identical to a short-term RPS signalled in the SPS, which should also be disallowed.

- The current semantics of signalling the poc_lsb_lt[ i ] and lt_idx_sps[ i ] require the LSBs to be signalled in a non-increasing order. The restriction made sense when POC LSBs were differentially coded using ue(v). Since the coding of poc_lsb_lt[ i ] was changed to u(v), the restriction is less efficient; it also adds complexity because it requires more conditions to be checked in the derivation of DeltaPocMSBCycleLt[ i ] (see Eqn. 7-37). Consider the following example. Let MaxPicOrderCntLsb be 32 and current picture have a POC of 311. Let three LTRPs be signalled (explicitly in the slice header) for the current picture– with POC values 84, 170, and 308. As can be seen from Table 3 and Table 4, removing the non-increasing constraint and additional condition in Eqn. 7-37 result in fewer bits to signal the LTRPs. Furthermore, the non-increasing constraint also disallows encoders to put LTRPs in optimal order to avoid RPLM signalling.

Table 3: Syntax element values (with the non-increasing constraint) for LTRPs for current picture 
with POC 331 = 32*10 + 11

|  | i = 0 | i = 1 | i = 2 |
| --- | --- | --- | --- |
| LTRPs | POC 308 | POC 84 | POC 170 |
| POC split up | 32*9 + 20 | 32*2 + 20 | 32*5 + 10 |
| DeltaPocMSBCycleLt[ i ] | 1 | 8 | 5 |
| poc_lsb_lt[ i ] | 20 | 20 | 10 |
| delta_poc_msb_cycle_lt[ i ] | 1 | 7 | 5 |
| # of bits for MSB cycle | 3 | 7 | 5 |

Table 4: Syntax element values (without the non-increasing constraint) for LTRPs for current picture 
with POC 331 = 32*10 + 11

|  | i = 0 | i = 1 | i = 2 |
| --- | --- | --- | --- |
| LTRPs | POC 308 | POC 170 | POC 84 |
| POC split up | 32*9 + 20 | 32*5 + 10 | 32*2 + 20 |
| DeltaPocMSBCycleLt[ i ] | 1 | 5 | 8 |
| poc_lsb_lt[ i ] | 20 | 10 | 20 |
| delta_poc_msb_cycle_lt[ i ] | 1 | 4 | 3 |
| # of bits for MSB cycle | 3 | 5 | 5 |

- When there are two reference pictures in the DPB with the same LSB, and one of the pictures is signalled as an STRP and the other picture as an LTRP, the semantics mandate that the MSB information of the LTRP be signalled. However, in this case, the MSB information for the LTRP need not be signalled.

# 2	Proposal

A detailed description of the proposed changes is given below.

- Addition of restrictions that each LTRP signalled (explicitly or indexed) in the slice header shall be a distinct reference picture. Addition of these restrictions ensures the correctness of the note saying that a picture cannot be included in more than one of the five reference picture set lists.

- Addition of a restriction that an LTRP entry shall not be directly signalled in any slice header when an equivalent LTRP entry is included in the SPS. Such an LTRP entry may be easily indexed in the slice header using the syntax element lt_idx_sps[ i ]. This restriction is added to the semantics of the syntax element used_by_curr_pic_lt_flag[ i ].

- Addition of a restriction that disallows duplicate LTRP entries signalled in the SPS. Signalling the same LTRP entry in the SPS does not serve any advantage, and can be restricted by adding a constraint to the semantics of used_by_curr_pic_lt_sps_flag[ i ].

- Addition of a restriction that ensure no two short-term RPS candidates that are signalled in the SPS are identical, and a short-term RPS pattern shall not be explicitly signalled in the slice header if an identical pattern is signalled among the short-term RPS candidates in the SPS.

- Removal of the restriction that POC LSBs for LTRPs are signalled in a non-increasing order. In addition to the removal of this restriction in the semantics of lt_idx_sps[ i ] and poc_lsb_lt[ i ], a condition in the derivation of DeltaPocMSBCycleLt[ i ] is also removed.

- Not counting STRPs in determination of sending MSB cycle for LTRPs. When there are n (n > 1) reference pictures in the DPB with the same LSB, and n - 1 of those pictures are signalled as STRPs and the nth picture is signalled as an LTRP, then the MSB cycle information of the LTRP is not signalled. The proposed change not only implements this feature, but also simplifies the list of constraints after Eqn. 8-5, from 12 items to 9 items.

The proposed changes to the syntax, semantics, and the decoding process are highlighted below with corresponding colour codings.

  - Sequence parameter set semantics

used_by_curr_pic_lt_sps_flag[ i ] equal to 0 specifies that the i-th candiate long-term reference picture specified in the sequence parameter set is not used for reference by a picture that includes in its reference picture set the i-th candidate long-term reference picture specified in the sequence parameter set. For any i and j in the range of 0 to num_long_term_ref_pics_sps – 1, inclusive, if i is not equal to j and lt_ref_pic_poc_lsb_sps[ i ] is equal to lt_ref_pic_poc_lsb_sps[ j ], then used_by_curr_pic_lt_sps_flag[ i ] shall not be equal to used_by_curr_pic_lt_sps_flag[ j ].

  - Slice header syntax

| … |  |
| --- | --- |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |

  - Slice header semantics

lt_idx_sps[ i ] specifies an index into the list of candidate long-term reference pictures specified in the active sequence parameter set for identification of the picture that is included in the long-term reference pictur set of the current picture. The value of lt_idx_sps[ i ] shall be in the range of 0 to num_long_term_ref_pics_sps − 1, inclusive. The number of bits used to represent lt_idx_sps[ i ] is equal to Ceil( Log2( num_long_term_ref_pics_sps ) ). For any values of j and k in the range of 0 to num_long_term_sps − 1, inclusive, when j is less than k, lt_ref_pic_poc_lsb_sps[ lt_idx_sps[ j ] ] shall not be less than lt_ref_pic_poc_lsb_sps[ lt_idx_sps[ k ] ].

poc_lsb_lt[ i ] specifies the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. The length of the poc_lsb_lt[ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. For any values of j and k in the range of num_long_term_sps to num_long_term_sps + num_long_term_pics − 1, inclusive, if j is less than k, poc_lsb_lt[ j ] shall not be less than poc_lsb_lt[ k ].

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th candidate long-term reference picture to be included in the long-term reference picture set of the current picture is not used for reference by the current picture. For any value of j in the range of 0 to num_long_term_ref_pics_sps – 1, inclusive, if poc_lsb_lt[ i ] equals lt_ref_pic_poc_lsb_sps[ j ], used_by_curr_pic_lt_flag[ i ] shall not equal used_by_curr_pic_lt_sps_flag[ j ].

The variables PocLsbLt[ i ] and UsedByCurrPicLt[ i ] are derived as follows.

–	If i is less than num_long_term_sps, PocLsbLt[ i ] is set equal to lt_ref_pic_poc_lsb_sps[ lt_idx_sps[ i ] ] and UsedByCurrPicLt[ i ] is set equal to used_by_curr_pic_lt_sps_flag[ lt_idx_sps[ i ] ].

–	Otherwise, PocLsbLt[ i ] is set equal to poc_lsb_lt[ i ] and UsedByCurrPicLt[ i ] is set equal to used_by_curr_pic_lt_flag[ i ].

delta_poc_msb_present_flag[ i ] equal to 1 specifies that delta_poc_msb_cycle_lt[ i ] is present. delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt[ i ] is not present. delta_poc_msb_present_flag[ i ] shall be equal to 1 when there is more than one reference picture in the decoded picture buffer, excluding those pictures that are signalled as short-term reference picture for the current picture, with the least significant bits of the picture order count value equal to poc_lsb_lt[ i ].

delta_poc_msb_cycle_lt[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. When not present, the value of delta_poc_msb_cycle_lt[ i ] is inferred to be 0.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if( i  = =  0  | |  i = =  num_long_term_sps  | |  PocLsbLt[ i − 1 ]  ! =  PocLsbLt[ i ] )
	DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]
else							(7-1)
	DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ]

The value of DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb + pic_order_cnt_lsb − PocLsbLt[ i ] shall be in the range of 1 to 224 − 1, inclusive.

  - Changes to the decoding process for reference picture set

…

Five lists of picture order count values are constructed to derive the reference picture set; PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll with NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll number of elements, respectively.

- If the current picture is an IDR picture, PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll are all set to empty, and NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll are all set to 0.

- Otherwise, the following applies for derivation of the five lists of picture order count values and the numbers of entries.

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPositivePics[ StRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ StRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k		(8-2)
for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {
	pocLt = PocLsbLt[ i ]
	if( delta_poc_msb_present_flag[ i ] )
		pocLt += PicOrderCntVal − 	DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb − pic_order_cnt_lsb
	if( UsedByCurrPicLt[ i ] ) {
		PocLtCurr[ j ] = pocLt
		CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]
	} else {
		PocLtFoll[ k ] = pocLt
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]
	}
}
NumPocLtCurr = j
NumPocLtFoll = k

where PicOrderCntVal is the picture order count of the current picture as specified in subclause Error: Reference source not found.

NOTE 1 – A value of StRpsIdx in the range from 0 to num_short_term_ref_pic_sets − 1, inclusive, indicates that a short-term reference picture set from the active sequence parameter set is being used, where StRpsIdx is the index of the short-term reference picture set to the list of short-term reference picture sets in the order in which they are signalled in the sequence parameter set. StRpsIdx equal to num_short_term_ref_pic_sets indicates that a short-term reference picture set explicitly signalled in the slice header is being used.

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStFoll[ j ].

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStFoll[ j ].

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

The reference picture set consists of five lists of reference pictures; RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetStFoll, RefPicSetLtCurr and RefPicSetLtFoll. The variable NumPocTotalCurr is set equal to NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr.

It is a requirement of bitstream conformance that the following applies to the value of NumPocTotalCurr:

- If the current picture is a BLA or CRA picture, the value of NumPocTotalCurr shall be equal to 0.

- Otherwise, when the current picture contains a P or B slice, the value of NumPocTotalCurr shall not be equal to 0.

NOTE 2 – RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr contains all reference pictures that may be used in inter prediction of the current picture and that may be used in inter prediction of one or more of the pictures following the current picture in decoding order. RefPicSetStFoll and RefPicSetLtFoll consists of all reference pictures that are not used in inter prediction of the current picture but may be used in inter prediction of one or more of the pictures following the current picture in decoding order.

The derivation process for the reference picture set and picture marking are performed according to the following ordered steps, where DPB refers to the decoded picture buffer as described in Annex Error: Reference source not found:

- The following applies:

for( i = 0; i < NumPocLtCurr; i++ )
	if( !CurrDeltaPocMsbPresentFlag[ i ] )
		if( there is a long-term reference picture picX in the DPB [Ed. (JB): Should be made more precise. (GJS): Seems roughly OK to me.]
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] 
				and with PicOrderCntVal not in any of PocStCurrBefore[ ], 
				PocStCurrAfter[ ], and PocStFoll[ ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	else
		if( there is a long-term reference picture picX in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
										(8-3)
for( i = 0; i < NumPocLtFoll; i++ )
	if( !FollDeltaPocMsbPresentFlag[ i ] )
		if( there is a long-term reference picture picX in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] 
					and with PicOrderCntVal not in any of PocStCurrBefore[ ], 
					PocStCurrAfter[ ], and PocStFoll[ ] )
				RefPicSetLtFoll[ i ] = picY
		else 
				RefPicSetLtFoll[ i ] = "no reference picture"
	else
		if( there is a long-term reference picture picX in the DPB
					with PicOrderCntVal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
			else if( there is a short-term reference picture picY in the DPB
					with PicOrderCntVal equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else
				RefPicSetLtFoll[ i ] = "no reference picture"

- All reference pictures included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "used for long-term reference", and the following constraints apply.

For each i in the range of 0 to NumPocLtCurr – 1, inclusive, it is a requirement of bitstream conformance that the following conditions apply:

  - There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PicOrderCntVal of RefPicSetLtCurr[ i ] is equal to PocStCurrBefore[ j ].

  - There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PicOrderCntVal of RefPicSetLtCurr[ i ] is equal to PocStCurrAfter[ j ].

  - There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PicOrderCntVal of RefPicSetLtCurr[ i ] is equal to PocStFoll[ j ].

  - There shall be no j in the range of 0 to NumPocLtCurr – 1, inclusive, where j is not equal to i, for which PicOrderCntVal of RefPicSetLtCurr[ i ] is equal to PicOrderCntVal of RefPicSetLtCurr[ j ].

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when RefPicSetLtFoll[ i ] is not equal to “no reference picture”, it is a requirement of bitstream conformance that the following conditions apply:

  - There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PicOrderCntVal of RefPicSetLtFoll[ i ] is equal to PocStCurrBefore[ j ].

  - There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PicOrderCntVal of RefPicSetLtFoll[ i ] is equal to PocStCurrAfter[ j ].

  - There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PicOrderCntVal of RefPicSetLtFoll[ i ] is equal to PocStFoll[ j ].

  - There shall be no j in the range of 0 to NumPocLtCurr – 1, inclusive, for which PicOrderCntVal of RefPicSetLtFoll[ i ] is equal to PicOrderCntVal of RefPicSetLtCurr[ j ].

  - There shall be no j in the range of 0 to NumPocLtFoll – 1, inclusive, where j is not equal to i and RefPicSetLtFoll[ j ] is not equal to “no reference picture”, for which PicOrderCntVal of RefPicSetLtFoll[ i ] is equal to PicOrderCntVal of RefPicSetLtFoll[ j ].

- The following applies:

for( i = 0; i < NumPocStCurrBefore; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrBefore[ i ])
		RefPicSetStCurrBefore[ i ] = picX
	else
		RefPicSetStCurrBefore[ i ] = "no reference picture"

for( i = 0; i < NumPocStCurrAfter; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrAfter[ i ])
		RefPicSetStCurrAfter[ i ] = picX
	else
		RefPicSetStCurrAfter[ i ] = "no reference picture"	(8-4)

for( i = 0; i < NumPocStFoll; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStFoll[ i ])
		RefPicSetStFoll[ i ] = picX
	else
		RefPicSetStFoll[ i ] = "no reference picture"

- All reference pictures in the decoded picture buffer that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference".

NOTE 3 – There may be one or more reference pictures that are included in the reference picture set but not present in the decoded picture buffer. Entries in RefPicSetStFoll or RefPicSetLtFoll that are equal to "no reference picture" should be ignored. An unintentional picture loss should be inferred for each entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr that is equal to "no reference picture".

It is a requirement of bitstream conformance that the reference picture set is restricted as follows:

- There shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr for which one or more of the following are true.

  - The entry is equal to "no reference picture".

  - The entry is a picture that has nal_unit_type equal to TRAIL_N, TSA_N or STSA_N and TemporalId equal to that of the current picture.

  - The entry is a picture that has TemporalId greater than that of the current picture.

- When the current picture is a TSA picture, there shall be no picture included in the reference picture set with TemporalId equal to the TemporalId of the current picture.

- When the current picture is an STSA picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that has TemporalId equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, an STSA picture that has TemporalId equal to that of the current picture, there shall be no picture that has TemporalId equal to that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that preceded the STSA picture in decoding order.

- When the current picture is a CRA picture, there shall be no picture included in the reference picture set that precedes, in decoding order, any preceding RAP picture in decoding order (when present).

- When the current picture is a trailing picture, there shall be no picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that was generated by the decoding process for generating unavailable reference pictures as specified in subclause Error: Reference source not found.

- When the current picture is a trailing picture, there shall be no picture in the reference picture set that precedes the associated RAP picture in output order or decoding order.

- When the current picture is a DLP picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is any of the following types of pictures.

  - A TFD picture.

  - A picture that was generated by the decoding process for generating unavailable reference pictures as specified in subclause Error: Reference source not found.

  - A picture that precedes the associated RAP picture in decoding order.

- When the sps_temporal_id_nesting_flag is equal to 1, the following applies. Let tIdA be the value of TemporalId of the current picture picA. Any picture picB with TemporalId equal to tIdB that is less than or equal to tIdA shall not be included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr of picA when there exists a picture picC with TemporalId equal to tIdC that is less than tIdB, which follows the picture picB in decoding order and precedes the picture picA in decoding order.

- For any two different values of idxA and idxB in the range of 0 to num_short_term_ref_pic_sets, inclusive, one or more of the following shall be true.

  - NumNegativePics[ idxA ] is not equal to NumNegativePics[ idxB ].

  - NumPositivePics[ idxA ] is not equal to NumPositivePics[ idxB ].

  - When NumNegativePics[ idxA ] and NumPositivePics[ idxA ] equal NumNegativePics[ idxB ] and NumPositivePics[ idxB ], respectively, there exists either an i in the range of 0 to NumNegativePics[ idxB ] – 1, inclusive, for which UsedByCurrPicS0[ idxA ][ i ] is not equal to UsedByCurrPicS0[ idxB ][ i ] or DeltaPocS0[ idxA ][ i ] is not equal to DeltaPocS0[ idxB ][ i ], or there exists a j in the range of 0 to NumPositivePics[ idxB ] - 1, inclusive, for which UsedByCurrPicS1[ idxA ][ j ] is not equal to UsedByCurrPicS1[ idxB ][ j ] or DeltaPocS1[ idxA ][ j ] is not equal to DeltaPocS1[ idxB ][ j ], or both such i and j.

NOTE 4 – No two short-term reference picture set candidates signalled in the sequence parameter set can be identical, and a short-term reference picture set pattern cannot be explicitly signalled in the slice header if an identical candidate is present in the candidate list in the sequence parameter set.

NOTE 6 – A picture cannot be included in more than one of the five reference picture set lists.

# 3	Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).