| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0124 |
| --- | --- |

| Title: | AHG9: Reference picture list modification with truncation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ying Chen 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current HEVC draft, reference picture list modification requires signaling of each entry for a reference picture list. In this proposal, the flexibility of signalling less number of entries is proposed with reportedly minor syntax changes. Two methods with the same syntax table are proposed. The first method provides similar functionalities as AVC reference picture list modification, while the second method is further simplified to just bring pictures to the beginning of the reference picture list with less decoding process changes.

# Introduction

The current design in the latest HEVC draft has to signal all entries in a reference picture list once reference picture list modification (RPLM) is needed. This design is less flexible than AVC, wherein the number of commands for RPLM can be less than the number of entries in a reference picture list. This may result in unnecessary signaling of bits in many cases, e.g. in the following cases:

- Example 1: Five pictures in the initial list are to be put in the final list, and only the first two entries in the initial list need to be swapped to get the final list. For the AVC design, only two commands are needed while for the design in HEVC WD8, five commands are needed.

- Example 2: Five pictures are to be put in the final list. A picture that is at the end of the initial list or is not included in the initial list needs to be added at the first entry of the final list, the last entry is removed and the position of the other entries have their positions shifted by one. This is a typical case, e.g., in MVC.

When the spatial resolution is smaller than the maximum value allowed by the level definition, it is possible that more reference pictures are used compared to the case where the maximum spatial resolution is in use.

This document proposes to have a simple syntax change, without introducing new syntax elements to enable the functionality of terminating the loop of RPLM.

Based on the same syntax change, the first method provides functionalities similar to H.264/AVC but with much simpler decoding processes; the second method, however, does not remove duplicate entries in the list.

# Proposals

## Reference picture list modification syntax

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 && NumPocTotalCurr > 1 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1 && ( i && list_entry_l0[ i - 1 ] < NumPocTotalCurr ); i++ ) |  |
| list_entry_l0[ i ] | u(v) |
| } |  |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 && NumPocTotalCurr > 1 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1 && ( i && list_entry_l1[ i - 1 ] < NumPocTotalCurr ); i++ ) |  |
| list_entry_l1[ i ] | u(v) |
| } |  |
| } |  |

### Reference picture list modification semantics

list_entry_lX[ i ] (with X equal to 0 or 1) specifies the index of the reference picture in RefPicSetCurrTempListX to be placed at the current position of reference picture list LX (with X being 0 or 1). The length of the list_entry_lX[ i ] syntax element is Ceil( Log2( NumPocTotalCurr + 1 ) ) bits. The value of list_entry_lX[ i ] shall be in the range of 0 to NumPocTotalCurr − 1, inclusive. If the syntax element list_entry_lX[ i ] is not present, it is inferred to be equal to 0.

NOTE - When list_entry_lX[ i ] is equal to NumPocTotalCurr, the reference picture list modification syntax for RefPicListX ends.

## Method 1

The modification is performed by first checking whether an entry is already present in the default list of RefPicIdxTempListX, and if present, the entry is removed and the other entries after the target position are shifted by 1 till the removed position is filled. The following example shows how the modification process works. Assume that RefPicListX contains five distinct pictures, regardless their POC values and whether there are short-term or long-term pictures. Assume that these are the only pictures in the reference picture set that are used for reference by the current picture. A representation of the list is given below.

| Reference pictures used by current picture | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| RefPicIdxTempListX[ ] | 0 | 1 | 2 | 3 | 4 |

A temporary RefPicIdxTempListX[ ] is initially assigned as RefPicIdxTempListX[ j ] = j. The list is used to store the relative indices of the pictures in the reference picture subsets containing those pictures that may be used by the current picture for reference.

Assume that the picture D has to be brought to the beginning of the list, with the relative positions of the other entries unchanged. The list_entry_l0[ ] elements signalled are list_entry_l0[ 0 ] = 3 and list_entry_l0[ 1 ] = 5.

First, it is checked whether index 3 is present in the list RefPicIdxTempListX[ ]. Once index 3 is found, the entries in the RefPicIdxTempListX[ ] are shifted, as shown below. Then list_entry_l0[ 1 ] equal to 5 is parsed, which indicates the loop is terminated. This process actually results in changes of only RefPicIdxTempListX[ ].

| 0 | 1 | 2 | 3 | 4 |  | 3 | 0 | 1 | 2 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Modified RefPicIdxTempListX

By checking the entries in RefPicIdxTempListX[ ], the RefPicListX is formed as below.

| D | A | B | C | E |
| --- | --- | --- | --- | --- |

### Changes to the derivation process of reference picture list

#### 8.3.4.2	Initialization process for reference picture lists

This process is invoked when decoding a P or B slice header.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(8-1)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
}

If ref_pic_list_modification_flag_l0 equals 1, subclause 8.3.4.3 is invoked with RefPicIdxTempList0 as output.

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			(8-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? 
						RefPicListTemp0[ RefPicIdxTempList0list_entry_l0[ rIdx ] ] : RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(8-3)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
}

If ref_pic_list_modification_flag_l1, subclause 8.3.4.3 is invoked with RefPicIdxTempList1 as output.

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(8-4)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ?
						RefPicListTemp1[ RefPicIdxTempList1list_entry_l1[ rIdx ] ] :RefPicListTemp1[ rIdx ]

#### 8.3.4.3	Modification process for reference picture lists

This decoding process is invoked only when ref_pic_list_modification_flag_lX ( with X being equal to 0 or 1 ) equal to 1.

Output of this process is a list RefPicIdxTempListX.

–	RefPicIdxTempListX is set as follows: RefPicIdxTempListX[ j ]= j % NumPocTotalCurr.

## for ( i = 0 ; list_entry_lX[ i ] < NumPocTotalCurr; i++) {
	existPos= num_ref_idx_lX _active_minus1
	for( j = i ; j <= num_ref_idx_lX_active_minus1 && existPos = = num_ref_idx_lX_active_minus1; j++ )
		if ( RefPicIdxTempListX[ j ] = = list_entry_lX[ i ] ) 	existPos = j
	for( cIdx = existPos; cIdx > i; cIdx- - )
		RefPicIdxTempListX[ cIdx ] = RefPicIdxTempListX[ cIdx – 1 ]
	RefPicIdxTempListX[ i ] = list_entry_lX[ i ]
}

## Method 2

The modification is performed by placing the pictures pointed by list_entry_lX[ ] in the target locations, and shifting the rest of the contents in the list to the right. We consider the same example as given in Section 2.2. Assume that the picture D has to be brought to the beginning of the list, and the rest of the entries shifted to the right. The signaling is the same as earlier: list_entry_l0[ 0 ] = 3 and list_entry_l0[ 1 ] = 5. All the entries are shifted to the right by one position, and picture D is placed in position 0. The following figure represents the shifting operation for the above modification, and the final modified list.

As shown in the example, the decoding process for the method 2 is simpler while it is not as flexible as the method 1.

| A | B | C | D | E |  | D | A | B | C | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Modified RefPicListX

D

### Changes to the derivation process of reference picture list

#### 8.3.4.2	Initialization process for reference picture lists

This process is invoked when decoding a P or B slice header.

If the syntax element list_entry_lX[ i ] is not present, it is inferred to be equal to (i = = 0) ? 0 : (list_entry_lX[ i – 1 ] + 1)% NumPocTotalCurr.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(8-5)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx ≤ num_ref_idx_l0_active_minus1; rIdx++)			(8-6)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? 
						RefPicListTemp0[ list_entry_l0[ rIdx ] % NumPocTotalCurr ] : RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(8-7)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx ≤ num_ref_idx_l1_active_minus1; rIdx++)			(8-11)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? 
						RefPicListTemp1[ list_entry_l1[ rIdx ] % NumPocTotalCurr ] : RefPicListTemp1[ rIdx ]

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).