| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0125 |
| --- | --- |

| Title: | AHG9: On video parameter set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes some changes to the VPS syntax as well as the profile_tier_level( ) and operation_point( ) syntax structures included in the VPS syntax. Bitrate and picture rate information are added to the VPS to assist session negotiation and content selection in HEVC application systems. The syntax elements for signalling of profile space, tier, compatible profiles, and profile-related constraints for temporal sub-layers are removed from the profile_tier_level( ) syntax structure. The operation point syntax is changed to support a simple operation point mode for which only one value of nuh_reserved_zero_6bits needs to be signalled. When multiple values of the nuh_reserved_zero_6bits are signalled for one operation point, they are differentially coded. The changing operation point signalling is asserted to be more efficient, particular for typical scalability coding scenarios with linear layer dependency. Within one VPS, duplicate copies of operation points and duplicate copies of hrd_parameters( ) syntax structure are disallowed.

# Introduction

The current VPS design, including the syntax structures included in the VPS, has the following shortcomings:

- While the information of profile, tier, and level that is important for session negotiation and content selection is signalled in the VPS, other information that is also important for the same purpose, such as bitrate and picture rate, is not. Furthermore, the latest HEVC design does not support the signalling of bitrate and picture rate information, which is supported in AVC and its SVC and MVC extensions by the sub-sequence layer characteristics SEI message, scalability information SEI message, and view scalability information SEI message.

- Temporal subsets for a coded video sequence are allowed to signal a set of profile space, tier, compatible profiles, and profile-related constraints that is different from that of the entire coded video sequence. However, it does not make sense, at least to the authors, for a temporal subset to use a different profile space, and we don't see good use cases for temporal subsets to have the other values different than the entire coded video sequence, either.

- Currently, the set of nuh_reserved_zero_6bits values (i.e. layer IDs) for each operation point, except for the first one, is signalled by explicitly signal each of the layer IDs included in the coded video sequence to be decoded. However, in most scalability coding scenarios the layer dependency relationship is linear, and signalling of only the target layer ID would be sufficient. Furthermore, for non-linear layer dependency relationship, which is typical for multiview coding scenarios, wherein multiple layer IDs are signalled, they can be differentially coded, which is simple and efficient, similarly as e.g. coding of POC values in short-term reference picture set signalling. Lastly, it is allowed to have duplicate sets of layer IDs signalled.

- It is allowed to have duplicate hrd_parameters( ) syntax structures with the same content.

We propose the following changes to address the above shortcomings:

- Optional signalling of bitrate and picture rate information for each temporal sub-layer is included in the VPS.

- Syntax elements for signalling of profile space, tier, compatible profiles, and profile-related constraints for temporal sub-layers are removed.

- A simple operation point mode, for which only one value of nuh_reserved_zero_6bits needs to be signalled, is defined. The signalling of operation points is changed to enable signalling of only one value of nuh_reserved_zero_6bits for operation points with the simple operation point mode. When multiple values of the nuh_reserved_zero_6bits are signalled for one operation point, they are differentially coded. Furthermore, duplicate copies of operation points are disallowed.

- Duplicate copies of hrd_parameters( ) syntax structure with the same content is disallowed to be signalled in a VPS.

# Proposal

The proposed text changes are as follows.

## Definitions

...

note: A term used to prefix informative remarks. This term is used exclusively in an informative context.

operation point: An operation point is identified by a set of nuh_reserved_zero_6bits values, denoted as OpLayerIdSet, and a TemporalId value, denoted as OpTid, and the associated bitstream subset derived as the output of the sub-bitstream extraction process as specified in subclause Error: Reference source not found with OpTid and OpLayerIdSet as inputs is independently decodable.

output order: The order in which the decoded pictures are output from the decoded picture buffer in case the decoded pictures are to be output from the decoded picture buffer. The output order of a picture is specified by the picture order count value, regardless of whether the picture is to be output.

...

should: A term used to refer to behaviour of an implementation that is encouraged to be followed under anticipated ordinary circumstances, but is not a mandatory requirement for conformance to this Specification.

simple operation point mode: An operation point mode in which for each operation point the OpLayerIdSet includes and only includes a particualr value of nuh_reserved_zero_6bits and all other values of nuh_reserved_zero_6bits less than the particualr value of nuh_reserved_zero_6bits.

slice: An integer number of coding tree blocks ordered consecutively in the tile scan. The division of each picture into slices is a partitioning. The coding tree block addresses are derived from the first coding tree block address in a slice (as represented in the slice header).

...

## Video parameter set RBSP syntax and semantics

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_zero_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| bitrate_framerate_info( 0, vps_max_temporal_layers_minus1 ) |  |
| vps_reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| vps_simple_op_mode_flag[ i ] | u(1) |
| if( i > 0 ) |  |
| operation_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- vps_simple_op_mode_flag[ i ] equal to 1 specifies that the simple operation point mode is in use for the i-th operation_point_layer_ids( ) synax structure. vps_simple_op_mode_flag[ i ] equal to 0 specifies that the simple operation piont mode is not in use for the the i-th operation_point( ) synax structure.

- Any two instances of the syntax structure hrd_parameters( i, vps_max_sub_layers_minus1 ) and hrd_parameters( j, vps_max_sub_layers_minus1 ), where i is not equal to j, shall not have the same content.

## Profile, tier and level syntax and semantics

| profile_tier_level( ProfilePresentFlag, MaxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( ProfilePresentFlag ) { |  |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( i = 0; i < 32; i++ ) |  |
| general_profile_compatibility_flag[ i ] | u(1) |
| general_reserved_zero_16bits [Ed. (GJS): Adjust semantics accordingly.] | u(16) |
| } |  |
| general_level_idc | u(8) |
| for( i = 0; i < MaxNumSubLayersMinus1; i++ ) { |  |
| if( ProfilePresentFlag ) |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( sub_layer_profile_present_flag[ i ] ) |  |
| sub_layer_profile_idc[ i ] | u(5) |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

sub_layer_profile_present_flag[ i ] equal to 1, when ProfilePresentFlag is equal to 1, specifies that profile information is present in the profile_tier_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i. sub_layer_profile_present_flag[ i ] equal to 0 specifies that profile information is not present in the profile_tier_level( ) syntax structure for the representations of the sub-layer with TemporalId equal to i. When not present, the value of sub_layer_profile_present_flag[ i ] is inferred to be equal to 0.

sub_layer_level_present_flag[ i ] equal to 1 specifies that level information is present in the profile_tier_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i. sub_layer_level_present_flag[ i ] equal to 0 specifies that level information is not present in the profile_tier_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i.

sub_layer_profile_idc[ i ] and sub_layer_level_idc[ i ] have the same semantics as general_profile_idc and general_level_idc, respectively, but apply to the representation of the sub-layer with TemporalId equal to i.

## Bitrate and picture rate information syntax and semantics

| bitrate_picrate_info( TempLevelLow, TempLevelHigh ){ |  |
| --- | --- |
| for( i = TempLevelLow; i <= TempLevelHigh; i++ ) { |  |
| bitrate_info_present_flag[ i ] | u(1) |
| pic_rate_info_present_flag[ i ] | u(1) |
| if( bitrate_info_present_flag[ i ] ) { |  |
| avg_bitrate[ i ] | u(16) |
| max_bitrate [ i ] | u(16) |
| } |  |
| if( pic_rate_info_present_flag[ i ] ) { |  |
| constant_pic_rate_idc[ i ] | u(2) |
| avg_pic_rate[ i ] | u(16) |
| } |  |
| } |  |
| } |  |

bitrate_info_present_flag[ i ] equal to 1 specifies that the bit rate information for the i-th sub-layer is present. bitrate_info_present_flag[ i ] equal to 0 specifies that the bit rate information for the i-th sub-layer is not present.

pic_rate_info_present_flag[ i ] equal to 1 specifies that picture rate information for the i-th sub-layer is present. pic_rate_info_present_flag[ i ] equal to 0 specifies that picture rate information for the i-th sub-layer is not present.

avg_bitrate[ i ] indicates the average bit rate of the representation of the i-th sub-layer. The average bit rate for the representation of the i-th sub-layer in bits per second is given by BitRateBPS( avg_bitrate[ i ] ) with the function BitRateBPS( ) being specified by

BitRateBPS( x ) = ( x & ( 214 − 1 ) ) * 10( 2 + ( x >> 14 ) )

The average bit rate is derived according to the access unit removal time specified in Annex Error: Reference source not found of this Specification. In the following, bTotal is the number of bits in all NAL units of the representation of the i-th sub-layer, t1 is the removal time (in seconds) of the first access unit to which the video parameter set applies, and t2 is the removal time (in seconds) of the last access unit (in decoding order) to which the video parameter set applies.

With x specifying the value of avg_bitrate[ i ], the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

( x & ( 214 − 1 ) )  = =  Round( bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x >> 14 ) ) ) )

–	Otherwise (t1 is equal to t2), the following condition shall be true:

( x & ( 214 − 1 ) )  = =  0

max_bitrate_layer[ i ] indicates an upper bound for the bitrate of the representation of the i-th sub-layer in any one-second time window of access unit removal time as specified in Annex Error: Reference source not found. The upper bound for the bitrate in bits per second is given by BitRateBPS( max_bitrate_layer[ i ] ). The bit rate values are derived according to the access unit removal time specified in Annex Error: Reference source not found of this Specification. In the following, t1 is any point in time (in seconds), t2 is set equal to t1 + 1 ÷ 100, and bTotal is the number of bits in all NAL units of that belong to access units with a removal time greater than or equal to t1 and less than t2. With x specifying the value of max_bitrate_layer[ i ], the following condition shall be obeyed for all values of t1:

( x & ( 214 − 1 ) )  >=  bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x >> 14 ) ) )

constant_pic_rate_idc[ i ] indicates whether the picture rate of the representation of the i-th sub-layer is constant. In the following, a temporal segment tSeg is any set of two or more consecutive access units, in decoding order, of the representation of the i-th sub-layer, fTotal( tSeg ) is the number of pictures in the temporal segment tSeg, t1( tSeg ) is the removal time (in seconds) of the first access unit (in decoding order) of the temporal segment tSeg, t2( tSeg ) is the removal time (in seconds) of the last access unit (in decoding order) of the temporal segment tSeg, and avgFR( tSeg ) is the average frame rate in the temporal segment tSeg, which is given by:

avgFR( tSeg)  = =  Round( fTotal( tSeg ) * 256  ( t2( tSeg ) − t1( tSeg ) ) )

If the representation of the i-th sub-layer does only contain one access unit or the value of avgFR( tSeg ) is constant over all temporal segments of the representation of the i-th sub-layer, the picture rate is constant; otherwise, the picture rate is not constant.

constant_pic_rate_idc[ i ] equal to 0 indicates that the picture rate of the representation of the i-th sub-layer is not constant. constant_pic_rate_idc[ i ] equal to 1 indicates that the picture rate of the representation of the i-th sub-layer is constant. constant_pic_rate_idc[ i ] equal to  2 indicates that the picture rate of the representation of the i-th sub-layer may or may not be constant. The value of constant_pic_rate_idc[ i ] shall be in the range of 0 to 2, inclusive.

avg_pic_rate[ i ] indicates the average picture rate, in units of picture per 256 seconds, of representation of the i-th sub-layer. With fTotal being the number of pictures in the representation of the i-th sub-layer, t1 being the removal time (in seconds) of the first access unit to which the video parameter set applies, and t2 being the removal time (in seconds) of the last access unit (in decoding order) to which the video parameter set applies, the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

avg_pic_rate[ i ]  = =  Round( fTotal * 256  ( t2 − t1 ) )

–	Otherwise (t1 is equal to t2), the following condition shall be true:

avg_pic_rate[ i ]  = =  0

## Operation point syntax and semantics

| operation_point( opIdx ) { | Descriptor |
| --- | --- |
| op_first_present_layer_id[ opIdx ] | u(6) |
| if( !vps_simple_op_mode_flag[ opIdx ] ) { |  |
| op_num_layer_id_values_minus1[ opIdx ] | ue(v) |
| for( i = 1; i <= op_num_layer_id_values_minus1[ opIdx ]; i++ ) |  |
| op_layer_id_delta_minus1[ opIdx ][ i ] | ue(v) |
| } |  |
| } |  |

The operation_point( opIdx ) syntax structure specifies the set of nuh_reserved_zero_6bits values included in the OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set applies.

op_first_present_layer_id[ opIdx ], when vps_simple_op_mode_flag[ opIdx ] is equal to 0, specifies the first (i.e. the 0-th) value of nuh_reserved_zero_6bits included in the OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set applies. When vps_simple_op_mode_flag[ opIdx ] is equal to 1, op_first_present_layer_id[ opIdx ] specifies the greatest value of nuh_reserved_zero_6bits included in the OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set applies.

op_num_layer_id_values_minus1[ opIdx ] plus 1, when vps_simple_op_mode_flag[ opIdx ] is equal to 0, specifies the number of nuh_reserved_zero_6bits values included in the OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set appliesoperationlayer_id_values_minus1[ opIdx ] shall be less than or equal to 63.

In bitstreams conforming to this Specification, op_num_layer_id_values_minus1[ opIdx ] shall be equal to 0. Although the value of op_num_layer_id_values_minus1[ opIdx ] is required to be equal to 0 in this version of this Specification, decoders shall allow other values to appear in the op_num_layer_id_values_minus1[ opIdx ] syntax.

op_layer_id_delta_minus1[ opIdx ][ i ] specifies the difference between the i-th value and the (i − 1)-th value of nuh_reserved_zero_6bits included in the OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set applies.. The value of op_layer_id_delta_minus1[ opIdx ][ i ] shall be in the range of 0 to 63, inclusive.

The variable NumOpLayerIdsMinus1[ opIdx ] is derived as follows.

if( vps_simple_op_mode_flag[ opIdx ] )
	NumOpLayerIdsMinus1[ opIdx ] = op_first_present_layer_id[ opIdx ]
else			(7-X)
	NumOpLayerIdsMinus1[ opIdx ] = op_num_layer_id_values_minus1[ opIdx ]

NumOpLayerIdsMinus1[ 0 ] is inferred to be equal to 0.

The variables OpLayerId[ opIdx ][ i ], for i in the range of 0 to NumOpLayerIdsMinus1[ opIdx ], inclusive, are derived as follows.

OpLayerId[ opIdx ][ 0 ] = vps_simple_op_mode_flag[ opIdx ] ? 0 : op_first_present_layer_id[ opIdx ]
for( i = 1; i <= NumOpLayerIdsMinus1[ opIdx ]; i++ )
	if( vps_simple_op_mode_flag[ opIdx ] )
		OpLayerId[ opIdx ][ i ] = i	(7-X)
	else
		OpLayerId[ opIdx ][ i ] = OpLayerId[ opIdx ][ i − 1 ] + op_layer_id_delta_minus1[ opIdx ][ i ] + 1

The value of OpLayerId[ 0 ][ 0 ] is inferred to be equal to 0.

No value of OpLayerId[ opIdx ][ i ] shall be equal to OpLayerId[ opIdx ][ j ] when i is not equal to j and both i and j are in the range of 0 to NumOpLayerIdsMinus1[ opIdx ], inclusive.

Any two sets OpLayerId[ opIdx1 ] and OpLayerId[ opIdx2 ], where opIdx1 is not equal to opIdx2, shall not include the same sets of nuh_reserved_zero_6bits values.

The OpLayerIdSet of the operation points to which the opIdx-th hrd_parameters( ) syntax structure in the video parameter set applies is set to include and only include the nuh_reserved_zero_6bits values equal to OpLayerId[ opIdx ][ i ], for i in the range of 0 to NumOpLayerIdsMinus1[ opIdx ], inclusive.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).