| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0126r3 |
| --- | --- |

| Title: | AHG9: On HRD and related general issues |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports a list of problems or shortcomings related to the HRD specification and HRD related general topics, such as the description of the general decoding process, bitstream conformance requirements and level constraints. Solutions for the reported issues are summarized in the contribution document, and proposed spec text changes for the solutions, in relative to JCTVC-K0030v2, are enclosed in the same zip file of the contribution document.

In revision 1 of this contribution, a condition is removed from the applicable_operation_points( ) syntax structure, as described in Section 3.

In revision 2 of this contribution, problem 2h and the corresponding proposal was removed from this contribution. In the proposed spec text changes, instead of removing the last bitstream conformance requirement in subclause C.4 of the draft HEVC spec, the formulation of the bitstream conformance requirement was corrected.

In revision 3 of this contribution, the proposed solutions for problem 1c and 1d were modified according to the initial review of these items and offline discussions afterwards. The modified solutions were summarized in subsections 2.3 and 2.4, respectively of this contribution, and the modified spec text changes were included in the attachment enclosed in the same zip file of the contribution document. Furthermore, an acknowledgement to individuals for their reviews, discussions and suggestions to the proposed text changes was added.

# Problems or shortcomings

The current HRD specification and the related parts have the following problems or shortcomings:

- On general HRD behaviors and HRD related general topics

  - There is no definition of operation point, and the operation point concept currently is used without considering TemporalId.

  - The process for bitstream conformance tests is not very clear, e.g. regarding generation of the bitstream subset for each test and selection of HRD parameters, and description text is scattered in different places. Furthermore, the number of bitstream conformance tests is not clearly specified.

  - Currently, only the hrd_parameters( ) syntax structures in the video parameter set may be selected for HRD operations, while the possibly present hrd_parameters( ) syntax structure in the sequence parameter set is never selected. This requires parsing and handling of the video parameter set even when decoding a bitstream that does not contain nuh_reserved_zero_6bits greater than 0 (e.g. a bitstream conforming only the currently specified Main profile).

  - Each hrd_parameters( ) syntax structure, except for the first one, in the VPS is associated with an operation_point( ) syntax structure. The selection of an hrd_parameters( ) syntax structure for use in the HRD operations is based on the associated operation_point( ) syntax structure. Corresponding to each selected hrd_parameters( ) syntax structure, a set of buffering period SEI messages and picture timing SEI messages is also needed in the HRD operations. However, when the operation_point( ) syntax structure for one hrd_parameters( ) syntax structure specifies multiple values of nuh_reserved_zero_6bits, it is not possible to find a corresponding set of buffering period SEI messages and picture timing SEI messages. This is because, currently there is no mechanism specified for signalling of a buffering period or picture timing SEI message that applies to an operation point with multiple values of nuh_reserved_zero_6bits. One possible solution is to include something like the MVC scalable nesting SEI message as specified in Annex H of AVC. However, the MVC scalable nesting SEI message has the following disadvantages when applied to HEVC. Firstly, it does not utilize nuh_reserved_zero_6bits and nuh_temporal_id_plus1 in the NAL unit header, which can be used for association of a buffering period or picture timing SEI message to certain operation points, because SEI NAL units in H.264/AVC only has a one-byte NAL unit header. Secondly, each nested SEI message can only be associated with one operation point.

  - According to the current general decoding process in subclause 8.1, when the decoding process is invoked for HRD bitstream conformance testing, the values of TargetDecLayerIdSet and HighestTid, which determine the bitstream to be decoded, are not properly set.

  - The parameters sps_max_num_reorder_pics[ i ], sps_max_dec_pic_buffering[ i ], and cpb_cnt_minus1[ i ] are not properly selected at various places in texts for HRD operations, bitstream conformance requirements, and level restrictions.

  - The output of the general decoding process in subclause 8.1 is currently specified for the decoded current picture. However, it also talks about the generation of the bitstream to be decoded. It would be more logical if the input of the process is a bitstream and the output is a list of decoded pictures, such that it applies better to the part of the process that specifies the bitstream to be decoded. Other parts are specified to apply to each picture individually.

  - Subclause 8.1 includes the following paragraph:

The decoding process is specified such that all decoders shall produce numerically identical results. Any decoding process that produces identical results to the process described herein conforms to the decoding process requirements of this Specification.

The first sentence specifies a requirement of on the decoding process. However, the spec only specifies decoder conformance (in addition to bitstream conformance), which only requires that the (cropped) output is numerically identical to output by the HRD. Furthermore, a conforming decoder may output more pictures than as specified by the decoder conformance requirement by not inferring no_output_of_prior_pics_flag equal to 1 in some cases. Therefore, is the requirement specified by the first sentence needed?

The "decoding process requirements of this Specification" mentioned in the second sentence is not clear.

- On sub-picture level CPB operations

  - The definition of DU does not consider non-VCL NAL units with nal_unit_type equal to UNSPEC0, EOS_NUT, EOB_NUT, in the range of RSV_NVCL44..RSV_NVCL47, or in the range of UNSPEC48..UNSPEC63.

  - When the CPB operates at sub-picture level (i.e. when SubPicCpbFlag is equal to 1), the signalling and derivation of decoding unit (DU) CPB removal times based on picture timing SEI messages are not error resilient. This is because of the signalling of the duration between CPB removal of the first decoding unit in the current AU and the last DU in the previous AU in decoding order. If the picture timing SEI message of the previous AU is lost, then the CPB removal time of the first DU in the current AU cannot be correctly derived. Furthermore, except for the last DU of the current AU, for which the CPB removal time is derived as equal to that of the current AU, each of the CPB removal time of all other DUs in the current AU relies on the CPB removal time of its previous DU in decoding order. Thus, if the picture timing SEI message of the previous AU is lost, the CPB removal time of every DU in the current AU, except for the last DU, would not be correctly derived.

  - Currently the sub-picture level CPB operation is specified in a way that it always utilizes picture timing SEI messages and never utilizes sub-picture timing (SPT) SEI messages. Furthermore, the DU CPB removal delay signalled in an SPT SEI message is similar as AU CPB removal delay, which is not consistent with the DU CPB removal delay signalling in picture timing SEI message.

  - Sub-picture level CPB parameters in picture timing SEI messages and SPT SEI messages may be both present, redundantly.

  - The picture timing SEI message includes the syntax element num_nalus_in_du_minus1[ i ] to signal the number of NAL units included in a DU. However, based on this signalling, when any single NAL unit in the AU but not in the last DU is lost, the decoder would not be able to correctly derive which sets of NAL units belong to which DUs.

  - The association of NAL units to each DU based the positions of SPT SEI messages does not allow any particular SPT SEI message to be repeated within the associated DU. However, when a DU includes multiple slices, which are transported in more than one packet in an error-prone environment, it may be beneficial to include a copy of the SPT SEI message in each packet, to enable recover the DU decoding time in case other packets of the same DU are lost.

  - In the CPB removal time derivation process, when sub_pic_cpb_params_present_flag is equal to 1, the derivation of CPB removal time uses the final arrival times and the nominal removal times for both cases with SubPicCpbFlag equal to 0 (when the CPB operates at AU level) and with SubPicCpbFlag equal to 1 (when the CPB operates at sub-picture level). However, those used values are derived for only one of the two cases and hene are not available for the other case.

# Summary of the proposed solutions

The proposed solutions are summarized below in this section. Required spec text changes, in relative to JCTVC-K0030v2, for all the proposed solutions are jointly implemented in the attachment enclosed in the same zip file of this document.

## Solution for problem 1a

Problem: There is no definition of operation point, and the operation point concept currently is used without considering TemporalId.

Add the following definition, which also covers TemporalId:

operation point: An operation point is identified by a set of nuh_reserved_zero_6bits values, denoted as OpLayerIdSet, and a TemporalId value, denoted as OpTid, and the associated bitstream subset derived as the output of the sub-bitstream extraction process as specified in subclause Error: Reference source not found with OpTid and OpLayerIdSet as inputs is independently decodable.

## Solution for problem 1b

Problem: The process for bitstream conformance tests is not very clear, e.g. regarding generation of the bitstream subset for each test and selection of HRD parameters, and description text is scattered in different places. Furthermore, the number of bitstream conformance tests is not clearly specified.

The process for each bitstream conformance test is now clearly specified in 7 steps, summarized in below:

- Selection of an operation point under test

- Generation of the sub-bitstream under test

- Selection of one instance of the hrd_parameters( ) syntax structure and one instance of the sub_layer_hrd_parameters( ) syntax structure

- Selection of the HRD initialization point

- Section of SEI messages including timing information

- Selection of a value of SchedSelIdx

- Selection of the initial CPB removal delay and delay offset parameters when the selected HRD initialization point has TFD access units

The texts describing the process of bitstream conformance tests are all included in subclause C.1, and subclause C.4 only includes the list of bitstream conformance requirements.

The number of bitstream conformance tests carried out is equal to N1 * N2 * N3 * ( N4 * 2 + N5), where the values of N1, N2, N3, N4 and N5 are summarized as follows.

–	N1 is the number of operation points contained in the bitstream under test.

–	If BitstreamToDecode is a Type I bitstream, N2 is equal to 1, otherwise N2 is equal to 2.

–	N3 is equal to the selected cpb_cnt_minus1[ ] + 1.

–	N4 and N5 are the numbers of two sets of access units associated with buffering period SEI messages, one set with TFD pictures, and the other not.

## Solution for problem 1c

Problem: The hrd_parameters( ) syntax structure in the SPS is never selected for use with HRD operations.

The operation points to which the hrd_parameters( ) syntax structure in a sequence parameter set is specified to be the operation points for which OpLayerIdSet includes all values of nuh_reserved_zero_6bits present in the bitstream.

## Solution for problem 1d

Problem: There is no mechanism specified for signalling of a buffering period, picture timing or sub-picture timing SEI message that applies to an operation point with multiple values of nuh_reserved_zero_6bits.

The scalable nesting SEI message proposed in JCTVC-K0180v2 is proposed to be applied to specify the set of operation points a buffering period SEI message, picture timing SEI message or SPT SEI message applies to. In the scalable nesting SEI message design in JCTVC-K0180v2, the syntax elements nuh_reserved_zero_6bits and nuh_temporal_id_plus1 in the NAL unit header of SEI NAL units are utilized in specifying the applicable operation points, and the information conveyed in one SEI message may be shared by multiple operation points.

## Solution for problem 1e

Problem: When the decoding process is invoked for HRD bitstream conformance testing, the values of TargetDecLayerIdSet and HighestTid, which determinate the bitstream to be decoded, are not properly set.

In subclause 8.1, the following is specified. When the decoding process is invoked for a bitstream conformance test, the values of TargetDecLayerIdSet and HighestTid (changed to TargetDecHighestTid for editorial consistency) are set to the set of nuh_reserved_zero_6bits values and the greatest TemporalId value present in the sub-bitstream corresponding to the operation point under test for a specific bitstream conformance test.

## Solution for problem 1f

Problem: The parameters sps_max_num_reorder_pics[ i ], sps_max_dec_pic_buffering[ i ], and cpb_cnt_minus1[ i ] are not properly selected at various places in texts for HRD operations, bitstream conformance requirements, and level restrictions.

The parameters sps_max_num_reorder_pics[ i ], sps_max_dec_pic_buffering[ i ], and cpb_cnt_minus1[ i ] in the HRD operations, bitstream conformance requirements and level restrictions are consistently selected with i equal to the specified value of TargetDecHighestTid.

## Solution for problem 1g

Problem: The output of the general decoding process in subclause 8.1 is currently specified as the decoded current picture while it talks about the generation of the bitstream to be decoded.

The input of the process is specified as a bitstream and the output is a list of decoded pictures. Firstly the bitstream to be decoded is decided, and then the other parts are specified to apply to each picture individually.

## Solution for problem 1h

Problem: The decoding process requirements seem not clear and unnecessary.

The paragraph, repeated below for conveneince, are removed from the general decoding process in subclause 8.1.

The decoding process is specified such that all decoders shall produce numerically identical results. Any decoding process that produces identical results to the process described herein conforms to the decoding process requirements of this Specification.

## Solution for problem 2a

Problem: The definition of DU does not consider non-VCL NAL units with nal_unit_type equal to UNSPEC0, EOS_NUT, EOB_NUT, in the range of RSV_NVCL44..RSV_NVCL47, or in the range of UNSPEC48..UNSPEC63.

The definition of decoding unit is changed as follows, with addition of two definitons of "associated VCL NAL unit" and "associated non-VCL NAL unit". The multi-sentence editorial problem for this definition is also solved.

associated non-VCL NAL unit: An assoicated non-VCL NAL unit of a VCL NAL unit is one of the non-VCL NAL units for which the VCL NAL unit is their associated VCL NAL unit.

associated VCL NAL unit: The most recent preceding VCL NAL unit in decoding order for non-VCL NAL units with nal_unit_type equal to UNSPEC0, EOS_NUT, EOB_NUT, FD_NUT, in the range of RSV_NVCL44..RSV_NVCL47, or in the range of UNSPEC48..UNSPEC63, or the the first subsequent VCL NAL unit in decoding order for non-VCL NAL units with nal_unit_type equal to other values.

decoding unit: An access unit when SubPicCpbFlag is equal to 0 or a subset of an access unit when SubPicCpbFlag is equal to 1, consisting of one or more VCL NAL units in an access unit and the associated non-VCL NAL units.

## Solution for problem 2b

Problem: If the picture timing SEI message of the previous AU is lost, the CPB removal time of every DU in the current AU, except for the last DU, would not be correctly derived.

Instead of signalling the duration between CPB removal of the first DU in the current AU and the last DU in the previous AU, for each current DU, the duration between CPB removal of the next DU in decoding order in the same AU and the current DU is signalled. For the last DU in the current AU, same as currently specified, the CPB removal time is derived as equal to that of the entire AU. Therefore, the derivation of DU CPB removal times is based on the AU CPB removal time and the signalled sub-picture level CPB parameters. This way, loss of the picture timing SEI message in the previous AU would not cause incorrect derivation of DU CPB removal times for the current AU.

## Solution for problem 2c

Problem: Sub-picture timing (SPT) SEI messages are never used, and the DU CPB removal delay signalling in the SPT SEI message is not consistent with the signalling in the picture timing SEI message.

The DU CPB removal time derivation process is specified to use the DU removal timing information either in picture timing SEI messages or SPT SEI messages.

The duration between CPB removal of the last DU in the AU and the DU associated with the SPT SEI message is signalled, and the length in bits for the syntax element is the same as the syntax element in the picture timing SEI message for DU CPB removal delay.

## Solution for problem 2d

Problem: Sub-picture level CPB parameters in picture timing SEI messages and SPT SEI messages may be both present, redundantly.

The flag sub_pic_cpb_params_in_pic_timing_sei_flag is signalled in the hrd_parameters( ) syntax structure to control the presence of DU CPB parameters, to be either in picture timing SEI messages or SPT SEI messages, but never in both. This flag also controls whether to use the DU removal timing information from picture timing SEI messages or SPT SEI messages.

## Solution for problems 2e and 2f

Problem 2e: For the association of NAL units to DUs based on num_nalus_in_du_minus1[ i ], when any single NAL unit in the AU but not in the last DU is lost, the decoder would not be able to correctly derive which sets of NAL units belongs to which DUs

Problem 2f: Duplication of SPT SEI messages within the associated DU is not possible.

The two problems are both solved by signalling of DU index in the SPT SEI message. Since now it also includes non-timing information, the name is changed to DU information SEI message.

The association of NAL units to DUs is then based on DU index values. This way, received NAL units can always be correctly associated to DUs, and DU SEI messages (formerly SPT SEI messages) can be repeated within a DU.

## Solution for problems 2g

Problem: Unavailable values are used for derivation of the CPB removal times when sub_pic_cpb_params_present_flag is equal to 1.

When sub_pic_cpb_params_present_flag is equal to 1, the CPB arrival times and the CPB nominal removal times for both AU level and sub-picture level are required, regardless of the value of SubPicCpbFlag. Therefore, the CBP arrival times and nominal removal times for both AU level and sub-picture level are derived regardless of the value of SubPicCpbFlag, while the CPB removal times are derived only for AU level when SubPicCpbFlag is equal to 0 and only for sub-picture level when SubPicCpbFlag is equal to 1.

# Changes made in revision 1 of this document

In JCTVC-K0126r1, the following syntax structure in the provided spec text changes

| applicable_operation_points( ) { |  |
| --- | --- |
| num_applicable_ops_minus1 | ue(v) |
| if( num_applicable_ops_minus1 > 0) |  |
| default_op_applicable_flag | u(1) |
| mumOpsSignalled = default_op_applicable_flag ? num_applicable_ops_minus1 : num_applicable_ops_minus1 + 1 |  |
| for( i = 0; i < mumOpsSignalled; i++ ) { |  |
| operation_point_layer_ids( i ) |  |
| op_temporal_id[ i ] | u(3) |
| } |  |
| } |  |

is changed to

| applicable_operation_points( ) { |  |
| --- | --- |
| num_applicable_ops_minus1 | ue(v) |
| default_op_applicable_flag | u(1) |
| mumOpsSignalled = default_op_applicable_flag ? num_applicable_ops_minus1 : num_applicable_ops_minus1 + 1 |  |
| for( i = 0; i < mumOpsSignalled; i++ ) { |  |
| operation_point_layer_ids( i ) |  |
| op_temporal_id[ i ] | u(3) |
| } |  |
| } |  |

where the condition "if( num_applicable_ops_minus1 > 0)" (the third row) was removed from the syntax table.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Acknowledgement

Miska M. Hannuksela of Nokia is thanked for the review and suggestions to the text changes for general HRD description in subclause C.1, particularly those parts related to the description of bitstream conformance tests, including selection of HRD parameters and related SEI messages for each bitstream conformance test. Sachin Deshpande of Sharp Labs of America and Kimihiko Kazui of Fujitsu are thanked for the reviews, discussions and suggestions to the text changes related to signalling and derivation sub-picture level CPB removal timing. Thomas Schierl of Fraunhofer HHI is thanks for reviewing and confirming the text changes related to sub-picture timing SEI message. Adarsh K. Ramasubramonian of Qualcomm is thanked for the review and reports of several typos in the text changes.