| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0127 |
| --- | --- |

| Title: | AHG9: Signalling of bitstream and elementary stream properties |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Adarsh K. Ramasubramonian 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes a bitstream properties SEI message and an elementary stream properties SEI message, for signalling of the consistence of various properties, such as profile, tier and level, spatial resolution, and so on, across all coded video sequences in a bitstream or an elementary stream.

# Introduction

## Bitstream and elementary stream

HEVC specifies the concept of bitstream and elementary stream (ES), defined as follows.

bitstream: A sequence of bits that forms the representation of coded pictures and associated data forming one or more coded video sequences. Bitstream is a collective term used to refer either to a NAL unit stream or a byte stream.

elementary stream: A sequence of one or more bitstreams.

An elementary stream that consists of two or more bitstreams would typically have been formed by splicing together two or more bitstreams (or parts thereof). When an elementary stream contains multiple bitstreams, except for the last bitstream, at the end of each of the other bitstreams there must be an end of bitstream (EOS) NAL unit.

## Bitstream and elementary stream properties

A bitstream may contain multiple coded video sequences (CVSs) that are of different profiles, possibly with additional restrictions as signalled by general_profile_compatibility_flag[ i ] for i in the range of 0 to 31, inclusive, and general_reserved_zero_16bits. Thus, given a bitstream, simply inspecting the profile information included in the active SPS for the first CVS is not sufficient to know whether a decoder conforming to a particular profile can decode the entire bitstream. To ensure that a decoder can decode the entire bitstream, all CVSs in the bitstream must be inspected to find out whether all the CVSs conform to the same profile. Such an inspection is not simple at all as virtually all NAL units in the bitstream may have to be checked to find out all the active SPSs, which involves finding out SPS NAL units, EOS NAL units, NAL units of RAP pictures, and even SEI NAL units, and parsing into buffering period SEI messages and/or slice headers of NAL units of RAP pictures to find out which SPS is active for the CVS.

The situation is similar for the following types of bitstream level information, though for some of them the complexity involved in finding out the information from the bitstream is not as high:

- Whether tier and level remains unchanged for all CVSs in the bitstream.

- Whether all CVSs in the bitstream have the same spatial resolution.

- Whether all CVSs in the bitstream have the same maximum bitrate.

- Whether all CVSs in the bitstream have the same maximum picture rate.

- Whether all CVSs in the bitstream have the same color format.

- Whether all CVSs in the bitstream have the same sample aspect ratio.

- Whether all CVSs in the bitstream have the same set of bit depths.

- Whether the presence of parameter sets in the bitstream enables full random accessibility, i.e. it is possible to perform random access at the position of each RAP access unit by discarding all access units before the RAP access unit (and to correctly decode the RAP picture and all the subsequent non-TFD pictures in decoding order). This requires that, when the above random access is performed at any RAP access unit, each parameter set is available in the bitstream when it needs to be activated.

- Whether in the bitstream at least one VPS is updated, whether in the bitstream at least one SPS is updated, and whether in the bitstream at least one PPS is updated.

- Whether all VPSs are present at the beginning of the bitstream (i.e. within the first access unit in the bitstream), whether all SPSs are present at the beginning of the bitstream, and whether all PPSs are present at the beginning of the bitstream.

- Whether all CVSs in the bitstream contain only progressive frames coded in frame pictures, i.e. the scan type is progressive (not interlaced), and there is no field picture.

- Whether all CVSs in the bistream contain no frame packing arrangement SEI messages, i.e. the video segment represented by each CVS in the bitstream is not frame-packed.

- Whether all CVSs in the bitstream have the same configuration of tiles.

- Whether all CVSs in the bitstream have wavefront parallel processing (WPP) enabled.

Similarly, an ES may contain multiple bitstreams and consequently may also contain multiple CVSs. The CVSs in an ES may also have different profiles and/or the above items listed.

## Usefulness for signalling of bitstream and elementary stream properties

The following ES-level and bitstream-level information are important for session negotiation, capability exchange and content selection etc. in video applications: profile, tier, level, spatial resolution, bitrate, picture rate, colour format, sample aspect ratio, and bit depth. Knowing that any of such information being consistent accoss CVSs would significantly simplify the process to get the exact information, for example as mentioned earlier for profile.

In both the draft HEVC file format (becoming a part of ISO/IEC 14496-15, which currently includes the file format specifications for AVC and its SVC and MVC extensions) and MPEG-2 systems extension for HEVC carriage, full random accessibility (see item 8 above) is required. Thus, if it is known that full random accessibility is supported, the composer for HEVC files and MPEG-2 TS content does not have to inspect the stream to find it out.

Knowing that whether parameter set update is present, senders and receivers would know whether they need to take additional measures to avoid hard-to-detect loss of updated parameter sets, and decoders know whether they need to check whether a parameter set with an existing ID is simply a repetition by bit-by-bit comparison.

Knowing all parameter sets are at the beginning, the server or sender can simply take all the parameter sets from the beginning and send them out-of-band, without the need for worry about those possibly repeated ones in later access units. Also, it would be easier to compose HEVC files according to the file format without the need for checking whether this condition holds, as in the file format such information is signalled in the sample entry.

Knowing whether tiles configuration and/or support of WPP are the same across CVSs provides may further assist workload allocation in the multi-threaded decoding, in addition to the same use provided by tiles_fixed_structure_flag at the CVS level.

# Proposal

This document proposes to signal whether the above information stays unchanged across all CVSs in an ES or a bitstream, by using an ES properties SEI message and a bitstream properties SEI message, respectively. The syntax and semantics of the proposed SEI messages are provided in the following subsections.

## Elementary stream properties SEI message

| elementary_stream_properties( payloadSize ) { | Descriptor |
| --- | --- |
| es_profile_unchanging_flag | u(1) |
| es_tier_level_unchanging_flag | u(1) |
| es_spatial_resolution_unchanging_flag | u(1) |
| es_max_bitrate_unchanging_flag | u(1) |
| es_max_pic_rate_unchanging_flag | u(1) |
| es_colour_format_unchanging_flag | u(1) |
| es_aspect_ratio_unchanging_flag | u(1) |
| es_bit_depth_unchanging_flag | u(1) |
| es_full_random_access_enabled_flag | u(1) |
| es_all_vps_in_first_au_flag | u(1) |
| if(es_all_vps_in_first_au_flag ) |  |
| es_no_vps_set_update_flag | u(1) |
| es_all_sps_in_first_au_flag | u(1) |
| if(es_all_sps_in_first_au_flag ) |  |
| es_no_sps_set_update_flag | u(1) |
| es_all_pps_in_first_au_flag | u(1) |
| if(es_all_pps_in_first_au_flag ) |  |
| es_no_pps_set_update_flag | u(1) |
| es_progressive_frames_only_flag | u(1) |
| es_unpacked_frames_only_flag | u(1) |
| es_tiles_unchanging_flag | u(1) |
| es_wpp_unchanging_flag | u(1) |
| es_properties_sei_extension_flag | u(1) |
| } |  |

The elementary stream properties SEI message provides information applicable for the entire elementary stream containing the SEI message. The elementary stream properties SEI message may only be present in the first access unit of an elementary stream. The SEI NAL unit containing the elementary stream properties SEI message shall not contain other SEI messages. The values of nuh_temporal_id_plus1 in an SEI NAL unit containing an elementary stream properties SEI message shall be equal to 1.

es_profile_unchanging_flag equal to 1 indicates that the values of general_profile_idc, general_profile_compatibility_flag[ i ] for i in the range of 0 to 31, inclusive, and general_reserved_zero_16bits are respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded. es_profile_unchanging_flag equal to 0 indicates that the values of general_profile_idc, general_profile_compatibility_flag[ i ] for i in the range of 0 to 31, inclusive, and general_reserved_zero_16bits are not respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded.

es_tier_level_unchanging_flag equal to 1 indicates that the values of general_tier_flag and general_level_idc are respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded. es_tier_level_unchanging_flag equal to 0 indicates that the values of general_tier_flag and general_level_idc are not respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded.

es_spatial_resolution_unchanging_flag equal to 1 indicates that the values of pic_width_in_luma_samples and pic_height_in_luma_samples are respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded. es_spatial_resolution_unchanging_flag equal to 0 indicates that the values of pic_width_in_luma_samples and pic_height_in_luma_samples are not respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded.

es_max_bitrate_unchanging_flag equal to 1 indicates that the maximum bitrate is identical for all coded video sequences in the elementary stream. es_max_bitrate_unchanging_flag equal to 0 indicates that the maximum bitrate is not identical for all coded video sequences in the elementary stream.

es_max_pic_rate_unchanging_flag equal to 1 indicates that the maximum picture rate is identical for all coded video sequences in the elementary stream. es_max_pic_rate_unchanging_flag equal to 0 indicates that the maximum picture rate is not identical for all coded video sequences in the elementary stream.

es_colour_format_unchanging_flag equal to 1 indicates that the value of chroma_format_idc is identical for all sequence parameter sets that are activated when the elementary stream is decoded. es_colour_format_unchanging_flag equal to 0 indicates that the value of chroma_format_idc is not identical for all sequence parameter sets that are activated when the elementary stream is decoded.

es_aspect_ratio_unchanging_flag equal to 1 indicates that the value of aspect_ratio_idc is identical for all sequence parameter sets that are activated when the elementary stream is decoded, and, when aspect_ratio_idc is equal to Extended_SAR, the values of sar_width and sar_height are respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded. es_aspect_ratio_unchanging_flag equal to 0 indicates that the value of aspect_ratio_idc is not identical for all sequence parameter sets that are activated when the elementary stream is decoded, or, the values of sar_width and sar_height are not respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded.

es_bit_depth_unchanging_flag equal to 1 indicates that the values of bit_depth_luma_minus8 and bit_depth_chroma_minus8 are respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded. es_bit_depth_unchanging_flag equal to 0 indicates that the values of bit_depth_luma_minus8 and bit_depth_chroma_minus8 are not respectively identical for all sequence parameter sets that are activated when the elementary stream is decoded.

es_full_random_access_enabled_flag equal to 1 indicates that it is possible to perform random access at the position of each RAP access unit in the elementary stream by discarding all access units before the RAP access unit and to correctly decode the RAP picture and all the subsequent non-TFD pictures in decoding order, i.e. for each RAP access unit, when all access units before the RAP access unit are discarded and the remaining part of the elementary stream is decoded, each parameter set is still available in the elementary stream when it needs to be activated.

es_all_vps_in_first_au_flag equal to 1 indicates that all video parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream. es_all_vps_in_first_au_flag equal to 0 indicates that not all video parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream.

NOTE 1 – When es_all_vps_in_first_au_flag is equal to 1, there may still be video parameter sets repeated in other access units in the elementary stream, e.g. for error resilience purposes, however, there won't be any video parameter set update.

es_no_vps_update_flag equal to 0 indicates that the content of at least one video parameter set in the elementary stream is changed without changing the video_parameter_set_id in the video parameter set NAL unit. es_no_vps_update_flag equal to 1 indicates that the content of no video parameter set in the elementary stream is changed without changing the video_parameter_set_id in the video parameter set NAL unit.

es_all_sps_in_first_au_flag equal to 1 indicates that all sequence parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream. es_all_sps_in_first_au_flag equal to 0 indicates that not all sequence parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream.

NOTE 2 – When es_all_sps_in_first_au_flag is equal to 1, there may still be sequence parameter sets repeated in other access units in the elementary stream, e.g. for error resilience purposes, however, there won't be any sequence parameter set update.

es_no_sps_update_flag equal to 1 indicates that the content of at least one sequence parameter set in the elementary stream is changed without changing the seq_parameter_set_id in the sequence parameter set NAL unit. es_no_sps_update_flag equal to 0 indicates that the content of no sequence parameter set in the elementary stream is changed without changing the seq_parameter_set_id in the sequence parameter set NAL unit.

es_all_pps_in_first_au_flag equal to 1 indicates that all picture parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream. es_all_pps_in_first_au_flag equal to 0 indicates that not all picture parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream.

NOTE 3 – When es_all_pps_in_first_au_flag is equal to 1, there may still be picture parameter sets repeated in other access units in the elementary stream, e.g. for error resilience purposes, however, there won't be any picture parameter set update.

es_no_pps_update_flag equal to 1 indicates that the content of at least one picture parameter set in the elementary stream is changed without changing the pic_parameter_set_id in the picture parameter set NAL unit. es_no_pps_update_flag equal to 0 indicates that the content of no picture parameter set in the elementary stream is changed without changing the pic_parameter_set_id in the picture parameter set NAL unit.

es_progressive_frames_only_flag equal to 1 indicates that the value of field_seq_flag is equal to 0 for all sequence parameter sets that are activated when the elementary stream is decoded, and there is no field indication SEI message in the elementary stream. es_progressive_frames_only_flag equal to 0 indicates that there is at least one field indication SEI message in the elementary stream.

es_unpacked_frames_only_flag equal to 1 indicates that there is no frame packing arrangement SEI message in the elementary stream. es_unpacked_frames_only_flag equal to 0 indicates that there is at least one frame packing arrangement SEI message in the elementary stream.

es_tiles_unchanging_flag equal to 1 indicates that the value of tiles_enabled_flag is identical for all picture parameter sets that are activated when the elementary stream is decoded, and when the value of tiles_enabled_flag is equal to 1, the value of tiles_fixed_structure_flag is equal to 1 for all sequence parameter sets that are activated when the elementary stream is decoded, and all the picture parameter sets that are activated when the elementary stream is decoded have the set of values for the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width[ i ], row_height[ i ] and loop_filter_across_tiles_enabled_flag, when present. es_tiles_unchanging_flag equal to 0 indicates that the value of tiles_enabled_flag is not identical for all picture parameter sets that are activated when the elementary stream is decoded, or the value of tiles_fixed_structure_flag is not equal to 1 for all sequence parameter sets that are activated when the elementary stream is decoded, or all the picture parameter sets that are activated when the elementary stream is decoded do not have the set of values for the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width[ i ], row_height[ i ] and loop_filter_across_tiles_enabled_flag, when present.

es_wpp_unchanging_flag equal to 1 indicates that the value of entropy_coding_sync_enabled_flag is identical for all picture parameter sets that are activated when the elementary stream is decoded. es_wpp_unchanging_flag equal to 0 indicates that the value of entropy_coding_sync_enabled_flag is not identical for all picture parameter sets that are activated when the elementary stream is decoded.

es_properties_sei_extension_flag equal to 0 indicates that no additional data follows within the elementary stream properties SEI message. It is a requirement of bitstream conformance that the value of es_properties_sei_extension_flag shall be equal to 0. The value 1 for es_properties_sei_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of es_properties_sei_extension_flag in an elementary stream properties SEI message and shall ignore all data that follows within an elementary stream properties SEI message after the value 1 for es_properties_sei_extension_flag.

## Bitstream properties SEI message

| bitstream_properties( payloadSize ) { | Descriptor |
| --- | --- |
| bitstream_profile_unchanging_flag | u(1) |
| bitstream_tier_level_unchanging_flag | u(1) |
| bitstream_spatial_resolution_unchanging_flag | u(1) |
| bitstream_max_bitrate_unchanging_flag | u(1) |
| bitstream_max_pic_rate_unchanging_flag | u(1) |
| bitstream_colour_format_unchanging_flag | u(1) |
| bitstream_aspect_ratio_unchanging_flag | u(1) |
| bitstream_bit_depth_unchanging_flag | u(1) |
| bitstream_full_random_access_enabled_flag | u(1) |
| bitstream_all_vps_in_first_au_flag | u(1) |
| if( bitstream_all_vps_in_first_au_flag ) |  |
| bitstream_no_vps_update_flag | u(1) |
| bitstream_all_sps_in_first_au_flag | u(1) |
| if(bitstream_all_sps_in_first_au_flag ) |  |
| bitstream_no_sps_set_update_flag | u(1) |
| bitstream_all_pps_in_first_au_flag | u(1) |
| if(bitstream_all_pps_in_first_au_flag ) |  |
| bitstream_no_pps_set_update_flag | u(1) |
| bitstream_progressive_frames_only_flag | u(1) |
| bitstream_unpacked_frames_only_flag | u(1) |
| bitstream_tiles_unchanging_flag | u(1) |
| bitstream_wpp_unchanging_flag | u(1) |
| bitstream_properties_sei_extension_flag | u(1) |
| } |  |

The bitstream properties SEI message provides information applicable for the entire bitstream containing the SEI message. The bitstream properties SEI message may only be present in the first access unit of a bitstream. The SEI NAL unit containing the bitstream properties SEI message shall not contain other SEI messages. The values of nuh_temporal_id_plus1 in an SEI NAL unit containing a bitstream properties SEI message shall be equal to 1.

bitstream_profile_unchanging_flag equal to 1 indicates that the values of general_profile_idc, general_profile_compatibility_flag[ i ] for i in the range of 0 to 31, inclusive, and general_reserved_zero_16bits are respectively identical for all sequence parameter sets that are activated when the bitstream is decoded. bitstream_profile_unchanging_flag equal to 0 indicates that the values of general_profile_idc, general_profile_compatibility_flag[ i ] for i in the range of 0 to 31, inclusive, and general_reserved_zero_16bits are not respectively identical for all sequence parameter sets that are activated when the bitstream is decoded.

bitstream_tier_level_unchanging_flag equal to 1 indicates that the values of general_tier_flag and general_level_idc are respectively identical for all sequence parameter sets that are activated when the bitstream is decoded. bitstream_tier_level_unchanging_flag equal to 0 indicates that the values of general_tier_flag and general_level_idc are not respectively identical for all sequence parameter sets that are activated when the bitstream is decoded.

bitstream_spatial_resolution_unchanging_flag equal to 1 indicates that the values of pic_width_in_luma_samples and pic_height_in_luma_samples are respectively identical for all sequence parameter sets that are activated when the bitstream is decoded. bitstream_spatial_resolution_unchanging_flag equal to 0 indicates that the values of pic_width_in_luma_samples and pic_height_in_luma_samples are not respectively identical for all sequence parameter sets that are activated when the bitstream is decoded.

bitstream_max_bitrate_unchanging_flag equal to 1 indicates that the maximum bitrate is identical for all coded video sequences in the bitstream. bitstream_max_bitrate_unchanging_flag equal to 0 indicates that the maximum bitrate is not identical for all coded video sequences in the bitstream.

bitstream_max_pic_rate_unchanging_flag equal to 1 indicates that the maximum picture rate is identical for all coded video sequences in the bitstream. bitstream_max_pic_rate_unchanging_flag equal to 0 indicates that the maximum picture rate is not identical for all coded video sequences in the bitstream.

bitstream_colour_format_unchanging_flag equal to 1 indicates that the value of chroma_format_idc is identical for all sequence parameter sets that are activated when the bitstream is decoded. bitstream_colour_format_unchanging_flag equal to 0 indicates that the value of chroma_format_idc is not identical for all sequence parameter sets that are activated when the bitstream is decoded.

bitstream_aspect_ratio_unchanging_flag equal to 1 indicates that the value of aspect_ratio_idc is identical for all sequence parameter sets that are activated when the bitstream is decoded, and, when aspect_ratio_idc is equal to Extended_SAR, the values of sar_width and sar_height are respectively identical for all sequence parameter sets that are activated when the bitstream is decoded. bitstream_aspect_ratio_unchanging_flag equal to 0 indicates that the value of aspect_ratio_idc is not identical for all sequence parameter sets that are activated when the bitstream is decoded, or, the values of sar_width and sar_height are not respectively identical for all sequence parameter sets that are activated when the bitstream is decoded.

bitstream_bit_depth_unchanging_flag equal to 1 indicates that the values of bit_depth_luma_minus8 and bit_depth_chroma_minus8 are respectively identical for all sequence parameter sets that are activated when the bitstream is decoded. bitstream_bit_depth_unchanging_flag equal to 0 indicates that the values of bit_depth_luma_minus8 and bit_depth_chroma_minus8 are not respectively identical for all sequence parameter sets that are activated when the bitstream is decoded.

bitstream_full_random_access_enabled_flag equal to 1 indicates that it is possible to perform random access at the position of each RAP access unit in the bitstream by discarding all access units before the RAP access unit and to correctly decode the RAP picture and all the subsequent non-TFD pictures in decoding order, i.e., for each RAP access unit, when all access units before the RAP access unit are discarded and the remaining part of the bitstream is decoded, each parameter set is still available in the bitstream when it needs to be activated.

bitstream_all_parameter_sets_in_first_au_flag equal to 1 indicates that all parameter sets that are activated when the bitstream is decoded are present in the first access unit in bitstream. bitstream_all_parameter_sets_in_first_au_flag equal to 0 indicates that not all parameter sets that are activated when the bitstream is decoded are present in the first access unit in bitstream.

NOTE 1 – When bitstream_all_parameter_sets_in_first_au_flag is equal to 1, there may still be parameter sets repeated in other access units in the bitstream, e.g. for error resilience purposes, however, there won't be any parameter set update.

bitstream_no_parameter_set_update_flag equal to 1 indicates that the content of at least one video parameter set, sequence parameter set, or picture paramete set in the bitstream is changed without changing the video_parameter_set_id, seq_parameter_set_id, or pic_parameter_set_id in the parameter set NAL unit. bitstream_no_parameter_set_update_flag equal to 0 indicates that the content of no parameter set is changed without changing the video_parameter_set_id, seq_parameter_set_id, or pic_parameter_set_id in the parameter set NAL unit.

bitstream_all_sps_in_first_au_flag equal to 1 indicates that all sequence parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream. bitstream_all_sps_in_first_au_flag equal to 0 indicates that not all sequence parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream.

NOTE 2 – When bitstream_all_sps_in_first_au_flag is equal to 1, there may still be sequence parameter sets repeated in other access units in the elementary stream, e.g. for error resilience purposes, however, there won't be any sequence parameter set update.

bitstream_no_sps_update_flag equal to 0 indicates that the content of at least one sequence parameter set in the elementary stream is changed without changing the seq_parameter_set_id in the sequence parameter set NAL unit. bitstream_no_sps_update_flag equal to 1 indicates that the content of no sequence parameter set in the elementary stream is changed without changing the seq_parameter_set_id in the sequence parameter set NAL unit.

bitstream_all_pps_in_first_au_flag equal to 1 indicates that all picture parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream. bitstream_all_pps_in_first_au_flag equal to 0 indicates that not all picture parameter sets that are activated when the elementary stream is decoded are present in the first access unit in elementary stream.

NOTE 3 – When bitstream_all_pps_in_first_au_flag is equal to 1, there may still be picture parameter sets repeated in other access units in the elementary stream, e.g. for error resilience purposes, however, there won't be any picture parameter set update.

bitstream_no_pps_update_flag equal to 1 indicates that the content of at least one picture parameter set in the elementary stream is changed without changing the pic_parameter_set_id in the picture parameter set NAL unit. bitstream_no_pps_update_flag equal to 0 indicates that the content of no picture parameter set in the elementary stream is changed without changing the pic_parameter_set_id in the picture parameter set NAL unit.

bitstream_progressive_frames_only_flag equal to 1 indicates that the value of field_seq_flag is equal to 0 for all sequence parameter sets that are activated when the bitstream is decoded, and there is no field indication SEI message in the bitstream. bitstream_progressive_frames_only_flag equal to 0 indicates that there is at least one field indication SEI message in the bitstream.

bitstream_unpacked_frames_only_flag equal to 1 indicates that there is no frame packing arrangement SEI message in the bitstream. bitstream_unpacked_frames_only_flag equal to 0 indicates that there is at least one frame packing arrangement SEI message in the bitstream.

bitstream_tiles_unchanging_flag equal to 1 indicates that the value of tiles_enabled_flag is identical for all picture parameter sets that are activated when the bitstream is decoded, and when the value of tiles_enabled_flag is equal to 1, the value of tiles_fixed_structure_flag is equal to 1 for all sequence parameter sets that are activated when the bitstream is decoded, and all the picture parameter sets that are activated when the bitstream is decoded have the set of values for the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width[ i ], row_height[ i ] and loop_filter_across_tiles_enabled_flag, when present. bitstream_tiles_unchanging_flag equal to 0 indicates that the value of tiles_enabled_flag is not identical for all picture parameter sets that are activated when the bitstream is decoded, or the value of tiles_fixed_structure_flag is not equal to 1 for all sequence parameter sets that are activated when the bitstream is decoded, or all the picture parameter sets that are activated when the bitstream is decoded do not have the set of values for the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width[ i ], row_height[ i ] and loop_filter_across_tiles_enabled_flag, when present.

bitstream_wpp_unchanging_flag equal to 1 indicates that the value of entropy_coding_sync_enabled_flag is identical for all picture parameter sets that are activated when the bitstream is decoded. bitstream_wpp_unchanging_flag equal to 0 indicates that the value of entropy_coding_sync_enabled_flag is not identical for all picture parameter sets that are activated when the bitstream is decoded.

bitstream_properties_sei_extension_flag equal to 0 indicates that no additional data follows within the bitstream properties SEI message. It is a requirement of bitstream conformance that the value of bitstream_properties_sei_extension_flag shall be equal to 0. The value 1 for bitstream_properties_sei_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of bitstream_properties_sei_extension_flag in a bitstream properties SEI message and shall ignore all data that follows within a bitstream properties SEI message after the value 1 for bitstream_properties_sei_extension_flag.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).