| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0128r2 |
| --- | --- |

| Title: | AHG9: Signalling of regions of interest and gradual decoding refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes the following:

- A sub-picture level SEI message, for signalling of the association of NAL units to different regions for region-of-interest (ROI) coding and/or gradual decoding refresh (GDR) coding.

- A sequence-level SEI message, for signalling of the position and size of each ROI.

- Two enabling flags in the SPS, one for ROI and the other for GDR, for control of syntax elements in the proposed sub-picture level SEI message.

Revision 1 of JCTVC-K0128 provides spec text changes only for the GDR aspect, without changing to the SPS syntax, in section 3 of JCTVC-K0128.

Revisions 2 and 3 of JCTVC-K0128 provides updated spec text changes only for the GDR aspect, without changing to the SPS syntax, in section 4 of JCTVC-K0128, in response to comments and suggestions from Miska M. Hannuksela of Nokia (thanks!).

# Introduction

A region of interest (ROI), which is a subset of the picture region, may be independently coded throughout a temporal duration, such that only data units required for decoding a desired ROI need to be transmitted.

A video sequence may also be encoded in a way that random access from a non-intra picture is possible, and after a few pictures, the entire picture region can be correctly decoded at a position and afterwards in display/output order. This behavior is referred to as gradual decoding refresh (GDR). GDR provides both random accessibility and enhanced error resilience. When random accessing from a GDR starting point, the so-called foreground region (also referred to as the refreshed region) gradually increase until it covers the entire picture regions, and the data units not needed for decoding the foreground region do not need to be transmitted.

Currently, there is no mechanism for signalling of ROIs in HEVC. In AVC, ROI coding can be signalled using slice group type 2, which has also been supported in AVC's SVC extension.

The recovery point SEI message in HEVC may be used to signal a GDR starting point and the corresponding recovery point. However, there is no way to signal which NAL units are needed if random access is performed at a GDR starting point.

This document proposes a sub-picture level SEI message for signalling of the association of NAL units to different regions when ROI coding and/or GDR are supported, such that it can be known which NAL units need to be transmitted when a particular ROI is requested or when performing random access at a GDR starting point.

In addition, a sequence-level SEI message is proposed, for signalling of the position and size of each ROI.

To control signalling of syntax elements in the sub-picture level SEI message, two enabling flags are proposed to be signalled in the SPS, one for ROI and the other for GDR.

Syntax and semantics for the proposed changes are provided in the following section.

# Proposal

## Independent sub-picture and gradual decoding refresh information SEI message syntax and semantics

| isp_gdr_info( payloadSize ) { | Descriptor |
| --- | --- |
| if( indenpendent_sub_picture_enabled_flag ) |  |
| isp_id | ue(v) |
| if( gradual_decoding_refresh_enabled_flag ) |  |
| gdr_foreground_flag | u(1) |
| } |  |

The indenpendent sub-picture (ISP) and gradual decoding refresh (GDR) information SEI message provides the identifier of the ISP that contains the slices the current SEI message applies to, and/or whether the slices the current SEI message applies to belongs to the foreground region of the current picture, for which the access unit contains the current SEI message.

If the ISP and GDR information SEI message is not the last ISP and GDR information SEI message in the access unit, the slices to which the SEI message applies consists, in decoding order, of all slices that succeed the SEI NAL unit containing the SEI message in decoding order and precede the next the SEI NAL unit containing an ISP and GDR information SEI message in decoding order. Otherwise, the slices to which an ISP and GDR information SEI message applies consists of all slices that succeed the the SEI NAL unit containing the SEI message in decoding order.

The slices that an ISP and GDR information SEI message applies to are also referred to as the slices associated with the ISP and GDR information SEI message.

isp_id specifies the identifer of the independent sub-picture the slices associated with the current SEI message belongs to. The value of isp_id shall be in the range of 0 to 255, inclusive.

An independent sub-picture (ISP) identified by a particular value ispID include and only include all slices associated with ISP and GDR information SEI messages having isp_id equal to ispID in the same picture. The region represented by an ISP shall be a rectangular region. And regions repesented by different ISPs in a picture shall not overlap. An ISP shall be independently decodable from other slices not in the ISP in the same picture, i.e. when a slice belongs to an ISP, and has dependent_slice_flag equal 1, the previous slice, in decoding order, that has dependent_slice_flag equal to 0 shall belong to the same ISP.

Within a coded video sequence, when indenpendent_sub_picture_enabled_flag is equal to 1, independent sub-pictures identified by a particualr isp_id from all the coded pictures are collectively referred to as an ISP sequence. An ISP sequence shall be independently decodable from coded slices not belonging to the ISP sequence, i.e., in decoding of a slice in an ISP, no samples from a slice not belonging to the ISP sequence shall be used in the inter prediction process.

gdr_foreground_flag equal to 1 indicates that the slices associated with the current SEI message belong to the forground region in the current picture. gdr_foreground_flag equal to 0 indicates that the slices associated with the current SEI message belong to the background region in the current picture.

The forground region in a picture include and only include all slices associated with ISP and GDR information SEI messages having gdr_foreground_flag equal to 1 in the picture, and other slices belong to the background region of the picture. The foreground region shall be independently decodable from the background region in the same picture, i.e. when a slice belongs to the foreground region, and has dependent_slice_flag equal 1, the previous slice, in decoding order, that has dependent_slice_flag equal to 0 shall also belong to the foreground region.

When gradual_decoding_refresh_enabled_flag is equal to 1 and the current picture is a RAP picture, the presence of a recovery point SEI message associated with the current picture indicates all of the following:

- The foreground region in the picture contains intra-coded coding blocks only.

- The foreground regions in the set of pictures starting from the associated picture to the recovery point, inclusive, are independently decodable from coded slices that belong to coded pictures preceding the associated picture in decoding order or that belong to the background regions in the same set of pictures.

- All slices in the pictures following the recovery point in decoding order shall be independently decodable from slices that belong to coded pictures preceding the associated picture in decoding order or that belong to the background regions in the same set of pictures.

## Independent sub-picture sequence information SEI message syntax and semantics

| isp_seq_info( payloadSize ) { | Descriptor |
| --- | --- |
| num_isps_minus1 | ue(v) |
| for( i = 0; i <= num_isps_minus1; i++ ) { |  |
| isp_top_left[ i ] | ue(v) |
| isp_bottom_right[ i ] | ue(v) |
| } |  |
| } |  |

The independent sub-picture sequence information SEI message provides the number of ISPs in the coded video sequence containing the independent sub-picture sequence information SEI message as well as the ISPs positions and sizes.

num_isps_minus1 plus 1 specifies the number of ISPs in each picture in the coded video sequence containing the independent sub-picture sequence information SEI message. The value of num_isps_minus1 shall be in the range of 0 to 255, inclusive.

isp_top_left[ i ] and isp_bottom_right[ i ] specify the top-left and bottom-right corners, respectively, of the ISP with isp_id equal to i in each picuture in the coded video sequence. top_left[ i ] and bottom_right[ i ] are coding tree block addresses in coding tree block raster scan of a picture. The ISP with top-left and bottom-right corners specified by top_left[ i ] and bottom_right[ i ], respectively, shall be inside pictures in the coded video sequence and shall not overlap with any other ISP.

## Sequence parameter set RBSP syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| indenpendent_sub_picture_enabled_flag | u(1) |
| gradual_decoding_refresh_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

indenpendent_sub_picture_enabled_flag equal to 1 specifies the presence of the isp_id syntax element in ISP and GDR information SEI messages in the coded video sequence. indenpendent_sub_picture_enabled_flag equal to 0 specifies the absence of the isp_id syntax element in ISP and GDR information SEI messages in the coded video sequence.

gradual_decoding_refresh_enabled_flag equal to 1 specifies the presence of the gdr_foreground_flag syntax element ISP and GDR information SEI messages in the coded video sequence. gradual_decoding_refresh_enabled_flag equal to 0 specifies the absence of the gdr_foreground_flag syntax element ISP and GDR information SEI messages in the coded video sequence.

# Cleaned spec text changes for GDR signalling

| gdr_info( payloadSize ) { | Descriptor |
| --- | --- |
| gdr_foreground_flag | u(1) |
| } |  |

The gradual decoding refresh (GDR) information SEI message indicates whether the slices the current SEI message applies to belong to the foreground region of the current picture.

If the GDR information SEI message is not the last GDR information SEI message in the access unit, the slices to which the SEI message applies consists, in decoding order, of all slices that succeed the SEI NAL unit containing the SEI message in decoding order and precede the next the SEI NAL unit containing a GDR information SEI message in decoding order. Otherwise, the slices to which a GDR information SEI message applies consists of all slices that succeed the the SEI NAL unit containing the SEI message in decoding order.

The slices that a GDR information SEI message applies to are also referred to as the slices associated with the GDR information SEI message.

Let gdrAuSet associated with a recovery point SEI message be the set of access units starting from the access unit containing the recovery point SEI message to the access unit that is the recovery point as indicated by the recovery point SEI message, inclusive. GDR information SEI messages shall not be present in an access unit that is not included in the gdrAuSet associated with a recovery point SEI message. When one access unit that is included in the gdrAuSet associated with a recovery point SEI message contains one or more GDR information SEI messages, all access units except for the access unit that is the recovery point included in the gdrAuSet shall contain one or more GDR information SEI messages.

gdr_foreground_flag equal to 1 indicates that the slices associated with the current SEI message belong to the forground region in the current picture. gdr_foreground_flag equal to 0 indicates that the slices associated with the current SEI message belong to the background region in the current picture.

The forground region in a picture include and only include all slices associated with the GDR information SEI messages having gdr_foreground_flag equal to 1 in the picture, and other slices belong to the background region of the picture.

When a slice belongs to the foreground region and has dependent_slice_flag equal 1, the previous slice in decoding order that has dependent_slice_flag equal to 0 shall also belong to the foreground region.

When the gdrAuSet associated with a recovery point SEI message contain GDR information SEI messages, the following applies:

- The foreground region in the first picture included in the gdrAuSet in decoding order contains intra-coded coding blocks only.

- The coded slices belonging to the foreground regions in the gdrAuSet are independently decodable from coded slices that belong to coded pictures preceding the first picture included in the gdrAuSet in decoding order or that belong to the background regions in the gdrAuSet.

- All slices in the pictures that follow all pictures in the gdrAuSet in decoding order are independently decodable from slices that belong to coded pictures preceding the first picture included in the gdrAuSet in decoding order or that belong to the background regions in the gdrAuSet.

# Further refined spec text changes for GDR signalling

| gdr_info( payloadSize ) { | Descriptor |
| --- | --- |
| gdr_foreground_flag | u(1) |
| } |  |

The gradual decoding refresh (GDR) information SEI message indicates whether the slices the current SEI message applies to belong to the foreground region (defined below) of the current picture.

An access unit that is not a RAP access unit and that contains a recovery point SEI message is referred to as a gradual decoding refresh (GDR) access unit and the corresponding picture is referred to as a GDR picture. The access unit corresponding to the indicated recovery point is referred to as the recovery point access unit and the corresponding picture is referred to as the recovery point picture.

Let gdrPicSet be the set of pictures starting from an GDR picture to the recovery point picture, inclusive, in decoding order. When the decoding process is started with an GDR access unit, the foreground region in each of the gdrPicSet is indicated to be correct or approximately correct in content, and the foreground region in the recovery point picture covers the entire picture region.

If the GDR information SEI message is not the last GDR information SEI message in the access unit, the slices to which the SEI message applies consist, in decoding order, of all slices that succeed the SEI NAL unit containing the SEI message and precede the next the SEI NAL unit containing a GDR information SEI message. Otherwise, the slices to which a GDR information SEI message applies consist of all slices, in decoding order, that succeed the the SEI NAL unit containing the SEI message and precede the next access unit.

The slices that a GDR information SEI message applies to are also referred to as the slices associated with the GDR information SEI message.

Let gdrAuSet be the set of access units corresponding to gdrPicSet. A gdrAuSet and the corresponding gdrPicSet are referred to as associated with the recovery point SEI message contained in the GDR access unit.

A GDR information SEI message shall only be present in an access unit that is included in a gdrAuSet associated with a recovery point SEI message. When one access unit that is included in a gdrAuSet contains one or more GDR information SEI messages, all access units in the gdrAuSet except for the recovery point access unit shall contain one or more GDR information SEI messages.

gdr_foreground_flag equal to 1 indicates that the slices associated with the current SEI message belong to the foreground region in the current picture. gdr_foreground_flag equal to 0 indicates that the slices associated with the current SEI message do not belong to the foreground region in the current picture.

The foreground region in a picture include and only include all slices associated with the GDR information SEI messages having gdr_foreground_flag equal to 1 in the picture, and other slices belong to the background region of the picture.

When a slice belongs to the foreground region and has dependent_slice_flag equal 1, the previous slice in decoding order that has dependent_slice_flag equal to 0 shall also belong to the foreground region.

When a gdrAuSet contains GDR information SEI messages, the following applies:

- The foreground region in the first picture included in the corresponding gdrPicSet in decoding order contains intra-coded coding blocks only.

- Let gdrForegroundSliceSet be all slices that belong to the foreground regions in the gdrPicSet. For each picture included in the gdrPicSet, the syntax elements in gdrForegroundSliceSet are constrained so that they cause the inter prediction process to be constrained as follows: No sample or motion vector value outside the gdrForegroundSliceSet, and no sample value at a fractional sample position that is derived using one or more sample values outside the gdrForegroundSliceSet is used for inter prediction of any sample or motion vector within the gdrForegroundSliceSet.

- For each of the pictures that follow the recovery point picture in decoding order, the syntax elements are constrained so that they cause the inter prediction process to be constrained as follows: No sample or motion vector value in a picture preceding the GDR picture in decoding order or outside the gdrForegroundSliceSet, and no sample value at a fractional sample position that is derived using one or more sample values in a picture preceding the GDR picture order or outside the gdrForegroundSliceSet is used for inter prediction of any sample or motion vector.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).