| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, China, October 2012 | Document: JCTVC-K0130 |
| --- | --- |

| Title: | Support of weighted prediction in the HEVC deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input contribution |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis Apple Inc. Cupertino CA | Tel: Email: | +1 408 228 7983 atourapis@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution presents some possible shortcomings of the HEVC deblocking filter process and suggests methods for their resolution. In particular, it is identified that the HEVC deblocking filter does not properly consider the presence of weighted prediction during the deblocking filter strength derivation.

# Introduction

The deblocking filtering process is an essential component of modern video codecs such as MPEG-4 AVC/H.264 and HEVC, since it can substantially reduce blocking artifacts and improve subjective quality during the video encoding process. The process commonly involves a determination of the deblocking filter strength across block boundaries based on a variety of motion and/or texture/residual information whereas the process is followed by an adaptive filtering process controlled primarily by the filtering strength and the actual sample values of the processed edge.

The new HEVC video coding standard adopts a deblocking filter mechanism that is somewhat similar to the one used in MPEG-4 AVC, primarily in terms of the deblocking filter strength derivation. In particular, the process first involves examining if either of the two neighboring blocks across an edge are intra coded. If so, then a stronger deblocking filter strength, i.e. Bs strength 2, is selected for processing these blocks. Otherwise, additional conditions based on the residual or motion characteristics of the two blocks are utilized to derive the deblocking filter strength. In particular, if any of the two blocks contains any residual information, then deblocking filter strength equal to 1 is used. If not, then the deblocking filter strength is set to 1 if a different number or different “actual” references are used for prediction, or if the motion vectors of the two blocks defer by more than 4 quarter samples in distance across either direction. Otherwise, deblocking filter strength equal to 0, which implies that no deblocking is to be performed, is used.

Unfortunately, a severe shortcoming exists with this model, as was also in the case of MPEG-4 AVC, where no consideration is made for the use of different weighting parameters in two neighboring blocks. Since weighted prediction may in fact primarily impact the DC value of a block, there could be cases where blockiness may occur, especially at lower bitrates/higher quantization parameter values.

In this contribution we present two alternative solutions that could be used to alleviate this problem, and therefore improve objective and subjective performance in the presence of weighted prediction. One of the proposed methods could also considerably reduce the complexity of the deblocking filter strength derivation, if that is desired.

# Deblocking filter with weighted prediction considerations

## Proposed Method A

A very simple solution that would indirectly enable the consideration of weighted prediction parameters during the deblocking filtering strength derivation process is to remove the clarification from the HEVC text that references are compared based on the pictures that are being referenced, instead of the reference index used. In particular, we propose that the deblocking filter derivation for strength 1, in addition to the consideration of the presence or not of any transform coefficients in either block, could be based on whether the two blocks use a different number of references, a single reference but a different reference list, or, in addition to the motion vector condition, if the same prediction lists are used but a different reference index is used for a particular list in one block versus the other block. One could argue that in HEVC the use of the same reference in different lists may be less useful compared to AVC and thus this solution would likely have very little impact, in any to any real system. In fact, this could potentially better justify why one may wish to use the same reference in both lists. However, this modification would immediately allow the use of deblocking across blocks that use the same picture as reference, have no residual information, but utilize different weighting parameters. Furthermore, this would greatly simplify the text as well as any reference software implementation. It should be noted that using this method no actual determination of the actual pictures that are referenced needs to be made, which can impose a slight penalty in some systems.

A drawback of this method is that this may also impact deblocking across slice partitions, especially if different picture list modification instructions were used for two adjacent slices. However, from our experience, we have found that in most systems it is less likely that one may use different picture list modification instructions for each slice in the same picture, whereas the stronger deblocking strength may in fact help in reducing the slice partitioning artifacts that could occur at lower bitrates.

### Detailed Edits

In 8.7.2.3, adjust the text as follows:

    - If one or more of the following conditions are true, the variable bS[ xDi ][ yDj ] is set equal to 1.

    - For the prediction of the luma prediction block containing the sample p0 a different number of motion vectors, or different prediction lists, or different reference indices for the same list, are used than for the prediction of the luma prediction block containing the sample q0.

NOTE 2 – The number of motion vectors that are used for the prediction of a luma prediction block with lop left luma sample covering ( xB, yB ), is equal to PredFlagL0[ xB, yB ] + PredFlagL1[ xB, yB ].

    - The motion vectors corresponding to each list used for each block have an absolute difference between the horizontal or vertical component of the motion vectors greater than or equal to 4 in units of quarter luma samples.

    - Otherwise (none of the conditions above is true), the variable bS[ xDi ][ yDj ] is set equal to 0.

## Proposed Method B

An alternative but more complex method is to actually explicitly consider the use of weighting prediction parameters within the deblocking filter strength derivation. In particular, without making any further modifications in the existing process, we also consider, the case where the number of references and the pictures referenced are the same, whether different weighting parameters (weight or offset) were used. In this scenario, regardless of the amount of the difference, deblocking filter strength equal to 1 is again used.

### Detailed Edits

In 8.7.2.3, adjust the text as follows:

    - If one or more of the following conditions are true, the variable bS[ xDi ][ yDj ] is set equal to 1.

    - For the prediction of the luma prediction block containing the sample p0 different reference pictures, or a different number of motion vectors, or different weighting parameters are used than for the prediction of the luma prediction block containing the sample q0.

NOTE 3 – The determination of whether the reference pictures used for the two luma prediction blocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 4 – The number of motion vectors that are used for the prediction of a luma prediction block with lop left luma sample covering ( xB, yB ), is equal to PredFlagL0[ xB, yB ] + PredFlagL1[ xB, yB ].

    - One motion vector is used to predict the luma prediction block containing the sample p0 and one motion vector is used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 4 in units of quarter luma samples.

    - Two motion vectors and two different reference pictures are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same two reference pictures are used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two luma prediction blocks for the same reference picture is greater than or equal to 4 in units of quarter luma samples,

    - Two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample q0 and all of the following conditions are true:

    - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two luma prediction bocks is greater than or equal to 4 in quarter luma samples or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two luma prediction blocks is greater than or equal to 4 in units of quarter luma samples,

    - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the luma prediction block containing the sample p0 and the list 1 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma samples or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the luma prediction block containing the sample p0 and list 0 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma samples.

    - Otherwise (none of the conditions above is true), the variable bS[ xDi ][ yDj ] is set equal to 0.

# Simulation Results

The test conditions specified in JCTVC-I1100 were used to evaluate our proposal. Only the first method was tested since, given that since no weighted prediction is used in the default coding conditions, no performance impact is expected in that scenario.

For the Low delay P only case, no performance difference was observed, as expected. This is due to the fact that the software does not utilize in this scenario multiple slices, and there is only a single list, therefore not having to distinguish references in different lists we propose to adopt a solution that takes in account the weighted prediction parameters during the deblocking filter strength derivation process.

For the low delay B and the Random Access encoding cases very small differences between the proposed and original methods were observed. The results are shown in Table 1 and Table 2. Insignificant impact in encoding and decoding was also observed.

Table 1. Results of the proposed deblocking method of Low Delay B Encoding

|  |  | BD-rate (piecewise cubic) |  |  | BD-rate (cubic) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |
| Class B |  | 0.1% | 0.3% | -0.2% | 0.1% | 0.2% | 0.0% |
| Class C |  | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% |
| Class D |  | 0.0% | 0.2% | -0.1% | 0.0% | 0.3% | 0.0% |
| Class E |  | 0.0% | 0.3% | 0.5% | 0.1% | 0.3% | 0.5% |
| Class F |  | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
| All |  | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |

Table 2. Results of the proposed deblocking method of Random Access Encoding

|  |  | BD-rate (piecewise cubic) |  |  | BD-rate (cubic) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| Class A |  | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class B |  | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |  |
| Class F |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| All |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

# Proposal

This contribution proposes the adoption of a solution that takes in account the weighted prediction parameters during the deblocking filter strength derivation process.

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).