| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, China, October 2012 | Document: JCTVC-K0131 |
| --- | --- |

| Title: | Deblocking filter display preference SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input contribution |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis, Athanasios Leontaris Apple Inc. Cupertino CA | Tel: Email: | +1 408 228 7983 atourapis@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution extends the Deblocking filter display preference SEI message to also properly support the use of SAO in the HEVC specification. The current specification copies the syntax and behavior of the H.264/MPEG-4 AVC SEI message, without, unfortunately considering the new tools and further flexibility that has been introduced in the HEVC specification. The existing text may also be in error since it suggests that the display output can either be immediately prior or after the deblocking process, bypassing essentially the SAO process. Finally, we also discuss possibilities of how we can enhance multi-reference prediction by making this process normative instead of optional.

# Introduction

| deblocking_filter_display_preference( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| deblocking_display_preference_cancel_flag | 5 | u(1) |
| if( !deblocking_display_preference_cancel_flag ) { |  |  |
| display_prior_to_deblocking_preferred_flag | 5 | u(1) |
| dec_frame_buffering_constraint_flag | 5 | u(1) |
| deblocking_display_preference_repetition_period | 5 | ue(v) |
| } |  |  |
| } |  |  |

Table 1. Deblocking filter display preference syntax

The deblocking filter display preference SEI message was adopted in the H.264/MPEG-4 AVC standard as a mechanism that could allow for improved subjective quality in certain video coding scenarios, wile not compromising prediction/coding performance. The SEI message was especially useful for encoding of High Definition movie material that may have been characterized by considerable noise, e.g. film grain noise. The idea was that under such scenarios it may be undesirable to perform any deblocking for display, since that may end up having some adverse effect onto the video material, because of the denoising characteristics of the deblocking filter process. At the same time, deblocking would still be desirable since that can help improve the quality of the prediction signal for subsequent pictures, whereas the denoising characteristics of the filter could be in fact exploited during the coding process. The syntax of this SEI message is shown in Table 1. This SEI message was also adopted in the HEVC specification without, however, any modifications or enhancements.

Unfortunately, HEVC has introduced an additional post-filtering mechanism, namely the sample adaptive offset (SAO) method. A third post-filtering method, i.e. the Adaptive Loop Filter (ALF), was also part of the same specification but was removed at the Stockholm meeting. These new post-filtering mechanisms introduce severe complications in but also possibilities for this SEI message.

More specifically, it is observed that the current semantics of this SEI message suggest that the display output can be selected only prior or immediately after the de-blocking process. No consideration of the SAO process is considered, which appears to be incorrect. Furthermore, it may in fact be desirable that the display output is selected at any point within the in-loop post-filter process of HEVC. That is, we may wish to output an image prior to deblocking and the SAO process, after deblocking but prior to the SAO process, or after all processes have been performed, while the reference image will still be generated using all available post-filter processes. This allows us considerable more flexibility in the encoding process since it also enables the SAO process to be best optimized not for display but for prediction.

An additional post-filter mechanism, named the Adaptive Loop Filter (ALF) process, was previously included in the HEVC specification, but has since been removed due to its questionable impact in video quality performance and its substantial complexity increase. Nevertheless, it is yet unclear if such or a similar post-filter process may be re-introduced in the current standard or its future extensions. It is therefore recommended that any extension to the deblocking display preference SEI message is designed with that in mind. Furthermore, the SEI message should likely be renamed to “In-loop filter display preference SEI message” to better account for other in-loop filtering mechanisms.

# In-loop filter display preference SEI message

## Proposed Methods

A very simple extension of the deblocking display preference SEI message is shown in Table 2. The display_prior_to_deblocking_preferred  flag is replaced by a display_preference_filter_entry_point syntax element, which is of ue(v) form. This flag allows the encoder to signal from which point within the in-loop pipeline the display process should be performed. The syntax provides full support to the current specification that uses only 2 in-loop filter mechanisms, but is flexible enough for future extensions.

| in_loop_filter_display_preference( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| in_loop_filter_display_preference_cancel_flag | 5 | u(1) |
| if( !in_loop_filter_display_preference_cancel_flag ) { |  |  |
| display_preference_filter_entry_point | 5 | ue(v) |
| dec_frame_buffering_constraint_flag | 5 | u(1) |
| in_loop_filter_display_preference_repetition_period | 5 | ue(v) |
| } |  |  |
| } |  |  |

Table 2. In-loop filter display preference syntax

For example, if display_preference_filter_entry_point is set to 0, then the display can be performed prior to any in-loop filtering is performed. If 1, then the display is selected between the deblocking filter and the SAO process, whereas a value of 2 signals that the display process is performed after all in-loop processes are performed. It should be noted that the dependencies between the in-loop filters need to be carefully considered when designing this signaling in order to avoid an increase in complexity by processing the same samples multiple times.

In a further extension, it may be desirable, apart from signaling the display entry point position, to also signal within the same SEI message additional post-filter mechanisms that could be used for enhancing the image quality for display. Such signaling could be done for example using the syntax provided in Table 3, where additional syntax elements dictate how post-filtering should be performed for the image after it has been extracted from the display entry point of the decoder. For example, if we may have selected that the display entry point is after deblocking but prior to SAO, the post-filtering parameters could include “display-only” SAO information. This allows one to optimize the in-loop SAO parameters for only prediction, while the display-only SAO parameters would be optimized for only display. This basically allows us to decouple somewhat the display from the prediction process and further improve coding performance for some scenarios.

| deblocking_filter_display_preference( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| deblocking_display_preference_cancel_flag | 5 | u(1) |
| if( !deblocking_display_preference_cancel_flag ) { |  |  |
| display_preference_filter_entry_point | 5 | ue(v) |
| if (display_preference_filter_entry_point != 0) { |  |  |
| for (i = 0; i < total_filters + 1 - display_preference_filter_entry_point; i++) { |  |  |
| display_preference_filter_flag[ i ] | 5 | u(1) |
| if (display_preference_filter_flag[ i ] == 1) |  |  |
| signal_display_preference_filter_params( i ) |  |  |
| } |  |  |
| } |  |  |
| dec_frame_buffering_constraint_flag | 5 | u(1) |
| deblocking_display_preference_repetition_period | 5 | ue(v) |
| } |  |  |
| } |  |  |

Table 3. In-loop filter display preference syntax

## Proposed scenantics - In-loop filter display preference SEI message semantics

This SEI message provides the decoder with an indication of whether the display of the cropped result of the deblocking filter process specified in subclause 8.7.2 or of the cropped result of the sample adaptive offset specified in subclause 8.7.3 or of the cropped result of the picture construction process prior to the deblocking filter process specified in subclause 8.6.5 is preferred by the encoder for the display of each decoded picture that is output.

NOTE 1 – The display process is not specified in this Recommendation | International Standard. The means by which an encoder determines what to indicate as its preference expressed in a deblocking filter display preference SEI message is also not specified in this Recommendation | International Standard, and the expression of an expressed preference in a deblocking filter display preference SEI message does not impose any requirement on the display process.

in_loop_filter_display_preference_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous in-loop filter display preference SEI message in output order. in_loop_display_preference_cancel_flag equal to 0 indicates that a display_preference_filter_entry_point and in_loop_filter_display_preference_repetition_period follow.

NOTE 2 – In the absence of the in-loop filter display preference SEI message, or after the receipt of an in-loop filter display preference SEI message in which in_loop_filter_display_preference_cancel_flag is equal to 1, the decoder should infer that the display of the cropped result of the sample adaptive offset filter process specified in subclause 8.7.3 is preferred over the display of the cropped result of the picture construction process at any point prior to the sample adaptive offset filter process, such as points 8.7.2 or 8.6.5, for the display of each decoded picture that is output.

display_preference_filter_entry_point specifies the position preferred for the display process. A value of 0 indicates that the encoder preference for the display process (which is not specified in this Recommendation | International Standard) is to display the cropped result of the picture construction process prior to the deblocking filter process specified in subclause 8.6.5. A value of 1 indicates that the encoder preference for the display process is to display  the cropped result after the deblocking process but prior to the sample adaptive offset, specified in subclaure 8.7.2. Finally, if the value is equal to 2, then the display process is performed based on the cropping and output processes specified in Annex C.

NOTE 3 – The presence or absence of the in_loop filter display preference SEI message and the value of display_preference_filter_entry_point does not affect the requirements of the decoding process specified in this Recommendation | International Standard. Rather, it only provides an indication of when, in addition to fulfilling the requirements of this Recommendation | International Standard for the decoding process, enhanced visual quality might be obtained by performing the display process (which is not specified in this Recommendation | International Standard) in an alternative fashion. Encoders that use the in-loop filter display preference SEI message should be designed with an awareness that unless the encoder restricts its use of the DPB capacity specified in Annex A and subclauses G.10 and H.10 for the profile and level in use, some decoders may not have sufficient memory capacity for the storage of the result of the picture construction process prior to the in-loop filter process specified in subclause 8.6.5 or 8.7.2 in addition to the storage of the result of the in-loop filter process specified in subclause 8.7.3 when reordering and delaying pictures for display, and such decoders would therefore not be able to benefit from the preference indication. By restricting its use of the DPB capacity, an encoder can be able to use at least half of the DPB capacity specified in Annex A and subclauses G.10 and H.10 while allowing the decoder to use the remaining capacity for storage of unfiltered pictures that have been indicated as preferable for display until the output time arrives for those pictures.

dec_frame_buffering_constraint_flag equal to 1 indicates that the use of the frame buffering capacity of the HRD decoded picture buffer (DPB) as specified by max_dec_frame_buffering has been constrained such that the coded video sequence will not require a decoded picture buffer with more than Max( 1, max_dec_frame_buffering ) frame buffers to enable the output of the decoded filtered or unfiltered pictures, as indicated by the deblocking filter display preference SEI messages, at the output times specified by the dpb_output_delay of the picture timing SEI messages. dec_frame_buffering_constraint_flag equal to 0 indicates that the use of the frame buffering capacity in the HRD may or may not be constrained in the manner that would be indicated by dec_frame_buffering_constraint_flag equal to 1. For purposes of determining the constraint imposed when dec_frame_buffering_constraint_flag is equal to 1, the quantity of frame buffering capacity used at any given point in time by each frame buffer of the DPB that contains a picture shall be derived as follows.

– If both of the following criteria are satisfied for the frame buffer, the frame buffer is considered to use two frame buffers of capacity for its storage.

– The frame buffer contains a frame or one or more fields that is marked as "used for reference", and

– The frame buffer contains a picture for which both of the following criteria are fulfilled:

– The HRD output time of the picture is greater than the given point in time.

– It has been indicated in an in-loop filter display preference SEI message that the encoder preference

for the picture is for the display process to display the cropped result of the picture construction process

prior to the deblocking filter process specified in subclause 8.6.5 or prior to the sample adaptive offset process specified in subclause 8.7.2  rather than the cropped result of the

sample adaptive offset filter process specified in subclause 8.7.3.

– Otherwise, the frame buffer is considered to use one frame buffer of DPB capacity for its storage.

When dec_frame_buffering_constraint_flag is equal to 1, the frame buffering capacity used by all of the frame buffers in the DPB that contain pictures, as derived in this manner, shall not be greater than

Max( 1, max_dec_frame_buffering ) during the operation of the HRD for the coded video sequence.

The value of dec_frame_buffering_constraint_flag shall be the same in all deblocking filter display preference SEI messages of the coded video sequence.

in_loop_filter_display_preference_repetition_period specifies the persistence of the in-loop filter display preference SEI message and may specify a picture order count interval within which another in-loop filter display preference message or the end of the coded video sequence shall be present in the bitstream. The value of in_loop_display_preference_repetition_period shall be in the range 0 to 16 384, inclusive.

in_loop_filter_display_preference_repetition_period equal to 0 specifies that the in-loop filter display preference SEI message applies to the current decoded picture only.

in_loop_filter_display_preference_repetition_period equal to 1 specifies that the in-loop filter display preference SEI message persists in output order until any of the following conditions are true:

– A new coded video sequence begins.

– A picture in an access unit containing a in-loop filter display preference SEI message is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ).

in_loop_display_preference_repetition_period greater than 1 specifies that the in-loop filter display preference SEI message persists until any of the following conditions are true:

– A new coded video sequence begins.

– A picture in an access unit containing a in-loop filter display preference SEI message is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + in_loop_display_preference_repetition_period.

in_loop_display_preference_repetition_period greater than 1 indicates that another in-loop filter display preference SEI message shall be present for a picture in an access unit that is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + in_loop_display_preference_repetition_period; unless the bitstream ends or a new coded video sequence begins without output of such a picture.

# Further Observations

The in-loop filter display preference process essentially creates and temporarily stores multiple versions of the same picture, one for display and another for prediction. However, one may argue that it may be possible to further exploit the presence of these multiple images within the prediction process. That is, it may be possible but also desirable to use the image targeted for display as an additional motion prediction hypothesis within the coding loop. Some areas, in particular, may prefer to use prediction samples without any deblocking and SAO performed on them, i.e. areas with considerable texture or some types of noise, whereas other areas may prefer to use for prediction either samples generated only with deblocking or generated while using both the deblocking and SAO processes. Although this has not been tested, it is quite possible that this strategy could further improve the performance of HEVC or its extensions and it may be desirable to explore this further. This, obviously would suggest that the process specified by this SEI message would have to be mandated as part of the actual coding process, and not be optional any more.

# Proposal

In this document we have proposed extending the deblocking display preference SEI message to better support the additional in-loop filtering mechanisms specified in HEVC. Furthermore, we suggest that there could be further benefits by considering this process as normative instead of optional within the codec.

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Since film grain noise is not commonly temporally correlated, one could have to compensate both for the noise that would exist in the reference signal, as well as for the noise that exists in the current image being encoded.