| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0132 |
| --- | --- |

| Title: | Unified coefficient scan for 8x8 TU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-8.0, three scan patterns including diagonal, horizontal, and vertical are used for 8x8 transform units (TUs) when mode dependent coefficient scan (MDCS) is applied. The scan patterns can be divided into scanning across coefficient groups (CGs) and scanning within CGs. In this contribution, the scanning order across CGs of horizontal scan is unified to be the same as those of vertical and diagonal scans. Simulation results reportedly show no significant differences in BD-rates and run times in comparison with HM-8.0.

# Introduction

In High Efficiency Video Coding test model version 8.0 (HM-8.0), transform coefficients are coded transform unit (TU) by TU using context-based adaptive binary arithmetic coding (CABAC). Within one TU, coefficients are further divided into coefficient groups (CGs). Each CG consists of 16 coefficients. CGs within the TU and coefficients within CGs will be scanned according to the predefined scan pattern. The five syntax elements related to coefficient significance map, levels, and signs are sequentially coded in each CG in five scan passes.

For 8x8 TU, there are three scan patterns being used when MDCS is applied, that is, diagonal, horizontal and vertical scans. In the 10th JCT-VC meeting in Stockholm, JCTVC-J0256 [1] was adopted to unify the shapes of CGs for these three scan patterns, as shown in Figure 1, where each CG is a 4x4 sub-block in TU. However, the order of CGs for horizontal scan is different from those for vertical and diagonal scans. In this contribution, we try to further unify all these three scan patterns to have the same order of CGs. The proposed method is implemented on top of HM-8.0. The rest of this contribution is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

- (b)		                (c)

Figure 1.  The current scan patterns for 8x8 TUs: (a) diagonal (b) horizontal (c) vertical

# Proposed method

The proposed scan patterns are shown in Figure 2. We divide the scan patterns of 8x8 TUs into two levels, where the first level scanning is associated with the scanning order of CGs and the second level scanning is associated with the scan pattern within each CG. With our proposed scan patterns, the first level scanning is unified for the three scan patterns, that is, the upper-left sub-block is first scanned and then followed by lower-left, upper-right and lower-right CGs. However, for the second level scanning within CGs, MDCS with diagonal/horizontal/vertical scans can be still applied respectively.

Figure 2.  The proposed scan patterns for 8x8 TUs: (a) diagonal (b) horizontal (c) vertical

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-J1100 [2], and the software is based on HM-8.0. Simulation results of the unified scan patterns for 8x8 TUs are shown in Table 1. The results for low QPs (12, 17, 22, 27) are also provided in Table 2. It is shown that the modification does not cause any noticeable change in terms of BD-rates and run times.

Table 1.  Results of the unified scan patterns for 8x8 TU

Table 2.  Results of the unified scan patterns for 8x8 TU for low Qp (12, 17, 22, 27)

# Conclusions

In this contribution, unified MDCS scan patterns for 8x8 TUs are proposed for the coefficient coding in HM-8.0. The scanning order of coefficient groups (CGs) in horizontal scan is unified to be the same as those of vertical and diagonal scans. Simulation results reportedly show no significant differences in BD-rates and run times in comparison with HM-8.0.

# References

- J. Sole, R. Joshi, M. Karczewicz, “Removal of the 8x2/2x8 coefficient groups,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0256, July, 2012.

- F. Bossen, “HM 8 common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-J1100, July, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).