| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0136 |
| --- | --- |

| Title: | AHG9: High-level syntax cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents nine cleanups of the latest HEVC syntax in JCTVC-K0030. Some of the modifications avoid encoding and decoding unnecessary syntax elements in certain circumstances. The rest of the modifications prevent encoders from violating encoder-only and normative constraints by guiding encoders with explicitly specified bounds of syntax elements, which can reduce the complexity for practical decoders that still need to decode illegal bitstreams. In addition to descriptions of syntax cleanups, all modifications are implemented on top of HM-8.0-dev r2779 using different MACROs.

# Proposed modifications

## vps_temporal_id_nesting_flag

- In JCTVC-K0030 (the latest High Efficiency Video Coding text specification) [1] subclauses 7.3.2.1 and 7.4.2.1, vps_temporal_id_nesting_flag is always coded. However, the syntax element is required only when vps_max_sub_layer_minus1 is larger than 0. Hence, it is proposed to signal vps_temporal_id_nesting_flag when vps_max_sub_layer_minus1 is larger than 0.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_zero_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| if( vps_max_sub_layers_minus1 > 0 ) |  |
| vps_temporal_id_nesting_flag | u(1) |
| … |  |

## sps_temporal_id_nesting_flag

- In JCTVC-K0030 subclauses 7.3.2.2 and 7.4.2.2, sps_temporal_id_nesting_flag is always coded. However, the syntax element is required only when sps_max_sub_layer_minus1 is larger than 0. Hence, it is proposed to signal sps_temporal_id_nesting_flag when sps_max_sub_layer_minus1 is larger than 0.

| seq_parameter_set_rbsp( ) { |  |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| … |  |
| if (sps_max_sub_layers_minus1 > 0) |  |
| sps_temporal_id_nesting_flag | u(1) |
| … |  |
| } |  |

## sub_layer_profile_present_flag

- In JCTVC-K0030 sybclauses 7.3.3 and 7.4.3, sub_layer_profile_present_flag[ i ] is always coded. However, the syntax element is required only when ProfilePresentFlag is equal to 1. Hence, it is proposed to signal sub_layer_profile_present_flag[ i ] only when ProfilePresentFlag is equal to 1. In addition, it is also proposed to signal related syntax elements including sub_layer_profile_space[ i ], sub_layer_tier_flag[ i ], sub_layer_profile_idc[ i ], sub_layer_profile_compatibility_flag[ i ][ j ], and sub_layer_reserved_zero_16bits[ i ] only when sub_layer_proile_present_flag[ i ] is equal to 1.

| profile_tier_level( ProfilePresentFlag, MaxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i < MaxNumSubLayersMinus1; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( ProfilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_present_flag[ i ] |  |
| if( sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_reserved_zero_16bits[ i ] | u(16) |
| } |  |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

## inter_ref_pic_set_prediction_flag

In JCTVC-K0030 subclauses 7.3.7.2 and 7.4.7.2, when the first reference picture set (RPS) is coded (i.e., idx = 0), inter-RPS prediction cannot be applied and inter_ref_pic_set_prediction_flag shall be 0. However, inter_ref_pic_set_prediction_flag is always coded regardless of idx, and syntax-wise the illegal value still can happen (i.e., inter_ref_pic_set_prediction_flag = 1 when idx = 0). Hence, it is proposed to signal inter_ref_pic_set_prediction_flag when a non-first RPS is coded (i.e., idx != 0) and to infer inter_ref_pic_set_prediction_flag to 0 when the first RPS is coded (i.e., idx = 0).

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( idx != 0 ) |  |
| inter_ref_pic_set_prediction_flag | u(1) |
| … |  |

inter_ref_pic_set_prediction_flag equal to 1 specifies that the reference picture set of the current picture is predicted using another reference picture set in the active sequence parameter set. When idx is equal to 0 not present, the value of inter_ref_pic_set_prediction_flag shall be is inferred to be equal to 0.

## Transform block size syntax elements

In JCTVC-K0030 subclauses 7.3.2.2 and 7.4.2.2, log2_min_transform_block_size_minus2 is coded to derive the minimum transform unit (TU) size, which shall be smaller than the minimum luma coding unit (CU) size and shall not be smaller than 4x4. Besides, log2_diff_max_min_transform_block_size is coded to derive the maximum TU size, which shall not be larger than the luma coding tree block (CTB) size or 32x32. However, syntax-wise the constraints of the minimum and maximum TU sizes still can be violated. Hence, instead of signaling log2_min_transform_block_size_minus2, it is proposed to signal a new syntax element, log2_diff_min_cb_min_tb_size_minus1, which is the minimum luma CU size minus the minimum TU size and then further minus 1. In this way, the minimum TU size is always smaller than the minimum luma CU size. Moreover, when the Log2MinTrafoSize is smaller than 2, it is clipped to 2 on the decoder side to guarantee the minimum TU size is larger than or equal to 4x4. When the Log2MaxTrafoSize is larger than Min( Log2CtbSizeY, 5 ), where Log2CtbSizeY represents the luma CTB size, it is clipped to Min( Log2CtbSizeY, 5 ) on the decoder side to guarantee the maximum TU size is smaller than or equal to the smaller of the luma CTB size and 32x32.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_min_cb_min_tb_size_minus1 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| … |  |
| } |  |

log2_min_transform_block_size_minus2 specifies the minimum transform size.

The variable Log2MinTrafoSize is set equal to log2_min_transform_block_size_minus2 + 2. The bitstream shall not contain data that result in Log2MinTrafoSize greater than or equal to Log2MinCbSizeY.

log2_diff_min_cb_min_tb_size_minus1 specifies the minimum transform size.

The variable Log2MinTrafoSize is set equal to Max( 2, Log2MinCbSizeY –log2_diff_min_cb_min_tb_size_minus1 – 1 ).

log2_diff_max_min_transform_block_size specifies the difference between the maximum and minimum transform size.

The variable Log2MaxTrafoSize is set equal to log2_min_transform_block_size_minus2 + 2 + log2_diff_max_min_transform_block_size Min( Log2CtbSizeY, Min( 5, Log2MinTrafoSize + log2_diff_max_min_transform_block_size ) ).

The bitstream shall not contain data that result in Log2MaxTrafoSize greater than Min( Log2CtbSizeY, 5 ).

## PCM luma coding block size syntax elements

In JCTVC-K0030 subclauses 7.3.2.2 and 7.4.2.2, log2_min_pcm_luma_coding_block_size_minus3 is coded to derive the minimum PCM luma coding block size, which shall be within the range from the minimum luma CU size to the smaller value of the luma CTB size and 32x32. Besides, log2_diff_max_min_pcm_luma_coding_block_size is coded to derive the maximum PCM luma coding block size, which shall not be larger than the luma CTB size or 32x32. However, syntax-wise the constraints of the minimum and maximum PCM luma coding block sizes still can be violated. Hence, instead of signaling log2_min_pcm_luma_coding_block_size_minus3, it is proposed to signal a new syntax element, log2_diff_min_pcm_luma_cb_min_cb_size, which is the minimum PCM luma coding block size minus the minimum luma CU size. In this way, the minimum PCM luma coding block size is always larger than or equal to the minimum luma CU size. Moreover, when the Log2MinIpcmCbSizeY is larger than Min( Log2CtbSizeY, 5 ), it is clipped to Min( Log2CtbSizeY, 5 ) on the decoder side to guarantee the minimum PCM luma coding block size does not exceeed the luma CTB size or 32x32. When the Log2MaxIpcmCbSizeY is larger than Min( Log2CtbSizeY, 5 ), it is clipped to Min( Log2CtbSizeY, 5 ) on the decoder side to guarantee the maximum PCM luma coding block size is does not exceed the luma CTB or 32x32, either.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_min_pcm_luma_cb_min_cb_size | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| } |  |
| … |  |
| } |  |

log2_min_pcm_luma_coding_block_size_minus3 + 3 specifies the minimum size of coding blocks with pcm_flag equal to 1.

The variable Log2MinIpcmCbSizeY is set equal to log2_min_pcm_luma_coding_block_size_minus3 + 3. The variable Log2MinIpcmCbSizeY shall be in the range of Log2MinCbSizeY to Min( Log2CtbSizeY, 5 ), inclusive.

log2_diff_min_pcm_luma_cb_min_cb_size specifies the minimum size of coding blocks with pcm_flag equal to 1.

The variable Log2MinIpcmCbSizeY is set equal to Min( Log2CtbSizeY, Min( 5, Log2MinCbSizeY + log2_diff_min_pcm_luma_cb_min_cb ) ).

log2_diff_max_min_pcm_luma_coding_block_size specifies the difference between the maximum and minimum size of coding blocks with pcm_flag equal to 1.

The variable Log2MaxIpcmCbSizeY is set equal to log2_min_pcm_luma_coding_block_size_minus3 + 3 + log2_diff_max_min_pcm_luma_coding_block_size Min( Log2CtbSizeY, Min( 5, Log2MinIpcmCbSizeY + log2_diff_max_min_pcm_luma_coding_block_size ) ). The variable Log2MaxIpcmCbSizeY shall be equal or less than Min( Log2CtbSizeY, 5 ).

## PCM sample bit depth syntax elements

In JCTVC-K0030 subclauses 7.3.2.2 and 7.4.2.2, the PCM luma sample bit depth and the PCM chroma sample bit depth are coded and shall not exceed the coded luma sample bit depth and the coded chroma sample bit depth, respectively. However, syntax-wise, the bit depth constraints still can be violated. Hence, it is proposed to clip the PCM sample bit depth to the coded sample bit depth on the decoder side when the PCM sample bit depth exceeds the coded sample bit depth.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| … |  |
| } |  |

pcm_sample_bit_depth_luma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of luma component. The value of pcm_sample_bit_depth_luma_minus1 + 1 shall be less than or equal to the value of BitDepthY.

PCMBitDepthY = Min( 1 + pcm_sample_bit_depth_luma_minus1, BitDepthY )

pcm_sample_bit_depth_chroma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of chroma components. The value of pcm_sample_bit_depth_chroma_minus1 + 1 shall be less than or equal to the value of BitDepthC.

PCMBitDepthC = Min( 1 + pcm_sample_bit_depth_chroma_minus1, BitDepthC )

## Tile-related syntax elements in picture parameter set

In JCTVC-K0030 subclauses 7.3.2.3, and 7.4.2.3, several tile-related syntax elements are coded in picture parameter set (PPS). However, a portion of tile-related syntax elements in PPS is useless when the picture has only one tile. Hence, it is proposed to signal uniform_spacing_flag, column_width_minus1, row_height_minus1, and loop_filter_across_tiles_enabled_flag when the number of tile columns or the number of tile rows is larger than 1.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if( num_tile_columns_minus1 > 0 ¦¦ num_tile_rows_minus1 > 0) { |  |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

## Tile-related syntax elements in slice header

In JCTVC-K0030 subclauses 7.3.7.1, and 7.4.7.1, several tile-related syntax elements are coded in slice header. However, a portion of tile-related syntax elements in slice header is useless when the picture has only one tile. Hence, it is proposed to signal num_entry_point_offsets, offset_len_minus1, and entry_point_offset when the number of tile columns or the number of tile rows is larger than 1.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( tiles_enabled_flag && (num_tile_columns_minus1 > 0 ¦¦ num_tile_rows_minus1 > 0) ) ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

# References

- B. Bross, G. J. Sullivan, T. K. Tan, “Proposed editorial improvements for High Efficiency Video Coding (HEVC) text specification draft 8,” Document of Joint Collaborative Team on Video Coding, JCTVC-K0030, October, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).