| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0138 |
| --- | --- |

| Title: | Non-CE1: Deblocking of Large Block Artifacts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports on the reasons for large block artifacts that are not smoothed by the HM8 deblocking filter in some video sequences and it proposes a solution that is a minor modification. The solution does not require the signalling of tuned deblocking adjustment parameters and, most importantly, it avoids replacing deblocking decisions and filters at the DIS stage, which have been a stable part of the HEVC deblocking filter for more than one year. It is reported that the primary reason for the large block artifacts is the clipping of the normal deblocking filter, while boundary filtering of the DC, horizontal and vertical intra modes also contributes to the artifacts. It is proposed to increase the normal filter clipping bounds for edges of the maximum transform blocks and to restrict boundary filtering. This method smooths the observed large block artifact edges, while it avoids oversmoothing in picture areas that are unaffected by these artifacts. Besides targeted smoothing, the method improves texture details compared with HM8.

# Introduction

Visual tests have demonstrated that certain video content coded with HM8 [1] suffers from large block artifacts that are transform size dependent. These artifacts are mainly visible at relatively high QP values (> 30) and at the boundaries of the maximum transform blocks/units (TB/TU).

First, this document reports on the problems that leave sharp edges of large block artifacts seemingly unfiltered by the HM8 deblocking filter. The primary problem is that the HM8 ‘normal’ or ‘weak’ luma deblocking filter clipping is too narrow to smooth the sharp edge discontinuities. The second problem is that the boundary filtering for DC, horizontal and vertical intra modes interacts with deblocking decisions, which has been reported earlier in [3] and is further studied in CE1 [4].

A solution is proposed that increases the normal filter clipping bounds for edges of maximum TBs and restricts boundary filtering.

This method avoids oversmoothing in picture areas that are unaffected by these artifacts. The following figures illustrate sharp and large block artifacts in the “Riverbed”, “Ducks Take Off”, “West Wind Easy”, and “Red Kayak” sequences [2]. In Figures 1 to 4, the maximum TB is the default size of 32x32; while in Figures 5 and 6 the maximum TB size is 16x16. This maximum TB size is encoder implementation dependent.

Figure 1 “Riverbed” sequence coded with HM8.0 (LD-B, max TB=32x32, QP32, frame #2): sharp block artifact edges are visible

Figure 2 “Ducks Take Off” sequence coded with HM8.0 (LD-B, max TB=32x32, QP37, frame #7): sharp block artifact edges are visible

Figure 3 “West Wind Easy” coded with HM8.0 (LD-B, max TB=32x32, QP32, frame #6): sharp block artifact edges are visible

Figure 4 “Red Kayak” coded with HM8.0 (LD-B, max TB=32x32, QP37, frame #7): sharp block artifact edges are visible

Figure 5 “Riverbed” sequence coded with HM8.0 (LD-B, max TB=16x16, QP32, frame #2): sharp block artifact edges are visible

Figure 6 “Ducks Take Off” sequence coded with HM8.0 (LD-B, max TB=16x16, QP37, frame #199): sharp block artifact edges are visible

# Causes of large block artifacts

### Narrow clipping of the normal deblocking filter

The following summarizes the normal deblocking filter [1]:

  - Normal filter delta:

    -  =  ( 9(q0-p0) - 3(q1-p1) + 8 )/16

  - If (  < 10tc )

    - δ = Clip3( -c1, c1, )

    - p0’ = Clip( p0 + δ )

    - q0’ = Clip( q0 - δ )

    - If modify p1:

      - p = Clip3( -c2, c2, ( (p2 + p0 + 1)/2 - p1 + δ)/2 )

      - p1’ = Clip( p1 + p )

    - If modify q1:

      - q = Clip3( -c2, c2, ( (q2 + q0 + 1)/2 - q1 - δ)/2 )

      - q1’ = Clip( q1 + q )

    - Filter clipping thresholds

      - c1 = tC and c2 = tC / 2

The problem is that the filter clipping intervals [-c1, c1] and [-c2, c2] are too narrow to smooth sharp edge discontinuities as illustrated in Fig. 5. Experiments have demonstrated that the observed large block artifact edges are smoothed if normal filter clipping intervals are increased, as is illustrated in Fig. 6.

Figure 7 The sharp edge discontinuity (magenta) is not smoothed enough by the normal deblocking filter (green).

Figure 8 The sharp edge discontuinity (magenta) is smoothed by the normal deblocking filter if clipping intervals are increased (green).

### Boundary filtering for DC, horizontal and vertical intra modes

The observation that these methods interact with deblocking filtering has been made in [3] and is further studied in [4].

# Proposal

## Increased clipping bounds for normal filtering at Max TB boundaries

The following is proposed:

- If an edge is a maximum TB edge, then the boundary strength (Bs) is set to 3 and the normal filter is configured with clipping bounds: c1 = 4tC and c2 = 2tC

If Bs is equal to 3, the default intra tC offset (=2) is not applied for luma, while it still is applied for chroma. Chroma filtering is applied to intra block edges as in HM8.

## Restriction on boundary filtering for DC, horizontal and vertical intra modes

It is proposed to restrict the filtering to block sizes below 32x32.

# Results

## Examples of improved subjective quality on CE1 type 1 sequences

The following and additional examples are included in the accompanying slide deck for easy comparison.

### Max TB = 32x32

Figure 9 “Riverbed” coded with proposed method (LD-B, QP32, max TB=32x32, frame #2) for comparison with Fig. 1. The sharp edges of the large block artifacts are smoothed.

Figure 10 “Ducks Take Off” coded with proposed method (LD-B, QP37, max TB=32x32, frame #7) for comparison with Fig. 2. The sharp edges of the large block artifacts are smoothed.

Figure 11 “West Wind Easy” coded with proposal (LD-B, max TB=32x32, QP32, frame #6) for comparison with Fig. 3. The sharp edges of the large block artifacts are smoothed.

Figure 12 “Red Kayak” coded with proposal (LD-B, max TB=32x32, QP37, frame #7) for comparison with Fig. 4. The sharp edges of the large block artifacts are smoothed.

### Max TB = 16x16

Figure 13 “Riverbed” coded with proposed method (LD-B, QP32, max TB=16x16, frame #2) for comparison with Fig. 5. The sharp edges of the large block artifacts are smoothed.

Figure 14 “Ducks Take Off” coded with proposed method (LD-B, QP37, max TB=16x16, frame #199) for comparison with Fig. 6. The sharp edges of the large block artifacts are smoothed.

## Examples of improved subjective quality on CTC sequences

The following figures illustrate improved detail of the proposal compared with HM8.0.

Figure 15 “Kimono” coded with HM8.0 (LD-B, QP32, max TB 32x32, frame #137)

Figure 16 “Kimono” coded with proposal (LD-B, QP32, max TB 32x32, frame #137) for comparison with Fig. 11. Improved sharpness of the face is visible.

## Objective Results

### CTC Sequences

Table 1 summarizes the BD-rate results for the proposal under CTC. The encoding times are measured in a cluster computing environment and are approximate. The decoding times are measured on a single PC.

In Table 2 the macro RESTRICT_INTRA_BOUNDARY_SMOOTHING is disabled, which means HM8 intra prediction is unmodified. This test illustrates that BD-rates are similar to the case with the restriction enabled (Table 1).

Table 1 BD-rate results for proposal

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.65% | 0.14% | 0.16% | 0.73% | 0.17% | 0.20% |
| Class B | 0.22% | 0.07% | 0.07% | 0.28% | 0.09% | 0.08% |
| Class C | 0.07% | 0.06% | 0.03% | 0.08% | 0.06% | -0.01% |
| Class D | 0.09% | 0.03% | 0.02% | 0.10% | 0.05% | 0.06% |
| Class E | 0.13% | 0.02% | 0.03% | 0.16% | 0.06% | 0.02% |
| Class F | 0.07% | 0.02% | 0.01% | 0.11% | -0.09% | 0.05% |
| Overall | 0.21% | 0.06% | 0.06% | 0.25% | 0.06% | 0.07% |
|  | 0.21% | 0.06% | 0.06% | 0.25% | 0.06% | 0.07% |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.25% | -0.33% | 0.08% | 0.24% | -0.03% | -0.24% |
| Class B | 0.08% | 0.04% | 0.17% | 0.12% | 0.12% | 0.18% |
| Class C | 0.08% | 0.03% | 0.16% | 0.09% | 0.10% | -0.01% |
| Class D | 0.06% | -0.04% | 0.10% | 0.05% | -0.03% | -0.26% |
| Class E |  |  |  |  |  |  |
| Class F | 0.10% | -0.14% | -0.09% | 0.12% | 0.09% | 0.14% |
| Overall | 0.11% | -0.08% | 0.09% | 0.12% | 0.05% | -0.03% |
|  | 0.10% | -0.09% | 0.09% | 0.12% | 0.04% | -0.05% |
| Enc Time[%] | 101% |  |  | 96% |  |  |
| Dec Time[%] | 104% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.06% | 0.24% | 0.19% | 0.09% | 0.15% | 0.33% |
| Class C | 0.12% | 0.04% | -0.03% | 0.07% | -0.07% | 0.00% |
| Class D | 0.00% | 0.31% | -0.13% | -0.01% | -0.21% | -0.42% |
| Class E | 0.17% | 0.20% | 0.99% | 0.00% | -0.28% | 0.22% |
| Class F | 0.23% | 0.16% | -0.09% | 0.09% | 0.47% | -0.45% |
| Overall | 0.11% | 0.19% | 0.15% | 0.05% | 0.03% | -0.06% |
|  | 0.11% | 0.18% | 0.16% | 0.06% | 0.04% | -0.04% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.06% | 0.12% | 0.06% | -0.04% | 0.17% | -0.38% |
| Class C | 0.06% | 0.04% | 0.25% | 0.02% | -0.23% | -0.20% |
| Class D | 0.04% | 0.24% | 0.02% | 0.08% | -0.47% | -0.11% |
| Class E | 0.10% | 0.51% | -0.81% | -0.01% | -0.52% | 0.04% |
| Class F | 0.14% | 0.43% | 1.11% | 0.15% | 0.26% | -0.23% |
| Overall | 0.05% | 0.25% | 0.17% | 0.04% | -0.13% | -0.20% |
|  | 0.05% | 0.23% | 0.19% | 0.04% | -0.17% | -0.09% |
| Enc Time[%] | 99% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2 BD-rate results for proposal with restriction in boundary filtering disabled

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.54% | 0.00% | 0.00% | 0.61% | 0.00% | 0.00% |
| Class B | 0.26% | 0.00% | 0.00% | 0.29% | 0.00% | 0.00% |
| Class C | 0.07% | 0.00% | 0.00% | 0.07% | 0.00% | 0.00% |
| Class D | 0.08% | 0.00% | 0.00% | 0.09% | 0.00% | 0.00% |
| Class E | 0.16% | 0.00% | 0.00% | 0.17% | 0.00% | 0.00% |
| Class F | 0.10% | 0.00% | 0.00% | 0.10% | 0.01% | 0.00% |
| Overall | 0.21% | 0.00% | 0.00% | 0.23% | 0.00% | 0.00% |
|  | 0.20% | 0.00% | 0.00% | 0.22% | 0.00% | 0.00% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | % |  |  | % |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.28% | -0.03% | -0.10% | 0.20% | -0.17% | -0.03% |
| Class B | 0.10% | 0.02% | 0.03% | 0.13% | 0.01% | 0.01% |
| Class C | 0.06% | 0.00% | 0.01% | 0.09% | 0.03% | -0.12% |
| Class D | 0.09% | 0.01% | -0.01% | 0.06% | -0.03% | -0.08% |
| Class E |  |  |  |  |  |  |
| Class F | 0.17% | 0.02% | 0.04% | 0.16% | 0.05% | 0.03% |
| Overall | 0.14% | 0.00% | 0.00% | 0.13% | -0.02% | -0.03% |
|  | 0.13% | 0.01% | -0.01% | 0.12% | -0.02% | -0.06% |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | % |  |  | % |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.08% | 0.18% | -0.08% | 0.11% | 0.08% | 0.25% |
| Class C | 0.15% | 0.01% | 0.07% | 0.07% | 0.09% | 0.05% |
| Class D | -0.02% | 0.19% | 0.02% | 0.05% | -0.14% | -0.29% |
| Class E | 0.18% | 0.18% | 0.61% | 0.08% | -0.18% | 0.31% |
| Class F | 0.18% | 0.15% | 0.42% | 0.07% | -0.26% | -0.82% |
| Overall | 0.11% | 0.14% | 0.17% | 0.08% | -0.07% | -0.10% |
|  | 0.11% | 0.09% | 0.20% | 0.08% | -0.09% | -0.10% |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | % |  |  | % |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.03% | -0.03% | -0.13% | -0.04% | 0.06% | -0.46% |
| Class C | 0.07% | 0.07% | 0.20% | 0.06% | -0.02% | -0.07% |
| Class D | 0.07% | 0.35% | -0.39% | 0.05% | 0.10% | 0.15% |
| Class E | 0.10% | 0.57% | 0.13% | 0.08% | -0.38% | -0.11% |
| Class F | 0.18% | 0.13% | 0.32% | 0.10% | 0.30% | -0.43% |
| Overall | 0.07% | 0.19% | 0.01% | 0.04% | 0.03% | -0.20% |
|  | 0.07% | 0.14% | 0.01% | 0.05% | -0.04% | -0.14% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | % |  |  | % |  |  |

### CE1 Sequences

The following objective results are obtained with CE1 sequences under test conditions described in [4]. The accompanying files are referenced due to the large amount of data.

- Type 1, CTC, LD-B / RA, QP=32, 37, 41:

  - File: “results_Type1_HM8_vs_nonCE1_QC.xlsx”

- Type 1, CTC + max deblocking offsets=6, LD-B / RA, QP=32, 37, 41:

  - File: “results_Type1_HM8_MaxOffset_vs_nonCE1_QC.xlsx”

- Type 1, CTC + max transform=16x16, LD-B / RA, QP=32, 37, 41:

  - File: “results_Type1_HM8_maxTU16_vs_nonCE1_QC.xlsx”

- Type 1, CTC + max transform=16x16, max deblocking offsets=6, LD-B / RA, QP=32, 37, 41:

  - File: “results_Type1_HM8_maxTU16_MaxOffset_vs_nonCE1_QC.xlsx”

- Type 2, CTC, LD-B, QP=27, 32, 37:

  - File: “results_Type2_HM8_vs_nonCE1_QC.xlsx”

- Type 2, CTC + max transform=16x16, LD-B, QP=27, 32, 37:

  - File: “results_Type2_HM8_maxTU16_vs_nonCE1_QC.xlsx”

# Conclusion

Minor modifications are proposed to the deblocking filter to reduce the large block artifacts observed in certain video content. In summary, it is proposed to adjust the normal deblocking filter clipping for maximum transform edges and to restrict boundary filtering for DC, horizontal and vertical intra modes. The following are key benefits of the proposal:

- Preserves HM8 deblocking filter decision conditions and filters.

- Tuning of signalled deblocking adjustment parameters is not required.

- Significant reduction of sharp edges of large block artifacts without oversmoothing other picture areas.

- Improved subjective quality by additional texture detail is observed.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” 10th JCT-VC meeting, Stockholm, Sweden, July 2012, Doc. JCTVC-J1003_d7

[2] http://media.xiph.org/video/derf/

[3] A. Norkin, “AHG6: Reduction of block artifacts in HEVC for large blocks,” 10th JCT-VC Meeting, Stockholm, Sweden, July 2012, Doc. JCTVC-J0286

[4] A. Norkin, T. Suzuki, M. Budagavi, G. Van der Auwera, “Description of Core Experiment 1: Deblocking Filter,” 10th JCT-VC Meeting, Stockholm, Sweden, July 2012, Doc. JCTVC-J1101

# DIS Text Changes

The following DIS text changes are based on [1].

#### “8.7.2.3 Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma picture sample array recPictureL,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a two-dimensional array of size (nS)x(nS), edgeFlags.

Output of this process is:

–	a two-dimensional array of size (nS)x(nS), bS specifying the boundary filtering strength.

The boundary filtering strength array bS for the current coding unit is derived as follows.

The variables xDi, yDj, xN and yN are derived as follows.

- If edgeType is equal to EDGE_VER, xDi is set equal to ( i << 3 ), yDj is set equal to ( j << 2 ), xN is set equal to (1 << ( log2CbSize − 3 )) − 1 and yN is set equal to (1 << ( log2CbSize − 2 )) − 1.

- Otherwise (edgeType is equal to EDGE_HOR), xDi is set equal to ( i << 2 ), yDj is set equal to ( j << 3 ), xN is set equal to (1 << ( log2CbSize − 2 )) − 1 and yN is set equal to (1 << ( log2CbSize − 3 )) − 1.

For xDi with i=0..xN, the following applies.

For yDj with j=0..yN, the following applies.

- If edgeFlags[ xDi ][ yDj ] is equal to 1, the sample values are derived as follows.

- If edgeType is equal to EDGE_VER, sample p0 = recPictureL[ xC + xDi − 1 ][ yC + yDj ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

- Otherwise (edgeType is equal to EDGE_HOR), sample p0 = recPictureL[ xC + xDi ][ yC + yDj − 1 ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

Depending on p0 and q0, the variable bS[ xDi ][ yDj ] is derived as follows.

  - The variables log2TrafoSizeQ and log2TrafoSizeP are set equal to the log2TrafoSize values of the transform units which include the sample q0 and p0, respectively.

  - If the block edge is also a transform block edge and Log2MaxTrafoSize is greater than 3 and one of the following conditions is true then the variable bS[ xDi ][ yDj ] is set equal to 3:

    - If the sample p0 is in the luma coding block of a coding unit coded with intra prediction mode and log2TrafoSizeP is equal to Log2MaxTrafoSize

    - The sample q0 is in the luma coding block of a coding unit coded with intra prediction mode and log2TrafoSizeQ is equal to Log2MaxTrafoSize

    - If the sample p0 is in the luma coding block of a coding unit coded with inter prediction mode and the sample p0 is in a luma transform block which contains non-zero transform coefficient level and log2TrafoSizeP is equal to Log2MaxTrafoSize

    - If the sample q0 is in the luma coding block of a coding unit coded with inter prediction mode and the sample q0 is in a luma transform block which contains non-zero transform coefficient level and log2TrafoSizeQ is equal to Log2MaxTrafoSize

- Otherwise, if the sample p0 or q0 is in the luma coding block of a coding unit coded with intra prediction mode, the variable bS[ xDi ][ yDj ] is set equal to 2.

- Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a luma transform block which contains non-zero transform coefficient level.

- Otherwise, the following applies.

    - When edgeType is equal to EDGE_HOR and yC + yDj − 1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), sample p0 = recPictureL[ xL ][ yC + yDj – 1 ] where xL is equal to ((( xC + xDi ) >> 3) << 3) + ((( xC + xDi ) >> 3) & 1) * 7.

    - If one or more of the following conditions are true, the variable bS[ xDi ][ yDj ] is set equal to 1.

    - For the prediction of the luma prediction block containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the luma prediction block containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two luma prediction blocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a luma prediction block with lop left luma sample covering ( xB, yB ), is equal to PredFlagL0[ xB, yB ] + PredFlagL1[ xB, yB ].

    - One motion vector is used to predict the luma prediction block containing the sample p0 and one motion vector is used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 4 in units of quarter luma frame samples.

    - Two motion vectors and two different reference pictures are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same two reference pictures are used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two luma prediction blocks for the same reference picture is greater than or equal to 4 in units of quarter luma frame samples,

    - Two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample p0 and two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample q0 and all of the following conditions are true:

    - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two luma prediction bocks is greater than or equal to 4 in quarter luma frame samples or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two luma prediction blocks is greater than or equal to 4 in units of quarter luma frame samples,

    - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the luma prediction block containing the sample p0 and the list 1 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma frame samples or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the luma prediction block containing the sample p0 and list 0 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma frame samples.

    - Otherwise (none of the conditions above is true), the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise (edgeFlags[ xDi ][ yDj ] is equal to 0), the variable bS[ xDi ][ yDj ] is set equal to 0.”

##### “8.7.2.4.3 Decision process for luma block edge

Inputs of this process are:

–	a luma picture sample array recPictureL,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma block relative to the top left sample of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a variable bS specifying the boundary filtering strength.

Outputs of this process are:

–	variables dE, dEp, dEq containing decisions,

–	variables β, tC.

If edgeType is equal to EDGE_VER, the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +i ][ yC + yB + k ]		(5-1)

pi,k = recPictureL[ xC + xB – i − 1 ][ yC + yB + k ]	(5-2)

Otherwise (edgeType is equal to EDGE_HOR), the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +k ][ yC + yB + i ]		(5-3)

pi,k = recPictureL[ xC + xB +k ][ yC + yB – i − 1 ]		(5-4)

The variables QPQ and QPP are set equal to the QPY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 )	(5-5)

The value of the variable β′ is determined as specified in X based on the luma quantization parameter Q derived as:

Q = Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) )	(5-6)

where beta_offset_div2 is the value of the variable beta_offset_div2 for the slice that contains sample q0,0.

The variable β is derived as:

β = β′*(1<<(BitDepthY − 8))	(5-7)

The value of the variable tC′ is determined as specified as X based on the luma quantization parameter Q derived as:

Q = Clip3( 0, 53, qPL + 2*( bS − 1) + ( tc_offset_div2 << 1) + tc_offset_MaxTrafoSize)	(5-8)

where tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0 and if bS is equal to 3, then tc_offset_MaxTrafoSize is equal to -4, otherwise 0.

The variable tC is derived as:

tC = tC′*(1<<(BitDepthY − 8))	(5-9)

If bS is equal to 3, then the variables c0, c1 are derived as:

c0 = tC << 2, c1 = tC << 1,

otherwise, c0 = tC, c1 = tC >> 1

Depending on edgeType, the following applies:

….”

##### “8.7.2.4.7 Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	a variable dE,

–	variables dEp1 and dEq1 containing decisions to filter samples p1 and q1 respectively,

–	a variable tC

–	variables c0, c1.

Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp − 1, j = 0..nDq − 1

Depending on dE, the following applies:

–	If the variable dE is equal to 2, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = Clip3( p0−2*tc, p0+2*tc, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(5-10)

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )	(5-11)

p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )	(5-12)

q0’ = Clip3( q0−2*tc, q0+2*tc, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(5-13)

q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )	(5-14)

q2’= Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )	(5-15)

–	Otherwise, nDp and nDq are set equal to 0 and the following weak filtering applies:

 = ( 9 * ( q0 −  p0 ) − 3 * ( q1 − p1 ) + 8 ) >> 4		(5-16)

  - When abs() is less than tc*10, the following ordered steps apply:

    - The filtered sample values p0’ and q0’ are specified as follows:

 = Clip3( −tc, tc, -c0, c0,  )		(5-17)

p0’ = Clip1Y( p0 +  )		(5-18)

q0’ = Clip1Y( q0 −  )		(5-19)

    - When dEp1 is equal to 1, the filtered sample value p1’ is specified as follows:

p = Clip3( −(tc >> 1), tc >> 1, -c1, c1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(5-20)

p1’ = Clip1Y( p1 + p )		(5-21)

    - When dEq1 is equal to 1, the filtered sample value q1’ is specified as follows:

q = Clip3( −(tc >> 1), tc >> 1, -c1, c1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(5-22)

q1’ = Clip1Y( q1 + q )		(5-23)

    - nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

…”

##### “8.7.2.4.5 Filtering process for chroma block edge

Inputs of this process are:

–	a chroma picture sample array s’,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma block relative to the top left sample of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a variable bS specifying the boundary filtering strength.

Output of this process is:

–	the modified chroma picture sample array s’.

If edgeType is equal to EDGE_VER, the values pi and qi with i = 0..1 and k = 0..3 are derived as follows:

qi,k = s’[ xC + xB +i ][ yC + yB + k ]	(5-24)

pi,k = s’[ xC + xB – i − 1 ][ yC + yB + k ]	(5-25)

Otherwise (edgeType is equal to EDGE_HOR), the sample values pi and qi with i = 0..1 and k = 0..3 are derived as follows:

qi,k = s’[ xC + xB +k ][ yC + yB + i ]	(5-26)

pi,k = s’[ xC + xB +k ][ yC + yB – i − 1 ]	(5-27)

The variables QPQ and QPP are set equal to the QPY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable QPC is determined as specified in X based on the index qPi derived as:

qPi = ( ( QPQ + QPP + 1 ) >> 1 )	(5-28)

The value of the variable tC′ is determined as specified as X based on the chroma quantization parameter Q derived as:

Q = Clip3( 0, 53, QPC + 2*(bS − 1) + (tc_offset_div2 << 1 ) + tc_offset_MaxTrafoSize)	(5-29)

The variable tC is derived as:

tC = tC′*(1<<(BitDepthY − 8))	(5-30)

where tc_offset_div2 is the value of the variable tc_offset_div2 for the slice that contains sample q0,0 and if bS is equal to 3, then tc_offset_MaxTrafoSize is equal to -2, otherwise 0.

If q0,0 or p0,0 is in the luma coding block of a coding unit coded with intra prediction mode, depending on edgeType, the following applies:

–	If edgeType is equal to EDGE_VER, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The filtering process for a chroma sample as specified in subclause X is invoked with the sample values pi,k, qi,k, with i = 0..1, and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB ][ yC + yB + k ] = q0’	(5-31)

s’[ xC + xB − 1 ][ yC + yB + k ] = p0’	(5-32)

–	Otherwise (edgeType is equal to EDGE_HOR), for each sample location ( xC + xB + k, yC + yB ), k = 0..3, the following ordered steps apply:

- The filtering process for a chroma sample as specified in subclause X is invoked with the sample values pi,k, qi,k, with i = 0..1, and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k ][ yC + yB ] = q0’	(5-33)

s’[ xC + xB +k ][ yC + yB − 1 ] = p0’	(5-34)

”

## Intra Prediction Changes

##### “8.4.4.2.5 Specification of Intra_DC (1) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process are:

–	predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as the following ordered steps:

- A variable dcVal is derived as:

dcVal = x'=0nS−1p[x'][−1]+y'=0nS−1p[−1][ y']+nS>>(k+1)	
where k=Log2(nT)

- Depending on the chroma component index cIdx, the following applies.

- If cIdx is equal to 0 and Log2(nT) is less than 5 the following applies.

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2*dcVal + p[ 0 ][ −1 ] + 2 ) >> 2	
predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3*dcVal + 2 ) >> 2, with x = 1..nT−1	
predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3*dcVal + 2 ) >> 2, with y = 1..nT−1	
predSamples[ x ][ y ] = dcVal, with x, y = 1..nT−1

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as

predSamples[ x ][ y ] = dcVal, with x, y = 0..nT−1

##### 8.4.4.2.6 Specification of Intra_Angular (2..34) prediction mode

…

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

  - The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( y + 1 )*intraPredAngle ) >> 5

iFact = ( ( y + 1 )*intraPredAngle ) && 31

  - Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ x+iIdx+1 ] + iFact*ref[ x+iIdx+2] + 16 ) >> 5

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ref[ x+iIdx+1 ]

  - When intraPartMode is equal to Intra_Angular (26), Log2(nT) is less than 5 and cIdx is equal to 0, the following filtering applies with x = 0, y = 0..nT−1.

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] – p[ −1 ][ −1 ] ) >> 1 ) )

–	Otherwise (intraPredMode is less than 18), the following ordered steps apply.

- The reference sample array ref[ x ], with x= −nT..2*nT is specified as follows.

ref[ x ] = p[ −1 ][ −1+x ], with x=0..nT

- If intraPredAngle is less than 0, the main reference sample array is extented as follows.

- When ( nT*intraPredAngle ) >>5 is less than −1,

ref[ x ] = p[ −1+( ( x*invAngle+128 )>>8 ) ][ −1 ], with x=( nT*intraPredAngle ) >>5..−1

- Otherwise,

ref[ x ] = p[ −1 ][ −1+x ], with x=nT+1..2*nT

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

- The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( x + 1 )*intraPredAngle ) >> 5

iFact = ( ( x + 1 )*intraPredAngle ) && 31

- Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ y+iIdx+1 ] + iFact*ref[ y+iIdx+2] + 16 ) >> 5

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ref[ y+iIdx+1 ]

- When intraPartMode is equal to Intra_Angular (10), Log2(nT) is less than 5 and cIdx is equal to 0, the following filtering applies with x = 0..nT−1, y = 0.

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] – p[ −1 ][ −1 ] ) >> 1 )

…”

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).