| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0139 |
| --- | --- |

| Title: | Contouring artefact and solution |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | TK Tan Y. Suzuki 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +65 97993861 ttk@pacific.net.sg suzukiyos@rd.nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc |  |  |

_____________________________

# Abstract

This contribution reports the problem of visible contouring artefacts found in some sequences under the common test conditions.  This contribution further identifies that the problem is due to the propagation of blocking artefact from the reference samples during the process of creating the intra predicted sample array predSamples in the 32x32 blocks.  This contribution proposes a simple solution to avoid this artefact by replacing the intra smoothing filter with a bi-linear interpolation of the reference samples in the 32x32 intra prediction when the conditions for contouring artefact is detected.  This solution is reported to reduce the contouring artefacts without causing any other subjective artefacts and has no significant impact in the average BD rate or simulation time for all the common conditions settings.

# Introduction

Contouring artefacts were found in sequences having flat areas with gradual illumination change.  Two clear examples of the artefacts are found in the first frame of the “Johnny” and “KristenAndSara” sequences as shown in Figure 1 and Figure 2 below.  Since the background is static in these sequences, the artefacts were not refreshed and hence propagated to the rest of the sequence in the low delay main coding configuration.

The source for the contouring was identified as originating from a discontinuity in the reference samples located at the 32x32 block boundary that were used for producing the intra predicted samples array predSamples.  When the intra prediction mode with a direction other than horizontal or vertical is selected, the discontinuity is propagated to sample locations that are no longer on the block boundary.  This is illustrated in Figure 3, where the discontinuity at reference samples above the current block to be encoded is propagated diaginally down.

The deblocking filter which act on the block boundary can no longer remove such discontinuities in the final output picture.  Also note that for 32x32 blocks, the intra smoothing filter is already enabled for all directions.

Attempts to remove this artefact by the following methods have failed or resulted in increase in bit rate.

- Disabling 32x32 intra prediction did not work as 16x16 intra prediction block can also propagate the artefact in the same manner.

- Reducing the quantization step size did not work as the contouring is also visible at qp = 32 and qp = 27.

Figure 1: First frame of Johnny sequence coded with low delay main configuration at QP=37

Figure 2: First frame of KristenAndSara sequence coded with low delay main configuration at QP=37

Figure 3: Propagation of discontinuity at block boundary to sample locations within the predicted sample array.

# Proposed solution

The proposed solution is to modify the intra smoothing filter in the 32x32 blocks when the conditions for contouring artefacts are detected.

Let N=32 when the block size is 32x32.

Let the bottom left reference sample be BL=ref[0];  the above left reference sample be AL=ref[2N]; and the above right reference sample be AR=ref[4N], as shown in Figure 3, be the key reference samples.

Let THRESHOLD be the threshold values used for determining the criteria for interpolating the reference samples, respectively.

The following two equations define the Boolean variable Interpolate_Above and Interpolate_Left that are derived by comparing the key reference samples to the thresholds.

Interpolate_Above = abs(AL+AR–2*ref[3N]) < THRESHOLD_ABOVE	(1)

Interpolate_Left = abs(BL+AL–2*ref[N]) < THRESHOLD_LEFT		(2)

If these criteria (1) and (2) are satisfied, then the process of bilinear interpolation is performed as follows to form the smoothed reference samples, ref’[i].

ref’[0] = ref[0]      		 						(3)

ref’[i] = BL + i*(AL-BL+N)/2N, 		(i=1 to 2N-1)			(4)

ref’[2N] = ref[2N]      		 					(5)

ref’[2N+i] = AL + i*(AR-AL+N)/2N, 	(i=1 to 2N-1)			(6)

ref’[4N] = ref[4N]      							(7)

Otherwise (criteria (1) and (2) are not satisfied), the process of applying the intra smoothing as specified in sub clause 8.4.4.2.3 is performed by applying the 121 filter to reference samples to form the smoothed reference samples, ref’[i].

The bi-linear interpolation of equation (4) and (6) can be graphically depicted as in Figure 4 below.  The bi-linear interpolation process is merely interpolating the values between the above left reference sample (AL=ref[2N]) and the two extreme reference samples of the above right and bottom left (AR=ref[4N] and BL=ref[0]).

Figure 4: Bi-linear interpolation of reference samples using the corner samples.

When AL, AR and ref[3N] forms a straight line, then the term (AL+AR–2*ref[3N]) is 0.  Similarly, when BL, AL and ref[N] forms a straight line, the term (BL+AL–2*ref[N]) is 0.  Hence the above equations (1) and (2) compares the magnitude of the deviation of ref[3N] from the line joining AL to AR and ref[N] from the line joining BL to AL to the thresholds, respectively.  Interpolate_Above and Interpolate_Left become true, if the deviations calculated are below the threshold THRESHOLD.

# Results

The objective results for the common conditions are shown below.  The results show no significant impact of the proposed solution in BD-Rate or simulation time.

Table 1: Objective results of the proposal compared to HM8.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.07% | 0.13% | 0.09% | 0.02% | 0.05% | -0.05% |
| Class B | 0.09% | 0.14% | 0.13% | 0.01% | 0.03% | 0.04% |
| Class C | 0.02% | 0.05% | 0.06% | 0.01% | 0.02% | -0.01% |
| Class D | 0.01% | 0.04% | -0.02% | 0.00% | -0.01% | 0.01% |
| Class E | 0.12% | 0.28% | 0.23% | -0.06% | 0.10% | 0.05% |
| Class F | 0.04% | 0.00% | 0.06% | 0.03% | 0.01% | -0.03% |
| Overall | 0.06% | 0.10% | 0.09% | 0.01% | 0.03% | 0.00% |
|  | 0.06% | 0.11% | 0.09% | 0.01% | 0.03% | -0.01% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 95% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.10% | 0.01% | -0.05% | -0.05% | 0.13% | -0.46% |
| Class B | 0.05% | 0.07% | 0.01% | 0.00% | 0.08% | 0.07% |
| Class C | -0.01% | 0.15% | 0.09% | -0.01% | -0.07% | -0.08% |
| Class D | 0.02% | -0.14% | 0.13% | 0.00% | -0.08% | -0.06% |
| Class E |  |  |  |  |  |  |
| Class F | -0.03% | -0.08% | -0.11% | 0.06% | 0.15% | 0.11% |
| Overall | 0.03% | 0.00% | 0.01% | 0.00% | 0.04% | -0.08% |
|  | 0.03% | -0.01% | -0.01% | 0.00% | 0.02% | -0.12% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.02% | 0.14% | 0.17% | -0.01% | -0.07% | 0.23% |
| Class C | 0.00% | 0.07% | -0.09% | -0.04% | 0.03% | 0.17% |
| Class D | -0.07% | 0.38% | -0.38% | -0.01% | -0.09% | -0.61% |
| Class E | -0.11% | -0.03% | -0.50% | -0.15% | 0.16% | -1.24% |
| Class F | 0.07% | 0.35% | -0.11% | 0.01% | 0.74% | -0.44% |
| Overall | -0.01% | 0.19% | -0.15% | -0.03% | 0.14% | -0.31% |
|  | -0.01% | 0.20% | -0.15% | -0.03% | 0.13% | -0.31% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | -0.02% | 0.16% | 0.00% | 0.01% | -0.28% |
| Class C | 0.02% | 0.10% | 0.36% | -0.06% | -0.09% | -0.05% |
| Class D | 0.01% | 0.43% | -0.29% | 0.00% | -0.18% | -0.28% |
| Class E | -0.15% | 0.07% | -1.15% | -0.17% | -0.29% | -0.85% |
| Class F | 0.20% | 0.47% | 0.71% | -0.07% | 0.14% | 0.42% |
| Overall | 0.03% | 0.21% | 0.02% | -0.05% | -0.07% | -0.18% |
|  | 0.03% | 0.16% | 0.04% | -0.05% | -0.12% | -0.16% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |

The subjective results show that the contouring artefacts were removed from the sequences shown in Figure 5 and Figure 6 below.

Figure 5: First frame of Johnny sequence coded with low delay main configuration at QP=37, with the bi-linear interpolation of intra reference sample.

Figure 6: First frame of KristenAndSara sequence coded with low delay main configuration at QP=37, with the bi-linear interpolation of intra reference sample.

# Draft Text

8.4.4.2.3	Filtering process of neighbouring samples

Inputs to this process are:

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform block size.

Output of this process are:

–	filtered samples pF[ x ][ y ],. with x, y = −1..2*nT−1.

The variable intThreshold is set to 8.

The variable interpolateFlag is derived as follows.

–	If all of the following conditions are true, interpolateFlag is set equal to 1

–	nT is equal 32

–	Abs(p[ −1 ][ −1 ]+ p[ 2*nT−1 ][ −1 ]–2* p[ nT−1 ][ −1 ]) < intThreshold

–	Abs(p[ −1 ][ −1 ]+ p[ −1 ][ 2*nT−1 ]–2* p[ −1 ][ nT−1 ]) < intThreshold

Otherwise, interpolateFlag is set equal to 0.

The variable filterFlag is derived as follows.

–	If one or more of the following conditions are true, filterFlag is set equal to 0

–	intraPredMode is equal to Intra_DC

–	nT is equal 4

–	interpolateFlag is equal 1

Otherwise, the following applies.

–	The variable minDistVerHor is set equal to Min( Abs( intraPredMode − 26 ), Abs( intraPredMode − 10 ) ).

–	The variable intraHorVerDistThres[ nT ] is specified in Table 8 3.

–	The variable filterFlag is derived as follows.

–	If minDistVerHor is larger than intraHorVerDistThresh[ nT ], filterFlag is set equal to 1,

–	Otherwise, filterFlag is set equal to 0.

Table 8 3 – Specification of intraHorVerDistThres[ nT ] for various transform block sizes

|  | nT = 8 | nT = 16 | nT = 32 |
| --- | --- | --- | --- |
| intraHorVerDistThresh[ nT ] | 7 | 1 | 0 |

When interpolateFlag is equal to 1, the filtered sample values pF[ x, y ] with x = −1..63 and y = −1..63 are derived as follows.

pF[ −1 ][ 63 ] = p[ −1 ][ 63 ]									(8 28)

pF[ 63 ][ −1 ] = p[ 63 ][ −1 ]									(8 29)

pF[ −1 ][ y ] = p[ −1 ][ −1 ] + (y+1)*(p[ −1 ][ 63 ] − p[ −1 ][ −1 ] + 32 ) >> 6 for y = 0..62	(8 30)

pF[ −1 ][ −1] = p[ −1 ][ −1]									(8 31)

pF[ x ][ −1 ] = p[ −1 ][ −1 ] + (x+1)*(p[ 63 ][ −1 ] − p[ −1 ][ −1 ] + 32 ) >> 6 for x = 0..62	(8 32)

When filterFlag is equal to 1, the filtered sample values pF[ x, y ] with x = −1..nT*2−1 and y = −1..nT*2−1  are derived as follows.

pF[ −1 ][ nT*2−1 ] = p[ −1 ][ nT*2−1 ]								(8 28)

pF[ nT*2−1 ][ −1 ] = p[ nT*2−1 ][ −1 ]								(8 29)

pF[ −1 ][ y ] = ( p[ −1 ][ y+1 ] + 2*p[ −1 ][ y ] + p[ −1 ][ y−1 ] + 2 ) >> 2 for y = nT*2−2..0	(8 30)

pF[ −1 ][ −1] = ( p[ −1 ][ 0 ] + 2*p[ −1 ][ −1] + p[ 0 ][ −1 ] + 2) >> 2				(8 31)

pF[ x ][ −1 ] = ( p[ x−1 ][ −1 ] + 2*p[ x ][ −1 ] + p[ x+1 ][ −1 ] + 2 ) >> 2 for x = 0..nT*2−2	(8 32)

# HM software modification

--- source/Lib/TLibCommon/TComPattern.cpp	(revision 2715)

+++ source/Lib/TLibCommon/TComPattern.cpp	(working copy)

@@ -285,6 +285,40 @@

piFilterBuf[l++] = piAdiTemp[1 + i];

}

+#if BI_LINEAR_REF

+  Int IbrBlkSize = 1 << pcCU->getSlice()->getSPS()->getIbrLog2BlkSize();

+  Int IbrThreshold = pcCU->getSlice()->getSPS()->getIbrThreshold();

+  Int BL = piFilterBuf[0];

+  Int TL = piFilterBuf[uiCuHeight2];

+  Int TR = piFilterBuf[iBufSize-1];

+  bool bilinear_left = (abs(BL+TL-2*piFilterBuf[uiCuHeight]) < IbrThreshold)? true : false;

+  bool bilinear_above  = (abs(TL+TR-2*piFilterBuf[uiCuHeight2+uiCuHeight]) < IbrThreshold)? true : false;

+

+  if (uiCuWidth>=IbrBlkSize && (bilinear_left && bilinear_above))

+  {

+    Int shift = g_aucConvertToBit[uiCuWidth] + 3;  // log2(uiCuHeight2)

+    piFilterBufN[0] = piFilterBuf[0];

+    piFilterBufN[uiCuHeight2] = piFilterBuf[uiCuHeight2];

+    piFilterBufN[iBufSize - 1] = piFilterBuf[iBufSize - 1];

+    for (i = 1; i < uiCuHeight2; i++)

+    {

+      piFilterBufN[i] = ((uiCuHeight2-i)*BL + i*TL + uiCuHeight) >> shift;

+    }

+    for (i = 1; i < uiCuWidth2; i++)

+    {

+      piFilterBufN[uiCuHeight2 + i] = ((uiCuWidth2-i)*TL + i*TR + uiCuWidth) >> shift;

+    }

+  }

+  else {

+    // 1. filtering with [1 2 1]

+    piFilterBufN[0] = piFilterBuf[0];

+    piFilterBufN[iBufSize - 1] = piFilterBuf[iBufSize - 1];

+    for (i = 1; i < iBufSize - 1; i++)

+    {

+      piFilterBufN[i] = (piFilterBuf[i - 1] + 2 * piFilterBuf[i]+piFilterBuf[i + 1] + 2) >> 2;

+    }

+  }

+#else

// 1. filtering with [1 2 1]

piFilterBufN[0] = piFilterBuf[0];

piFilterBufN[iBufSize - 1] = piFilterBuf[iBufSize - 1];

for (i = 1; i < iBufSize - 1; i++)

{

piFilterBufN[i] = (piFilterBuf[i - 1] + 2 * piFilterBuf[i]+piFilterBuf[i + 1] + 2) >> 2;

}

+#endif

# Conclusions

This contribution identified a deficiency in the current HEVC specification of the intra prediction that results in visible contouring artefacts in smooth areas of the coded sequences.  This contribution proposes a simple solution that is localized to the sub clause 8.4.4.2.3 on the filtering process of neighbouring samples.  The simple solution was able to improve the subjective results by removing the contouring artefact and have no impact on the objective results.  This contribution proposes to correct the deficiency in the current HEVC specification by adopting the simple change to the sub clause 8.4.4.2.3 is suggested in section 4 of this contribution.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).