| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0140 |
| --- | --- |

| Title: | AHG9: Simple HEVC stream editing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution presents high-level syntax and semantics for simple HEVC bitstream editing. This is the follow-up proposal of JCTVC-J0137 at the 10th JCT-VC meeting.

This contribution contains three topics. The first topic is a modifier of cpb_removal_delay and dpb_output_delay. The value of the modifier is intended to be set by an editing process which removes leading pictures from the bitstream.

The second topic relates to the restriction imposed when fixed_pic_rate_flag is equal to one. It is proposed that this restriction is not applied at a sequence boundary.

The third topic is a SEI message for indicating a possible editing point in a bitstream. When the bitstream is chopped up into two pieces at the beginning of a coded picture associated with this SEI message, and the former piece is connected by another bitstream starting with a RAP picture, it is assured that resulting bitstream is still conforming to the restriction imposed when fixed_pic_rate_flag is equal to one.

# Introduction

In JCTVC-J0137 [2] at the 10th JCT-VC meeting it was proposed to enhance the HRD specification in order to realize simple editing of an HEVC bitstream. Three topics were discussed in JCTVC-J0137. The first topic is two modifiers of cpb_removal_delay and dpb_output_delay referred when TFD (Tagged for discard) pictures are removed. The second topic is an SEI message for indicating a possible editing point at which the output timings of all decoded pictures are continuous after bitstream editing. The third topic is a parameter which indicates the continuity of the HRD at the boundary of two connected bitstreams.

The first topic was concluded as “further study” because the proposed modifiers cannot be applied to the case when a DLP (Decodable leading picture) picture, whose concept was introduced in draft specification text version 8 [1], follows a CRA picture together with a TFD picture. The second topic was also concluded as “further study” because the priority of the discussion on new SEI messages was decided to be lower than other version 1 related issues. The third topic was concluded as “no action” because it was decided to clarify the restriction on the continuity of the HRD at a sequence boundary.

This contribution presents high-level syntax and semantics for simple HEVC bitstream editing. This is the follow-up proposal of JCTVC-J0137 "New high-level syntax for simple HEVC stream editing" at the 10th JCT-VC meeting.

This contribution contains three topics. The first topic is a modifier of cpb_removal_delay and dpb_output_delay. The purpose of this modifier is to reduce the number of operations for bitstream editing including the removal of leading pictures. According to the current specification, the editing process has to change the values of cpb_removal_delay and dpb_output_delay of all pictures following a BLA picture when the leading pictures are removed from the bitstream. On the contrary, the editing process only has to change the values of the modifiers in a limited number of coded pictures. Unlike the modifiers in JCTVC-J0137, the proposed modifier can handle the case when DLA pictures and TFD pictures are interleaved in the bitstream.

The second topic relates to the restriction imposed when fixed_pic_rate_flag is equal to one. The current specification requires that the interval of output timings of any two succeeding pictures is equal to tc in Equation C-1 when the video sequence has a constant picture rate. This is too restrictive when two bitstreams with different value of sps_max_num_reoder_pics are concatenated. The interval of output timings between the last picture in the former bitstream in output order and the first picture in the latter bitstream in output order may be larger than tc. It is proposed that the restriction is not applied at a sequence boundary. This topic was not included in JCTVC-J0137.

The third topic is an SEI message for indicating a possible editing point in a bitstream. When the bitstream with sps_max_num_reoder_pics not equal to one is chopped up into two pieces at arbitrary position, the interval of output timings of two succeeding pictures in the former bitstream may be larger than tc. When the bitstream is chopped up into two pieces at the beginning of a coded picture associated with this SEI message, and the former piece is connected by another bitstream starting with a RAP picture, it is assured that resulting bitstream is still conforming to the restriction imposed when fixed_pic_rate_flag is equal to one. The proposed SEI message is basically the same as proposed in JCTVC-J0137.

# Modifier of cpb_removal_delay and dpb_output_delay for removal of leading picture

## Problem statement

Draft specification text version 8 specifies two NAL unit types, TFD (Tagged for discard) and DLA (Decodable leading picture), for a leading picture which precedes a RAP (Random access point) picture in output order and follows the RAP picture in decoding order. A TFD picture is not decodable when the decoding process starts from the immediately preceding CRA picture because the TFD picture refers to a picture which precedes the CRA picture in the decoding order. On the other hand, A DLA picture is decodable even when the decoding process starts from the CRA picture because the DLA picture only refers to the CRA picture (and possibly other DLA pictures).

When an HEVC bitstream editing process (implemented in such as a splicer) connects two HEVC bitstreams, the process changes the NAL unit type of the first CRA picture in the second bitstream from CRA to BLA (Broken link access), and generally removes all TFD pictures which immediately follow the CRA picture in decoding order since those TFD pictures are not decodable. In this case, the process has to change the CPB removal times and DPB output times of all pictures which follow the BLA picture in decoding order to maintain the continuity of the HRD after removing the TFD pictures.

In the current specification, the CPB removal time of a picture is derived by adding the delay signalled by the parameter cpb_removal_delay of the picture timing SEI to the CPB removal time of the immediately preceding picture associated with a buffering period SEI. The DPB output time of the picture is derived by adding the delay signalled by the parameter dpb_output_delay of the picture timing SEI to the CPB removal time of the picture. This means the process has to change the values of cpb_removal_delay and dpb_output_delay of all pictures from the BLA picture to the next picture associated with another buffering period SEI.

Figure 1 – Example of bitstream editing

Figure 1 shows an example of bitstream editing. In this example, two bitstreams BSA and BSB are concatenated at the editing points (marked as red lines), then bitstream BSC is generated. TFD pictures B12, B10, B9, B11, B14, B13 and B15 are removed from bitstream BSc since those are not decodable. In order to keep bitstream BSC conforming to the HRD specification, the values of cpb_removal_delay of picture I16 and its following pictures until the picture associated with another buffering period SEI have be changed properly (as highlighted in red number). For example, the value of cpb_removal_delay of picture B24 has to be changed from ‘8’ to ‘1’. Otherwise, overflow of CPB might occur at picture B24 or its following pictures.

Moreover, when fixed_pic_rate_flag is equal to 1, improper DPB output time makes bitstream BSC violate the restriction on to,dpb( n ) (as specified in Equation C-17). For this reason, the value of dpb_output_delay of picture I16 has to be changed from ‘10’ to ‘3’. Otherwise, to,dpb( n ) of picture B30 becomes not equal to tc (as specified in Equation C-1), and the DPB output timing of picture I16 overlaps with that of another picture following picture I16 in output order.

In JCTVC-J0137 two syntax elements, alt_cpb_removal_delay_offset and alt_dpb_output_delay in a buffering period SEI message are proposed in order to reduce the burden on an HEVC bitstream editing process by not imposing the process to the values of change cpb_removal_delay and dpb_output_delay of all pictures whose CPB removal time and DPB output time are affected by the removal of TFD pictures. It is intended that the values of those two syntax elements are set according to the number of TFD pictures by an encoder. A decoder modifies the values of cpb_removal_delay and dpb_output_delay of all pictures from the BLA picture to the next picture associated with another buffering period SEI using the values of those two syntax elements. Specifically, the values of cpb_removal_delay of all pictures from the BLA picture to the picture associated with a buffering period SEI are subtracted by the value of alt_cpb_removal_delay_offset. Also, the value of dpb_output_delay of the BLA picture is replaced with the value of alt_dpb_output_delay.

For the case shown in Figure 1, alt_cpb_removal_delay_offset and alt_dpb_output_delay are set to 7 and 3, respectively.

But it was found during the presentation of JCTVC- J0137 that the proposed two syntax elements are not applicable to the case when leading pictures consist both of TFD picture(s) and DLA picture(s).

Figure 2 – Example of bitstream editing when TFD picture and DLP picture are interleaved

Figure 2 shows an example of bitstream editing in which the proposed scheme in JCTVC-J0137 cannot be applied. Unlike the example in Figure 1, the leading pictures of picture I16 are composed of TFD pictures B12, B10, B9, B11 and B13, and DLP pictures B14 and B13. This kind of situation happens for example when there is a scene change inside a “GOP”. In this example, a scene change happens between picture B13 and B14. If TFD pictures and DLP pictures are interleaved, the proper values for modifying cpb_removal_delay of picture B14 and the following pictures (i.e. B15, B24, …) are not equal. Moreover, dpb_output_delay of picture B14 shall also be changed since there is TFD picture B13 which follows picture B14 in output order in bitstream BSB.

This contribution presents a syntax element in order to solve the aforementioned problem. Two variations are proposed. The first variation is a syntax element in a picture timing SEI message. The editing process is intended to change the value of the syntax element in each picture which precedes the removed leading pictures in decoding order. The second variation is syntax elements in a buffering period SEI message. Similar to the first variation, the editing process is intended to change the value of the syntax elements in the BLA picture.

Any of two variations can handle the case when the editing process arbitrarily selects pictures for removal regardless those are TFD pictures or DLA pictures.

## Proposal

### Variation 1: syntax element in PTSEI

The first variation is a syntax element in a picture timing SEI. The semantics of the syntax element specifies the procedure to modify the values of cpb_removal_delay and dpb_output_delay in the decoding process.

The proposed syntax is the following. It is noted that the proposed syntax is based on the syntax specified in JCTVC-K0030 [3].

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( CpbDpbDelaysPresentFlag ) { |  |
| num_removed_tfds | u(8) |
| cpb_removal_delay | u(v) |
| dpb_output_delay | u(v) |
| if( sub_pic_cpb_params_present_flag ) { |  |
| num_decoding_units_minus1 | ue(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| common_du_cpb_removal_delay_flag | u(1) |
| if( common_du_cpb_removal_delay_flag ) |  |
| common_du_cpb_removal_delay_minus1 | u(v) |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !common_du_cpb_removal_delay_flag ) |  |
| du_cpb_removal_delay[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The proposed semantic is the following.

num_removed_tfds is used, with num_removed_tfds in the picture timing SEI of previous BLA access unit, to modify the values of cpb_removal_delay and dpb_output_delay in the same picture timing SEI message. When num_removed_tfds is not present, it is inferred to be equal to 0.

Let PrevNumRemovedTfds be the value of num_removed_tfds of previous BLA access unit. If the current access unit is a BLA access unit, PrevNumRemovedTfds is set to num_removed_tfds.

The variables CpbRemovalDelayCurr and DpbOutputDelayCurr are derived as follows:

- If PrevNumRemovedTfds is equal to 0, CpbRemovalDelayCurr and DpbOutputDelayCurr are derived as:

CpbRemovalDelayCurr = cpb_removal_delay

DpbOutputDelayCurr = dpb_output_delay

- Otherwise (PrevNumRemovedTfds is not equal to 0), CpbRemovalDelayCurr and DpbOutputDelayCurr are derived as:

CpbRemovalDelayCurr = cpb_removal_delay – PrevNumRemovedTfds + num_removed_tfds

DpbOutputDelayCurr = dpb_output_delay – num_removed_tfds

In Annex C, CpbRemovalDelayCurr and DpbOutputDelayCurr are used instead of cpb_removal_delay and dpb_output_delay, respectively.

In the examples in Figure 1 and Figure 2, the values of num_removed_tfds are set as follows:

Example in Figure 1:

| Picture | cpb_removal_delay (original) | dpb_output_delay (original) | num_removed_tfds | cpb_removal_delay (modified) | cpb_removal_delay (modified) |
| --- | --- | --- | --- | --- | --- |
| I16 | - | 10 | 7 | - | 3 |
| B24 | 8 | 10 | 0 | 1 | 10 |
| B20 | 9 | 5 | 0 | 2 | 5 |
| B18 | 10 | 2 | 0 | 3 | 2 |
| B17 | 11 | 0 | 0 | 4 | 0 |
| B19 | 12 | 1 | 0 | 5 | 1 |
| B22 | 13 | 3 | 0 | 6 | 3 |
| B21 | 14 | 1 | 0 | 7 | 1 |
| B23 | 15 | 2 | 0 | 8 | 2 |

Example in Figure 2:

| Picture | cpb_removal_delay (original) | dpb_output_delay (original) | num_removed_tfds | cpb_removal_delay (modified) | cpb_removal_delay (modified) |
| --- | --- | --- | --- | --- | --- |
| I16 | - | 10 | 5 | - | 5 |
| B14 | 5 | 3 | 1 | 1 | 2 |
| B15 | 7 | 2 | 0 | 2 | 2 |
| B24 | 8 | 10 | 0 | 3 | 10 |
| B20 | 9 | 5 | 0 | 4 | 5 |
| B18 | 10 | 2 | 0 | 5 | 2 |
| B17 | 11 | 0 | 0 | 6 | 0 |
| B19 | 12 | 1 | 0 | 7 | 1 |
| B22 | 13 | 3 | 0 | 8 | 3 |
| B21 | 14 | 1 | 0 | 9 | 1 |
| B23 | 15 | 2 | 0 | 10 | 2 |

The value of num_removed_tfds of picture A is intended to be derived by the editing process as follows:

Let picture B[i] (i = [0, N-1]) be a picture which follows picture A in decoding order and is removed from a bitstream through the editing process.

Let DecInterval[i] (i = [0, N-1]) be the variable which is derived as follows:

DecInterval[i] = CpbRemovalDelayCurr – CpbRemovalDelayPrev

Where CpbRemovalDelayCurr and CpbRemovalDelayPrev are the values of cpb_removal_delay of picture B[i] and cpb_removal_delay of the picture immediately precedes picture B[i] in decoding order, respectively. Note that DecInterval[i] is equal to the number of pictures between those two pictures when fixed_pic_rate_flag is equal to one.

The value of num_removed_tfds for picture A is derived as:

numremovedtfds=i=0N−1DecInterval[i]

### Variation 2: syntax element in BPSEI

The second variation is syntax elements in a buffering timing SEI. The semantics of the syntax elements specifies the procedure to modify the values of cpb_removal_delay and dpb_output_delay in the decoding process.

The proposed syntax is the following.

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| seq_parameter_set_id | ue(v) |
| if( !sub_pic_cpb_params_present_flag ) |  |
| rap_cpb_params_present_flag | u(1) |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag ) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| if( sub_pic_cpb_params_present_flag ¦ ¦ rap_cpb_params_present_flag) { |  |
| initial_alt_cpb_removal_delay[ SchedSelIdx ] | u(v) |
| initial_alt_cpb_removal_delay_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| if ( NalHrdBpPresentFlag ¦¦ VclHrdBpPresentFlag ) |  |
| { |  |
| ptsei_overwrite_flag | u(1) |
| if ( ptsei_overwrite_flag ) |  |
| num_entries_minus1 | ue(v) |
| for ( i = 0; i <= num_entries_minus1+1; i ++ ) { |  |
| alt_cpb_removal_delay_offset[ i ] | ue(v) |
| alt_dpb_output_delay_offset[ i ] |  |
| } |  |
| } |  |
| } |  |

The proposed semantic is the following.

<Buffering period SEI message semantics>

ptsei_overwrite_flag equal to 1 specifies syntax elements num_entries_minus1, alt_cpb_removal_delay_offset and alt_dpb_output_delay_offset are present in the bitstream. When ptsei_overwrite_flag is not present, it is inferred to be equal to 0.

When ptsei_overwrite_flag is equal to 1, cpb_removal_delay and dpb_output_delay in the picture timing SEI message in all access units, from the access unit associated with this buffering period SEI message to the next access unit with another buffering period SEI message, are changed using the values of alt_cpb_removal_delay_offset and alt_dpb_output_delay_offset.

num_entries_minus1 plus 1 specifies the number of alt_cpb_removal_delay_offset and alt_dpb_output_delay_offset in a buffering period SEI. When ptsei_overwrite_flag is equal to 0, it is inferred to be equal to 0.

alt_cpb_removal_delay_offset specifies the offset of cpb_removal_delay of an access unit. When ptsei_overwrite_flag is equal to 0, alt_cpb_removal_delay_offset[0] is inferred to be equal to 0.

alt_dpb_output_delay_offset specifies the offset of dpb_output_delay of an access unit. When ptsei_overwrite_flag is equal to 0, alt_dpb_output_delay_offset[0] is inferred to be equal to 0.

<Picture timing SEI message semantics>

If the current access unit is a BLA access unit, the variables CpbRemovalDelayCurr and DpbOutputDelayCurr are derived as follows:

Let PtseiOverwriteFlagPrev and PtseiOverwriteFlagCurr be the value of ptsei_overwrite_flag of the previous BLA access unit and the value of ptsei_overwrite_flag of the current access unit, respectively.

Let AltCpbRemovalDelayOffset[] be the array of alt_cpb_removal_delay_offset of the previous BLA access unit.

Let AltDpbOutputDelayOffset[] be the array of alt_dpb_output_delay_offset of the current access unit.

Let NumEntriesMinus1 be the value of num_entries_minus1 of the previous BLA access unit.

- If PtseiOverwriteFlagPrev is equal to 0, CpbRemovalDelayCurr is derived as:

CpbRemovalDelayCurr = cpb_removal_delay

- Otherwise (PtseiOverwriteFlagPrev is equal to 1), CpbRemovalDelayCurr is derived as:

CpbRemovalDelayCurr = cpb_removal_delay
 – AltCpbRemovalDelayOffset[ NumEntriesMinus1 ]

- If PtseiOverwriteFlagCurr is equal to 0, DpbOutputDelayCurr is derived as:

DpbOutputDelayCurr = dpb_output_delay

- Otherwise (PtseiOverwriteFlagCurr is equal to 1), DpbOutputDelayCurr is derived as:

DpbOutputDelayCurr = dpb_output_delay – AltDpbOutputDelayOffset[ 0 ]

Otherwise (the current access unit is not a BLA access unit), the variables CpbRemovalDelayCurr and DpbOutputDelayCurr are derived as follows:

Let PtseiOverwriteFlagPrev be the value of ptsei_overwrite_flag of the previous BLA access unit.

Let AltCpbRemovalDelayOffset[] be the array of alt_cpb_removal_delay_offset of the current access unit.

Let AltDpbOutputDelayOffset[] be the array of alt_dpb_output_delay_offset of the current access unit.

Let NumEntriesMinus1 be the value of num_entries_minus1 of the previous BLA access unit.

Let k be the number of access units between the previous BLA access unit and the current access unit, including the current access unit.

- If PtseiOverwriteFlagPrev is equal to 0, CpbRemovalDelayCurr and DpbOutputDelayCurr are derived as:

CpbRemovalDelayCurr = cpb_removal_delay

DpbOutputDelayCurr = dpb_output_delay

- Otherwise (PtseiOverwriteFlagPrev is equal to 1), CpbRemovalDelayCurr and DpbOutputDelayCurr are derived as:

CpbRemovalDelayCurr = cpb_removal_delay
 – AltCpbRemovalDelayOffset[ min( k, NumEntriesMinus1 ) ]

DpbOutputDelayCurr = dpb_output_delay

– AltDpbOutputDelayOffset[ min( k, NumEntriesMinus1 ) ]

In Annex C, CpbRemovalDelayCurr and DpbOutputDelayCurr are used instead of cpb_removal_delay and dpb_output_delay, respectively.

In the examples in Figure 1 and Figure 2, the values of num_entries_minus1, alt_cpb_removal_delay_offset[] and alt_dpb_output_delay_offset[] of picture I16 (BLA picture associated with a buffering period SEI) are set as follows:

Example in Figure 1:

num_entries_minus1 = 1

alt_cpb_removal_delay_offset[0] = 0

alt_dpb_output_delay_offset [0] = 7

alt_cpb_removal_delay_offset[1] = 7

alt_dpb_output_delay_offset [0] = 0

Example in Figure 2:

num_entries_minus1 = 2

alt_cpb_removal_delay_offset[0] = 0

alt_dpb_output_delay_offset [0] = 5

alt_cpb_removal_delay_offset[1] = 4

alt_dpb_output_delay_offset [1] = 1

alt_cpb_removal_delay_offset[2] = 5

alt_dpb_output_delay_offset [2] = 0

# Relaxation of constraint related to fixed_pic_rate_flag

## Problem statement

When the value of fixed_pic_rate_flag of HRD parameters is equal to 1, it is required that any picture in a bitstream shall satisfy the following equation.

Dto,dpb( n ) = to,dpb( nn ) − to,dpb( n ) = tc * ( pic_duration_in_tcs_minus1[ i ] + 1 )

When two bitstream with different sps_max_num_reoder_pics[] are concatenated, the last picture in output order of the first bitstream may not satisfy the above constraint.

Figure 3 – Example of bitstream editing when two bitstreams have different sps_max_num_reoder_pics

Figure 3 shows an example of bitstream editing in which two bitstreams before editing have different values of sps_max_num_reorder_pics. Specifically, sps_max_num_reorder_pics of bitstream BSB (i.e. 3) is larger than that of bitstream BSA (i.e. 2). In this case, Dto,dpb() of the last picture in output order in bitstream BSA (i.e. picture I32) is not equal to tc. This means that the edited bitstream does not conform to the HRD specification even though two bitstreams before editing do.

One possible way to avoid above situation is to insert an end_of_bitstream NAL at the end of coded picture B31 instead of an end_of_sequence NAL since the restriction on Dto,dpb() is only applied inside a bitstream. However, it cannot be expected that a decoder which decodes bitstream BSC outputs decoded pictures with minimum gap at a bitstream boundary since typical decoder resets its decoding process when at the bitstream boundary.

Other way is to change the values of dpb_output_timing of all following pictures. Apprantly this is the burden to the editing process.

In order to realize simple bitstream editing, the restriction of Dto,dpb() shall not be applied at a sequence boundary.

## Proposal

It is proposed to change the semantics of fixed_pic_rate_flag so that the restriction of Dto,dpb() is not applied to a picture whose following picture in output order belongs to different sequence.

The proposed change in fixed_pic_rate_flag is the following:

fixed_pic_rate_flag[ i ] equal to 1 indicates that, when HighestTid is equal to i, the temporal distance between the HRD output times of any two consecutive pictures in output order is constrained as follows. fixed_pic_rate_flag[ i ] equal to 0 indicates that no such constraints apply to the temporal distance between the HRD output times of any two consecutive pictures in output order.

When fixed_pic_rate_flag[ i ] is not present, it is inferred to be equal to 0.

When HighestTid is equal to i and, fixed_pic_rate_flag[ i ] is equal to 1 for a coded video sequence containing picture n and picture n in a coded video sequence is not the last picture of the coded video sequence in output order, the value computed for to,dpb( n ) as specified in Equation Error: Reference source not found shall be equal to tc * ( pic_duration_in_tcs_minus1[ i ] + 1 ), wherein tc is as specified in Equation Error: Reference source not found (using the value of tc for the coded video sequence containing picture n) when one or more of the following conditions are true for the following picture nn that is specified for use in Equation Error: Reference source not found:

–	picture nn is in the same coded video sequence as picture n.

–	picture nn is in a different coded video sequence and fixed_pic_rate_flag[ i ] is equal to 1 in the coded video sequence containing picture nn, the value of num_units_in_tick  time_scale is the same for both coded video sequences, and the value of pic_duration_in_tc_minus1[ i] is the same for both coded video sequences.

# SEI message for indicating editing point

An SEI message proposed in this section is basically the same as proposed in JCTVC-J0137. This SEI message indicates an editing point in a bitstream. If a bitstream is chopped up just before the coded picture with this SEI message, and is connected by another bitstream starting with a RAP picture, the resulting bitstream is assured to satisfy the constraint of fixed_pic_rate_flag equal to 1.

## Problem statement

When two bitstreams with the same picture size and the same fixed picture rate are to be concatenated, many applications would require that all coded pictures in the concatenated bitstream are decoded correctly and displayed continuously. But the continuous display of all coded pictures is depended on the last coded picture in the connected bitstream. Figure 4 shows the example of discontinuous display of pictures.

Figure 4 – Discontinued, interleaved picture output due to editing in improper position

In the example of Figure 4, the editing point in bitstream BSA is different from that in Figure 1. Specifically, the editing point is B29, and picture B29 and B31 are not included in bitstream BSC, even though those following picture in output order (i.e. picture I32) is included. In this case, discontinuity in output timing between pictures B28, B30 and I32 occurs. Moreover, if the values of cpb_removal_delay and dpb_output_delay of all pictures following B30 are the same as described in the example of the first proposal, overlap of output timings of two pictures (i.e. picture B17 and I32) and interleaving of output timings of pictures (i.e. picture B30, I32, I16 and B17) occur. One possible way to avoid this problem is to change the values of dpb_output_delay of picture B30 and I32 from 3 and 10 to 2 and 8, respectively. But this operation is the additional burden to an editing apparatus. Moreover, the capturing intervals of picture B28, B30 and I32 are not preserved in the output pictures.

Considering the above problem, it is required to put a constraint on a possible editing point in a bitstream in order to realize a simple bitstream editing without causing discontinuity in output timing. In the example of Figure 4, such editing points are B40, B24 and B32 in bitstream BSB.

It is not impossible for an editing process to find such editing point in a bitstream. However it is the additional burden to the editing process since the editing process needs to parse the bitstream to find the editing point.

## Proposal

It is proposed to introduce an editing point SEI message in order to indicate possible editing points. When the editing process concatenates a bitstream starting from a CRA picture to a bitstream terminated just before a picture associated with an editing point SEI message, all coded pictures in the generated bitstream are guaranteed to be output continuously with the possible exception at the boundary of a sequence. If the second proposal in this contribution is applied, the resulting bitstream satisfies the restriction on fixed_pic_rate_flag equal to 1.

The proposed syntax is the following.

| editing_point( payloadSize ) { | Descriptor |
| --- | --- |
| if( NalHrBpPresentFlag ¦¦ VclHrdBpPresentFlag ) |  |
| cpb_status_present_flag | u(1) |
| if( cpb_status_present_flag ) { |  |
| if( NalHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| cpb_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| if( VclHrdBpPresentFlag ) { |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| cpb_offset[ SchedSelIdx ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The proposed semantics is the following.

The editing point SEI message assists the process of bitstream editing in determining which access unit in a bitstream is suitable as an editing point. When the picture rate of the bitstream is constant (i.e. fixed_pic_rate_flag is equal to 1, and the constraint on to,dpb in Equation C-17 is satisfied at every picture of the bitstream), and is cut into two sub-bitstreams just before an access unit with the editing point SEI, the picture rate of the former sub-bitstream is also constant.

NOTE: When sps_max_num_reorder_pics[ 0 ] of the bitstream is not equal to 0 and the bitstream is cut into two sub-bitstream s at an access unit without the editing point SEI message, the former sub-bitstream may contain a coded picture which does not satisfy the constraint on to,dpb in Equation C-17.

When an access unit is associated with an editing point SEI message, the output timing of the access unit shall not be earlier than any access units preceding the access unit.

cpb_status_present_flag equal to 1 specifies the presence of the cpb_offset syntax element.

cpb_offset specifies the position of the SchedSelIdx-th CPB at the moment when the access unit immediately preceding an access unit associated with the editing point SEI message is removed from the CPB.

NOTE: The process of bitstream editing may refer to cpb_offset when the bitstream, whose last access unit is associated with the editing point SEI message, is connected by another bitstream. In order the generated bitstream to conform to the HRD specification, cpb_removal_delay of the first access unit in the latter bitstream shall be modified properly so that no CPB overflow or underflow occurs at or after the editing point. cpb_offset assists the process of deriving the proper value of cpb_removal_delay in determining the position of the CPB buffer after decoding the last access unit.

The position of the SchedSelIdx-th CPB in unit of bit is calculated by Ceil( cpb_offset[ SchedSelIdx ]  90,000 * bit_rate[ SchedSelIdx ] ).

The syntax elements have a length in bits given by initial_cpb_removal_delay_length_minus1 + 1, and are in units of a 90 kHz clock.

# References

[1] "High Efficiency Video Coding (HEVC) text specification draft 8", JCTVC-J1003, July 2012, Stockholm Sweden

[2] K.Kazui et al., "AHG9: New high-level syntax for simple HEVC stream editing", JCTVC-J0137, July 2012, Stockholm Sweden

[3] "Proposed Editorial Improvements for High efficiency video coding (HEVC) Text Specification Draft 8, JCTVC-K0030, October 2012, Shanghai China

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

In this case BSC is regarded as an elementary stream, rather than a bitstream.