| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0141 |
| --- | --- |

| Title: | AHG9: Low delay display hint SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution presents the low delay display hint SEI message which assists a decoder in determining the possible earliest V-sync timing for outputting decoded pictures in the ultra-low delay mode. This contribution is the follow-up proposal of JCTVC-J0136 at the 10th JCT-VC meeting.

# Introduction

In JCTVC-J0136 [2] at the 10th JCT-VC meeting it was proposed to enhance the specification regarding the sub-picture based CPB operation. The second item in JCTVC-J0136 is a low delay display hint SEI message, which indicates the possible V-sync timing for outputting decoded pictures. It was concluded as “further stufy” because the priority of the discussion on new SEI messages was decided to be lower than other version 1 related issues.

This contribution presents an SEI message which assists a decoder in determining the possible V-sync timing for outputting decoded pictures in the ultra-low delay mode This is the follow-up proposal of JCTVC-J0136 "AHG9: Improvement of HRD for sub-picture based operation" at the 10th JCT-VC meeting.

# SEI as a hint of early displaying of a decoded picture

## Problem statement

In order to realize interoperable ultra-low delay video transmission, HEVC has introduced the concept of “decoding unit” in the HRD specification. A decoding unit is a part of an access unit and consists of a number of NAL units. The CPB removal time of each decoding unit, which is earlier than that of the access unit containing the decoding unit, is derived from the parameters in the picture timing SEI message. When a decoder starts the decoding process of each decoding unit at its CPB removal time instead of at the CPB removal time of the access unit, the decoder can output each line of the reconstructed picture earlier because typical decoder needs one picture interval to decode the access unit.

In order to minimize the displaying delay, as well as the decoding delay, the decoder has to align the timing of outputting each line in the reconstructed picture to the timing when the line becomes reconstructed. The detail is explained in the following.

Figure 1 – Decoder timing model

Figure 1 shows the timing model of a decoder. In this timing model, the input bitstream has the CPB removal time of decoding unit n tr, n(n). The video output signal has its vertical synchronization timing V-sync(B). In addition to those two timing, the output from the decoding process conceptually has the vertical synchronization timing V-sync(A). The timing of V-sync(A) is such earliest timing in which each line in the picture is assured to be reconstructed before its scanning for output.

Figure 2 – Timing of tr,n, V-sync(A) and V-sync(B)

Figure 2 show the timing of tr,n, V-sync(A) and V-sync(B). The delay between tr,n() of the access unit (m-1) and the V-sync(A) of picture m is referred as to Δt. When the timings of V-sync(A) and V-sync(B) are identical as in case 1, the display delay becomes minimum. When the timing of V-sync(B) is slightly earlier than that of V-sync(A) as in case 2, the display delay becomes maximum, which is equal to the picture interval tc. Therefore, the decoder has to align V-sync(B) to V-sync(A) in order to minimize the display delay.

The problem is that it is not easy to for the decoder to determine the proper value of Δt, since it is highly dependent to the structure of decoding units in the access unit.

Figure 3 – The relationship between decoding unit structure and Δt

Figure 3 shows example of the dependency of Δt on the structure of decoding units. Both of two cases in this example have the same number of decoding units (i.e. 4) and each decoding unit is supposed to contain the same number of CTBs and bits.

In case 1, there is only one tile in a picture and each decoding unit do not share the same line with other decoding unit. In this case, the delay between the output and the decoding of each line is almost constant. Thus Δt can be set to (Δtr,n + C), where Δtr,n is the interval of the arrival time of each decoding unit and C is the interval corresponding to one CTB line.

On the contrary, in case 2, there are two vertical tiles and two decoding units share the same line. In this case, the output timing of each line has to be later than the decoding time of the 3rd decoding unit. Thus Δt has to be (3*Δtr,n + C/2).

Since the timing of V-sync(A) has to be determined before outputting the first line in the first decoded picture, the decoder has to set the value of Δt before parsing VCL NAL units in the first decoding unit in the bitstream. But it is hard to derive the proper value of Δt without parsing the whole decoding units in the first access unit.

## Proposal

It is proposed to introduce the low delay display hint SEI message which explicitly signals the value of Δt in order to minimize the displaying delay.

The proposed syntax of the low delay display hint SEI is the following.

| low_delay_display_hint( payloadSize ) { | Descriptor |
| --- | --- |
| display_delay_hint | u(v) |
| decoding_delay_hint_present_flag | u(1) |
| if (decoding_delay_hint_present_flag ) |  |
| decoding_delay_hint | u(v) |
| } |  |

The proposed semantics of the low delay display hint SEI is the following.

The low delay display hint SEI message assists a decoder in determining the V-sync timing for outputting decoded pictures. The low delay display hint SEI message signals the possible earliest time relative to the arrival time of the first bit of a bitstream to the CPB buffer, when a decoder can start outputting the first line of the first decoded picture to an external display device, with the following assumption on a decoder.

- The display order of a pixel is progressive raster scan.

- The decoding interval of each coding tree block is constant, and the decoding time of a whole picture is equal to the output interval of the picture.

If present, the low delay display hint SEI message shall be associated to all access units associated with the buffering period SEI message.

NOTE: It is recommended that a decoder takes the architecture of the decoder into consideration for determining the V-sync timing for outputting decoded pictures. For example, the V-sync timing can be set earlier than that is signalled in the bitstream when the decoding interval of a coded tree block is shorter than the assumption above.

display_delay_hint indicates the possible earliest time when a decoder can start displaying decoded pixels without any loss of pixels through the display process, supposing that the display order is progressive raster scan. The syntax element is a fixed length code whose length in bits is given by cpb_removal_delay_length_minus1 + 1.

The possible earliest time tearliest_display, relative to the arrival time of the first bit of a bitstream to the CPB buffer, is derived by the following equation.

tearliest_display = display_delay_hint  90 000

decoding_delay_hint_present_flag equal to 1 signals that decoding_delay_hint is present in the bitstream.

decoding_delay_hint indicates the possible earliest time when a decoder can start the decoding process of access units in the bitstream without causing an underflow or an overflow of the CPB, assuming that the decoder decodes one access unit in one picture period. The syntax element is a fixed length code whose length in bits is given by cpb_removal_delay_length_minus1 + 1

The possible earliest time tearliest_decode is derived by the following equation.

tearliest_decode = decoding_delay_hint  90 000

# References

[1] "High Efficiency Video Coding (HEVC) text specification draft 8", JCTVC-J1003, July 2012, Stockholm Sweden

[2] K.Kazui et al., "AHG9: Improvement of HRD for sub-picture based operation", JCTVC-J0136, July 2012, Stockholm Sweden

[3] Proposed Editorial Improvements for High efficiency video coding (HEVC) Text Specification Draft 8, JCTVC-K0030, October 2012, Shanghai China

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).