| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0142 |
| --- | --- |

| Title: | AHG9: Modification of SEIs specified in AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

- Abstract

In this contribution it is proposed to modify the definitions of several SEIs in HEVC draft specification text version 8, whose syntax and semantics are specified in the AVC specification, in order to adapt those SEIs to the HEVC specification.

Introduction

In draft specification text version 8[3], the syntax and semantics of the following SEIs are the same as specified in the AVC specification [1][2].

- Pan-scan rectangle SEI (*)

- Filler payload SEI

- User data registered by Rec. ITU-T T.35 SEI

- User data unregistered SEI

- Scene information SEI

- Full-frame snapshot SEI

- Progressive refinement segment start SEI (*)

- Progressive refinement segment end SEI

- Film grain characteristics SEI

- Deblocking filter display preference SEI (*)

- Post-filter hint SEI

- Frame packing arrangement SEI (*)

In the definition of those SEIs associated with (*), the concepts such as "frame_num" and "MB", which are not relevant to the HEVC specification, are used. Therefore it is not appropriate to specify those SEIs by just referring to the AVC specification.

In this contribution it is proposed to modify those SEIs, whose syntax and semantics are specified in the AVC specification, in order to adapt those SEIs to the HEVC specification.

Proposal

In this section, proposed modification on pan-scan rectangle SEI, progressive refinement segment start SEI, deblocking filter display preference SEI and frame packing arrangement SEI is described. The changes in those SEIs from the AVC specification are recorded in this document.

Clause 3: Definition

Problem

The terms relating to interlaced video, such as "field" and "frame", are used in the semantic part of some SEIs. However, those terms are not defined in the clause 3 "Definitions".

Proposed changes

Insert in subclause 3:

field: An assembly of alternate rows of a frame. A frame is composed of two fields, a top field and a bottom field.

NOTE – field is not referred in the decoding process.

frame: A frame consists of two fields, a top field and a bottom field.

NOTE – frame is not referred in the decoding process.

In the end of the definition of "picture", add:

NOTE – When both field_pic_flag and progressive_source_flag are equal to 0, a picture is a frame consisting of two fields, a top field and a bottom field. When field_pic_flag is equal to 0 and progressive_source_flag is equal to 1, a picture is a progressive frame. When field_pic_flag is equal to 1, a picture consists of one field, either a top field or a bottom field.

Pan-scan rectangle SEI

Problem

The terms "field" and "frame", which are not used in the decoding process, are used in the semantics of pan_scan_cnt_minus1. It shall be linked with the syntax elements specified in field indication SEI.

The variable PicWidthInMbs (and its specific number '16') and frame_crop_yyy_offset (where yyy is either "left", "right", "top" or "bottom"), used in the semantics of pan_scan_rect_left_offset, pan_scan_rect_right_offset, pan_scan_rect_top_offset and pan_scan_rect_bottom_offset, are not defined in HEVC.

Proposed change

Replace D.1.3 by

| pan_scan_rect( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| pan_scan_rect_id | 5 | ue(v) |
| pan_scan_rect_cancel_flag | 5 | u(1) |
| if( !pan_scan_rect_cancel_flag ) { |  |  |
| pan_scan_cnt_minus1 | 5 | ue(v) |
| for( i = 0; i <= pan_scan_cnt_minus1; i++ ) { |  |  |
| pan_scan_rect_left_offset[ i ] | 5 | se(v) |
| pan_scan_rect_right_offset[ i ] | 5 | se(v) |
| pan_scan_rect_top_offset[ i ] | 5 | se(v) |
| pan_scan_rect_bottom_offset[ i ] | 5 | se(v) |
| } |  |  |
| pan_scan_rect_repetition_period | 5 | ue(v) |
| } |  |  |
| } |  |  |

Replace D.2.3 by:

The pan-scan rectangle SEI message syntax elements specify the coordinates of a rectangle relative to the cropping rectangle of the sequence parameter set. Each coordinate of this rectangle is specified in units of one-sixteenth sample spacing relative to the luma sampling grid.

pan_scan_rect_id contains an identifying number that may be used to identify the purpose of the pan-scan rectangle (for example, to identify the rectangle as the area to be shown on a particular display device or as the area that contains a particular actor in the scene). The value of pan_scan_rect_id shall be in the range of 0 to 232 − 2, inclusive.

Values of pan_scan_rect_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of pan_scan_rect_id from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of pan_scan_rect_id in the range of 256 to 511 or in the range of 231 to 232 − 2 shall ignore (remove from the bitstream and discard) it.

pan_scan_rect_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous pan-scan rectangle SEI message in output order. pan_scan_rect_cancel_flag equal to 0 indicates that pan-scan rectangle information follows.

pan_scan_cnt_minus1 specifies the number of pan-scan rectangles that are present in the SEI message. pan_scan_cnt_minus1 shall be in the range of 0 to 2, inclusive. pan_scan_cnt_minus1 equal to 0 indicates that a single pan-scan rectangle is present that applies to the decoded picture. If the decoding picture consists of two fields (i.e. field_pic_flag is equal to 0 and progressive_source_flag is equal to 0), the single pan-scan rectangle applies to two fields. pan_scan_cnt_minus1 shall be equal to 0 when the current picture is a field (i.e. field_pic_flag is equal to 1) or a progressive frame (i.e field_pic_flag is equal to 0 and progressive_source_flag is equal to 1). pan_scan_cnt_minus1 equal to 1 indicates that two pan-scan rectangles are present, the first of which applies to the first field of the picture in output order and the second of which applies to the second field of the picture in output order. pan_scan_cnt_minus1 equal to 2 indicates that three pan-scan rectangles are present, the first of which applies to the first field of the picture in output order, the second of which applies to the second field of the picture in output order, and the third of which applies to a repetition of the first field as a third field in output order.

pan_scan_rect_left_offset[ i ], pan_scan_rect_right_offset[ i ], pan_scan_rect_top_offset[ i ], and pan_scan_rect_bottom_offset[ i ], specify, as signed integer quantities in units of one-sixteenth sample spacing relative to the luma sampling grid, the location of the pan-scan rectangle. The values of each of these four syntax elements shall be in the range of −231 + 1 to 231 − 1, inclusive.

The pan-scan rectangle is specified, in units of one-sixteenth sample spacing relative to a luma frame sampling grid, as the region with frame horizontal coordinates from 16*CropUnitX * pic_crop_left_offset + pan_scan_rect_left_offset[ i ] to 16 * ( CtbSizeY * PicWidthInCtbsY − CropUnitX * pic_crop_right_offset ) + pan_scan_rect_right_offset[ i ] − 1 and with vertical coordinates from 16 *CropUnitY * pic_crop_top_offset + pan_scan_rect_top_offset[ i ] to 16 * ( CtbSizeY * PicHeightInCtbsY − CropUnitY * pic_crop_bottom_offset ) + pan_scan_rect_bottom_offset[ i ] − 1, inclusive. The value of 16 * CropUnitX * pic_crop_left_offset + pan_scan_rect_left_offset[ i ] shall be less than or equal to 16 * ( CtbSizeY * PicWidthInCtbsY − CropUnitX * pic_crop_right_offset ) + pan_scan_rect_right_offset[ i ] − 1; and the value of 16 * CropUnitY * pic_crop_top_offset + pan_scan_rect_top_offset[ i ] shall be less than or equal to 16 * ( CtbSizeY * PicHeightInCtbsY − CropUnitY * pic_crop_bottom_offset ) + pan_scan_rect_bottom_offset[ i ] − 1.

When the pan-scan rectangular area includes samples outside of the cropping rectangle, the region outside of the cropping rectangle may be filled with synthesized content (such as black video content or neutral grey video content) for display.

pan_scan_rect_repetition_period specifies the persistence of the pan-scan rectangle SEI message and may specify a picture order count interval within which another pan-scan rectangle SEI message with the same value of pan_scan_rect_id or the end of the coded video sequence shall be present in the bitstream. The value of pan_scan_rect_repetition_period shall be in the range of 0 to 16 384, inclusive. When pan_scan_cnt_minus1 is greater than 0, pan_scan_rect_repetition_period shall not be greater than 1.

pan_scan_rect_repetition_period equal to 0 specifies that the pan-scan rectangle information applies to the current decoded picture only.

pan_scan_rect_repetition_period equal to 1 specifies that the pan-scan rectangle information persists in output order until any of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing a pan-scan rectangle SEI message with the same value of pan_scan_rect_id is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ).

pan_scan_rect_repetition_period equal to 0 or equal to 1 indicates that another pan-scan rectangle SEI message with the same value of pan_scan_rect_id may or may not be present.

pan_scan_rect_repetition_period greater than 1 specifies that the pan-scan rectangle information persists until any of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing a pan-scan rectangle SEI message with the same value of pan_scan_rect_id is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + pan_scan_rect_repetition_period.

pan_scan_rect_repetition_period greater than 1 indicates that another pan-scan rectangle SEI message with the same value of pan_scan_rect_id shall be present for a picture in an access unit that is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + pan_scan_rect_repetition_period; unless the bitstream ends or a new coded video sequence begins without output of such a picture.

Progressive refinement segment start SEI

Problem

The variables relating to frame_num, which is not used in HEVC, are used in the semantics of progressive refinement segment start SEI.

Proposed change

Replace D.1.10 by:

| progressive_refinement_segment_start( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| progressive_refinement_id | 5 | ue(v) |
| num_refinement_steps_minus1 | 5 | ue(v) |
| } |  |  |

Replace D.2.10 by:

The progressive refinement segment start SEI message specifies the beginning of a set of consecutive coded pictures that is labelled as the current picture followed by a sequence of one or more pictures of refinement of the quality of the current picture, rather than as a representation of a continually moving scene.

The tagged set of consecutive coded pictures shall continue until one of the following conditions is true. When a condition below becomes true, the next slice to be decoded does not belong to the tagged set of consecutive coded pictures:

–	The next slice to be decoded belongs to an IDR picture.

–	num_refinement_steps_minus1 is greater than 0 and the pic_order_cnt_lsb of the next slice to be decoded is (currPicOrderCntLsb + num_refinement_steps_minus1 + 1) % MaxPicOrderCntLsb, where currPicOrderCntLsb is the value of pic_order_cnt_lsb of the picture in the access unit containing the SEI message.

–	num_refinement_steps_minus1 is 0 and a progressive refinement segment end SEI message with the same progressive_refinement_id as the one in this SEI message is decoded.

The decoding order of pictures within the tagged set of consecutive pictures should be the same as their output order.

progressive_refinement_id specifies an identification number for the progressive refinement operation. progressive_refinement_id shall be in the range of 0 to 232 − 2, inclusive.

Values of progressive_refinement_id in the range of 0 to 255, inclusive, and in the range of 512 to 231 − 1, inclusive, may be used as determined by the application. Values of progressive_refinement_id in the range of 256 to 511, inclusive, and in the range of 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of progressive_refinement_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore (remove from the bitstream and discard) it.

num_refinement_steps_minus1 specifies the number of reference frames in the tagged set of consecutive coded pictures as follows:

–	If num_refinement_steps_minus1 is equal to 0, the number of reference frames in the tagged set of consecutive coded pictures is unknown.

–	Otherwise, the number of reference frames in the tagged set of consecutive coded pictures is equal to num_refinement_steps_minus1 + 1.

num_refinement_steps_minus1 shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

Deblocking filter display preference SEI

Problem

Unlike in AVC, sample adaptive offset process is performed after deblocking filter process. Those two filter process should be considered jointly for display preference.

Subclauses referred in the semantics of deblocking filter display preference SEI are not appropriate.

The name of syntax corresponding to "max_dec_frame_buffering" is changed in the HEVC specification.

Proposed change

Replace D.1 with:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 6 ) |  |
| recovery_point( payloadSize ) |  |
| else if( payloadType = = 9 ) |  |
| scene_info( payloadSize ) |  |
| else if( payloadType = = 15 ) |  |
| full_frame_snapshot( payloadSize ) |  |
| else if( payloadType = = 16 ) |  |
| progressive_refinement_segment_start( payloadSize ) |  |
| else if( payloadType = = 17 ) |  |
| progressive_refinement_segment_end( payloadSize ) |  |
| else if( payloadType = = 19 ) |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 20 ) |  |
| inloop_filter_display_preference( payloadSize ) |  |
| else if( payloadType = = 22 ) |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 23 ) |  |
| tone_mapping_info( payloadSize ) |  |
| else if( payloadType = = 45 ) |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) [Ed. (GJS): Check numbering w.r.t. AVC.] |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 128 ) |  |
| sop_description( payloadSize ) |  |
| else if( payloadType = = 129 ) |  |
| field_indication( payloadSize ) |  |
| else if( payloadType = = 130) |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 131) |  |
| active_parameter_sets( payloadSize ) |  |
| else if( payloadType = = 132 ) |  |
| sub_pic_timing( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

Replace the title and content of D.1.13 by:

D.1.13	Inloop filter display preference SEI message syntax

| inloop_filter_display_preference( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| inloop_display_preference_cancel_flag | 5 | u(1) |
| if( !inloop_display_preference_cancel_flag ) { |  |  |
| display_prior_to_inloop_preferred_flag | 5 | u(1) |
| dec_pic_buffering_constraint_flag | 5 | u(1) |
| inloop_display_preference_repetition_period | 5 | ue(v) |
| } |  |  |
| } |  |  |

Replace the title and content of D.2.13 by:

D.2.13	Inloop filter display preference SEI message semantics

This SEI message provides the decoder with an indication of whether the display of the cropped result of the in-loop filter process specified in subclause 8.7 or of the cropped result of the picture construction process prior to the in-loop process specified in subclause 8.6.5 is preferred by the encoder for the display of each decoded picture that is output.

NOTE 1 – The display process is not specified in this Recommendation | International Standard. The means by which an encoder determines what to indicate as its preference expressed in an inloop filter display preference SEI message is also not specified in this Recommendation | International Standard, and the expression of an expressed preference in an inloop filter display preference SEI message does not impose any requirement on the display process.

inloop_display_preference_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous inloop filter display preference SEI message in output order. inloop_display_preference_cancel_flag equal to 0 indicates that a display_prior_to_inloop_preferred_flag and inloop_display_preference_repetition_period follow.

NOTE 2 – In the absence of the inloop filter display preference SEI message, or after the receipt of an inloop filter display preference SEI message in which inloop_display_preference_cancel_flag is equal to 1, the decoder should infer that the display of the cropped result of the in-loop filter process specified in subclause 8.7 is preferred over the display of the cropped result of the picture construction process prior to the in-loop filter process specified in subclause 8.6.5 for the display of each decoded picture that is output.

inloop_prior_to_deblocking_preferred_flag equal to 1 indicates that the encoder preference is for the display process (which is not specified in this Recommendation | International Standard) to display the cropped result of the picture construction process prior to the in-loop filter process specified in subclause 8.6.5 rather than the cropped result of the in-loop filter process specified in subclause 8.7 for each picture that is cropped and output as specified in Annex C. display_prior_to_inloop_preferred_flag equal to 0 indicates that the encoder preference is for the display process (which is not specified in this Recommendation | International Standard) to display the cropped result of the in-loop filter process specified in subclause 8.7 rather than the cropped result of the picture construction process prior to the in-loop filter process specified in subclause 8.6.5 for each picture that is cropped and output as specified in Annex C.

NOTE 3 – The presence or absence of the inloop filter display preference SEI message and the value of display_prior_to_inloop_preferred_flag does not affect the requirements of the decoding process specified in this Recommendation | International Standard. Rather, it only provides an indication of when, in addition to fulfilling the requirements of this Recommendation | International Standard for the decoding process, enhanced visual quality might be obtained by performing the display process (which is not specified in this Recommendation | International Standard) in an alternative fashion. Encoders that use the inloop filter display preference SEI message should be designed with an awareness that unless the encoder restricts its use of the DPB capacity specified in Annex A for the profile and level in use, some decoders may not have sufficient memory capacity for the storage of the result of the picture construction process prior to the inloop filter process specified in subclause 8.6.5 in addition to the storage of the result of the in-loop filter process specified in subclause 8.7 when reordering and delaying pictures for display, and such decoders would therefore not be able to benefit from the preference indication. By restricting its use of the DPB capacity, an encoder can be able to use at least half of the DPB capacity specified in Annex A while allowing the decoder to use the remaining capacity for storage of unfiltered pictures that have been indicated as preferable for display until the output time arrives for those pictures.

dec_pic_buffering_constraint_flag equal to 1 indicates that the use of the picture buffering capacity of the HRD decoded picture buffer (DPB) as specified by sps_max_dec_pic_buffering[ 0 ] has been constrained such that the coded video sequence will not require a decoded picture buffer with more than Max( 1, sps_max_dec_pic_buffering[ 0 ] ) picture buffers to enable the output of the decoded filtered or unfiltered pictures, as indicated by the inloop filter display preference SEI messages, at the output times specified by the dpb_output_delay of the picture timing SEI messages. dec_pic_buffering_constraint_flag equal to 0 indicates that the use of the picture buffering capacity in the HRD may or may not be constrained in the manner that would be indicated by dec_pic_buffering_constraint_flag equal to 1.

For purposes of determining the constraint imposed when dec_pic_buffering_constraint_flag is equal to 1, the quantity of picture buffering capacity used at any given point in time by each picture buffer of the DPB that contains a picture shall be derived as follows:

–	If both of the following criteria are satisfied for the picture buffer, the picture buffer is considered to use two picture buffers of capacity for its storage.

–	The picture buffer contains a picure that is marked as "used for reference", and

–	The picture buffer contains a picture for which both of the following criteria are fulfilled:

–	The HRD output time of the picture is greater than the given point in time.

–	It has been indicated in an inloop filter display preference SEI message that the encoder preference for the picture is for the display process to display the cropped result of the picture construction process prior to the in-loop filter process specified in subclause 8.6.5 rather than the cropped result of the in-loop filter process specified in subclause 8.7.

–	Otherwise, the picture buffer is considered to use one picture buffer of DPB capacity for its storage.

When dec_pic_buffering_constraint_flag is equal to 1, the picture buffering capacity used by all of the picture buffers in the DPB that contain pictures, as derived in this manner, shall not be greater than Max( 1, sps_max_dec_pic_buffering[ 0 ] ) during the operation of the HRD for the coded video sequence.

The value of dec_pic_buffering_constraint_flag shall be the same in all inloop filter display preference SEI messages of the coded video sequence.

inloop_display_preference_repetition_period specifies the persistence of the inloop filter display preference SEI message and may specify a picture order count interval within which another inloop filter display preference message or the end of the coded video sequence shall be present in the bitstream. The value of inloop_display_preference_repetition_period shall be in the range 0 to 16 384, inclusive.

inloop_display_preference_repetition_period equal to 0 specifies that the inloop filter display preference SEI message applies to the current decoded picture only.

inloop_display_preference_repetition_period equal to 1 specifies that the inloop filter display preference SEI message persists in output order until any of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing an inloop filter display preference SEI message is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ).

inloop_display_preference_repetition_period greater than 1 specifies that the inloop filter display preference SEI message persists until any of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing an inloop filter display preference SEI message is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + inloop_display_preference_repetition_period.

inloop_display_preference_repetition_period greater than 1 indicates that another inloop filter display preference SEI message shall be present for a picture in an access unit that is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + inloop_display_preference_repetition_period; unless the bitstream ends or a new coded video sequence begins without output of such a picture.

Frame packing arrangement SEI

Problem

There are AVC-specific terms for interlaced tools.

Proposed change

Replace D.1.16 by:

| frame_packing_arrangement( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| frame_packing_arrangement_id | 5 | ue(v) |
| frame_packing_arrangement_cancel_flag | 5 | u(1) |
| if( !frame_packing_arrangement_cancel_flag ) { |  |  |
| frame_packing_arrangement_type | 5 | u(7) |
| quincunx_sampling_flag | 5 | u(1) |
| content_interpretation_type | 5 | u(6) |
| spatial_flipping_flag | 5 | u(1) |
| frame0_flipped_flag | 5 | u(1) |
| field_views_flag | 5 | u(1) |
| current_frame_is_frame0_flag | 5 | u(1) |
| frame0_self_contained_flag | 5 | u(1) |
| frame1_self_contained_flag | 5 | u(1) |
| if( !quincunx_sampling_flag && frame_packing_arrangement_type != 5 ) { |  |  |
| frame0_grid_position_x | 5 | u(4) |
| frame0_grid_position_y | 5 | u(4) |
| frame1_grid_position_x | 5 | u(4) |
| frame1_grid_position_y | 5 | u(4) |
| } |  |  |
| frame_packing_arrangement_reserved_byte | 5 | u(8) |
| frame_packing_arrangement_repetition_period | 5 | ue(v) |
| } |  |  |
| frame_packing_arrangement_extension_flag | 5 | u(1) |
| } |  |  |

Replace D.2.16 by:

This SEI message informs the decoder that the output cropped decoded picture contains samples of multiple distinct spatially packed constituent frames that are packed into one frame using an indicated frame packing arrangement scheme. This information can be used by the decoder to appropriately rearrange the samples and process the samples of the constituent frames appropriately for display or other purposes (which are outside the scope of this Specification).

This SEI message may be associated with pictures that are either frames (i.e. field_pic_flag is equal to 0) or fields (i.e. field_pic_flag is equal to 1). The frame packing arrangement of the samples is specified in terms of the sampling structure of a frame in order to define a frame packing arrangement structure that is invariant with respect to whether a picture is a single field of such a packed frame or is a complete packed frame.

frame_packing_arrangement_id contains an identifying number that may be used to identify the usage of the frame packing arrangement SEI message. The value of frame_packing_arrangement_id shall be in the range of 0 to 232 − 2, inclusive.

Values of frame_packing_arrangement_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of frame_packing_arrangement_id from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore (remove from the bitstream and discard) all frame packing arrangement SEI messages containing a value of frame_packing_arrangement_id in the range of 256 to 511 or in the range of 231 to 232 − 2, and bitstreams shall not contain such values.

frame_packing_arrangement_cancel_flag equal to 1 indicates that the frame packing arrangement SEI message cancels the persistence of any previous frame packing arrangement SEI message in output order. frame_packing_arrangement_cancel_flag equal to 0 indicates that frame packing arrangement information follows.

frame_packing_arrangement_type indicates the type of packing arrangement of the frames as specified in Table D-8.

Table D-8 – Definition of frame_packing_arrangement_type

| Value | Interpretation |
| --- | --- |
| 0 | Each component plane of the decoded frames contains a "checkerboard" based interleaving of corresponding planes of two constituent frames as illustrated in Figure D-1. |
| 1 | Each component plane of the decoded frames contains a column based interleaving of corresponding planes of two constituent frames as illustrated in Figure D-2 and Figure D-3. |
| 2 | Each component plane of the decoded frames contains a row based interleaving of corresponding planes of two constituent frames as illustrated in Figure D-4 and Figure D-5. |
| 3 | Each component plane of the decoded frames contains a side-by-side packing arrangement of corresponding planes of two constituent frames as illustrated in Figure D-6, Figure D-7, and Figure D-10. |
| 4 | Each component plane of the decoded frames contains a top-bottom packing arrangement of corresponding planes of two constituent frames as illustrated in Figure D-8 and Figure D-9. |
| 5 | The component planes of the decoded frames in output order form a temporal interleaving of alternating first and second constituent frames as illustrated in Figure D-11. |

NOTE 1 – Figure D-1 to Figure D-10 provide typical examples of rearrangement and upconversion processing for various packing arrangement schemes. Actual characteristics of the constituent frames are signalled in detail by the subsequent syntax elements of the frame packing arrangement SEI message. In Figure D-1 to Figure D-10, an upconversion processing is performed on each constituent frame to produce frames having the same resolution as that of the decoded frame. An example of the upsampling method to be applied to a quincunx sampled frame as shown in Figure D-1 or Figure D-10 is to fill in missing positions with an average of the available spatially neighbouring samples (the average of the values of the available samples above, below, to the left and to the right of each sample to be generated). The actual upconversion process to be performed, if any, is outside the scope of this Specification.

NOTE 2 – The sample aspect ratio (SAR) indicated in the VUI parameters should indicate the output picture shape for the packed decoded frame output by a decoder that does not interpret the frame packing arrangement SEI message. In the examples shown in Figure D-1 to Figure D-10, the SAR produced in each upconverted colour plane would be the same as the SAR indicated in the VUI parameters, since the illustrated upconversion process produces the same total number of samples from each constituent frame as existed in the packed decoded frame.

NOTE 3 – When the output time of the samples of constituent frame 0 differs from the output time of the samples of constituent frame 1 (i.e., when field_views_flag is equal to 1 or frame_packing_arrangement_type is equal to 5) and the display system in use presents two views simultaneously, the display time for constituent frame 0 should be delayed to coincide with the display time for constituent frame 1. (The display process is not specified in this Recommendation | International Standard.)

NOTE 4 – When field_views_flag is equal to 1 or frame_packing_arrangement_type is equal to 5, the value 0 for fixed_frame_rate_flag is not expected to be prevalent in industry use of this SEI message.

NOTE 5 – frame_packing_arrangement_type equal to 5 describes a temporal interleaving process of different views.

All other values of frame_packing_arrangement_type are reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that the bitstreams shall not contain such other values of frame_packing_arrangement_type.

quincunx_sampling_flag equal to 1 indicates that each colour component plane of each constituent frame is quincunx sampled as illustrated in Figure D-1 or Figure D-10, and quincunx_sampling_flag equal to 0 indicates that the colour component planes of each constituent frame are not quincunx sampled.

When frame_packing_arrangement_type is equal to 0, it is a requirement of bitstream conformance that quincunx_sampling_flag shall be equal to 1. When frame_packing_arrangement_type is equal to 5, it is a requirement of bitstream conformance that quincunx_sampling_flag shall be equal to 0.

NOTE 6 – For any chroma format (4:2:0, 4:2:2, or 4:4:4), the luma plane and each chroma plane is quincunx sampled as illustrated in Figure D-1 when quincunx_sampling_flag is equal to 1.

content_interpretation_type indicates the intended interpretation of the constituent frames as specified in -Table D-1. Values of content_interpretation_type that do not appear in -Table D-1 are reserved for future specification by ITU-T | ISO/IEC.

For each specified frame packing arrangement scheme, there are two constituent frames that are referred to as frame 0 and frame 1.

Table D-1 – Definition of content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Unspecified relationship between the frame packed constituent frames |
| 1 | Indicates that the two constituent frames form the left and right views of a stereo view scene, with frame 0 being associated with the left view and frame 1 being associated with the right view |
| 2 | Indicates that the two constituent frames form the right and left views of a stereo view scene, with frame 0 being associated with the right view and frame 1 being associated with the left view |

NOTE 7 – The value 2 for content_interpretation_type is not expected to be prevalent in industry use of this SEI message. However, the value was specified herein for purposes of completeness.

spatial_flipping_flag equal to 1, when frame_packing_arrangement_type is equal to 3 or 4, indicates that one of the two constituent frames is spatially flipped relative to its intended orientation for display or other such purposes.

When frame_packing_arrangement_type is equal to 3 or 4 and spatial_flipping_flag is equal to 1, the type of spatial flipping that is indicated is as follows:

–	If frame_packing_arrangement_type is equal to 3, the indicated spatial flipping is horizontal flipping.

–	Otherwise (frame_packing_arrangement_type is equal to 4), the indicated spatial flipping is vertical flipping.

When frame_packing_arrangement_type is not equal to 3 or 4, it is a requirement of bitstream conformance that spatial_flipping_flag shall be equal to 0. When frame_packing_arrangement_type is not equal to 3 or 4, the value 1 for spatial_flipping_flag is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is not equal to 3 or 4, decoders shall ignore the value 1 for spatial_flipping_flag.

frame0_flipped_flag, when spatial_flipping_flag is equal to 1, indicates which one of the two constituent frames is flipped.

When spatial_flipping_flag is equal to 1, frame0_flipped_flag equal to 0 indicates that frame 0 is not spatially flipped and frame 1 is spatially flipped, and frame0_flipped_flag equal to 1 indicates that frame 0 is spatially flipped and frame 1 is not spatially flipped.

When spatial_flipping_flag is equal to 0, it is a requirement of bitstream conformance that frame0_flipped_flag shall be equal to 0. When spatial_flipping_flag is equal to 0, the value 1 for spatial_flipping_flag is reserved for future use by ITU-T | ISO/IEC. When spatial_flipping_flag is equal to 0, decoders shall ignore the value of frame0_flipped_flag.

field_views_flag equal to 1 indicates that all pictures in the current coded video sequence are coded as field pictures (i.e. field_pic_flag is equal to 1).  All fields of a particular parity are considered a first constituent frame and all fields of the opposite parity are considered a second constituent frame.  When frame_packing_arrangement_type is not equal to 2, it is a requirement of bitstream conformance that the field_views_flag shall be equal to 0. When frame_packing_arrangement_type is not equal to 2, the value 1 for field_views_flag is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is not equal to 2, decoders shall ignore the value of field_views_flag.

current_frame_is_frame0_flag equal to 1, when frame_packing_arrangement is equal to 5, indicates that the current decoded frame is constituent frame 0 and the next decoded frame in output order is constituent frame 1, and the display time of the constituent frame 0 should be delayed to coincide with the display time of constituent frame 1. current_frame_is_frame0_flag equal to 0, when frame_packing_arrangement is equal to 5, indicates that the current decoded frame is constituent frame 1 and the previous decoded frame in output order is constituent frame 0, and the display time of the constituent frame 1 should not be delayed for purposes of stereo-view pairing.

When frame_packing_arrangement_type is not equal to 5, the constituent frame associated with the upper-left sample of the decoded frame is considered to be consitutuent frame 0 and the other constituent frame is considered to be constituent frame 1. When frame_packing_arrangement_type is not equal to 5, it is a requirement of bitstream conformance that current_frame_is_frame0_flag shall be equal to 0. When frame_packing_arrangement_type is not equal to 5, the value 1 for current_frame_is_frame0_flag is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is not equal to 5, decoders shall ignore the value of current_frame_is_frame0_flag.

frame0_self_contained_flag equal to 1 indicates that no inter prediction operations within the decoding process for the samples of constituent frame 0 of the coded video sequence refer to samples of any constituent frame 1. frame0_self_contained_flag equal to 0 indicates that some inter prediction operations within the decoding process for the samples of constituent frame 0 of the coded video sequence may or may not refer to samples of some constituent frame 1. When frame_packing_arrangement_type is equal to 0 or 1, it is a requirement of bitstream conformance that frame0_self_contained_flag shall be equal to 0. When frame_packing_arrangement_type is equal to 0 or 1, the value 1 for frame0_self_contained_flag is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is equal to 0 or 1, decoders shall ignore the value of frame0_self_contained_flag. Within a coded video sequence, the value of frame0_self_contained_flag in all frame packing arrangement SEI messages shall be the same.

frame1_self_contained_flag equal to 1 indicates that no inter prediction operations within the decoding process for the samples of constituent frame 1 of the coded video sequence refer to samples of any constituent frame 0. frame1_self_contained_flag equal to 0 indicates that some inter prediction operations within the decoding process for the samples of constituent frame 1 of the coded video sequence may or may not refer to samples of some constituent frame 0. When frame_packing_arrangement_type is equal to 0 or 1, it is a requirement of bitstream conformance that frame1_self_contained_flag shall be equal to 0. When frame_packing_arrangement_type is equal to 0 or 1, the value 1 for frame1_self_contained_flag is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is equal to 0 or 1, decoders shall ignore the value of frame1_self_contained_flag. Within a coded video sequence, the value of frame1_self_contained_flag in all frame packing arrangement SEI messages shall be the same.

When quincunx_sampling_flag is equal to 0 and frame_packing_arrangement_type is not equal to 5, two (x, y) coordinate pairs are specified to determine the indicated luma sampling grid alignment for constituent frame 0 and constituent frame 1, relative to the upper left corner of the rectangular area represented by the samples of the corresponding constituent frame.

NOTE 9 – The location of chroma samples relative to luma samples can be indicated by the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field syntax elements in the VUI parameters.

frame0_grid_position_x (when present) specifies the x component of the (x, y) coordinate pair for constituent frame 0.

frame0_grid_position_y (when present) specifies the y component of the (x, y) coordinate pair for constituent frame 0.

frame1_grid_position_x (when present) specifies the x component of the (x, y) coordinate pair for constituent frame 1.

frame1_grid_position_y (when present) specifies the y component of the (x, y) coordinate pair for constituent frame 1.

When quincunx_sampling_flag is equal to 0 and frame_packing_arrangement_type is not equal to 5 the (x, y) coordinate pair for each constituent frame is interpreted as follows:

–	If the (x, y) coordinate pair for a constituent frame is equal to (0, 0), this indicates a default sampling grid alignment specified as follows:

–	If frame_packing_arrangement_type is equal to 1 or 3, the indicated position is the same as for the (x, y) coordinate pair value (4, 8), as illustrated in Figure D-2 and Figure D-6.

–	Otherwise (frame_packing_arrangement_type is equal to 2 or 4), the indicated position is the same as for the (x, y) coordinate pair value (8, 4), as illustrated in Figure D-4 and Figure D-8.

–	Otherwise, if the (x, y) coordinate pair for a constituent frame is equal to (15, 15), this indicates that the sampling grid alignment is unknown or unspecified or specified by other means not specified in this Recommendation | International Standard.

–	Otherwise, the x and y elements of the (x, y) coordinate pair specify the indicated horizontal and vertical sampling grid alignment positioning to the right of and below the upper left corner of the rectangular area represented by the corresponding constituent frame, respectively, in units of one sixteenth of the luma sample grid spacing between the samples of the columns and rows of the constituent frame that are present in the decoded frame (prior to any upsampling for display or other purposes).

NOTE 10 – The spatial location reference information frame0_grid_position_x, frame0_grid_position_y, frame1_grid_position_x, and frame1_grid_position_y is not provided when quincunx_sampling_flag is equal to 1 because the spatial alignment in this case is assumed to be such that constituent frame 0 and constituent frame 1 cover corresponding spatial areas with interleaved quincunx sampling patterns as illustrated in Figure D-1 and Figure D-10.

NOTE 11 – When frame_packing_arrangement_type is equal to 2 and field_views_flag is equal to 1, it is suggested that frame0_grid_position_y should be equal to frame1_grid_position_y.

frame_packing_arrangement_reserved_byte is reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that the value of frame_packing_arrangement_reserved_byte shall be equal to 0. All other values of frame_packing_arrangement_reserved_byte are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore (remove from the bitstream and discard) the value of frame_packing_arrangement_reserved_byte.

frame_packing_arrangement_repetition_period specifies the persistence of the frame packing arrangement SEI message and may specify a frame order count interval within which another frame packing arrangement SEI message with the same value of frame_packing_arrangement_id or the end of the coded video sequence shall be present in the bitstream. The value of frame_packing_arrangement_repetition_period shall be in the range of 0 to 16 384, inclusive.

frame_packing_arrangement_repetition_period equal to 0 specifies that the frame packing arrangement SEI message applies to the current decoded frame only.

frame_packing_arrangement_repetition_period equal to 1 specifies that the frame packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new coded video sequence begins.

–	A frame in an access unit containing a frame packing arrangement SEI message with the same value of frame_packing_arrangement_id is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ).

frame_packing_arrangement_repetition_period equal to 0 or equal to 1 indicates that another frame packing arrangement SEI message with the same value of frame_packing_arrangement_id may or may not be present.

frame_packing_arrangement_repetition_period greater than 1 specifies that the frame packing arrangement SEI message persists until any of the following conditions are true:

–	A new coded video sequence begins.

–	A frame in an access unit containing a frame packing arrangement SEI message with the same value of frame_packing_arrangement_id is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + frame_packing_arrangement_repetition_period.

frame_packing_arrangement_repetition_period greater than 1 indicates that another frame packing arrangement SEI message with the same value of frame_packing_arrangement_frames_id shall be present for a frame in an access unit that is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + frame_packing_arrangement_repetition_period; unless the bitstream ends or a new coded video sequence begins without output of such a frame.

frame_packing_arrangement_extension_flag equal to 0 indicates that no additional data follows within the frame packing arrangement SEI message. It is a requirement of bitstream conformance that the value of frame_packing_arrangement_extension_flag shall be equal to 0. The value 1 for frame_packing_arrangement_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value 1 for frame_packing_arrangement_extension_flag in a frame packing arrangement SEI message and shall ignore all data that follows within a frame packing arrangement SEI message after the value 1 for frame_packing_arrangement_extension_flag.

# Figure D-1 – Rearrangement and upconversion of checkerboard interleaving 
(frame_packing_arrangement_type equal to 0)

# Figure D-2 – Rearrangement and upconversion of column interleaving 
with frame_packing_arrangement_type equal to 1, quincunx_sampling_flag equal to 0,
and (x, y) equal to (0, 0) or (4, 8) for both constituent frames

# Figure D-3 – Rearrangement and upconversion of column interleaving with
frame_packing_arrangement_type equal to 1, quincunx_sampling_flag equal to 0, 
(x, y) equal to (0, 0) or (4, 8) for constituent frame 0 and (x, y) equal to (12, 8) for constituent frame 1

# Figure D-4 – Rearrangement and upconversion of row interleaving with
frame_packing_arrangement_type equal to 2, quincunx_sampling_flag equal to 0,
and (x, y) equal to (0, 0) or (8, 4) for both constituent frames

Figure D-5 – Rearrangement and upconversion of row interleaving with
frame_packing_arrangement_type equal to 2, quincunx_sampling_flag equal to 0, 
(x, y) equal to (0, 0) or (8, 4) for constituent frame 0, and (x, y) equal to (8, 12) for constituent frame 1

# Figure D-6 – Rearrangement and upconversion of side-by-side packing arrangement with
frame_packing_arrangement_type equal to 3, quincunx_sampling_flag equal to 0,
and (x, y) equal to (0, 0) or (4, 8) for both constituent frames

Figure D-7 – Rearrangement and upconversion of side-by-side packing arrangement with frame_packing_arrangement_type equal to 3, quincunx_sampling_flag equal to 0,
(x, y) equal to (12, 8) for constituent frame 0, and (x, y) equal to (0, 0) or (4, 8) for constituent frame 1

# Figure D-8 – Rearrangement and upconversion of top-bottom packing arrangement with
frame_packing_arrangement_type equal to 4, quincunx_sampling_flag equal to 0,
and (x, y) equal to (0, 0) or (8, 4) for both constituent frames

Figure D-9 – Rearrangement and upconversion of top-bottom packing arrangement with frame_packing_arrangement_type equal to 4, quincunx_sampling_flag equal to 0,
(x, y) equal to (8, 12) for constituent frame 0, and (x, y) equal to (0, 0) or (8, 4) for constituent frame 1

# Figure D-10 – Rearrangement and upconversion of side-by-side packing arrangement with quincunx sampling
(frame_packing_arrangement_type equal to 3 with quincunx_sampling_flag equal to 1)

# Figure D-11 – Rearrangement of a temporal interleaving frame arrangement 
(frame_packing_arrangement_type equal to 5)

References

[1] Rec. ITU-T H.264 (in force), Advanced video coding for generic audiovisual services.

[2] ISO/IEC 14496-10: in force, Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding

[3] "High Efficiency Video Coding (HEVC) text specification draft 8", JCTVC-J1003, July 2012, Stockholm Sweden

[4] Proposed Editorial Improvements for High efficiency video coding (HEVC) Text Specification Draft 8, JCTVC-K0030, October 2012, Shanghai China

Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).