| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0144 |
| --- | --- |

| Title: | Reduce worst-case memory bandwidth for SCU size larger than 8x8 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

It is asserted that memory bandwidth for inter-prediction is a significant issue facing video decoder implementations.  The adoption of prohibiting 4x4 inter PUs and restricting 4x8 and 8x4 PUs to uni-directional inter-prediction, applicable when the CU size is equal to the SCU size of 8x8, have to some extent alleviated this situation. This contribution proposes to apply these restrictions for any SCU size. It is asserted that larger SCU sizes may be desirable for larger frame sizes and that the proposed restrictions will assist these cases.  Simulation results show luma 1.0% losses in RA_Main (0.6% in class A), 1.0% losses in RA_HE10 (0.6% in class A), 1.2% in LB_Main, 1.2% in LB_HE10 for PART_NxN removal from inter-prediction. Encoder run-times are reduced to 88-90%.

# Problem Statement

It is asserted that memory bandwidth for inter-prediction is a significant issue facing video decoder implementations.  The worst case of memory bandwidth is achieved when each prediction unit (PU) is the smallest possible size, i.e. each coding unit (CU) is the size of the smallest coding unit (SCU), and has the maximum number of partitions and randomised motion vectors. The adoption of restrictions applicable when the SCU size is 8x8 have to some extent alleviated this situation. We assert that the problem is present when the SCU size is increased, e.g. to 16x16.  Such a configuration is likely for large frame sizes.

# Description of the proposed change

The proposed changes are as follows:

- Remove the PART_NxN partition mode entirely from inter-predicted PUs.  Originally this restriction was introduced only for 4x4 PUs [1]. This removal also simplifies the binarisation by removing the distinction between the cases of a CU size equal to the SCU size of 8x8 versus larger sizes.

- Apply the restriction for PART_2NxN and PART_Nx2N partition modes to uni-directional inter-prediction when the CU size is equal to the SCU size regardless of the SCU size (See [2]).

These changes also result in reducing the line buffer of motion vectors from the above LCU row to at most two motion vectors per SCU (for any SCU size).

By introducing a bitstream restriction, such as SCU > 8x8 for levels above a specific point, the above changes reduce the worst case memory bandwidth for these levels.

# Experiment Results

Results are presented in Table 1 and Table 2 below. Simulations were performed under the common conditions [3]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Context initialization values were not updated in this simulation.

Also, a comparison between the SCU=8x8 case (common test conditions) and the SCU=16x16 case is presented in Table 3 for informational purposes.

Table 1. Simulation result with SCU=16x16: PART_NxN inter-prediction disabled.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.6% | 0.8% | 0.7% | 0.6% | 0.9% | 0.5% |
| Class B | 0.5% | 0.6% | 0.7% | 0.6% | 0.7% | 0.7% |
| Class C | 1.3% | 1.4% | 1.6% | 1.3% | 1.5% | 1.4% |
| Class D | 1.8% | 2.3% | 2.2% | 1.8% | 2.1% | 2.3% |
| Class E |  |  |  |  |  |  |
| Overall | 1.0% | 1.2% | 1.3% | 1.0% | 1.2% | 1.2% |
|  | 1.0% | 1.2% | 1.2% | 1.0% | 1.2% | 1.2% |
| Class F | 0.9% | 1.1% | 1.1% | 0.9% | 1.0% | 1.1% |
| Enc Time[%] | 89% |  |  | 90% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.6% | 0.7% | 0.8% | 0.6% | 0.7% | 0.7% |
| Class C | 1.6% | 1.9% | 1.9% | 1.7% | 2.1% | 2.3% |
| Class D | 2.2% | 2.7% | 2.8% | 2.2% | 2.4% | 3.3% |
| Class E | 0.5% | 0.5% | 0.2% | 0.5% | 0.2% | 1.0% |
| Overall | 1.2% | 1.5% | 1.5% | 1.2% | 1.4% | 1.8% |
|  | 1.2% | 1.5% | 1.4% | 1.2% | 1.3% | 1.8% |
| Class F | 1.6% | 1.7% | 2.1% | 1.8% | 1.8% | 1.7% |
| Enc Time[%] | 90% |  |  | 88% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

Table 2. Simulation result with SCU=16x16: PART_NxN inter-prediction disabled, PART_2NxN and PART_Nx2N restricted to uni-directional (when CU=SCU).

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.1% | 1.7% | 1.5% | 1.2% | 1.7% | 1.1% |
| Class B | 1.4% | 1.4% | 1.5% | 1.5% | 1.5% | 1.4% |
| Class C | 2.9% | 3.0% | 3.2% | 2.9% | 3.0% | 3.1% |
| Class D | 4.1% | 4.6% | 4.4% | 4.1% | 4.6% | 4.7% |
| Class E |  |  |  |  |  |  |
| Overall | 2.3% | 2.6% | 2.6% | 2.4% | 2.6% | 2.5% |
|  | 2.3% | 2.6% | 2.6% | 2.4% | 2.6% | 2.5% |
| Class F | 1.5% | 1.9% | 2.0% | 1.7% | 2.0% | 2.1% |
| Enc Time[%] | 85% |  |  | 85% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.6% | 1.7% | 2.2% | 1.6% | 1.9% | 1.7% |
| Class C | 3.2% | 3.3% | 3.4% | 3.4% | 3.7% | 3.8% |
| Class D | 4.5% | 4.6% | 4.9% | 4.6% | 5.2% | 5.3% |
| Class E | 1.2% | 1.2% | 0.7% | 1.1% | 1.2% | 1.2% |
| Overall | 2.7% | 2.8% | 2.9% | 2.7% | 3.1% | 3.0% |
|  | 2.7% | 2.7% | 2.8% | 2.7% | 3.1% | 3.0% |
| Class F | 2.2% | 2.0% | 2.3% | 2.5% | 2.8% | 2.9% |
| Enc Time[%] | 83% |  |  | 83% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3. Comparison of SCU=8x8 (common test conditions) with SCU=16x16 for informational purposes.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.5% | -0.3% | -0.7% | 1.5% | -0.5% | -0.8% |
| Class B | 2.4% | 0.1% | 0.2% | 2.4% | 0.0% | 0.0% |
| Class C | 5.2% | 3.8% | 4.0% | 5.2% | 3.7% | 3.9% |
| Class D | 6.1% | 4.3% | 4.6% | 6.1% | 4.3% | 4.7% |
| Class E | 4.1% | 0.7% | 0.8% | 4.2% | 0.4% | 0.3% |
| Overall | 3.8% | 1.7% | 1.8% | 3.8% | 1.6% | 1.6% |
|  | 3.8% | 1.6% | 1.7% | 3.8% | 1.5% | 1.6% |
| Class F | 16.5% | 13.7% | 14.2% | 16.4% | 13.5% | 14.3% |
| Enc Time[%] | 63% |  |  | 62% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.9% | 2.0% | 1.8% | 2.5% | 3.2% | 2.3% |
| Class B | 2.9% | 2.2% | 2.2% | 3.5% | 2.7% | 2.9% |
| Class C | 6.2% | 7.6% | 8.4% | 7.7% | 9.1% | 9.7% |
| Class D | 7.4% | 9.0% | 9.7% | 9.3% | 11.5% | 12.1% |
| Class E |  |  |  |  |  |  |
| Overall | 4.5% | 5.0% | 5.3% | 5.6% | 6.4% | 6.5% |
|  | 4.5% | 5.0% | 5.3% | 5.6% | 6.3% | 6.5% |
| Class F | 14.6% | 13.9% | 14.6% | 15.8% | 15.3% | 16.2% |
| Enc Time[%] | 78% |  |  | 70% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.8% | 1.2% | 1.1% | 2.8% | 1.2% | 1.5% |
| Class C | 5.5% | 6.2% | 6.9% | 5.5% | 6.2% | 7.4% |
| Class D | 6.9% | 8.6% | 8.6% | 7.0% | 7.8% | 8.6% |
| Class E | 2.9% | 0.7% | 2.6% | 2.9% | 1.6% | 1.9% |
| Overall | 4.5% | 4.2% | 4.7% | 4.5% | 4.2% | 4.8% |
|  | 4.5% | 4.2% | 4.7% | 4.5% | 4.1% | 4.8% |
| Class F | 11.5% | 12.9% | 13.2% | 11.4% | 13.2% | 12.5% |
| Enc Time[%] | 79% |  |  | 79% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Working Draft changes

Proposed text is applicable to HEVC WD version 8 [4].

## INTER_NxN removal from inter-prediction

7.4.8.1	General coding unit semantics

Table 7-4 – Name association to prediction mode and partitioning type

| - PredMode[ x0 ][ y0 ] | - part_mode | - IntraSplitFlag | PartMode |
| --- | --- | --- | --- |
| - MODE_INTRA | - 0 | - 0 | - PART_2Nx2N |
|  | - 1 | - 1 | - PART_NxN |
| - MODE_INTER | - 0 | - 0 | PART_2Nx2N |
|  | - 1 | - 0 | PART_2NxN |
|  | - 2 | - 0 | PART_Nx2N |
|  | - 3 | - 0 | PART_NxN |
|  | - 43 | - 0 | PART_2NxnU |
|  | - 54 | - 0 | PART_2NxnD |
|  | - 65 | - 0 | PART_nLx2N |
|  | - 76 | - 0 | PART_nRx2N |

7.3.9.1	General coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

8.5.1	Inter prediction process

–	Otherwise, if PartMode is equal to PART_NxN, the following ordered steps apply:

- The decoding process for prediction units in inter prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, 0 ), the size of the luma coding block nCSL, the width of the luma prediction block nPbW set equal to nCS1L, the height of the luma prediction block nPbH set equal to nCS1L as and a partition index partIdx set equal to 0 inputs, and the outputs are a (nCSL)x(nCSL) array predSamplesL and two (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( nCS1L, 0 ), the size of the luma coding block nCSL, the width of the luma prediction block nPbW set equal to nCS1L, the height of the luma prediction block nPbH set equal to nCS1L and a partition index partIdx set equal to 1 as inputs, and the outputs are the modified (nCSL)x(nCSL) array predSamplesL and the two modified (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( 0, nCS1L ), the size of the luma coding block nCSL, the width of the luma prediction block nPbW set equal to nCS1L, the height of the luma prediction block nPbH set equal to nCS1L and a partition index partIdx set equal to 2 as inputs, and the outputs are the modified (nCSL)x(nCSL) array predSamplesL and the two modified (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

- The decoding process for prediction units in inter prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the luma location ( xB, yB ) set equal to ( nCS1L, nCS1L ), the size of the luma coding block nCSL, the width of the luma prediction block nPbW set equal to nCS1L, the height of the luma prediction block nPbH set equal to nCS1L and a partition index partIdx set equal to 3 as inputs, and the outputs are the modified (nCSL)x(nCSL) array predSamplesL and the two modified (nCSC)x(nCSC) arrays predSamplesCb and predSamplesCr.

9.3.2.6 Binarization process for part_mode

Table 9-5 – Binarization for part_mode

| PredMode[ xC ][ yC ] | part_mode | PartMode | Bin string |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | cLog2CbSize > Log2MinCbSize |  | cLog2CbSize = = Log2MinCbSize |  |
|  |  |  | !amp_enabled_flag | amp_enabled_flag | cLog2CbSize = = 3 | cLog2CbSize > 3 |
| MODE_INTRA | 0 | PART_2Nx2N | - | - | 1 | 1 |
|  | 1 | PART_NxN | - | - | 0 | 0 |
| MODE_INTER | 0 | PART_2Nx2N | 1 | 1 | 1 | 1 |
|  | 1 | PART_2NxN | 01 | 011 | 01 | 01 |
|  | 2 | PART_Nx2N | 00 | 001 | 00 | 001 |
|  | 3 | PART_NxN | - | - | - | 000 |
|  | 43 | PART_2NxnU | - | 0100 | - | - |
|  | 54 | PART_2NxnD | - | 0101 | - | - |
|  | 65 | PART_nLx2N | - | 0000 | - | - |
|  | 76 | PART_nRx2N | - | 0001 | - | - |

## PART_2NxN, PART_Nx2N uni-directional restriction

Table 9-6 – Assignment of ctxIdxInc to syntax elements with context coded bins

| inter_pred_idc | ( nPbW + nPbH ) != 12(3 << (log2MinCbSize – 1)) ? ctDepth : 4 | 4 | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |

#### 9.3.2.9 Binarization process for inter_pred_idc

Input to this process is a request for a binarization for the syntax element inter_pred_idc, the width and the height of the current luma prediction block nPbW and nPbH.

Output of this process is the binarization of the syntax element.

The binarization for inter_pred_idc is given by Table 9-7.

Table 9-7 – Binarization for inter_pred_idc

| Value of inter_pred_idc | Name of inter_pred_idc | Bin string |  |
| --- | --- | --- | --- |
|  |  | ( nPbW + nPbH ) != 12(3 << (log2MinCbSize – 1)) | ( nPbW + nPbH ) = = 12(3 << (log2MinCbSize – 1)) |
| 0 | Pred_L0 | 00 | 0 |
| 1 | Pred_L1 | 01 | 1 |
| 2 | Pred_BI | 1 | - |

# Conclusion

Presented is the removal of PART_NxN from inter-prediction, to reduce worst case memory bandwidth.  This resulted in 1.0% losses in RA_Main (0.6% in class A), 1.0% losses in RA_HE10 (0.6% in class A), 1.2% in LB_Main, 1.2% in LB_HE10. In all cases the anchor was set to SCU size is equal to 16x16.  Further presented is a restriction on PART_Nx2N and PART_2NxN to be uni-predicted when CU size is equal to SCU size.  Encoder run-times are reduced to 88-90%.

# References

- T. Hellman, W. Wan, “AHG7: Reducing HEVC worst-case memory bandwidth by restricting bi-directional 4x8 and 8x4 prediction units”, Joint Collaborative Team on Video Coding, JCTVC-I0216, April 2012.

- M. Zhou, “AHG7: Disallow bi-predictive mode for 8x4 and 4x8 inter PUs”, Joint Collaborative Team on Video Coding, JCTVC-J0086, July 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-J1100, July 2012.

- B. Bross, W.-J. Han, J.-R. Ohm, G. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8”, Joint Collaborative Team on Video Coding, JCTVC-J1003, July 2012.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).