| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0145 |
| --- | --- |

| Title: | Consistent chroma QP derivation in the deblocking and inverse transform processes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA | Email: Email: | wwan@broadcom.com timhellman@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution recommends changing the chroma QP derivation process in the deblocking process to make it consistent with the QP derivation process used in the inverse transform process.  Specifically, it proposes to include the chroma QP offsets and QpBdOffset in the chroma block edge filtering process.  The contribution also proposes to remove slice-level chroma QP offsets, as it claims that these add a storage burden to single-pass deblocking implementations.

# Problem Statement

The HEVC quantization parameter (QP) is used in two places: the inverse transform, for coefficient scaling, and the deblocker, for filter edge strength determination.  Chroma QP is derived from Luma QP by an equation that produces an index into a lookup table.  The lookup table is identical for inverse transform and deblocking processes, but the equations are not.  The equations for the inverse transform, from [1], are:

qPiCb = Clip3( −QpBdOffsetC, 57, QPY + pic_cb_qp_offset + slice_cb_qp_offset )	(8-1)

qPiCr = Clip3( −QpBdOffsetC, 57, QPY + pic_cr_qp_offset + slice_cr_qp_offset )	(8-2)

However, the deblocking equation is:

qPi = ( ( QPQ + QPP + 1 ) >> 1 )	(8-3)

where QPQ and QPP are the QPY values on either side of the deblocking edge.  This produces an asymmetry in the standard which is undesirable, and also differs from H.264/AVC [2], which uses identical derivations for the transform and deblocking processes.

# Proposed Solution

The proposed solution is to make the deblocking equation consistent with that of the inverse transform by adding QP offsets and QpBdOffset to equation 8-298.  This is straightforward to do in the text, but produces an undesirable complication in single-pass deblocking implementations, due to the slice-level QP offsets first introduced in JCTVC-J0342 [3].  Single-pass encoders and decoders need to provide line buffer storage for all parameters involved in top LCU horizontal-edge filtering, because such filtering can only be performed once the lower LCU is available.

Any parameters that can change per slice must be allocated space in this buffer.  In the worst-case, every LCU can be in a different slice, so the amount of storage needed is determined by the maximum picture width divided by the smallest LCU size.  The amount of additional storage needed is on the order of 256 bytes which is not a large amount of memory, but it is preferable to avoid adding it unless a significant benefit can be obtained.  The solution proposed here is to remove the slice-level QP offsets.  It appears that the picture-level QP offsets in H.264/AVC are infrequently used, so the need for additional slice-level flexibility, given the cost in the deblocker, seems unwarranted.

The inverse transform equations then become:

qPiCb = Clip3( −QpBdOffsetC, 57, QPY + pic_cb_qp_offset )

qPiCr = Clip3( −QpBdOffsetC, 57, QPY + pic_cr_qp_offset )

Where qPixx is the index into the QPC lookup table.

For the deblocker, the conversion from QPY to QPC is done before averaging, as in H.264/AVC, using the same conversion equations as in the inverse transform.  The ‘Q’ and tC values derived from QPC are now computed independently for Cr and Cb, as in H.264/AVC.

# Conclusion

Using the same QP derivation in the inverse transform and the deblocker makes sense theoretically and intuitively, and is consistent with the precedent established in H.264/AVC.  Limiting chroma QP offsets to only picture-level adaptation allows this change to be made with no implementation cost.  Therefore, this contribution recommends adoption of these changes to deblocking QP derivation.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-J1003_d7, July 2012.

- Rec. ITU-T H.264 | ISO/IEC 14496-10, “Advanced video coding for generic audiovisual services”, 2012.

- G. Sullivan, S. Kanumuri, J.-Z. Xu, Y. Wu, “Chroma QP range extension”, Document of Joint Collaborative Team on Video Coding, JCTVC-J0342, July 2012

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

## Section 7.3.2.3 Picture parameter set RBSP syntax

| pic_cr_qp_offset | se(v) |
| --- | --- |
| pic_slice_level_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |

## Section 7.3.5.1

| slice_qp_delta | se(v) |
| --- | --- |
| if( pic_slice_level_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |

## Section 7.4.2.3 Picture parameter set RBSP semantics

pic_slice_level_chroma_qp_offsets_present_flag equal to 1 indicates that the slice_cb_qp_offset and slice_cr_qp_offset syntax elements are present in the associated slice headers.

pic_slice_level_chroma_qp_offsets_present_flag equal to 0 indicates that these syntax elements are not present in the associated slice headers.

## Section 7.4.5.1 General slice header semantics

slice_cb_qp_offset specifies a difference to be added to the value of pic_cb_qp_offset when determining the value of the QP’Cb quantization parameter. The value of slice_cb_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cb_qp_offset is not present, it is inferred to be equal to 0. The value of pic_cb_qp_offset + slice_cb_qp_offset shall be in the range of −12 to +12, inclusive.

## Section 8.6.1 Derivation process for quantization parameters

The variables qPCb and qPCr are set equal to the value of QPC as specified in Error: Reference source not found based on the index qPi equal to qPiCb and qPiCr derived as:

qPiCb = Clip3( −QpBdOffsetC, 57, QPY + pic_cb_qp_offset + slice_cb_qp_offset )	(8-4)

qPiCr = Clip3( −QpBdOffsetC, 57, QPY + pic_cr_qp_offset + slice_cr_qp_offset )	(8-5)

## Section 8.7.2.4.5 Filtering process for chroma block edge

…

The variables QPQ and QPP are set equal to the QPYC values that correspond to the QPY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively, as specified in subclause 8.6.1.

The variable QPC is determined as specified in Error: Reference source not found based on the index qPi derived as:

QPC = ( ( QPQ + QPP + 1 ) >> 1 )	(8-6)