| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0146 |
| --- | --- |

| Title: | AHG9: Comments on Coding and Display Formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Brian Heng Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Waqar Ahmad Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA | Email: Email: Email: | wwan@broadcom.com bheng@broadcom.com waqarahmad@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution notes that several coding functionalities that were present in H.264/AVC and widely utilized in practice are prohibited in the current HEVC text.  More specifically, the ability to use a coding format different than the desired display format has coding efficiency advantages and can be specified in a similar fashion to the H.264/AVC standard.  This contribution proposes to include the ability to signal top/bottom field cadence, 3:2 pulldown, and frame doubling/tripling with SEI messages in order to provide the HEVC standard more flexibility and clarity when using a coding format different from the desired display format.

# Discussion

The pic_struct field from the picture timing SEI message in the H.264/AVC standard [1] is often used in broadcast services to help indicate how the decoded video was intended to be displayed, especially when the desired display format is not the same as the coding format.  Table 1 provides a list of the functionalities provided by the pic_struct field in H.264/AVC and whether they are supported or not in the current HEVC text [2].

Table 1: Comparison of Coding and Display Format Functionality in H.264/AVC and HEVC

| H.264/AVC |  | HEVC |  |
| --- | --- | --- | --- |
| pic_struct | Indicated display of picture | Possible? | Comments |
| 0 | (progressive) frame | YES | field_pic_flag = 0 |
| 1 | top field | YES | field_pic_flag = 1 bottom_field_flag = 0 |
| 2 | bottom field | YES | field_pic_flag = 1 bottom_field_flag = 1 |
| 3 | top field, bottom field, in that order | NO | Note top_field_first flag is only signalled when progressive_source = 0. The progressive_source flag indicates the nature of content and not the intended display. |
| 4 | bottom field, top field, in that order | NO | Note top_field_first flag is only signalled when progressive_source = 0. The progressive_source flag indicates the nature of content and not the intended display. |
| 5 | top field, bottom field, top field repeated in that order | NO | See pic_struct = 3 and no ability to signal repeated (field) picture. |
| 6 | bottom field, top field, bottom field repeated in that order | NO | See pic_struct = 4 and no ability to signal repeated (field) picture. |
| 7 | frame doubling | NO | No ability to signal repeated (frame) picture. |
| 8 | frame tripling | NO | No ability to signal repeated (frame) picture. |
| 9-15 | reserved |  |  |

## Top/Bottom Field Cadence

The ability to encode progressive content within an interlaced stream is not possible if the relative ordering of the fields is not signaled.  That is, with the current HEVC text, a decoder cannot differentiate between the cases of pic_struct = 3 and pic_struct = 4. Note that the top_field_first flag is only signalled when progressive_source = 0. The progressive_source flag in the HEVC Field Indication SEI only indicates the nature of the original content and not the intended display format.  Even progressive content must have a specified field order for interlaced display in order to maintain consistent field cadence when coding mixed progressive/interlaced content.

## 3:2 pulldown in frame sequences

A common industry practice is to signal 3:2 pulldown in frame coded sequences (pic_struct = 5 and pic_struct = 6) for coding 24 frames per second content at 60 fields per second.  This is not possible with the current HEVC text.  Specifically, there is no mechanism analogous to the repeat_first_field flag which can be combined with frame coding and a proper field cadence as described in the previous section in order to enable 3:2 pulldown signalling.

## Frame doubling/tripling

Another common industry practice (which is also listed as an informative note in the H.264/AVC standard) is to utilize frame doubling (pic_struct = 7) to facilitate display of 25p coded video on a 50p display and display of 29.97p coded video on a 59.94p display as well as use of both frame doubling and frame tripling (pic_struct = 8) to facilitate display of 23.98p coded video on a 59.94 display.  There are also other useful scenarios for frame doubling and/or tripling.  This functionality is not possible in the current HEVC text without explicitly coding repeated frames.  While the additional bitrate of explicitly coding repeated frames may be relatively small, this is inefficient for both the encoding and decoding as well as unnecessary when frame doubling and tripling are mechanisms well understood and already present in existing video standards.

# Proposed Solution

This contribution proposes the following signaling that was permitted using the pic_struct field in H.264/AVC and widely utilized in practice to be added to SEI messages in HEVC:

- Signal field cadence in frame coded pictures intended for interlaced display.

- Signal 3:2 pulldown in frame coded pictures intended for interlaced display.

- Signal frame doubling/tripling for progressive display.

The HEVC Field indication SEI has some similarities to the H.264/AVC pic_struct element but this SEI appears to be intended only for interlaced and mixed progressive/interlaced sequences while the information provided by pic_struct is more generally applicable rather than just for.  It is suggested that a new SEI with the pic_struct functionality be defined to replace some/all of the elements of the Field indication SEI or equivalently, that additional elements be added in the Field indication SEI or other SEIs to enable the missing functionalities.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-J1003_d7, July 2012.

- Rec. ITU-T H.264 | ISO/IEC 14496-10, “Advanced video coding for generic audiovisual services”, 2012.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).