| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0148 |
| --- | --- |

| Title: | Cross-check of Rate control by R-lambda model for HEVC (JCTVC-K0103) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com 86-10-82171657 wenhao.zhang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of Rate control by R-lambda model for HEVC proposed by USTC in JCTVC-K0103. The software provided by the proponent was checked and tested. The cross-check result confirms the result reported by proponent.

# Introduction

JCTVC-K0103 proposes a rate control algorithm by R-lambda model for HEVC to replace the existing rate control in HM8.0, which is based on a unified RQ model. The proposed R-lambda model is shown in the following equation,

R=λα−β=α1⋅λβ1 	(1)

where α1 and β1 are parameters related to the video source. The basic idea of proposed rate control algorithm is tuning the lambda value to achieve the target bitrate.

Beside of the model in equation (1), JCTVC-K0103 also improves the bit location schemes at GOP level, picture level and LCU level.

# Experimental results

The platform used to test the encoder and decoder is Linux + GCC 4.2.2. The software package obtained from the proponent was successfully compiled and tested under the common test conditions [1]. The anchor is the HM8.0 with the unified RQ model rate control enabled. There was no mismatch between encoder and decoder. It is observed that the results produced by this cross-check have some mismatches compared with the results reported by the proponent. The BD-bitrate differences are always less than 0.2%. The proponent has investigated this issue and found it is caused by float precision of different platform. The performance data is shown in Table 1 to 2, and detailed results can be found in the attached excel sheets.

Table 1 Performance data of Rate control by R-lambda model with hierarchically bit allocation

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -24.1% | -26.8% | -27.0% | -22.0% | -4.4% | -3.8% |
| Class B | -26.8% | -33.1% | -35.0% | -20.2% | -6.8% | -9.5% |
| Class C | -26.5% | -32.1% | -31.6% | -7.6% | -3.5% | -4.7% |
| Class D | -20.3% | -33.7% | -33.0% | -0.4% | 2.8% | 0.9% |
| Class E |  |  |  |  |  |  |
| Overall | -24.6% | -31.5% | -31.8% | -13.0% | -3.2% | -4.6% |
|  | -24.2% | -31.5% | -31.9% | -12.8% | -3.3% | -4.4% |
| Class F | -37.7% | -37.3% | -38.1% | -19.7% | -13.4% | -15.5% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -23.1% | -14.3% | -14.6% | -51.5% | -27.2% | -26.3% |
| Class C | -10.7% | -10.4% | -10.1% | -23.3% | -7.6% | -6.5% |
| Class D | -6.9% | -12.3% | -11.1% | -6.8% | 9.5% | 9.7% |
| Class E | -22.5% | 6.0% | 12.3% | -31.6% | 2.5% | 9.8% |
| Overall | -15.8% | -9.0% | -7.6% | -29.5% | -7.6% | -5.6% |
|  | -15.6% | -9.5% | -8.3% | -29.4% | -8.5% | -6.5% |
| Class F | -3.7% | -1.2% | 2.5% | -12.6% | 0.6% | 2.6% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 2 Performance data of Rate control by R-lambda model with equally bit allocation

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -20.0% | 5.1% | 6.4% | -49.3% | -12.0% | -9.6% |
| Class C | -0.3% | 12.3% | 11.8% | -13.8% | 16.7% | 17.0% |
| Class D | 3.9% | 13.5% | 12.2% | 4.7% | 36.7% | 36.4% |
| Class E | -18.3% | 19.2% | 27.8% | -28.2% | 13.7% | 22.7% |
| Overall | -8.8% | 11.6% | 13.2% | -23.0% | 12.2% | 14.6% |
|  | -8.6% | 10.4% | 11.6% | -22.9% | 10.3% | 12.8% |
| Class F | -1.2% | 8.3% | 11.1% | -11.2% | 10.8% | 12.1% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# References

[1] F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-J1100, Stockholm, July 2012.