| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0149 |
| --- | --- |

| Title: | CE1: On deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal/Information |  |  |
| Author(s) or Contact(s): | Shuo Lu Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Shuo.Lu@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document reports performance of tools combination in CE1 which are intent to deal with block artifacts in sequences such as Riverbed/WestWindEasy etc. Results confirm that the proposed beta parameter can provide wider effective working range based on both HM8.0 and tools in CE1. It has no influence on Common Test Conditions but can enable encoder designers to optimize subjective quality with higher flexibility. It can be adopted as a revision of current HM8.0 or combining tools in CE1

# Introduction

In the 10th Stockholm meeting, blocky artifacts in sequences like Riverbed are reported and studied. Experts viewing test was performed to evaluate the performances of related proposals (J0090/J0096/J0181/J0286). Based on the results of the viewing test, CE1 was set up to study combinations of tools mostly from J0181 and J0286.

# Tools description

Tools to be tested in CE1 are described as below. In this document, Tool of 2.1/2.2/2.3/2.4/2.5/2.6 are involved. For tests of Tool 2.7 which is involved in test8 and test9 of CE1, one can refer to JCTVC_Kxxx for details.

## β table modification

During deblocking filter process, the following conditions are checked to make filtering decision

dp0 = | p2,0 − 2*p1,0 + p0,0 |;  dq0 = | q2,0 − 2*q1,0 + q0,0 |

dp3 = | p2,3 − 2*p1,3 + p0,3 |;  dq3 = | q2,3 − 2*q1,3 + q0,3 |

(s1)Filtering on an edge only takes place when

(dp0+dq0+dp3+dq3) < β									    (1)

Based on the result of (s1), the following conditions need to be checked further to determine the filtering type, strong filter or weak filter.

(s2)Strong filter will be chosen if the following conditions all hold:

2*(dpi+dqi) < (β >> 2);									    (2)

| p3,i – p0,i | + | q0,i – q3,i | < (β >>3);		(3)

| p0,i – q0,i | < ((5*tc + 1) >> 1) where i =0,3.		(4)

It can be observed that the value of β determines whether filtering is applied or not. To choose strong fitler, β and tc are working together to make the decision. Therefore, the changing rate of β should have similar trend with the changing rate of tc to make stronger filter be selected when QP is larger. The β-tc-QP relation with modified β is shown in Figure 1.

Figure1: β-tc-QP relation: blue line is tc, red line is β in hm8.0, green line is proposed β

HM8.0:

const UChar betatable_8x8[52] =

{ 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,6,7,8,9,10,11,12,13,14,15,16,17,18,20,22,24,26,28,30,32,34,36,38,40,42,44,46,48,50,52,54,56,58,60,62,64 };

To be tested:

const UChar betatable_8x8[52] =

{ 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,6,7,8,9,10,11,12,13,14,15,16,17,18,20,22,24,26,28,30,32,34,36,38,40,42,44,52,60,68,76,84,92,100,116,132,148 };

For DIS and also implementation, only the parameter table as shown in section 6 is modified. No additional computation or changes is involved.

## Intra smoothing restricted

Remove intra-boundary smoothing in DC, vertical, and horizontal intra prediction modes for blocks of larger size. Tests results in this document are obtained with applying intra-boundary smoothing for 8x8/16x16 blocks. One can refer to JCTVC_Kxxx for explicit explanation of this tool.

## Modification of strong filter decisions

HM8.0:

(|p3i - p0i| + |q0i – q3i|) < (β>>3)

((| p2,i − 2*p1,i + p0,i | + | q2,i − 2*q1,i + q0,i |) <<1) < (β>>2)

|p0i – q0i| < ((5*tC + 1)>>1)

To be tested:

((| p3,i − 2*p2,i + p1,i | + | q3,i − 2*q2,i + q1,i |) <<1) < (β>>2)

((| p2,i − 2*p1,i + p0,i | + | q2,i − 2*q1,i + q0,i |) <<1) < (β>>2)

|p0i – q0i| < (tC<<4)

## Modification of strong filter

HM8.0:

p0’ = Clip3( p0−2*tc, p0+2*tc, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )   
q0’ = Clip3( q0−2*tc, q0+2*tc, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )
p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 ) 
q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )
q2’ = Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )

To be tested:

tc2 = tc;

tc1 = (tc2*3+1)>>1;

tc3 = (tc2+1)>>1;

p0’ = Clip3( p0−2*tc2, p0+2*tc2, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )   
q0’ = Clip3( q0−2*tc2, q0+2*tc2, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )

p1' = Clip3(p1-tc1, p1+tc1, ((p3 + 2*p2+ 2*p1 + p0 + 2*q0 + 4)>>3))          
q1' = Clip3(q1-tc1, q1+tc1, ((q3 + 2*q2+ 2*q1 + q0+2*p0 + 4)>>3))
p2' = Clip3(p2-tc3, p2+tc3, ((3*p3 + 3*p2 + p1 + q0 + 4 )>>3))
q2' = Clip3(q2-tc3, q2+tc3, ((p0 + q1 + 3*q2 + 3*q3 +4 )>>3))

## Boundary strength modification for CU

The derivation of the boundary strength is modified in a way that for the edges that have boundary strength Bs = 2, Bs can be increased to 3 in case of CU larger or equal to 32x32 when the block boundary is on the 32x32 filtering grid.

## Intra tc_offset = 3

The default intra_tc_offset for intra-block boundaries is increased from 2 to 3 as an alternative tool of 2.5.

## Boundary strength modification for TU

Bs is increased from 2 to 3 at the intra TU boundary when one of the TUs has a maximum TU size. One can refer to JCTVC_Kxxx for report of tests on this tool.

# Tests performed in CE 1

The results for the following tests in CE1 are described in this document:

Test 1: Beta table modification

Test 3: Beta table modification + test2 (Strong filter decisions + Strong filter + Bs modification CU + Intra Smoothing Restricted)

Test 5: Beta table modification + test4 (Strong filter decisions + Strong filter + Intra smoothing restricted + Intra tc_offset = 3)

Test 6: Intra smoothing restricted

One can refer to JCTVC_Kxx for results of the other tests (test2/4/7/8/9) in CE1.

# Results

In CE1, two types of sequences are identified to evaluate the performance of deblocking filter. TypeⅠ sequences including riverbed/WestWindEasy/F.ChinaSpeed/DucksTakeOff/Redkayak, in which severe block noises are observed with current HM8.0, are used to demonstrate improvement in visual quality. TypeⅡsequences including B.Kimono/B.Cactus/F.BasketballDrillText/C.BQMall/E.Jonny are used to make sure that no visual degradation in Common Test Condition due to the change of deblocking filter.

Detailed objective performance of these two types of sequences is reported as below. The results show that proposal can keep or improve the objective evaluations of both types of sequences.

Test configurations are as below:

CTC :

Common Test Conditions as defined in JCTVC_J1100.

CTC with Offset6:

DeblockingFilterControlPresent	: 1

LoopFilterBetaOffset_div2		: 6

LoopFilterTcOffset_div2		: 6

CTC with MaxTransformSize 16 With Offset 6:

QuadtreeTULog2MaxSize		: 4

DeblockingFilterControlPresent	: 1

LoopFilterBetaOffset_div2		: 6

LoopFilterTcOffset_div2		: 6

## Objective performance

The results shows that comparing with HM8.0 under the same settings, there is no annoying loss on bitrate and PSNR.

Table1: Test1-CTC with Offset6 (typeⅠsequences)

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | Riverbed | 32 | hm8.0 | 5848.04 | 34.63 | 38.96 | 41.39 |
|  |  |  | CE_test1 | 5846.43 | 34.61 | 38.95 | 41.39 |
|  |  | 37 | hm8.0 | 2864.81 | 31.73 | 38.12 | 40.90 |
|  |  |  | CE_test1 | 2864.34 | 31.72 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1565.90 | 29.57 | 37.70 | 40.63 |
|  |  |  | CE_test1 | 1564.12 | 29.58 | 37.69 | 40.63 |
|  | WestWindEasy | 32 | hm8.0 | 3746.74 | 36.19 | 44.73 | 47.42 |
|  |  |  | CE_test1 | 3745.53 | 36.16 | 44.71 | 47.44 |
|  |  | 37 | hm8.0 | 1674.61 | 33.01 | 43.71 | 46.64 |
|  |  |  | CE_test1 | 1670.85 | 32.97 | 43.72 | 46.68 |
|  |  | 41 | hm8.0 | 853.89 | 30.75 | 42.95 | 45.85 |
|  |  |  | CE_test1 | 852.28 | 30.73 | 42.94 | 45.86 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1282.95 | 35.09 | 40.11 | 39.74 |
|  |  |  | CE_test1 | 1290.61 | 34.96 | 40.10 | 39.80 |
|  |  | 37 | hm8.0 | 615.75 | 31.82 | 38.63 | 38.01 |
|  |  |  | CE_test1 | 622.93 | 31.59 | 38.66 | 38.00 |
|  |  | 41 | hm8.0 | 316.77 | 29.32 | 37.66 | 36.40 |
|  |  |  | CE_test1 | 318.77 | 29.15 | 37.66 | 36.37 |
| RA-Main | DucksTakeOff | 32 | hm8.0 | 9211.76 | 29.53 | 32.44 | 36.73 |
|  |  |  | CE_test1 | 9205.94 | 29.48 | 32.44 | 36.73 |
|  |  | 37 | hm8.0 | 3951.16 | 27.25 | 31.53 | 36.27 |
|  |  |  | CE_test1 | 3949.74 | 27.21 | 31.53 | 36.27 |
|  |  | 41 | hm8.0 | 1993.11 | 25.68 | 30.95 | 35.98 |
|  |  |  | CE_test1 | 1992.20 | 25.67 | 30.96 | 35.98 |
|  | RedKayak | 32 | hm8.0 | 3708.75 | 35.60 | 41.82 | 44.92 |
|  |  |  | CE_test1 | 3709.50 | 35.57 | 41.82 | 44.92 |
|  |  | 37 | hm8.0 | 1715.45 | 32.92 | 41.01 | 44.12 |
|  |  |  | CE_test1 | 1714.93 | 32.89 | 41.01 | 44.13 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.43 | 43.48 |
|  |  |  | CE_test1 | 852.94 | 30.95 | 40.42 | 43.48 |

Table2: Test3-CTC with Offset6 (typeⅠsequences)

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | Riverbed | 32 | hm8.0 | 5848.04 | 34.63 | 38.96 | 41.39 |
|  |  |  | CE_test3 | 5848.27 | 34.50 | 38.95 | 41.39 |
|  |  | 37 | hm8.0 | 2864.81 | 31.73 | 38.12 | 40.90 |
|  |  |  | CE_test3 | 2864.92 | 31.66 | 38.13 | 40.88 |
|  |  | 41 | hm8.0 | 1565.90 | 29.57 | 37.70 | 40.63 |
|  |  |  | CE_test3 | 1562.47 | 29.56 | 37.69 | 40.64 |
|  | WestWindEasy | 32 | hm8.0 | 3746.74 | 36.19 | 44.73 | 47.42 |
|  |  |  | CE_test3 | 3749.26 | 36.03 | 44.71 | 47.47 |
|  |  | 37 | hm8.0 | 1674.61 | 33.01 | 43.71 | 46.64 |
|  |  |  | CE_test3 | 1671.47 | 32.88 | 43.63 | 46.67 |
|  |  | 41 | hm8.0 | 853.89 | 30.75 | 42.95 | 45.85 |
|  |  |  | CE_test3 | 852.23 | 30.66 | 43.05 | 45.98 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1282.95 | 35.09 | 40.11 | 39.74 |
|  |  |  | CE_test3 | 1294.05 | 34.92 | 40.11 | 39.78 |
|  |  | 37 | hm8.0 | 615.75 | 31.82 | 38.63 | 38.01 |
|  |  |  | CE_test3 | 626.23 | 31.52 | 38.59 | 37.98 |
|  |  | 41 | hm8.0 | 316.77 | 29.32 | 37.66 | 36.40 |
|  |  |  | CE_test3 | 321.17 | 29.10 | 37.62 | 36.38 |
| RA-Main | DucksTakeOff | 32 | hm8.0 | 9211.76 | 29.53 | 32.44 | 36.73 |
|  |  |  | CE_test3 | 9215.98 | 29.44 | 32.44 | 36.73 |
|  |  | 37 | hm8.0 | 3951.16 | 27.25 | 31.53 | 36.27 |
|  |  |  | CE_test3 | 3952.01 | 27.16 | 31.53 | 36.27 |
|  |  | 41 | hm8.0 | 1993.11 | 25.68 | 30.95 | 35.98 |
|  |  |  | CE_test3 | 1992.62 | 25.64 | 30.95 | 35.98 |
|  | RedKayak | 32 | hm8.0 | 3708.75 | 35.60 | 41.82 | 44.92 |
|  |  |  | CE_test3 | 3714.62 | 35.51 | 41.81 | 44.92 |
|  |  | 37 | hm8.0 | 1715.45 | 32.92 | 41.01 | 44.12 |
|  |  |  | CE_test3 | 1717.44 | 32.83 | 41.01 | 44.13 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.43 | 43.48 |
|  |  |  | CE_test3 | 854.54 | 30.91 | 40.43 | 43.48 |

Table3: Test5-CTC with Offset6 (typeⅠsequences)

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | Riverbed | 32 | hm8.0 | 5848.04 | 34.63 | 38.96 | 41.39 |
|  |  |  | CE_test5 | 5846.34 | 34.50 | 38.95 | 41.39 |
|  |  | 37 | hm8.0 | 2864.81 | 31.73 | 38.12 | 40.90 |
|  |  |  | CE_test5 | 2874.93 | 31.55 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1565.90 | 29.57 | 37.70 | 40.63 |
|  |  |  | CE_test5 | 1582.41 | 29.28 | 37.69 | 40.61 |
|  | WestWindEasy | 32 | hm8.0 | 3746.74 | 36.19 | 44.73 | 47.42 |
|  |  |  | CE_test5 | 3746.94 | 36.03 | 44.69 | 47.49 |
|  |  | 37 | hm8.0 | 1674.61 | 33.01 | 43.71 | 46.64 |
|  |  |  | CE_test5 | 1682.62 | 32.82 | 43.65 | 46.68 |
|  |  | 41 | hm8.0 | 853.89 | 30.75 | 42.95 | 45.85 |
|  |  |  | CE_test5 | 874.62 | 30.53 | 43.04 | 45.94 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1282.95 | 35.09 | 40.11 | 39.74 |
|  |  |  | CE_test5 | 1294.48 | 34.91 | 40.07 | 39.70 |
|  |  | 37 | hm8.0 | 615.75 | 31.82 | 38.63 | 38.01 |
|  |  |  | CE_test5 | 626.67 | 31.50 | 38.59 | 37.88 |
|  |  | 41 | hm8.0 | 316.77 | 29.32 | 37.66 | 36.40 |
|  |  |  | CE_test5 | 322.66 | 29.32 | 37.65 | 36.34 |
| RA-Main | DucksTakeOff | 32 | hm8.0 | 9211.76 | 29.53 | 32.44 | 36.73 |
|  |  |  | CE_test5 | 9215.79 | 29.44 | 32.44 | 36.73 |
|  |  | 37 | hm8.0 | 3951.16 | 27.25 | 31.53 | 36.27 |
|  |  |  | CE_test5 | 3959.25 | 27.17 | 31.53 | 36.27 |
|  |  | 41 | hm8.0 | 1993.11 | 25.68 | 30.95 | 35.98 |
|  |  |  | CE_test5 | 2003.52 | 25.59 | 30.94 | 35.97 |
|  | RedKayak | 32 | hm8.0 | 3708.75 | 35.60 | 41.82 | 44.92 |
|  |  |  | CE_test5 | 3714.29 | 35.51 | 41.81 | 44.92 |
|  |  | 37 | hm8.0 | 1715.45 | 32.92 | 41.01 | 44.12 |
|  |  |  | CE_test5 | 1720.37 | 32.77 | 41.01 | 44.12 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.43 | 43.48 |
|  |  |  | CE_test5 | 857.61 | 30.79 | 40.40 | 43.45 |

Table4: Test6-CTC (typeⅠsequences)

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | Riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE_test6 | 5861.04 | 34.64 | 38.96 | 41.40 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | CE_test6 | 2873.90 | 31.72 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | CE_test6 | 1569.20 | 29.57 | 37.70 | 40.64 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | CE_test6 | 3761.39 | 36.22 | 44.71 | 47.49 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | CE_test6 | 1680.29 | 33.01 | 43.64 | 46.64 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | CE_test6 | 857.20 | 30.75 | 43.04 | 46.05 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | CE_test6 | 1270.66 | 35.30 | 40.23 | 40.20 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | CE_test6 | 611.47 | 31.97 | 38.68 | 38.39 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | CE_test6 | 317.21 | 29.39 | 37.76 | 36.83 |
| RA-Main | DucksTakeOff | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | CE_test6 | 9221.79 | 29.57 | 32.50 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | CE_test6 | 3958.52 | 27.27 | 31.56 | 36.26 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | CE_test6 | 1996.42 | 25.69 | 30.96 | 35.97 |
|  | RedKayak | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | CE_test6 | 3713.96 | 35.65 | 41.81 | 44.92 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | CE_test6 | 1717.33 | 32.94 | 41.01 | 44.11 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | CE_test6 | 854.47 | 30.97 | 40.42 | 43.49 |

Please refer to attached excel file for results of CTC with MaxTransformSize 16 with Offset 6(test1/3/5/6) and CTC (test6).

## Subjective performance

Test1 is testing the perfomance of beta table modification, while test3/5 is testing its combiantion with the other tools which are discussed in test2 and test4 respectively.

Since modified beta values lie out of QP range in CTC, and its motivition is to extend the ability of deblocking filter as supplement when default performance is not satisfactory, the tests are all carried out with offset 6 which is the upper limit of deblocking filter. In Figure 2 to 6 (4.2.1), it can be observed that comparing with HM8.0, modified beta value make more cases be covered.

With same principle, test3/5 is assigned to confirm the adaptivity when deblocking filter itself is changed as in test2/4. In Figure 7 to 9 (4.2.2), it can be observed that cooperating with beta can make more block boundaries be processed.

The ability to control the properties of deblocking filter by transmitting nonzero offsets provides the encoder designer with the ability to optimize the subjective quality of the decoded video beyond that provided by use of the default tables. The results show that the beta modification in CE1 can extend the deblocking filter’s adaptive range to cover different encoder cases.

Tool Intra smoothing restricted which is separated from deblocking filter part is tested in test6.

Test6 is carried out under CTC and the subjective results shows that improvement of viewing quality is not sufficient as shown in Figure 10 (4.2.3)

### Test1-Subjective performance

Figure2: anchor (left) and test1 (right)                                             Riverbed 1th frame QP=37, LB-MAIN, BetaOffset=6, TcOffset=6

Figure3: anchor (left) and test1 (right)                                        DucksTakeOff 46th frame QP=41, RA-MAIN, BetaOffset=6, TcOffset=6

Figure4: anchor (left) and test1 (right)

RedKayak 73th frame QP=37, LB-MAIN, BetaOffset=6, TcOffset=6

Figure5: anchor (left) and test1 (right)

WestWindEasy 6th frame QP=37, LB-MAIN, BetaOffset=6, TcOffset=6

Figure6: anchor (left) and test1 (right)

F.ChinaSpeed 8th frame QP=37, LB-MAIN, BetaOffset=6, TcOffset=6

### Test3/5-Subjective performance

Figure7: test2 (left) and test3 (right)                                            DucksTakeOff 18th frame QP=37, LB-MAIN, BetaOffset=6, TcOffset=6

Figure8: test2 (left) and test3 (right)                                           Westwindeasy 14th frame QP=37, LB-MAIN, BetaOffset=6, TcOffset=6

Figure9: test2 (left) and test3 (right)                                           F.ChinaSpeed 49th frame QP=37, LB-MAIN, BetaOffset=6, TcOffset=6

### Test6-subjective performance

Figure10: HM8.0 (left) and test6 (right)

Riverbed 1th frame QP=37, LB-MAIN

# Conclusion

Several tests have been performed to confirm the effect of modified beta combining with both HM8.0 and tools proposed in CE1. The results show that the adaptive ability of deblocking filter can be extended with tested beta parameter. Therefore it enables the encoder designer to optimize the subjective quality with larger coverage range and flexibility. It is confirmed that proposed beta works effectively based on both HM8.0 and tools in CE1.

It is recommended to adopt this beta modification either as a revision of current HM8.0 or combining with tools in CE1.

# Proposed modification of Test 1

The red coloured values in the following table are proposed to be changed.

Table 8-1 – Derivation of threshold variables β′ and tC′ from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β′ | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC′ | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |  |  |  |
| β′ | 38 | 40 | 42 | 44 | 52 | 60 | 68 | 76 | 84 | 92 | 100 | 116 | 132 | 148 | - | - |  |  |  |
| tC′ | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 13 | 14 | 16 | 18 | 20 | 22 | 24 |  |  |  |

# Reference

[1] JCTVC-J1101, Description of Core Experiment 1: Deblocking filter

[2] JCTVC-J1100, HM 8 Common Test Conditions and Software Reference Configurations

[3] JCTVC-J0181, AHG6: On deblocking filter parameters

[4] JCTVC-Kxxx, CE1: Reduction of block artifacts in HEVC

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).