| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0150 |
| --- | --- |

| Title: | NonCE1: Simple improvement of Deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuo Lu Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Shuo.Lu@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

Severe block noises in some sequences are discussed during last meeting. This document proposes an adaption on thresholds usage to address the problem without changes on the current filter design since it has been proved rational and effective during all the past tests. Test results show that proposed thresholds can reduce block noises in the mentioned sequences while keeping even improving the quality of sequences in CTC. Changes on BD-rate comparing with HM8.0 are (0.1%, 0.0%, 0.0% and 0.0%) on Main profile and (0.2%, 0.1%, 0.1%, 0.0%) on HE10 configuration.

# Problem Statement

In the past 8th/9th/10th meeting, the blocky artifacts in sequences like Riverbed are reported and studied. In the last Stockholm meeting, some experts suggested to reduce blocky artifacts under default operation of deblocking filter.

## Introduction of Deblocking filter in HM

When using a block transform for residual coding, it is easier to get larger coding error near the block boundaries than in the interior of the block, which is called block noise if subjectively observed. Deblocking filter is designed to smooth these block boundaries. There are two types of filtering in HM, strong filter modifying three pixels of one side along the edges, and weak filter modifying two pixels.

To each edge between 8x8 blocks, a Boundary Strength (Bs) parameter is assigned a value from 0/1/2.Bs value determines the strength of the filtering performed on the edge. Bs = 0 means no filtering is applied on this specific edge. Bigger Bs means stronger filter is expected.

For edges with nonzero Bs, parameter tc and β is used in condition checks to make filtering decisions as follow.

dp0 = | p2,0 − 2*p1,0 + p0,0 |;  dq0 = | q2,0 − 2*q1,0 + q0,0 |

dp3 = | p2,3 − 2*p1,3 + p0,3 |;  dq3 = | q2,3 − 2*q1,3 + q0,3 |

(s1)Filtering on an edge only takes place when

(dp0+dq0+dp3+dq3) < β	(1)

Based on the result of (s1), the following conditions need to be checked further to determine the filtering type, strong filter or weak filter.

(s2)Strong filter will be chosen if the following conditions all hold:

2*(dpi+dqi) < (β >> 2);	(2)

| p3,i – p0,i | + | q0,i – q3,i | < (β >>3);	(3)

| p0,i – q0,i | < ((5*tc + 1) >> 1) where i =0,3.	(4)

Figure 1 block boundary

β value is used to evaluate the interior change of each block and tc value is used to evaluate the change along the edge. Comparing (1) and (2), one can see that (1) checks the continuity of the unit while (2) checks the continuity of each line with a tighter constraint. Condition (3) is to make sure that the strength of filtering is greater only when small changes in intensity occur on both side of the edge. Condition (4) checks changes in intensity along the edge. In current algorithm, if one of the blocks is intra-coded, Bs of this specific edge will be assigned to be 2. And a default offset value of 2 is used to derive tc to make strong filter easily be chosen.

## Problem Analysis

Figure 2 shows one sample of how deblocking filter is applying in sequence “Riverbed” under the encode condition of LB-Main/QP37. Green lines mark where weak filter are applied, and red lines mark where strong filters are applied. The other places are not filtered. It is observed that most of block noises lie on boundaries with no filter or weak filter. On such edges, if strong filter is chosen instead of weak filter, as shown in Figure 3, blocky artifacts can be reduced as expected. Thus the problem is that filter decision is not proper with current strong filter checking conditions.

Figure 2 HM8.0 Riverbed LB-Main QP37 frame1 (one intra-coded part)

Figure 3 Only strong filter (same position as in Figure2)

Two causes of the improper decision are studied:

First, for intra-coded blocks, deriving tc with offset 2 is not enough to lead to a stronger filter. As shown in Figure4, (a) shows the result without default tc offset for intra-coded blocks. Comparing with HM8.0, there is no obvious increment of the amount of strong filter at the intra boundaries. This’s because increment of the threshold value in (4) is not enough. For example, tc is changed from 4 to 5 at QP37 and sequentially the threshold value in (4) is only changed from 10 to 13. As shown in Figure5, the influence does not cover a big amount of condition (4) check. Also it does not deal with the failure of condition (2) and (3) which is essential for choosing strong filter.

Second, constrains in strong filter conditions are so tight that most edges are eliminated. As shown in Figure6, the left part of red line shows all the true cases of the particular condition, while the right part of red line shows all the false cases of the particular condition of current HM8.0. Although this is not a precise counting since the positions are not considered, it illustrates that for all three conditions, the proportions of true to false is low which is consistent with outcome of final decisions in Figure 7(a).

Figure4 Without default tc offset (same position as in Figure2)

## Proposal on Deblocking filter

As a conclusion of the above analysis, the following solution is proposed to satisfy two principles that Bs should be further associated with the choice of filtering outcome, and constrains in strong filter conditions should be relaxed:

Instead of using table-derived β and tc value directly, (Bs+1)* β and (Bs+1)* tc is passed to the strong filter decision function as follow:

Comparing with HM8.0, the proposed change is highlighted in yellow.

HM8.0:

2*(dpi+dqi) < (β >> 2);

| p3,i – p0,i | + | q0,i – q3,i | < (β >>3);

| p0,i – q0,i | < ((5*tc + 1) >> 1) where i =0,3.

Proposal:

β = (Bs+1)*β; tc = (Bs+1)*tc			    (5)

2*(dpi+dqi) < (β >> 2);

| p3,i – p0,i | + | q0,i – q3,i | < (β >>3);

| p0,i – q0,i | < ((5*tc + 1) >> 1) where i =0,3.

Proposal can make filtering decision according to the character of input. As shown in Figure5, since obvious blocky noises appears in Riverbed, proposal can make strong filter be chosen at such boundaries as 5(b), and better visual quality can be achieved as shown in 5(a).

As shown in Figure6, the left part of green line shows all the true cases of the particular condition, while the right part of green line shows all the false cases of the particular condition of proposal. It can be observed that condition (4) and (2) can eliminate worse cases as shown in 6(a) and 6(b) respectively, while a tighter condition (3), which exists specially in strong filter checking, can pick up edges only when strong filter is necessary as shown in 6(c).

For sequences like Kimono@QP32, since blocky noises are not severe, the amount of strong filter is constrained as shown in Figure8 and Figure8.

(a)                                   (b)

Figure5 Proposal (same position as in Figure2)

(a)                                       (b)

(c)

Figure6 histogram of quantized strong filter condition checking: Riverbed,QP37,LB-Main,frame 1

(a)HM8.0

(b)Proposal

Figure7 filtering outcome of Riverbed LB-Main QP37 frame1 (whole frame )

(a)                                        (b)

(c)

Figure8 histogram of quantized strong filter condition checking: Kimono, QP32, LB-Main, frame1

(a)HM8.0

(b)Proposal

Figure9 filtering outcome of Kimono LB-Main QP32 frame1 (whole frame)

# Results

## Objective performance

### Common Test Conditions

The results show that averagely there is 0.1% loss at AI-Main and 0.2% loss at AI-HE10 configuration. Overall proposal has stable performance and no big loss is introduced.

Table1 CTC results of proposal vs. HM8.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% |
| Class B | 0.2% | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  | 105% |  |  |
| Dec Time[%] | 103% |  |  | 96% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.3% | 0.0% | 0.0% | -0.3% |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.2% | 0.0% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 107% |  |  | 103% |  |  |
| Dec Time[%] | 93% |  |  | 87% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.1% | 0.1% | 0.2% |
| Class C | 0.1% | -0.1% | 0.2% | 0.1% | 0.2% | 0.1% |
| Class D | 0.0% | 0.2% | -0.2% | 0.0% | -0.3% | -0.3% |
| Class E | 0.1% | 0.3% | 0.7% | 0.3% | 0.2% | -0.2% |
| Class F | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | -1.0% |
| Overall | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | -0.2% |
|  | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 106% |  |  | 103% |  |  |
| Dec Time[%] | 92% |  |  | 83% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.1% | 0.0% | -0.1% | 0.2% | -0.5% |
| Class C | 0.0% | 0.0% | 0.2% | 0.0% | -0.2% | -0.1% |
| Class D | 0.0% | 0.5% | 0.0% | 0.1% | -0.2% | 0.0% |
| Class E | -0.2% | 0.2% | -0.2% | -0.2% | -0.3% | 0.0% |
| Class F | -0.1% | -0.2% | 0.8% | -0.1% | 1.0% | -0.1% |
| Overall | -0.1% | 0.1% | 0.2% | -0.1% | 0.1% | -0.2% |
|  | -0.1% | 0.1% | 0.2% | -0.1% | 0.0% | -0.2% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 89% |  |  | 82% |  |  |

Note: Current Enc/Dec Time is not precise

### Sequences in CE1

In CE1, two types of sequences are identified to evaluate the performance of deblocking filter. TypeⅠ sequences including riverbed/WestWindEasy/F.ChinaSpeed/DucksTakeOff(25fps)/Redkayak(570f), which with current HM8.0 severe block noises are observed, are used to demonstrate improvement in visual quality. TypeⅡsequences including B.Kimono/B.Cactus/F.BasketballDrillText/C.BQMall/E.Jonny are used to make sure that no visual degradation in Common Test Condition due to the change of deblocking filter.

Detailed objective performance of these two types of sequences is reported as below. The results show that proposal can keep or improve the objective evaluations of both types of sequences.

#### TypeⅠsequences in CE1

Table2: objective performance of typeⅠsequences in CE1

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | Riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | Proposal | 5850.89 | 34.64 | 38.96 | 41.40 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | Proposal | 2868.57 | 31.74 | 38.12 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | Proposal | 1564.63 | 29.58 | 37.69 | 40.63 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | Proposal | 3751.72 | 36.22 | 44.72 | 47.45 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | Proposal | 1674.61 | 33.02 | 43.72 | 46.65 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | Proposal | 853.37 | 30.77 | 42.96 | 45.83 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | Proposal | 1269.18 | 35.30 | 40.22 | 40.19 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | Proposal | 610.89 | 31.97 | 38.75 | 38.42 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | Proposal | 317.09 | 29.40 | 37.75 | 36.84 |
| RA-Main | DucksTakeOff | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | Proposal | 9215.33 | 29.57 | 32.50 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | Proposal | 3954.29 | 27.27 | 31.56 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | Proposal | 1993.78 | 25.70 | 30.96 | 35.98 |
|  | RedKayak | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | Proposal | 3707.87 | 35.65 | 41.82 | 44.91 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | Proposal | 1713.79 | 32.95 | 41.01 | 44.14 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | Proposal | 853.64 | 30.97 | 40.42 | 43.48 |

#### TypeⅡsequences in CE1

Table3: objective performance of typeⅡsequences in CE1

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | B.Kimono1 | 27 | hm8.0 | 2400.17 | 39.66 | 41.62 | 42.80 |
|  |  |  | Proposal | 2399.51 | 39.66 | 41.62 | 42.80 |
|  |  | 32 | hm8.0 | 1150.22 | 37.14 | 40.32 | 41.48 |
|  |  |  | Proposal | 1149.96 | 37.14 | 40.31 | 41.48 |
|  |  | 37 | hm8.0 | 562.99 | 34.54 | 39.50 | 40.80 |
|  |  |  | Proposal | 562.07 | 34.54 | 39.48 | 40.79 |
|  | B.Cactus | 27 | hm8.0 | 5722.74 | 36.70 | 38.88 | 41.41 |
|  |  |  | Proposal | 5719.30 | 36.70 | 38.88 | 41.42 |
|  |  | 32 | hm8.0 | 2569.40 | 34.56 | 37.92 | 39.77 |
|  |  |  | Proposal | 2568.34 | 34.56 | 37.92 | 39.77 |
|  |  | 37 | hm8.0 | 1267.91 | 32.26 | 37.18 | 38.58 |
|  |  |  | Proposal | 1264.80 | 32.25 | 37.18 | 38.56 |
|  | F.BasketballDrillText | 27 | hm8.0 | 1822.96 | 37.03 | 39.68 | 40.05 |
|  |  |  | Proposal | 1824.79 | 37.02 | 39.67 | 40.04 |
|  |  | 32 | hm8.0 | 880.51 | 34.00 | 37.36 | 37.63 |
|  |  |  | Proposal | 880.15 | 33.99 | 37.38 | 37.64 |
|  |  | 37 | hm8.0 | 456.94 | 31.31 | 35.76 | 35.96 |
|  |  |  | Proposal | 456.12 | 31.31 | 35.79 | 35.91 |
|  | C.BQMall | 27 | hm8.0 | 1869.00 | 37.24 | 40.91 | 41.99 |
|  |  |  | Proposal | 1869.19 | 37.24 | 40.90 | 41.97 |
|  |  | 32 | hm8.0 | 902.28 | 34.29 | 39.12 | 39.99 |
|  |  |  | Proposal | 902.06 | 34.28 | 39.14 | 40.02 |
|  |  | 37 | hm8.0 | 457.40 | 31.38 | 37.83 | 38.61 |
|  |  |  | Proposal | 457.22 | 31.38 | 37.81 | 38.63 |
|  | E.Johnny | 27 | hm8.0 | 419.06 | 41.14 | 46.15 | 46.80 |
|  |  |  | Proposal | 418.24 | 41.14 | 46.15 | 46.78 |
|  |  | 32 | hm8.0 | 185.82 | 39.00 | 44.54 | 45.12 |
|  |  |  | Proposal | 186.11 | 38.98 | 44.49 | 45.07 |
|  |  | 37 | hm8.0 | 97.17 | 36.49 | 43.15 | 43.59 |
|  |  |  | Proposal | 97.33 | 36.48 | 43.19 | 43.62 |

## Subjective performance

Based on the discussion of CE1, subjective study of both typeⅠand typeⅡsequences are reported. In 2.2.1, first typeⅠsequence are studied to show that proposal reduces block noises more effectively than current HM8.0. In 2.2.2, then typeⅡsequences are studied to make sure that no visual degradation is introduced in Common Test Conditions.

As shown in Figure 10 - Figure 17, it has been found that proposal generally has better or close quality comparing with HM8.0. The improvement is observed in most sequences of typeⅠand even sequences of typeⅡ. Also, block reduction can be observed when maximum transform size is limited to 16x16 as shown in Figure 14 - Figure 15.

### TypeⅠsequences in CE1

Figure10 hm8.0(left), proposal(right): RedKayak RA-Main QP37 frame 393

Figure11 hm8.0(left), proposal(right): Riverbed LB-Main QP37 frame 45

Figure 12 hm8.0(left), proposal(right): WestWindEasy LB-Main QP37 frame 26

Figure13 hm8.0(left), proposal(right): DucksTakeOff RA-Main QP37 frame 28

Figure 14 hm8.0(left), proposal(right): F.ChinaSpeed LB-Main QP37 MaxTrformSize=16 frame35

Figure15 hm8.0(left), proposal(right): Riverbed LB-Main QP37 MaxTrformSize=16 frame 83

### TypeⅡsequences in CE1

Figure16 hm8.0(left), proposal(right): B.Cactus LB-Main QP37 frame 2

Figure17 hm8.0(left), proposal(right): F.BasketballDrillText LB-Main QP37 frame 134

### Comparing with results in CE1

In last 10th Stockholm meeting, CE1 is set up to discuss tools on deblocking filter. These tools share the same goal of this proposal, that to increase the amount of strong filter. As described in JCTVC-Kxxx, the goal is achieved through modifying the strong filter condition and filter shape to turn on strong filter at intra blocks or modifying Bs value at CU boundaries or TU boundaries.

Figure 18 shows the filtering outcome of both proposal and test2 in CE1 (as represent). One can see that generally similar reduction of blocking artifects can be achieved.

(a)HM8.0

(b)Proposal on delbocking filter                  (c) test2 in CE1

Figure18 Proposal on deblocking filter vs. test2 in CE1 (same position as in Figure2)

# Combination with intra smoothing restriction in CE1

In test6 of CE1, restriction on intra smoothing is tested. Though it is not directly connected with deblocking filter, it reduces the intra-prediction filtering in DC, vertical and horizontal modes to blocks of size 16x16, 8x8 and 4x4 which influences the input of deblocking filter (JCTVC_J0286). Therefore the combination of this tool with the above proposal on deblocking fitler is further studied.

According to the results reported in JCTVC_K0149, this tool itself cannot bring significant improvement on subjective quality. However, the results show that the combination of these two tools can reduce blocky artifacts effectively, and better visual quality can be observed at some places in the tested sequences as shown in Figure 19-21.

## Objective performance of combination

### Common Test Conditions

The results show that averagely there is 0.1% loss at AI-Main and 0.2% loss at AI-HE10 configuration. Therefore, the combination has no harmful side influence on objective effectiveness.

Table4 CTC results of combination vs. HM8.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.1% | 0.2% | 0.3% | 0.2% | 0.2% |
| Class B | 0.2% | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% |
| Class C | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
|  | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
| Enc Time[%] | 104% |  |  | 115% |  |  |
| Dec Time[%] | 97% |  |  | 105% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.1% | 0.1% | 0.0% | -0.2% |
| Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | -0.1% |
| Class D | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.3% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | 0.1% | 0.1% | 0.1% |
| Overall | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | -0.1% |
|  | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 104% |  |  | 109% |  |  |
| Dec Time[%] | 90% |  |  | 91% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.3% | 0.1% | 0.1% | 0.3% |
| Class C | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% |
| Class D | 0.0% | 0.5% | 0.1% | 0.1% | -0.6% | 0.3% |
| Class E | 0.2% | 0.3% | 0.9% | 0.1% | 0.2% | -0.3% |
| Class F | 0.0% | 0.0% | -0.1% | 0.1% | 0.9% | -0.8% |
| Overall | 0.0% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% |
|  | 0.0% | 0.2% | 0.2% | 0.1% | 0.1% | -0.1% |
| Enc Time[%] | 105% |  |  | 109% |  |  |
| Dec Time[%] | 90% |  |  | 87% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.3% | 0.4% | -0.1% | 0.2% | -0.1% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.2% | 0.0% | 0.0% | -0.2% | 0.0% |
| Class E | -0.2% | 1.1% | -0.4% | -0.2% | -0.2% | 0.3% |
| Class F | 0.1% | 0.1% | 0.5% | 0.2% | 0.5% | -0.2% |
| Overall | 0.0% | 0.3% | 0.2% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.3% | 0.2% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  | 108% |  |  |
| Dec Time[%] | 93% |  |  | 85% |  |  |

### Sequences in CE1

Table5: combination: objective performance of typeⅠsequences in CE1

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | Riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | combination | 5857.27 | 34.65 | 38.96 | 41.40 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | combination | 2869.09 | 31.74 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | combination | 1566.48 | 29.59 | 37.69 | 40.63 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | combination | 3756.13 | 36.22 | 44.71 | 47.50 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | combination | 1676.92 | 33.03 | 43.65 | 46.65 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | combination | 853.63 | 30.75 | 43.05 | 46.04 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | combination | 1270.78 | 35.29 | 40.24 | 40.19 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | combination | 611.09 | 31.97 | 38.68 | 38.41 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | combination | 316.91 | 29.40 | 37.75 | 36.85 |
| RA-Main | DucksTakeOff | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | combination | 9218.29 | 29.57 | 32.49 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | combination | 3957.53 | 27.27 | 31.56 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | combination | 1997.35 | 25.70 | 30.96 | 35.97 |
|  | RedKayak | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | combination | 3713.00 | 35.66 | 41.82 | 44.91 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | combination | 1715.73 | 32.95 | 41.01 | 44.13 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | combination | 854.00 | 30.98 | 40.42 | 43.48 |

Table6: combination: objective performance of typeⅡsequences in CE1

| Condition | Sequence | QP | Item | Bitrate [kbps] | Ypsnr [db] | Upsnr [dB] | Vpsnr [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LB-Main | B.Kimono1 | 27 | hm8.0 | 2400.17 | 39.66 | 41.62 | 42.80 |
|  |  |  | combination | 2400.76 | 39.67 | 41.62 | 42.81 |
|  |  | 32 | hm8.0 | 1150.22 | 37.14 | 40.32 | 41.48 |
|  |  |  | combination | 1148.42 | 37.14 | 40.31 | 41.47 |
|  |  | 37 | hm8.0 | 562.99 | 34.54 | 39.50 | 40.80 |
|  |  |  | combination | 562.34 | 34.55 | 39.47 | 40.78 |
|  | B.Cactus | 27 | hm8.0 | 5722.74 | 36.70 | 38.88 | 41.41 |
|  |  |  | combination | 5719.00 | 36.70 | 38.88 | 41.40 |
|  |  | 32 | hm8.0 | 2569.40 | 34.56 | 37.92 | 39.77 |
|  |  |  | combination | 2567.43 | 34.56 | 37.92 | 39.77 |
|  |  | 37 | hm8.0 | 1267.91 | 32.26 | 37.18 | 38.58 |
|  |  |  | combination | 1265.17 | 32.25 | 37.15 | 38.54 |
|  | F.BasketballDrillText | 27 | hm8.0 | 1822.96 | 37.03 | 39.68 | 40.05 |
|  |  |  | combination | 1825.50 | 37.03 | 39.66 | 40.02 |
|  |  | 32 | hm8.0 | 880.51 | 34.00 | 37.36 | 37.63 |
|  |  |  | combination | 879.69 | 33.98 | 37.37 | 37.65 |
|  |  | 37 | hm8.0 | 456.94 | 31.31 | 35.76 | 35.96 |
|  |  |  | combination | 456.14 | 31.30 | 35.76 | 35.97 |
|  | C.BQMall | 27 | hm8.0 | 1869.00 | 37.24 | 40.91 | 41.99 |
|  |  |  | combination | 1869.05 | 37.24 | 40.91 | 41.99 |
|  |  | 32 | hm8.0 | 902.28 | 34.29 | 39.12 | 39.99 |
|  |  |  | combination | 902.26 | 34.28 | 39.12 | 40.04 |
|  |  | 37 | hm8.0 | 457.40 | 31.38 | 37.83 | 38.61 |
|  |  |  | combination | 457.42 | 31.39 | 37.85 | 38.61 |
|  | E.Johnny | 27 | hm8.0 | 419.06 | 41.14 | 46.15 | 46.80 |
|  |  |  | combination | 418.85 | 41.14 | 46.15 | 46.80 |
|  |  | 32 | hm8.0 | 185.82 | 39.00 | 44.54 | 45.12 |
|  |  |  | combination | 186.23 | 38.99 | 44.44 | 44.98 |
|  |  | 37 | hm8.0 | 97.17 | 36.49 | 43.15 | 43.59 |
|  |  |  | combination | 96.91 | 36.43 | 43.23 | 43.59 |

## Subjective performance of combination

### TypeⅠsequences in CE1

Figure19 proposal (left), combination(right): riverbed LB-Main QP37 frame 1

Figure20 proposal (left), combination (right): duckstakeoff RA-Main QP37 frame 95

Figure21 proposal (left), combination (right): WestWindEasy LB-Main QP37 frame 63

Figure22 proposal (left), combination (right): WestWindEasy LB-Main QP37 MaxTU16 frame 55

### TypeⅡsequences in CE1

Figure22 proposal (left), combination (right): B.Cactus LB-Main QP32 frame 91

# Conclusion

An adaptation on thresholds in deblocking filter is proposed to reduce the block artifacts in sequences like Riverbed/WestWindEasy etc. The motivation is to make filter decision according to the characters of input video. It optimized the usage of current deblocking filter instead of changing the design. And as shown in section6 (WD changes), proposal only adds computation of beta and tc value. No other condition checking or computing complexity is introduced. For combination with intra-smoothing constrain tool, the change part on that tool would be also added.

The subjective results show that block artifacts mentioned above can be reduced with proposed optimization, while no quality degradation on sequences from Common Test Condition. The objective results show that there is 0.1% and 0.2% loss at AI-Main and AI-HE10 respectively, and has negligible impact on coding efficiency at the other configurations.

It is recommended to adopt this proposal to the specification text and to the next version of HM software.

# Reference

[1] JCTVC-J1101, Description of Core Experiment 1: Deblocking filter

[2] JCTVC-J1100, HM 8 Common Test Conditions and Software Reference Configurations

[3] JCTVC-J0181, AHG6: On deblocking filter parameters

[4] JCTVC-K0149, CE1: On deblocking filter parameters

[5] JCTVC-K0186, CE1: Reduction of block artifacts in HEVC

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

## WD changes of proposal on deblocking filter

###### 8.7.2.4.3 Decision process for luma block edges

Abbrv.

- When d is less than β, the following ordered steps apply:

- The variable dqp is set equal to 2*dpq0.

- For the sample location ( xC + xB, yC + yB ), the decision process for a luma sample as specified in subclause Error: Reference source not found is invoked with sample values pi,0, qi,0 with i = 0..3, the variables dpq, (bS+1)*β and (bS+1)*tC as inputs and the output is assigned to the decision dSam0.

- The variable dqp is set equal to 2*dpq3.

- For the sample location ( xC + xB, yC + yB + 3), the decision process for a luma sample as specified in subclause Error: Reference source not found is invoked with sample values pi,3, qi,3 with i = 0..3, the variables dpq, (bS+1)*β and (bS+1)*tC as inputs and the output is assigned to the decision dSam3.

- The variable dE is set equal to 1.

- When dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- When dp is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp is set equal to 1.

- When dq is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq is set equal to 1.

## WD changes of intra smoothing tool in CE1 (Same as JCTVC-K0186)

##### 8.4.4.2.5 Specification of Intra_DC (1) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process are:

–	predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as the following ordered steps:

- A variable dcVal is derived as:

dcVal = x'=0nS−1p[x'][−1]+y'=0nS−1p[−1][ y']+nS>>(k+1)	(9-1)
where k=Log2(nT)

- Depending on the chroma component index cIdx, the following applies.

- If cIdx is equal to 0 and Log2(nT) is less than X, the following applies.

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2*dcVal + p[ 0 ][ −1 ] + 2 ) >> 2	(9-2)
predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3*dcVal + 2 ) >> 2, with x = 1..nT−1	(9-3)
predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3*dcVal + 2 ) >> 2, with y = 1..nT−1	(9-4)
predSamples[ x ][ y ] = dcVal, with x, y = 1..nT−1	(9-5)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as

predSamples[ x ][ y ] = dcVal, with x, y = 0..nT−1	(9-6)

##### 8.4.4.2.6 Specification of Intra_Angular (2..34) prediction mode

…

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

  - The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( y + 1 )*intraPredAngle ) >> 5		(9-7)

iFact = ( ( y + 1 )*intraPredAngle ) && 31		(9-8)

  - Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ x+iIdx+1 ] + iFact*ref[ x+iIdx+2] + 16 ) >> 5	(9-9)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ref[ x+iIdx+1 ]		(9-10)

  - When intraPartMode is equal to Intra_Angular (26), Log2(nT) is less than X, and cIdx is equal to 0, the following filtering applies with x = 0, y = 0..nT−1.

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] – p[ −1 ][ −1 ] ) >> 1 ) )	(9-11)

–	Otherwise (intraPredMode is less than 18), the following ordered steps apply.

- The reference sample array ref[ x ], with x= −nT..2*nT is specified as follows.

ref[ x ] = p[ −1 ][ −1+x ], with x=0..nT		(9-12)

- If intraPredAngle is less than 0, the main reference sample array is extented as follows.

- When ( nT*intraPredAngle ) >>5 is less than −1,

ref[ x ] = p[ −1+( ( x*invAngle+128 )>>8 ) ][ −1 ], with x=( nT*intraPredAngle ) >>5..−1	(9-13)

- Otherwise,

ref[ x ] = p[ −1 ][ −1+x ], with x=nT+1..2*nT		(9-14)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

- The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( x + 1 )*intraPredAngle ) >> 5		(9-15)

iFact = ( ( x + 1 )*intraPredAngle ) && 31		(9-16)

- Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ y+iIdx+1 ] + iFact*ref[ y+iIdx+2] + 16 ) >> 5	(9-17)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ref[ y+iIdx+1 ]		(9-18)

- When intraPartMode is equal to Intra_Angular (10), Log2(nT) is less than X, and cIdx is equal to 0, the following filtering applies with x = 0..nT−1, y = 0.

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] – p[ −1 ][ −1 ] ) >> 1 )	(9-19)