| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0151 |
| --- | --- |

| Title: | CE1:Cross check of test7 in CE1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shuo Lu Ohji Nakagami Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Shuo.Lu@jp.sony.com ohji.nakagami@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document reports the cress-check results of test7 in CE1 (JCTVC_J1101), which studies the performance of New Strong Filter + Decisions + Bs Modification on CU boundary. It is confirmed that source code exactly reflects the proposal. BD-rate matches with what provided by the proposer too.

# Introduction

In the 10th Stockholm meeting, CE1 was set up to study the proposals on reducing block artifacts. Test 7 is assigned to study the performance of deblocking filter related tools as below:

(1)Modification of strong filter decisions

HM8.0:

(|p3i - p0i| + |q0i – q3i|) < (β>>3)

((| p2,i − 2*p1,i + p0,i | + | q2,i − 2*q1,i + q0,i |) <<1) < (β>>2)

|p0i – q0i| < ((5*tC + 1)>>1)

Test7:

((| p3,i − 2*p2,i + p1,i | + | q3,i − 2*q2,i + q1,i |) <<1) < (β>>2)

((| p2,i − 2*p1,i + p0,i | + | q2,i − 2*q1,i + q0,i |) <<1) < (β>>2)

|p0i – q0i| < (tC<<4)

(2)Modification of strong filter

HM8.0:

p0’ = Clip3( p0−2*tc, p0+2*tc, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )   
q0’ = Clip3( q0−2*tc, q0+2*tc, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )
p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 ) 
q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )
q2’= Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )

Test7:

p0’ = Clip3( p0−2*tc, p0+2*tc, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )   
q0’ = Clip3( q0−2*tc, q0+2*tc, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )

tc2 = tc;

tc1 = (tc2*3+1)>>1;

tc3 = (tc2+1)>>1;

p1' = Clip3(p1-tc1, p1+tc1, ((p3 + 2*p2+ 2*p1 + p0 + 2*q0 + 4)>>3))          
q1' = Clip3(q1-tc1, q1+tc1, ((q3 + 2*q2+ 2*q1 + q0+2*p0 + 4)>>3))
p2' = Clip3(p2-tc3, p2+tc3, ((3*p3 + 3*p2 + p1 + q0 + 4 )>>3))
q2' = Clip3(q2-tc3, q2+tc3, ((p0 + q1 + 3*q2 + 3*q3 +4 )>>3))

(3)Boundary strength modification for CU

The derivation of the boundary strength is modified in a way that for the edges that have boundary strength Bs = 2, Bs can be increased to 3 in case of CU larger or equal to 32x32 when the block boundary is on the 32x32 filtering grid.

# Results

## Objective results

The Tests are run with the following Macro settings:

//CE1 deblocking

#define SONY_DEBLOCK_BETA_TAB			0

#define RESTRICT_INTRA_BOUNDARY_SMOOTHING	0

#if RESTRICT_INTRA_BOUNDARY_SMOOTHING

#define INTRA_FILT_MAX_BLOCK_SIZE			16

#endif

#define STRONG_FILTER_AND_DECISIONS			1

#define INCREASE_TC_OFFSET_INTRA			0

#define BS_MODIFICATION_CU				1

#define BS_MODIFICATION_MAX_TU			0

//end of CE1 deblocking

Type1 and Type2 sequences from CE1 are tested with different configuration as below:

CTC:

Common Test Conditions as defined in JCTVC-Jxxx

CTC with Offset6:

DeblockingFilterControlPresent	: 1

LoopFilterBetaOffset_div2		: 6

LoopFilterTcOffset_div2		: 6

CTC with MaxTransformSize 16:

QuadtreeTULog2MaxSize		: 4

CTC with MaxTransformSize 16 With Offset 6:

QuadtreeTULog2MaxSize		: 4

DeblockingFilterControlPresent	: 1

LoopFilterBetaOffset_div2		: 6

LoopFilterTcOffset_div2		: 6

CTC (type 1 sequences)

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9214.38 | 29.56 | 32.50 | 36.73 |
|  |  |  | CE1_test7 | 9213.32 | 29.57 | 32.49 | 36.73 |
|  |  | 37 | hm8.0 | 3954.07 | 27.26 | 31.55 | 36.27 |
|  |  |  | CE1_test7 | 3952.97 | 27.28 | 31.55 | 36.27 |
|  |  | 41 | hm8.0 | 1993.55 | 25.69 | 30.96 | 35.98 |
|  |  |  | CE1_test7 | 1992.91 | 25.70 | 30.96 | 35.98 |
|  | RedKayak,570fr | 32 | hm8.0 | 3709.59 | 35.65 | 41.82 | 44.91 |
|  |  |  | CE1_test7 | 3707.06 | 35.65 | 41.82 | 44.91 |
|  |  | 37 | hm8.0 | 1714.38 | 32.94 | 41.01 | 44.13 |
|  |  |  | CE1_test7 | 1712.61 | 32.95 | 41.01 | 44.13 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.42 | 43.49 |
|  |  |  | CE1_test7 | 852.47 | 30.97 | 40.42 | 43.47 |
| LB-Main | riverbed | 32 | hm8.0 | 5851.44 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE1_test7 | 5846.52 | 34.64 | 38.96 | 41.39 |
|  |  | 37 | hm8.0 | 2870.61 | 31.71 | 38.13 | 40.91 |
|  |  |  | CE1_test7 | 2865.69 | 31.74 | 38.13 | 40.90 |
|  |  | 41 | hm8.0 | 1566.17 | 29.55 | 37.69 | 40.63 |
|  |  |  | CE1_test7 | 1562.49 | 29.58 | 37.70 | 40.64 |
|  | WestWindEasy | 32 | hm8.0 | 3758.62 | 36.22 | 44.74 | 47.46 |
|  |  |  | CE1_test7 | 3753.43 | 36.22 | 44.72 | 47.46 |
|  |  | 37 | hm8.0 | 1679.87 | 33.01 | 43.71 | 46.65 |
|  |  |  | CE1_test7 | 1673.81 | 33.02 | 43.74 | 46.67 |
|  |  | 41 | hm8.0 | 854.87 | 30.75 | 42.93 | 45.86 |
|  |  |  | CE1_test7 | 852.04 | 30.77 | 42.93 | 45.85 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1270.60 | 35.30 | 40.24 | 40.18 |
|  |  |  | CE1_test7 | 1268.94 | 35.31 | 40.23 | 40.20 |
|  |  | 37 | hm8.0 | 612.05 | 31.97 | 38.75 | 38.42 |
|  |  |  | CE1_test7 | 611.08 | 31.97 | 38.75 | 38.41 |
|  |  | 41 | hm8.0 | 316.87 | 29.38 | 37.76 | 36.85 |
|  |  |  | CE1_test7 | 317.11 | 29.39 | 37.75 | 36.84 |

CTC (type 2 sequences)

| LB-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Kimono | 27 | hm8.0 | 2400.17 | 39.66 | 41.62 | 42.80 |
|  |  |  | CE1_test7 | 2398.92 | 39.66 | 41.62 | 42.81 |
|  |  | 32 | hm8.0 | 1150.22 | 37.14 | 40.32 | 41.48 |
|  |  |  | CE1_test7 | 1149.95 | 37.13 | 40.30 | 41.48 |
|  |  | 37 | hm8.0 | 562.99 | 34.54 | 39.50 | 40.80 |
|  |  |  | CE1_test7 | 561.94 | 34.55 | 39.49 | 40.78 |
|  | Cactus | 27 | hm8.0 | 5722.74 | 36.70 | 38.88 | 41.41 |
|  |  |  | CE1_test7 | 5723.00 | 36.70 | 38.88 | 41.42 |
|  |  | 32 | hm8.0 | 2569.40 | 34.56 | 37.92 | 39.77 |
|  |  |  | CE1_test7 | 2570.45 | 34.55 | 37.92 | 39.78 |
|  |  | 37 | hm8.0 | 1267.91 | 32.26 | 37.18 | 38.58 |
|  |  |  | CE1_test7 | 1266.31 | 32.23 | 37.18 | 38.58 |
|  | Bask.DrillText | 27 | hm8.0 | 1822.96 | 37.03 | 39.68 | 40.05 |
|  |  |  | CE1_test7 | 1824.58 | 37.03 | 39.69 | 40.05 |
|  |  | 32 | hm8.0 | 880.51 | 34.00 | 37.36 | 37.63 |
|  |  |  | CE1_test7 | 880.94 | 33.98 | 37.39 | 37.66 |
|  |  | 37 | hm8.0 | 456.94 | 31.31 | 35.76 | 35.96 |
|  |  |  | CE1_test7 | 456.00 | 31.30 | 35.77 | 35.96 |
|  | BQMall | 27 | hm8.0 | 1869.00 | 37.24 | 40.91 | 41.99 |
|  |  |  | CE1_test7 | 1869.14 | 37.24 | 40.91 | 41.97 |
|  |  | 32 | hm8.0 | 902.28 | 34.29 | 39.12 | 39.99 |
|  |  |  | CE1_test7 | 902.07 | 34.28 | 39.13 | 40.03 |
|  |  | 37 | hm8.0 | 457.40 | 31.38 | 37.83 | 38.61 |
|  |  |  | CE1_test7 | 457.68 | 31.38 | 37.86 | 38.60 |
|  | Johnny | 27 | hm8.0 | 419.06 | 41.14 | 46.15 | 46.80 |
|  |  |  | CE1_test7 | 417.80 | 41.14 | 46.19 | 46.78 |
|  |  | 32 | hm8.0 | 185.82 | 39.00 | 44.54 | 45.12 |
|  |  |  | CE1_test7 | 186.75 | 38.96 | 44.50 | 45.08 |
|  |  | 37 | hm8.0 | 97.17 | 36.49 | 43.15 | 43.59 |
|  |  |  | CE1_test7 | 97.40 | 36.45 | 43.17 | 43.58 |

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9211.76 | 29.53 | 32.44 | 36.73 |
|  |  |  | CE1_test7 | 9210.46 | 29.52 | 32.44 | 36.73 |
|  |  | 37 | hm8.0 | 3951.16 | 27.25 | 31.53 | 36.27 |
|  |  |  | CE1_test7 | 3946.07 | 27.25 | 31.53 | 36.27 |
|  |  | 41 | hm8.0 | 1993.11 | 25.68 | 30.95 | 35.98 |
|  |  |  | CE1_test7 | 1992.27 | 25.69 | 30.95 | 35.98 |
|  | RedKayak,570fr | 32 | hm8.0 | 3708.75 | 35.60 | 41.82 | 44.92 |
|  |  |  | CE1_test7 | 3707.88 | 35.58 | 41.82 | 44.93 |
|  |  | 37 | hm8.0 | 1715.45 | 32.92 | 41.01 | 44.12 |
|  |  |  | CE1_test7 | 1714.03 | 32.91 | 41.01 | 44.13 |
|  |  | 41 | hm8.0 | 852.89 | 30.96 | 40.43 | 43.48 |
|  |  |  | CE1_test7 | 852.91 | 30.96 | 40.43 | 43.48 |
| LB-Main | riverbed | 32 | hm8.0 | 5848.04 | 34.63 | 38.96 | 41.39 |
|  |  |  | CE1_test7 | 5841.63 | 34.56 | 38.96 | 41.39 |
|  |  | 37 | hm8.0 | 2864.81 | 31.73 | 38.12 | 40.90 |
|  |  |  | CE1_test7 | 2862.13 | 31.71 | 38.13 | 40.89 |
|  |  | 41 | hm8.0 | 1565.90 | 29.57 | 37.70 | 40.63 |
|  |  |  | CE1_test7 | 1562.24 | 29.57 | 37.69 | 40.62 |
|  | WestWindEasy | 32 | hm8.0 | 3746.74 | 36.19 | 44.73 | 47.42 |
|  |  |  | CE1_test7 | 3742.78 | 36.13 | 44.71 | 47.43 |
|  |  | 37 | hm8.0 | 1674.61 | 33.01 | 43.71 | 46.64 |
|  |  |  | CE1_test7 | 1669.42 | 32.98 | 43.73 | 46.69 |
|  |  | 41 | hm8.0 | 853.89 | 30.75 | 42.95 | 45.85 |
|  |  |  | CE1_test7 | 851.33 | 30.75 | 42.95 | 45.86 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1282.95 | 35.09 | 40.11 | 39.74 |
|  |  |  | CE1_test7 | 1283.76 | 35.07 | 40.09 | 39.81 |
|  |  | 37 | hm8.0 | 615.75 | 31.82 | 38.63 | 38.01 |
|  |  |  | CE1_test7 | 616.33 | 31.81 | 38.63 | 37.98 |
|  |  | 41 | hm8.0 | 316.77 | 29.32 | 37.66 | 36.40 |
|  |  |  | CE1_test7 | 317.70 | 29.30 | 37.63 | 36.39 |

CTC With Offset 6 (type1 sequences)

CTC With MaxTransformSize 16 (type1 sequences)

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9426.85 | 29.50 | 32.42 | 36.73 |
|  |  |  | CE1_test7 | 9426.97 | 29.51 | 32.42 | 36.73 |
|  |  | 37 | hm8.0 | 4076.49 | 27.17 | 31.52 | 36.23 |
|  |  |  | CE1_test7 | 4074.30 | 27.19 | 31.53 | 36.24 |
|  |  | 41 | hm8.0 | 2053.53 | 25.55 | 30.96 | 35.90 |
|  |  |  | CE1_test7 | 2051.50 | 25.57 | 30.96 | 35.91 |
|  | RedKayak,570fr | 32 | hm8.0 | 3758.10 | 35.47 | 41.61 | 44.50 |
|  |  |  | CE1_test7 | 3757.51 | 35.49 | 41.62 | 44.50 |
|  |  | 37 | hm8.0 | 1730.81 | 32.72 | 40.73 | 43.63 |
|  |  |  | CE1_test7 | 1729.72 | 32.73 | 40.74 | 43.65 |
|  |  | 41 | hm8.0 | 861.90 | 30.72 | 40.06 | 42.99 |
|  |  |  | CE1_test7 | 861.93 | 30.73 | 40.05 | 42.96 |
| LB-Main | riverbed | 32 | hm8.0 | 6047.81 | 34.46 | 38.98 | 41.23 |
|  |  |  | CE1_test7 | 6038.66 | 34.46 | 38.97 | 41.22 |
|  |  | 37 | hm8.0 | 2984.24 | 31.57 | 38.20 | 40.61 |
|  |  |  | CE1_test7 | 2973.21 | 31.59 | 38.20 | 40.61 |
|  |  | 41 | hm8.0 | 1618.58 | 29.40 | 37.66 | 40.15 |
|  |  |  | CE1_test7 | 1615.18 | 29.42 | 37.66 | 40.18 |
|  | WestWindEasy | 32 | hm8.0 | 3800.08 | 36.09 | 44.69 | 47.04 |
|  |  |  | CE1_test7 | 3795.48 | 36.09 | 44.68 | 47.04 |
|  |  | 37 | hm8.0 | 1708.76 | 32.92 | 43.33 | 45.94 |
|  |  |  | CE1_test7 | 1703.93 | 32.93 | 43.32 | 45.91 |
|  |  | 41 | hm8.0 | 871.40 | 30.65 | 42.65 | 45.05 |
|  |  |  | CE1_test7 | 868.36 | 30.66 | 42.66 | 45.02 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1281.75 | 35.25 | 40.20 | 40.13 |
|  |  |  | CE1_test7 | 1280.87 | 35.26 | 40.19 | 40.15 |
|  |  | 37 | hm8.0 | 618.77 | 31.92 | 38.61 | 38.38 |
|  |  |  | CE1_test7 | 617.58 | 31.93 | 38.60 | 38.37 |
|  |  | 41 | hm8.0 | 321.53 | 29.33 | 37.60 | 36.79 |
|  |  |  | CE1_test7 | 320.82 | 29.33 | 37.64 | 36.79 |

CTC With MaxTransformSize 16 (type2 sequences)

| LB-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Kimono | 27 | hm8.0 | 2452.67 | 39.59 | 41.65 | 42.85 |
|  |  |  | CE1_test7 | 2450.83 | 39.59 | 41.65 | 42.85 |
|  |  | 32 | hm8.0 | 1162.22 | 37.01 | 40.37 | 41.52 |
|  |  |  | CE1_test7 | 1162.30 | 37.01 | 40.35 | 41.51 |
|  |  | 37 | hm8.0 | 568.67 | 34.42 | 39.43 | 40.65 |
|  |  |  | CE1_test7 | 567.14 | 34.42 | 39.41 | 40.62 |
|  | Cactus | 27 | hm8.0 | 5827.61 | 36.68 | 38.88 | 41.41 |
|  |  |  | CE1_test7 | 5824.79 | 36.68 | 38.88 | 41.41 |
|  |  | 32 | hm8.0 | 2636.34 | 34.53 | 37.92 | 39.78 |
|  |  |  | CE1_test7 | 2635.66 | 34.52 | 37.92 | 39.77 |
|  |  | 37 | hm8.0 | 1305.55 | 32.22 | 37.14 | 38.50 |
|  |  |  | CE1_test7 | 1304.20 | 32.19 | 37.13 | 38.50 |
|  | Bask.DrillText | 27 | hm8.0 | 1843.11 | 36.91 | 39.66 | 40.05 |
|  |  |  | CE1_test7 | 1844.52 | 36.90 | 39.68 | 40.03 |
|  |  | 32 | hm8.0 | 890.14 | 33.96 | 37.39 | 37.67 |
|  |  |  | CE1_test7 | 890.27 | 33.91 | 37.41 | 37.69 |
|  |  | 37 | hm8.0 | 461.82 | 31.27 | 35.80 | 35.97 |
|  |  |  | CE1_test7 | 460.79 | 31.27 | 35.82 | 36.00 |
|  | BQMall | 27 | hm8.0 | 1880.60 | 37.23 | 40.87 | 41.94 |
|  |  |  | CE1_test7 | 1877.77 | 37.22 | 40.88 | 41.96 |
|  |  | 32 | hm8.0 | 909.97 | 34.25 | 39.11 | 40.01 |
|  |  |  | CE1_test7 | 909.02 | 34.25 | 39.12 | 40.00 |
|  |  | 37 | hm8.0 | 461.57 | 31.34 | 37.74 | 38.53 |
|  |  |  | CE1_test7 | 461.36 | 31.34 | 37.74 | 38.52 |
|  | Johnny | 27 | hm8.0 | 421.70 | 41.09 | 46.06 | 46.66 |
|  |  |  | CE1_test7 | 421.76 | 41.08 | 46.07 | 46.67 |
|  |  | 32 | hm8.0 | 186.77 | 38.88 | 44.34 | 44.76 |
|  |  |  | CE1_test7 | 187.73 | 38.84 | 44.36 | 44.76 |
|  |  | 37 | hm8.0 | 96.22 | 36.35 | 42.79 | 43.35 |
|  |  |  | CE1_test7 | 96.74 | 36.30 | 42.74 | 43.38 |

CTC With MaxTransformSize 16 with Offset 6 (type1 sequences)

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DucksTakeOff, 25fps | 32 | hm8.0 | 9420.71 | 29.44 | 32.34 | 36.73 |
|  |  |  | CE1_test7 | 9417.41 | 29.42 | 32.33 | 36.73 |
|  |  | 37 | hm8.0 | 4069.37 | 27.14 | 31.48 | 36.24 |
|  |  |  | CE1_test7 | 4067.43 | 27.14 | 31.48 | 36.24 |
|  |  | 41 | hm8.0 | 2051.17 | 25.54 | 30.93 | 35.91 |
|  |  |  | CE1_test7 | 2049.19 | 25.55 | 30.94 | 35.91 |
|  | RedKayak,570fr | 32 | hm8.0 | 3760.31 | 35.42 | 41.61 | 44.50 |
|  |  |  | CE1_test7 | 3757.49 | 35.39 | 41.62 | 44.51 |
|  |  | 37 | hm8.0 | 1729.76 | 32.70 | 40.73 | 43.62 |
|  |  |  | CE1_test7 | 1729.19 | 32.68 | 40.74 | 43.66 |
|  |  | 41 | hm8.0 | 862.36 | 30.72 | 40.06 | 42.98 |
|  |  |  | CE1_test7 | 861.59 | 30.72 | 40.06 | 42.99 |
| LB-Main | riverbed | 32 | hm8.0 | 6039.06 | 34.44 | 38.98 | 41.22 |
|  |  |  | CE1_test7 | 6026.69 | 34.33 | 38.97 | 41.24 |
|  |  | 37 | hm8.0 | 2976.38 | 31.59 | 38.20 | 40.61 |
|  |  |  | CE1_test7 | 2968.53 | 31.54 | 38.20 | 40.61 |
|  |  | 41 | hm8.0 | 1617.93 | 29.42 | 37.67 | 40.23 |
|  |  |  | CE1_test7 | 1611.34 | 29.40 | 37.66 | 40.21 |
|  | WestWindEasy | 32 | hm8.0 | 3787.16 | 36.05 | 44.66 | 47.07 |
|  |  |  | CE1_test7 | 3778.78 | 35.96 | 44.67 | 46.96 |
|  |  | 37 | hm8.0 | 1702.68 | 32.92 | 43.32 | 45.95 |
|  |  |  | CE1_test7 | 1696.45 | 32.87 | 43.34 | 45.95 |
|  |  | 41 | hm8.0 | 869.03 | 30.65 | 42.65 | 45.02 |
|  |  |  | CE1_test7 | 866.70 | 30.64 | 42.68 | 45.07 |
|  | F.ChinaSpeed | 32 | hm8.0 | 1295.91 | 35.05 | 40.08 | 39.73 |
|  |  |  | CE1_test7 | 1295.39 | 35.03 | 40.04 | 39.74 |
|  |  | 37 | hm8.0 | 623.37 | 31.76 | 38.51 | 37.98 |
|  |  |  | CE1_test7 | 623.54 | 31.75 | 38.49 | 37.97 |
|  |  | 41 | hm8.0 | 321.52 | 29.24 | 37.54 | 36.38 |
|  |  |  | CE1_test7 | 321.32 | 29.24 | 37.53 | 36.40 |

## Subjective results

Subjective quality is also studied. Generally it is observed that proposed tools can reduce the block noises slightly in some sequences, but the improvement is not good enough.

For sequence riverbed , WestWindEasy and DucksTakeOff, slightly better viewing quality can be observed, but cannot reach satisfactory level.

For sequence RedKayk(first 300 frames) and F.ChinaSpeed, similar results as anchor are observed.

| sequence | condition | Test7 vs. HM8.0, TU32x32 |
| --- | --- | --- |
| riverbed | LB-Main, QP37 | Better than anchor But still blocky noises remains |
| WestWindEasy | LB-Main, QP37 | Better than anchor at the moving grasses part, but at the background, the blocky noises remains. |
| F.ChinaSpeed | LB-Main, QP37 | Same as anchor |
| RedKayak | RA-Main, QP37 | Only the first 300 frames are evaluated, and have same quality as anchor |
| DucksTakeOff | RA-Main, QP37 | For still pictures, blocky noises are reduced. But for video, the effect is not obvious |
| riverbed | LB-Main, QP41 | Slightly better than anchor But still blocky noises remains Quality is not satisfactory |
| WestWindEasy | LB-Main, QP41 | Better than anchor at the moving grasses part, but at the background, blocky noises remains. |
| F.ChinaSpeed | LB-Main, QP41 | Same as anchor |
| RedKayak | RA-Main, QP41 | Only the first 300 frames are evaluated, and have same quality as anchor |
| DucksTakeOff | RA-Main, QP41 | Slightly better than anchor But still blocky noises remains especially in the later part of the sequence(only water surface part) Quality is not satisfactory |

# Patent rights declaration(s)

Sony Corporation does not have any current or pending patent rights relating to the technology described in this contribution.