| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0153 |
| --- | --- |

| Title: | GOP structures for Interlaced Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution compares the performance of HM8.0 for coding interlaced video in frame sequence and in field sequence with the 9 MPEG2 test sequences using both CTC GOP structures and proposed GOP structures. The CTC field GOP structures resulted in an average BDR of approximately 2% to 6% when compared to the CTC frame GOP structure as anchor. The proposed field GOP structures resulted in an average BDR of approximately -18% to -21%.

# Introduction

In JCTVC-J0466 [1], the coding efficiency of HM6.2 for coding interlaced video in frame sequence was compared with coding interlaced video in field sequence in random access configuration with a GOP size of 8. The GOP structure used in the comparisons is the common test conditions GOP structures except that the IntraPeriod was 96 for field sequences and 48 for frame sequences. JCTVC-J0466 demonstrated that some sequences, such as Mobile and Calendar, are significantly better to be encoded in frame sequence. And some sequences such as F1car are significantly better to be encoded in field sequence.

This contribution compares the performance of HM8.0 for coding interlaced video in frame sequence and in field sequence with the 9 MPEG2 test sequences. The common test conditions GOP structures are used in the comparison where IntraPeriod is approximately one second for frame sequences and field sequences. In particular, a GOP size of 8 is used for random access configurations, and a GOP size of 4 is used for low delay configurations.

This contribution also proposes new GOP structures for encoding video by HM in field sequences. For the 9 MPEG2 test sequences, when a video is more efficiently encoded in fields than in frames, the new GOP structures maintain the coding efficiency of fields. When a video is less efficiently encoded in fields than in frames, the new GOP structures reduced the coding efficiency gap between field sequence and frame sequence.

# Common test condition GOP structure

Figure 1 shows the reference picture list of the random access GOP structure specified in the common test condition (CTC) configuration files. In each row, the green box represents the current picture. The number inside the green box is the POC of the current picture. The yellow boxes in the same row are the corresponding pictures in the reference picture list. For example, at encoding order 2, the reference picture list of picture POC 4 is {POC 0, POC 8}. At encoding order 3, the reference list of picture POC 2 is {POC 0, POC4, POC8}. At encoding order 4, the reference list of picture POC 1 is {POC 0, POC 2, POC4, POC 8} but only POC 0, POC 2, POC 4 are active. This GOP structure has 8 pictures. The reference list in the second GOP is general, and it is used to define the reference pictures in encoder_randomaccess_main.cfg and encoder_randomaccess_he10.cfg.

One issue with the CTC random access GOP structure for field sequence coding is that the all reference pictures are even POC pictures. In other words, odd fields cannot be predicted from any odd field.

Similarly, Figure 2 shows the reference picture list of the low delay GOP structure specified in the CTC configuration files. It has a GOP size of 4 pictures. The reference list in the fifth GOP is general, and it is used to define the reference pictures in encoder_lowdelay_main.cfg,  encoder_lowdealy_he10.cfg, encoder_lowdelay_P_main.cfg, and encoder_lowdelay_P_he10.cfg.

One issue with the CTC low delay GOP structure for field sequence coding is that the reference picture list of an odd POC picture has only even POC pictures. In other words, odd fields cannot be predicted from any odd field.

Figure 1: Common test condition random access GOP structure. The third GOP is general, it is used to define the coding structure in the random access HM configuration files.

Figure 2: Common test condition low delay GOP structure. The fifth GOP is general, it is used to define the coding structure in the low delay HM configuration files.

# Proposed GOP structures for field sequences

To improve the coding efficiency of field sequences, we propose that any GOP structures, including depth-first and breadth-first GOP structures, for field sequence coding by HEVC should have both even and odd fields in the reference list. In particular this contribution proposes the GOP structure in Figure 3 for random access field sequences and the GOP structure in Error: Reference source not found for low delay field sequences.

As shown in Figure 3, the proposed GOP structure for random access field sequences has a GOP size of 16 field pictures. In contrast, the CTC GOP structure for random access field sequence has a GOP size of 8 pictures.

As shown in Error: Reference source not found, the proposed GOP structure for low delay field sequences has a GOP size of 8 field pictures. In contrast, the CTC GOP structure for low delay field sequence has a GOP size of 4 pictures.

# Proposed modifications to HM8.0

Since the proposed GOP structures for field sequence have twice the number of reference pictures in the reference picture list as the corresponding CTC GOP structures, the HM8.0 ran into memory allocation problems. In order for HM8.0 to support the proposed GOP structures, more memory was allocated to HM8.0 to support longer reference picture list as shown in the Appendix A. No other changes were made to HM8.0.

Figure 3: Proposed random access GOP structure for field sequences. The third GOP is general, it is used to define the coding structure in the random access HM configuration files.

Figure 4: Proposed low delay GOP structure for field sequences. The fifth GOP is general, it is used to define the coding structure in the low delay HM configuration files.

# Simulation Results

Table 1 compares the performance of HM8.0 for coding interlaced video in frame sequence and in field sequence with the 9 MPEG2 test sequences using the CTC configurations and the proposed configurations. The MPEG 2 test sequences are 150 frames (300 fields) long except the car sequence which is 140 frames (280 fields) long. The anchor is encoded with CTC GOP structure and frame sequences.

As shown in Table 1, the CTC field GOP structures resulted in average BDR of approximately 2% to 6% when compared to the CTC frame GOP structure. The proposed field GOP structures resulted in average BDR of approximately -18% to -21%.

In one extreme case, the BDR of mobile (mobile & calendar) with CTC field GOP structure is approximately 103% to 117% relative to CTC frame GOP structure. In contrast, the BDR of mobile (mobile & calendar) with proposed field GOP structure is approximately 5% to 9% relative to CTC frame GOP structure.

In summary, when CTC field GOP structure is better than CTC frame GOP structure, the propose field GOP structure has similar performance as the CTC field GOP structure. When the CTC field GOP structure is worse than the CTC frame GOP structure (i.e. flower, mobile, and tennis), the CTC field GOP structures resulted in BDR of approximately 13% to 117%, whereas the proposed field GOP structures resulted in approximate BDR of -11% to 9%.

# References

- A. Luthra and D. Baylon, “Performance of HEVC for interlaced video”, JCTVC-J0466, 10th Meeting: Stockholm, SE, 11–20 July 2012.

# Patent rights declaration(s)

Sony Electronics Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

|  | RA-Main |  |  |  |  |  |  | RA-HE10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Field-GOP-CTC |  |  | Field-GOP-New |  |  |  | Field-GOP-CTC |  |  | Field-GOP-New |  |  |
|  | BD-rate (piecewise cubic) |  |  | BD-rate (piecewise cubic) |  |  |  | BD-rate (piecewise cubic) |  |  | BD-rate (piecewise cubic) |  |  |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| ballet | -44.9% | -21.5% | -29.6% | -44.6% | -30.8% | -34.2% |  | -45.1% | -22.2% | -29.3% | -45.0% | -32.1% | -34.6% |
| bicycle | -24.8% | -30.0% | -19.2% | -23.6% | -27.2% | -20.8% |  | -24.9% | -29.6% | -18.4% | -23.6% | -27.3% | -20.7% |
| bus | -24.1% | -23.5% | -11.2% | -29.5% | -35.4% | -26.5% |  | -24.3% | -23.7% | -9.7% | -29.6% | -36.0% | -26.7% |
| car | -31.9% | -14.0% | -16.3% | -33.2% | -20.9% | -19.3% |  | -31.9% | -11.9% | -13.2% | -33.3% | -19.6% | -18.6% |
| cheer | -13.4% | 0.1% | -6.1% | -11.2% | -6.6% | -8.9% |  | -13.5% | 0.4% | -6.1% | -11.2% | -6.6% | -9.0% |
| flower | 13.4% | 70.3% | 53.2% | -11.0% | 7.2% | 3.0% |  | 13.2% | 68.0% | 51.7% | -11.1% | 6.9% | 2.4% |
| football | -47.5% | -57.7% | -56.7% | -46.7% | -56.8% | -53.4% |  | -47.7% | -57.5% | -56.5% | -46.8% | -56.4% | -53.8% |
| mobile | 117.1% | 237.2% | 217.4% | 9.1% | 29.4% | 28.6% |  | 117.0% | 233.4% | 216.0% | 8.9% | 29.2% | 27.9% |
| tennis | 75.9% | 144.7% | 106.7% | 1.9% | 20.4% | 13.5% |  | 75.8% | 143.8% | 106.9% | 1.9% | 20.1% | 13.0% |
| average | 2.2% | 34.0% | 26.5% | -21.0% | -13.4% | -13.1% |  | 2.1% | 33.4% | 26.8% | -21.1% | -13.5% | -13.3% |
|  | LB-Main |  |  |  |  |  |  | LB-HE10 |  |  |  |  |  |
|  | Field-GOP-CTC |  |  | Field-GOP-New |  |  |  | Field-GOP-CTC |  |  | Field-GOP-New |  |  |
|  | BD-rate (piecewise cubic) |  |  | BD-rate (piecewise cubic) |  |  |  | BD-rate (piecewise cubic) |  |  | BD-rate (piecewise cubic) |  |  |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| ballet | -35.0% | -24.2% | -27.1% | -39.2% | -30.0% | -33.7% |  | -35.6% | -23.5% | -27.4% | -40.1% | -30.9% | -34.7% |
| bicycle | -17.8% | -20.9% | -15.7% | -18.8% | -19.3% | -15.3% |  | -17.8% | -20.9% | -15.4% | -18.8% | -19.3% | -15.1% |
| bus | -22.9% | -22.0% | -16.3% | -28.7% | -29.8% | -25.5% |  | -23.1% | -22.8% | -16.4% | -28.8% | -30.2% | -26.6% |
| car | -22.2% | -12.2% | -10.1% | -25.7% | -15.7% | -13.4% |  | -22.3% | -10.4% | -5.9% | -25.7% | -15.2% | -12.2% |
| cheer | -1.4% | -1.2% | -1.4% | -5.9% | -5.6% | -5.5% |  | -1.1% | -0.8% | -1.0% | -5.7% | -6.0% | -5.6% |
| flower | 12.6% | 33.2% | 39.0% | -11.3% | 0.1% | -1.2% |  | 12.6% | 33.2% | 39.6% | -11.4% | -0.4% | -1.5% |
| football | -42.0% | -50.6% | -48.6% | -42.3% | -49.6% | -46.1% |  | -42.0% | -50.0% | -48.3% | -42.3% | -48.9% | -45.3% |
| mobile | 113.6% | 187.4% | 176.4% | 5.0% | 28.0% | 23.8% |  | 113.4% | 189.0% | 177.6% | 4.9% | 27.1% | 23.5% |
| tennis | 75.3% | 113.8% | 99.0% | -2.7% | 18.5% | 8.3% |  | 75.3% | 114.1% | 97.9% | -2.1% | 17.8% | 7.2% |
| average | 6.7% | 22.6% | 21.7% | -18.8% | -11.5% | -12.1% |  | 6.6% | 23.1% | 22.3% | -18.9% | -11.8% | -12.3% |
|  | LP-Main |  |  |  |  |  |  | LP-HE10 |  |  |  |  |  |
|  | Field-GOP-CTC |  |  | Field-GOP-New |  |  |  | Field-GOP-CTC |  |  | Field-GOP-New |  |  |
|  | BD-rate (piecewise cubic) |  |  | BD-rate (piecewise cubic) |  |  |  | BD-rate (piecewise cubic) |  |  | BD-rate (piecewise cubic) |  |  |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| ballet | -37.3% | -27.7% | -31.1% | -41.4% | -34.1% | -36.1% |  | -37.7% | -26.6% | -31.0% | -42.0% | -33.5% | -36.7% |
| bicycle | -19.8% | -22.1% | -17.2% | -20.7% | -19.7% | -15.7% |  | -19.7% | -22.2% | -16.5% | -20.7% | -20.0% | -15.9% |
| bus | -30.6% | -29.1% | -22.8% | -34.5% | -33.5% | -28.5% |  | -31.0% | -31.5% | -22.6% | -34.9% | -35.8% | -30.2% |
| car | -25.7% | -13.7% | -10.1% | -28.2% | -14.1% | -9.6% |  | -25.8% | -12.0% | -8.0% | -28.2% | -13.8% | -9.3% |
| cheer | -2.2% | -1.9% | -2.3% | -6.2% | -6.1% | -6.0% |  | -1.9% | -1.0% | -1.1% | -6.1% | -5.7% | -5.0% |
| flower | 6.3% | 30.1% | 33.2% | -10.7% | 5.2% | 4.2% |  | 6.2% | 30.0% | 34.1% | -10.8% | 4.6% | 3.6% |
| football | -42.9% | -51.4% | -49.1% | -43.1% | -50.1% | -45.9% |  | -42.6% | -50.8% | -48.6% | -42.7% | -49.5% | -45.6% |
| mobile | 103.0% | 180.2% | 169.8% | 8.2% | 37.1% | 32.3% |  | 103.2% | 180.0% | 169.3% | 7.9% | 35.0% | 30.7% |
| tennis | 68.3% | 111.1% | 91.6% | 2.0% | 28.9% | 15.5% |  | 67.9% | 108.0% | 92.2% | 1.6% | 23.4% | 14.8% |
| average | 2.1% | 19.5% | 18.0% | -19.4% | -9.6% | -10.0% |  | 2.1% | 19.3% | 18.6% | -19.5% | -10.6% | -10.4% |

Table 1: BDR of CTC field GOP structures and proposed field GOP structures for the encoding of field sequences. The anchor is CTC GOP structures with frame sequences.

# Appendix A

Two files in HM8.0 were modified to allocate more memory to support longer reference picture lists:

## Modifications to CommonDef.h

The following macros in HM8.0

#define MAX_NUM_REF                 4           ///< max. value of multiple reference frames

#define MAX_NUM_REF_LC              8           ///< max. value of combined reference frames

were replaced by

#define MAX_NUM_REF                 8           ///< max. value of multiple reference frames

#define MAX_NUM_REF_LC             16           ///< max. value of combined reference frames

## Modifications to TEncTop.cpp

The following codes in TEncTop::xInitPPS()

Int histogram[8];

for(Int i=0; i<8; i++)

{

histogram[i]=0;

}

for( Int i = 0; i < getGOPSize(); i++)

{

if(getGOPEntry(i).m_numRefPicsActive<8)

{

histogram[getGOPEntry(i).m_numRefPicsActive]++;

}

}

Int maxHist=-1;

Int bestPos=0;

for(Int i=0; i<8; i++)

{

if(histogram[i]>maxHist)

{

maxHist=histogram[i];

bestPos=i;

}

}

were parameterized by

Int histogram[MAX_NUM_REF+1];

for(Int i=0; i<=MAX_NUM_REF; i++)

{

histogram[i]=0;

}

for( Int i = 0; i < getGOPSize(); i++)

{

if(getGOPEntry(i).m_numRefPicsActive<=MAX_NUM_REF)

{

histogram[getGOPEntry(i).m_numRefPicsActive]++;

}

}

Int maxHist=-1;

Int bestPos=0;

for(Int i=0; i<=MAX_NUM_REF; i++)

{

if(histogram[i]>maxHist)

{

maxHist=histogram[i];

bestPos=i;

}

}

# Appendix B

## Proposed GOP structure for random access configuration files

#======== Coding Structure =============

IntraPeriod                   : 64          # Period of I-Frame ( -1 = only first)

DecodingRefreshType           : 1           # Random Accesss 0:none, 1:CDR, 2:IDR

GOPSize                       : 16          # GOP Size (number of B slice = GOPSize-1)

#        Type POC QPoffset QPfactor temporal_id #ref_pics_active ref_pic #ref_pics reference pictures             predict deltaRPS #ref_idcs reference idcs

Frame1:  B    1   1        0.3536   0           7                1       7         -1  -4  -5  -8  -9 -16 -17           0

Frame2:  B    16  1        0.442    0           6                1       6        -15 -16 -19 -20 -23 -24               1     -15        8         1 1 1 1 1 0 0 1

Frame3:  B    9   2        0.3536   0           4                1       6         -8  -9 -12 -13 -16   7               1       7        7         1 1 1 1 1 0 1

Frame4:  B    8   2        0.3536   0           4                1       6         -7  -8 -11 -12   1   8               1       1        7         1 1 1 1 0 1 1

Frame5:  B    5   3        0.3536   0           4                1       7         -4  -5  -8  -9   3   4  11           1       3        7         1 1 1 1 1 1 1 1

Frame6:  B    4   3        0.3536   0           4                1       8         -3  -4  -7  -8   1   4   5  12       1       1        8         1 1 1 1 1 1 1 1

Frame7:  B    3   4        0.68     0           4                1       9         -2  -3  -6  -7   1   2   5   6  13   1       1        9         1 1 1 1 1 1 1 1 1

Frame8:  B    2   4        0.68     0           4                0       9         -1  -2  -5   1   2   3   6   7  14   1       1       10         1 1 1 0 1 1 1 1 1 1

Frame9:  B    7   4        0.68     0           4                1       7         -2  -3  -6  -7   1   2   9           1      -5       10         1 1 0 0 1 1 1 1 1 0

Frame10: B    6   4        0.68     0           4                0       8         -1  -2  -5  -6   1   2   3  10       1       1        8         1 1 1 1 1 1 1 1

Frame11: B   13   3        0.3536   0           4                1       7         -4  -5  -8  -9 -12 -13   3           1      -7        9         1 1 1 1 0 1 1 1 0

Frame12: B   12   3        0.3536   0           4                1       8         -3  -4  -7  -8 -11 -12   1   4       1       1        8         1 1 1 1 1 1 1 1

Frame13: B   11   4        0.68     0           4                1       7         -2  -3 -10 -11   1   2   5           1       1        9         1 1 0 0 1 1 1 1 1

Frame14: B   10   4        0.68     0           4                0       8         -1  -2  -9 -10   1   2   3   6       1       1        8         1 1 1 1 1 1 1 1

Frame15: B   15   4        0.68     0           4                1       7         -2  -3  -6  -7 -14 -15   1           1      -5        9         1 1 1 1 0 1 1 1 0

Frame16: B   14   4        0.68     0           4                1       8         -1  -2  -5  -6 -13 -14   1   2       1       1        8         1 1 1 1 1 1 1 1

ListCombination               : 1           # Use combined list for uni-prediction in B-slices

## Proposed GOP structure for low delay configuration files

#======== Coding Structure =============

IntraPeriod                   : -1          # Period of I-Frame ( -1 = only first)

DecodingRefreshType           : 0           # Random Accesss 0:none, 1:CDR, 2:IDR

GOPSize                       : 8           # GOP Size (number of B slice = GOPSize-1)

#        Type POC QPoffset QPfactor temporal_id #ref_pics_active ref_pic #ref_pics reference pictures                 predict deltaRPS #ref_idcs reference idcs

Frame1:  B    1   1        0.578    0           8                1       8         -1 -2 -8  -9 -16 -17 -24 -25       0

Frame2:  B    2   3        0.4624   0           8                1       8         -1 -2 -9 -10 -17 -18 -25 -26       1      -1       9         1 0 1 1 1 1 1 1 1

Frame3:  B    3   3        0.4624   0           8                1       8         -1 -2 -3 -10 -11 -18 -19 -26       1      -1       9         1 1 1 1 1 1 1 0 1

Frame4:  B    4   2        0.4624   0           8                1       8         -1 -2 -3  -4 -11 -12 -19 -20       1      -1       9         1 1 1 1 1 1 1 0 1

Frame5:  B    5   2        0.4624   0           8                1       8         -1 -2 -4  -5 -12 -13 -20 -21       1      -1       9         1 0 1 1 1 1 1 1 1

Frame6:  B    6   3        0.4624   0           8                1       8         -1 -2 -5  -6 -13 -14 -21 -22       1      -1       9         1 0 1 1 1 1 1 1 1

Frame7:  B    7   3        0.4624   0           8                1       8         -1 -2 -6  -7 -14 -15 -22 -23       1      -1       9         1 0 1 1 1 1 1 1 1

Frame8:  B    8   1        0.578    0           8                1       8         -1 -2 -7  -8 -15 -16 -23 -24       1      -1       9         1 0 1 1 1 1 1 1 1

ListCombination               : 1           # Use combined list for uni-prediction in B-slices