| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0154 |
| --- | --- |

| Title: | Simplification of PicOrderCntMsb calculation and specification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Jun Xu, Ali Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the specification and calculation of PicOrderCntMsb in WD8 without changing the semantics in WD8. This contribution removes four arithmetic-logic operations and removes the implicit assumption that the subtraction in the calculation is based on unsigned integer arithmetic in WD8.  The proposed text is correct for both signed and unsigned integer arithmetic.

# Introduction

In WD8, the derivation of PicOrderCntMsb is specified with the implicit assumption that unsigned integer arithmetic is used. Consequently extra checking is included in the derivation to guarantee that subtraction of two unsigned integers produce meaningful results. In WD8 [1], we have

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( (prevPicOrderCntLsb − pic_order_cnt_lsb )   >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-1)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

The ( pic_order_cnt_lsb <  prevPicOrderCntLsb ) in the first “if” statement in (8-1) is there to quarantee that                 ( prevPicOrderCntLsb − pic_order_cnt_lsb ) is always non-negative. In fact, if signed arithmetic is used in the subtraction, the condition

(prevPicOrderCntLsb − pic_order_cnt_lsb )   >=  ( MaxPicOrderCntLsb / 2 ) )

implies

( pic_order_cnt_lsb <  prevPicOrderCntLsb ) ,

and therefore it is redundent for signed arithmetic.

Similarly, the (pic_order_cnt_lsb  >  prevPicOrderCntLsb ) in the second “if” statement in (8-1) is there to guarantee that (pic_order_cnt_lsb − prevPicOrderCntLsb )  is always non-negative. In fact, if signed arithmetics is used in the subtraction, the condition

(pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) )

implies

(pic_order_cnt_lsb  >  prevPicOrderCntLsb ),

and therefore it is redundent for signed arithmetic.

# Simplified derivation of PicOrderCntMsb

This contribution proposes an alternate derivation of PicOrderCntMsb so that the specification and computation is correct for both signed arithmetic and unsigned arithmetic. It also removed two “compare” operations and two “and” operations as in following proposed working draft text:

if( prevPicOrderCntLsb   >=  ( pic_order_cnt_lsb  + MaxPicOrderCntLsb / 2 ) ) 
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-2)
else if( pic_order_cnt_lsb  >  (prevPicOrderCntLsb  + MaxPicOrderCntLsb / 2 ) ) 
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

The above text  can be shown to be mathematically equivalentto the WD8 text.

# Results

The proposed modifications were implemented in a HM8.0 decoder. The decoder decoded all the HM8.0 anchor bitstreams from BBC. (i.e. ftp://ftp.kw.bbc.co.uk/hevc/hm-8.0-anchors/ ) without any error as reported by the MD5 sums.

# References

- B. Bross, W.J. Han, J.R. Ohm, G.J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8”, JCTVC-J1003_d7, 10th Meeting: Stockholm, SE, 11–20 July 2012.

# Patent rights declaration(s)

Sony Electronics Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).