| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0155 |
| --- | --- |

| Title: | Intra angular prediction blending |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Seishi Takamura Hiroshi Fujii Atsushi Shimizu Y-517A, 1-1 Hikari-no-oka, Yokosuka-shi, Kanagawa, Japan, 239-0847 | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This document introduces an intra angular prediction method using the reference pixels that is closer to the predicted pixels than the original reference pixels. In the proposed method, the predicted signals of the angular prediction mode are generated by blending the predicted signals of the selected angular prediction mode and that of the other mode which has the opposite prediction direction. The overall average coding gains against the HM8.0 anchor were 0.1%, 0.1%, 0.1% (Y, Cb, Cr in Main profile), 0.2%, 0.2% and 0.2% (Y, Cb, Cr in HE10 profile). The maximum coding gains were 0.4%, 0.5% and 0.5% (Y, Cb and Cr) for the sequence “Traffic” in the Main profile configuration. The average run-times of encoding and decoding were 102.5% and 100.4%, respectively. When the proposal was turned off for 4x4, the average coding efficiency was improved by 0.03% and the deterioration of the Class F sequences was alleviated. The encoding and decoding time were also faster by 1.6 points and 0.1 points, respectively.

# Introduction

In the intra prediction of HEVC, there are 35 intra prediction modes: planar (mode 0), DC (mode 1) and 33 angular modes (mode 2 to 34). The definition of the intra modes is the same among all block sizes. Finer prediction direction makes flexible predicted signals, which results in high coding efficiency. When the intra angular mode is selected, the value of weighted reference pixels is copied to the predicted pixels along the prediction direction as described in the equation (8-45) and (8-53) of the draft international standard (DIS) [1].

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ x+iIdx+1 ] + iFact*ref[ x+iIdx+2] + 16 ) >> 5

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ y+iIdx+1 ] + iFact*ref[ y+iIdx+2] + 16 ) >> 5

The array predSamples[x][y] and refMain[x, y] denote the predicted samples and the reference pixels, respectively. The parameter iIdx and iFact denote the index and the multiplication factor determined by the intra prediction mode. In this contribution, to make the predicted signals which reduce the prediction error energy, the proposal exploits other reference pixels located in the closer position to the predicted pixel. The proposal is based on the idea that the prediction error can be reduced when the distance between the predicted pixel and the reference pixel is short.

# Intra angular prediction blending

## Basic idea

HEVC employs 33 kinds of intra angular prediction. When mode 2 to 9 and mode 27 to 34 are selected, there are some decoded reference pixels that are located on the prediction direction. As shown in Fig. 1 that shows the case that the block size is 8x8 and the number of the intra mode is 34, the vertical line of the decoded reference pixels as well as the horizontal line can be used for the generation of the predicted signals. In the case of Fig. 1, the predicted pixels located at the bottom left corner are closer to the vertical line of the decoded reference pixels than the original horizontal line. Therefore, the coding efficiency can be improved by using the vertical line as well as the horizontal line to make the predicted signals. In the proposal, all decoded reference pixels on the other side are used for the intra angular prediction.

In the following explanation,  denotes the distance between the predicted pixel and the original reference pixel.  denotes the distance between the predicted pixel and the reference pixel on the opposite side. When the block size of the prediction unit (PU) is large, the distance ratio of  to  is quite large compared to the case that the block size of the PU is small. Therefore, it is guessed that the gain of the proposal could be larger when the block size of the PU is larger. The proposal would be effective for the large block sizes such as 32x32.

Figure 1: Conventional intra angular prediction (left) and proposed intra angular prediction (right) (Example of 8x8 and mode 34).

## Encoding algorithm

The detailed encoding algorithm is summarized as follows.

S1: Set  that indicates the class of the proposed intra angular modes.

S2: Input the value of the intra prediction mode. If the input value is included in , go to S3. Otherwise, the corresponding conventional intra prediction process is performed as defined in Working Draft 8 [1]. After the process is done, go to S4.

S3: Proposed intra angular prediction is done by using the reference pixels on the other side as well as the original reference pixels. After the proposed intra angular prediction is done, go to S4.

S4: Move on to the next prediction block. Repeat the loop (S1 to S4) until all the intra prediction process is completed.

## Parameter syntax related to the proposal

In the current implementation, the proposed encoder and decoder share the class of the intra angular modes (). Therefore, there is no additional syntax related to the proposed intra angular prediction.

## Decoder description

When the intraPredMode is included in , the predicted signals are modified by the following equation:

predSamples[ x ][ y ] = (w[ x ][ y ]*predSamples1[ x ][y ] + (1024–w[ x ][ y ])*predSamples2[ x ][ y ] + 512) >> 10

where predSamples[x][y] is the blended predicted signals. The array predSamples1[x][y] and predSamples2[x][y] denote the predicted signals generated by the selected intra angular prediction mode and the predicted signals by the intra angular prediction that has the opposite prediction direction, respectively. The array w[x][y] denotes the weight coefficients. All weight coefficients used in this contribution are described in the attached appendix file “JCTVC-K0155-Appendix.doc”. The weight coefficients are derived to minimize the prediction error energy for all the sequence defined in the common test condition [2]. Derived weight coefficients are modified by Gaussian filter.

# Experimental results

The proposal was incorporated in HM8.0. It is applied to luminance signal and is not applied to chrominance signals. The class of the proposed intra angular modes  includes mode 2 to 9 and mode 27 to 34. Weight coefficients are listed in the appendix. Experimental conditions are subject to the common test conditions [2]. From Table 1, the average coding gains were about 0.10%, 0.10% and 0.13% for Y, Cb and Cr of Main profile. In the case of the HE10 configuration, they were about 0.16%, 0.15% and 0.17% for Y, Cb and Cr. When the proposal is turned off for 4x4 and turned on for 8x8 to 32x32, the average coding gain was improved and the run-times of encoding and decoding were also improved as shown in Table 2. Especially for the Class F sequences, the deterioration of BD-rate was alleviated. Although the results of Table 2 were derived by turning off the proposal in syntax level, the same results of BD-rate will be derived by setting the value of the weight coefficient to 1024 for all 4x4 matrices. In that sense, unification will be guaranteed.

Table 1: Experimental results (anchor: HM8.0).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.19% | -0.26% | -0.25% | -0.27% | -0.35% | -0.32% |
| Class B | -0.09% | -0.10% | -0.12% | -0.16% | -0.14% | -0.15% |
| Class C | -0.13% | -0.09% | -0.10% | -0.16% | -0.13% | -0.19% |
| Class D | -0.14% | -0.10% | -0.12% | -0.16% | -0.11% | -0.12% |
| Class E | 0.06% | 0.08% | 0.00% | -0.04% | 0.03% | -0.07% |
| Overall | -0.10% | -0.10% | -0.13% | -0.16% | -0.15% | -0.17% |
|  | -0.10% | -0.10% | -0.13% | -0.16% | -0.14% | -0.17% |
| Class F | 1.57% | 1.15% | 1.16% | 1.51% | 1.05% | 1.07% |
| Enc Time[%] | 102.5% |  |  | 102.4% |  |  |
| Dec Time[%] | 100.4% |  |  | 100.3% |  |  |

Table 2: Experimental results (anchor: HM8.0, proposal is turned off for 4x4 PU).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.19% | -0.26% | -0.25% | -0.27% | -0.34% | -0.36% |
| Class B | -0.11% | -0.13% | -0.15% | -0.17% | -0.17% | -0.18% |
| Class C | -0.12% | -0.12% | -0.13% | -0.14% | -0.15% | -0.16% |
| Class D | -0.11% | -0.07% | -0.13% | -0.13% | -0.12% | -0.08% |
| Class E | -0.04% | -0.07% | -0.13% | -0.12% | -0.10% | -0.20% |
| Overall | -0.12% | -0.14% | -0.16% | -0.17% | -0.18% | -0.20% |
|  | -0.12% | -0.14% | -0.15% | -0.17% | -0.17% | -0.20% |
| Class F | 0.17% | 0.12% | 0.09% | 0.16% | 0.03% | 0.00% |
| Enc Time[%] | 100.8% |  |  | 100.9% |  |  |
| Dec Time[%] | 100.5% |  |  | 100.0% |  |  |

# Conclusion

In this contribution, modification of intra angular prediction was proposed. The proposal uses the reference pixels on the other side as well as the original reference pixels. Average coding gains against the anchor HM8.0 were 0.1%, 0.1%, 0.1% (Y, Cb, Cr in Main profile), 0.2%, 0.2% and 0.2% (Y, Cb, Cr in HE10 profile). The maximum coding gains were about 0.4%, 0.5% and 0.5% (Y, Cb and Cr) for the sequence “Traffic” in Main profile. The average run-times of encoding and decoding were 102.5% and 100.4%, respectively. When the proposal was turned off for 4x4, the average coding efficiency was improved by 0.03% and the deterioration of the Class F sequences was alleviated. The encoding and decoding time were also faster by 1.6 points and 0.1 points, respectively. It is recommended to incorporate the proposal into intra angular prediction of HEVC ver.2.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” JCTVC-J1003_d7, 10th meeting, Stockholm, SE, 11-20 July, 2012.

[2] F. Bossen, “HM 8 Common Test Conditions and Software Reference Configurations,” JCTVC-J1100, 10th JCT-VC meeting, Stockholm, SE, 11-20 July, 2012.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).