| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0156 |
| --- | --- |

| Title: | SAO encoder selection bug fix |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina, A. Alshin, J.H. Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Patrice Onno Guillaume Laroche Christophe Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com +33(0)299876800 patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr |
| Source: | Samsung Electronics Ltd. and Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution is about a correction of a SAO encoder bug which was introduced during the integration of J0044 in HM8.0. J0044 was about the addition of a new threshold for the Chroma component for the decision to switch on/off the SAO process at the slice level. The bug fix is a two-line modification in the HM8.0 encoder and does not affect the coding efficiency performance. In addition, it is asserted that the bug fix speeds-up the HM8.0 encoder and in average reduces the amount of bins to parse.

# Introduction

The slice level SAO on/off flag was included into the SAO design from the very first adoption of SAO to the draft of the HEVC specifications [1]. During the Stockholm meeting it was decided to have independent SAO on/off slice level flags for Luma and Chroma. But before I0184 adoption [3], HM s/w didn’t have any mechanism to decide about this flag.

During the 9th JCTVC meeting Geneva, I0184 contribution was adopted. This contribution presented a non-normative change which consists in a simple encoder modification for the SAO interleaving LCU mode. The adoption enables to turn on/off the SAO tool for the current slice based on a statistical criterion of the previous frame. By analysing the SAO usage (e.g. the rate of LCU where SAO is turned on) at frame level according to the frame depth, it was observed that SAO is often/always selected for frame of depth 0 and less selected for depth 1 and even less for frames of depth 2 especially at low bitrates.

The aim is to save bitrate allocated to signal the non-use of SAO at LCU level by signaling it at frame level instead. This modification also enables to reduce the encoding complexity by skipping the SAO process for some frames.

The principle of the encoder algorithm is the following: during the encoding of frame with a depth equal to 0 in the hierarchy, the number of times when sao_type_idx is equal to 0 (which means that SAO is not applied) for all components is counted. If this number of no SAO usage represents more than a certain amount (currently 75%) of all LCUs for all components, SAO is disabled for the following frames of depth 1 and depth 2, otherwise SAO is enabled for these depths.

Figure 1: Illustration of the frame depth for Random Access configurations.

# Detailed bug explanation

During the 10th JCTVC meeting in Stockholm, J0044 was adopted. This adoption suggests using two different thresholds in SAO slice level decision: one for the Luma component and one for the Chroma components.

The intention of J0044 was to extend the idea of I0184 by having different selection thresholds for the Luma and for the Chroma component and to independently enable/disable the SAO process for Luma and Chroma on slice level. During additional tests on HM8.0, it was noted that the implementation of J0044 was buggy compared to the original idea of I0184. But this was not the actual intention of J0044.

The following example shows the current behavior in HM8.0 that should not happen with the original idea of I0184. The following table is the typical illustration of SAO on/off slice level flags assignment in HM8.0 for RA configuration:

| depth=0: | slice_sao_luma_flag = true | slice_sao_chroma_flag= true |
| --- | --- | --- |
| depth=1: | slice_sao_luma_flag = true | slice_sao_chroma_flag= false |
| depth=2: | slice_sao_luma_flag = false | slice_sao_chroma_flag= false |
| depth=3: | slice_sao_luma_flag = true | slice_sao_chroma_flag= true |

The bad behavior is represented by the bold characters in the example above. It means that at depth=3, we have slice level SAO flags set to “true”. This means that SAO parameters are encoded for each LCU, but for each LCU, the encoder actually chooses not to use SAO. In addition, this bug results in increasing the encoding time for frames with depth = 3.

We observed that, at depth=3, for about 50% of pictures, we encode SAO off for first LCU in a slice and then we are using the mechanism of sao_merge_left_flag/ sao_merge_up_flag = true to disable SAO for the whole slice.

Due to this bug, the HM decoder parses useless SAO merge flags (which are encoded by using context modeling) for each LCU instead only one flag parsing in slice header. So the amount of bits to parse is also increased.

In order to speed up encoder and reduce average amount of SAO bits to parse Samsung and Canon suggest fixing this bug.

# Bug fix

The bug fix consists in two lines in the file “TEncSampleAdaptiveOffset.cpp” and the patch is represented here

#if SAO_ENCODING_CHOICE_CHROMA

#if SAO_ENCODING_CHOICE_CHROMA_BF

if (saoParam->bSaoFlag[0] == false) m_depthSaoRate[0][depth] = 1.0;

else m_depthSaoRate[0][depth] = numNoSao[0]/((Double)                         frameHeightInCU*frameWidthInCU);

if (saoParam->bSaoFlag[1] == false) m_depthSaoRate[1][depth] = 1.0;

else m_depthSaoRate[1][depth] = numNoSao[1]/((Double) frameHeightInCU*frameWidthInCU*2);

#else

m_depthSaoRate[0][depth] = numNoSao[0]/((Double) frameHeightInCU*frameWidthInCU);

m_depthSaoRate[1][depth] = numNoSao[1]/((Double) frameHeightInCU*frameWidthInCU*2);

#endif

# Results

This section presents the results of the bug fix. In terms of performance, there is no BDR gain. As explained above, SAO is often turned off for each LCU level by using the merge up/left flag mechanism.

In addition to the logic of this fix, the bug fix has benefits by skipping the SAO process for an entire frame and it avoids going through each LCU to perform the SAO filtering. Still I0184/J0044 for slice level SAO on/off flag are quite useful since they save encoding and parsing time to rapidly determine the values of the flags: slice_sao_luma_flag and slice_sao_chroma_flag.

Performance results under common test conditions for suggested bug fix are presented in Table 1 (there is no performance impact from this bug fix for all intra test). More details can be found in Excel spreadsheet attached to this contribution.

Table 1. Performance test results for suggested encoder bug fix.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | -0.1% | 0.0% | -0.2% | -0.2% |
| Class E | -0.2% | -0.5% | 0.4% | -0.4% | -0.4% | -0.3% |
| Class F | 0.3% | 0.3% | 0.1% | 0.1% | 0.3% | -0.8% |
| Overall | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E | -0.2% | 0.4% | -0.3% | -0.3% | -0.8% | 0.1% |
| Class F | 0.2% | 0.3% | 0.2% | 0.0% | 0.3% | 0.4% |
| Overall | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

# Some SAO slice level on/off flag statistics

During this contribution preparation, the statistics for SAO slice level on/off flag were collected and analysed. Since it might be useful for encoder manufacturer, we would like to publish these data. This section is informative and suggests no modification for HM s/w or HEVC specifications.

Both I0184 and J0044 suggest encoder fast decision algorithm for SAO on/off slice level flag. To use or not to use SAO for current slice, it is decided before SAO parameters are selected for each LCU. And if based of previous slice statistics is decided not to use SAO for current slice then SAO parameters search is skipped for all LCU in this slice.

In this section we’ll analyse how accurate this fast encoder decision is.

First we have collected statistics for SAO on/off slice level flag in HM8.0. These statistics are reported in Table 2 (first line). HM8.0 encoder chooses to apply SAO for Luma in 80% slices. For Chroma, SAO is applied for 39% of slices using HM8.0 encoder.

After the bug fix suggested in this contribution, the HM encoder chooses to use SAO less frequently: 74% for Luma and 32% for Chroma (2nd line in Table 2). It means that for 6% slices for Luma and for 7% slices for Chroma, the “HM + BF” encoder will not search SAO parameters. For the same amount of slices decoder will not parse SAO parameters for each LCU. As reported in section 4, suggested bug fix provides almost no performance change. This means that current SAO parameters syntax is very efficient.

With minimal HM s/w modification the following SAO on/off slice level flag decision algorithm was implemented. First both I0184 and J0044 were disabled. In that case, the HM encoder always analysed SAO parameters for each LCU. Then if for LCUs in current slice, it was selected not to use SAO for Luma or for Chroma then corresponding SAO slice level on/off flag was set to false. We assume that this is the most accurate decision for SAO on/off slice level flag. In this case, the modified HM encoder has one slice delay for making decision about SAO slice level on/off flag. The number of slices which use SAO in this encoder design are presented in Table 2 by the “slow SAO encoder” raw.

It is easy to notice that the both fast encoders (current HM8.0 and the HM8.0 with the suggested bug fix) make almost correct decision about SAO slice level on/off flag without actual SAO parameter search. This can explain why suggested bug fix provides no performance improvement compare to HM8.0.

Table 2. The hit-ratio of “SAO on” for the slice level flags.

| Encoder | slice_sao_luma_flag | slice_sao_chroma_flag |
| --- | --- | --- |
| HM8.0 | 80% | 39% |
| HM8.0 + bug fix | 74% | 32% |
| “slow SAO encoder” | 76% | 44% |

Performance-wise, the 3 encoders in Table 2 are very similar. Table 3 reports performance for “slow SAO encoder” in main profile test.

Table 3. Performance test results for “slow SAO encoder”.

|  | All Intra Main |  |  | Random Access Main |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% | -0.4% |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay P Main |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.1% | -0.3% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.2% |
| Class D | -0.1% | 0.0% | -0.1% | 0.0% | 0.2% | -0.5% |
| Class E | -0.4% | -0.1% | 0.4% | -0.3% | 0.0% | -0.4% |
| Class F | -0.1% | -0.3% | -0.4% | -0.1% | -0.2% | -0.1% |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusion

Samsung and Canon suggest fixing the s/w bug regarding the SAO on/off flag at slice level in order to speed-up encoder and to reduce the average amount of SAO bits to parse. It enables to preserve the original idea of not applying SAO for some frames in the temporal hierarchy. The patch is a two-line modification in the HM8.0 source code.

# References

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, and S. Lei, “CE13: Sample adaptive offset with LCU-independent decoding,” Document of Joint Collaborative Team on Video Coding, JCTVC-E049, March, 2011.

- Minhua Zhou, “AHG6: Independent luma and chroma SAO on/off control at slice level,” Joint Collaborative Team on Video Coding, JCTVC-J0087, July, 2012

- G. Laroche, T. Poirier, and P. Onno, “Non-CE1: Encoder modification for SAO interleaving mode,” Joint Collaborative Team on Video Coding, JCTVC-I0184, April, 2012.

- E. Alshina, A. Alshin, and J.H. Park, “Encoder modification for SAO,” Joint Collaborative Team on Video Coding, JCTVC-J0044, July, 2012.

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).