| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0157 |
| --- | --- |

| Title: | Lossless coding with residual sample-based prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This proposal describes a residue coding method for lossless intra coding in HEVC that incorporates sample-based prediction during the residual coding process. The proposed method does not modify the current intra prediction scheme and is implemented in a manner that is independent of intra prediction modes and color components. It is reported that the proposed scheme improves lossless intra coding performance by ~6.5%.

# Introduction

Lossless coding is achieved in the current HEVC design by bypassing the transform and quantization stage. It has been shown that sample-based prediction can improve coding performance [1][2].

In the proposal scheme, sample-based prediction is performed on the prediction residues without any modification to the existing intra prediction process. Also, the sample-based prediction process does not depend on the intra prediction mode and color component, i.e., the same prediction process is applied to all intra residual samples.

# Proposed method

Prediction

q

p3

p2

p1

p0

Figure 1. Intra Prediction.

The method described in [1] delays the sample-based prediction until after intra prediction. Fig. 1 shows a row of samples undergoing horizontal prediction. Let q be the reference sample, and p0, p1, p2 and p3 be the source samples to be coded (Fig. 1). After the block prediction, the residues obtained are:

.

The sample-based prediction is carried in [1] by computing on the above residues:

.

The resulting residues, r0’, r1’, r2’ and r3’, are then transmitted. The decoder is able to reconstruct the source pixels by reversing the above steps. It is clear from the above that the resulting residue is the same if the sample-based prediction were performed on the original source pixels instead.

The aforementioned scheme ensures that the existing intra prediction scheme can be preserved but requires a mode-dependent method of coding residues as sample-based prediction is only applied to residues generated by a subset of all intra prediction modes.

C

B

X

A

Figure 4. Residue being predicted, X, and its causal neighboring samples

The proposed scheme selects the predictor, P(X) by using the procedure described in [3] on the prediction residue:

Min(A,B) if C  >= Max(A,B)

Max(A,B) if C <= Min(A,B)

A+B-C, otherwise

P(X) =

If any neighboring samples are not available due to X being in the first row or column of a block, the same process is carried out with the missing values set to zero.

In this way, there is no dependence on the intra prediction type and color component as this residual pixel-based prediction is applied to all intra coded residual samples.

# Experimental results

The proposed scheme was implemented in HM8.0 and comparisons were made against HM8.0 in the transform bypass mode. TransquantBypassEnableFlag and CUTransquantBypassFlagValue were set to 1 and QP was set to 0 (for Main Profile experiments) and -12 (for HE10 experiments) in the configuration file. The tables below show the coding performance results for the All-intra Main and the All-intra HE10 configurations.

|  | AI - Main | kbps % |
| --- | --- | --- |
| Class A | Traffic | -11.9 |
| 4K | PeopleOnStreet | -13.2 |
|  | Nebuta | -11.5 |
|  | SteamLocomotive | -9.5 |
| Class B | Kimono | -5.6 |
| 1080p | ParkScene | -6.8 |
|  | Cactus | 0.0 |
|  | BasketballDrive | 1.0 |
|  | BQTerrace | -0.5 |
| Class C | BasketballDrill | 0.5 |
| WVGA | BQMall | -2.9 |
|  | PartyScene | -2.8 |
|  | RaceHorses | -8.2 |
| Class D | BasketballPass | -12.3 |
| WQVGA | BQSquare | -0.9 |
|  | BlowingBubbles | -3.4 |
|  | RaceHorses | -10.6 |
| ClassE | FourPeople | -10.8 |
| 720p | Johnny | -7.4 |
|  | KristenAndSara | -8.1 |
| ClassF | BasketballDrillText | -0.4 |
|  | ChinaSpeed | -8.7 |
|  | SlideEditing | -2.1 |
|  | SlideShow | -16.1 |
|  | All | -6.3 |
|  | Enc T % | 113 |
|  | Dec T % | 97 |

|  | AI - HE | kbps % |
| --- | --- | --- |
| Class A | Traffic | -11.9 |
| 4K | PeopleOnStreet | -12.8 |
|  | Nebuta | -9.3 |
|  | SteamLocomotive | -7.2 |
| Class B | Kimono | -6.7 |
| 1080p | ParkScene | -7.0 |
|  | Cactus | -1.1 |
|  | BasketballDrive | -0.9 |
|  | BQTerrace | -1.8 |
| Class C | BasketballDrill | -0.1 |
| WVGA | BQMall | -3.6 |
|  | PartyScene | -2.8 |
|  | RaceHorses | -8.7 |
| Class D | BasketballPass | -12.0 |
| WQVGA | BQSquare | -1.4 |
|  | BlowingBubbles | -3.4 |
|  | RaceHorses | -10.7 |
| ClassE | FourPeople | -11.0 |
| 720p | Johnny | -8.4 |
|  | KristenAndSara | -9.3 |
| ClassF | BasketballDrillText | -0.8 |
|  | ChinaSpeed | -12.0 |
|  | SlideEditing | -1.7 |
|  | SlideShow | -17.2 |
|  | All | -6.7 |
|  | Enc T % | 105 |
|  | Dec T % | 87 |

The results show that the proposed prediction scheme improves coding performance for most sequences.

# Conclusion

In this contribution, a lossless coding scheme that delays the sample-based prediction till the residue coding stage of the codec and carries out prediction in the residual domain is introduced. In this way, the existing intra prediction scheme in HEVC can be retained and remain unchanged while realizing the coding gains associated with sample-based prediction for lossless coding.

# References

- Y.-L. Lee, K.-H. Han, and G. J. Sullivan, “Improved lossless intra coding for H.264/MPEG-4 AVC,” IEEE Trans. Image Processing, pp. 2610-2615, Sep 2006.

- M. Zhou, “AHG22: Sample-based angular prediction (SAP) for HEVC lossless coding,” JCTVC-G093, Geneva, Nov 2011.

- S. A. Martucci, “Reversible compression of HDTV images using median adaptive prediction and arithmetic coding,” in Proc IEEE Int. Symp. Circuits Syst., pp. 1310-1313, 1990.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text modifications

### 8.6.2 Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nT specifying the size of the current transform block.

Output of this process is:

–	the (nT)x(nT) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP’Y			(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = QP’C	b		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = QP’C	r		(8-3)

The (nT)x(nT) array of residual samples r is derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].

- If cu_transquant_bypass_flag is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT – 1, y = 0..nT – 1 are derived as follows:

  - The variables A, B and C are derived by

A = ( x = = 0 ) ? 0 : r[ x-1 ][ y ]

B = ( y = = 0 ) ? 0 : r[ x ][ y-1 ]

C = ( ( x = = 0 ) | | ( y = = 0 ) ) ? 0 : r[ x-1 ][ y-1 ]

  - The residual sample r[ x ][ y ] is derived by

r[ x ][ y ] = Median( A , B , A + B – C ) + TransCoeffLevel[ x ][ y ][ cIdx ]

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, the (nT)x(nT) array of transform coefficients, the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a (nT)x(nT) array of scaled transform coefficients d.

- The (nT)x(nT) array of residual samples r is derived as follows.

- If transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are derived as:

r[ x ][ y ] = ( d[ x ][ y ] << 7 )		(8-4)

- Otherwise (transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, and the (nT)x(nT) array of scaled transform coefficients d as the inputs and the output is a (nT)x(nT) array of residual samples r.

- The variable shift is derived as:

shift = (cIdx = = 0) ? 20 – BitDepthY : 20 – BitDepthC	(8-5)

- The residual sample values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are modified as follows.

r[ x ][ y ] = ( r[ x ][ y ] + (1 << ( shift − 1) ) ) >> shift	(8-6)