| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0159 |
| --- | --- |

| Title: | AHG 9: On referenced and non-referenced DLP and TFD NAL units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Yongjoon Jeon Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com yongjoon.jeon@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the 10th JCTVC meeting, it was agreed to remove nal_ref_flag from NAL unit header and divide NAL unit types for coded slice of a non-TSA, non-STSA trailing picture, coded slice of a TSA picture, and coded slice of an STSA picture into referenced and non-referenced NAL unit types to replace the functionality that was provided by the removed flag. This contribution proposes to also divide NAL unit types for leading pictures (i.e., DLP and TFD NAL unit types) should also be divided into referenced and non-referenced. The rationale for this is that leading pictures actually behave similar to trailing pictures in decoding process such that when random access / splicing are not done.

# Problem Statement

In the 10th JCTVC meeting, it was agreed to remove nal_ref_flag from NAL unit header and divide NAL unit types for coded slice of a non-TSA, non-STSA trailing picture, coded slice of a TSA picture, and coded slice of an STSA picture into referenced and non-referenced NAL unit types to replace the functionality that was provided by the removed flag. With such modification, NAL units that are not needed for reference can still be recognized at system level and be removed, if needed, easily as illustrated in Figure 1.

Figure 1 – Simple bitstream extraction by removing non-referenced NAL units

It is observed that leading picture NAL unit types (i.e., DLP and TFD NAL unit types) should also be divided into referenced and non-referenced types considering that leading pictures actually behave similar to trailing pictures in decoding process when random access / splicing are not done. Figure 2 illustrates the possible problem faced by bitstream extraction when dealing with leading pictures. Since there is no information for which one of those pictures are not needed for reference, it is difficult to determine which NAL units can safely be extracted / removed.

Figure 2 – Problem when needing to remove non-referenced leading pictures

# Proposed Modification

To deal with the above problem, we propose to assign different types of NAL units for leading pictures that are needed for reference and leading pictures that are not needed for reference.

With this proposed solution, the changes are:

- DLP_R  Referenced Decodable Leading Picture. Coded slice of a Decodable Leading picture that is needed for reference.

- DLP_N  Non-referenced Decodable Leading Picture. Coded slice of a Decodable Leading picture that is not needed for reference.

- TFD_R  Referenced Tagged for discard. Coded slice of a leading picture that cannot be decoded without error when trailing pictures the preceed its RAP pictures are not available and is needed for reference.

- TFD_N  Non-referenced Tagged for discard. Coded slice of a leading picture that cannot be decoded without error when trailing pictures the preceed its RAP pictures are not available and is not needed for reference.

The number of NAL unit type given to DLP_R, DLP_N, TFD_R, and TFD_N can be of any available NAL unit types that are not used by other types yet.

The following are applicable to TFD and DLP NAL units:

- When the value of nal_unit_type is equal to TFD_N or DLP_N, the decoded picture is not included in any of RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr of any picture with the same value of TemporalId.

- A coded picture with nal_unit_type equal to TFD_N or DLP_N may be discarded without affecting the decodability of other pictures with the same vlaue of TemporalId

- A coded picture with nal_unit_type equal to TFD_N or DLP_N can be treated similar to TRAIL_N, TSA_N, or STSA_N, unless in the situation where decoding started from the random access point associated to the leading pictures.

- A coded picture with nal_unit_type equal to TFD_R or DLP_R can be treated similar to TRAIL_R, TSA_R, or STSA_R, unless in the situation where decoding started from the random access point associated to the leading pictures.

Figure 3 illustrates bitstream extraction when dealing with leading pictures. With the introduction referenced and non-referenced leading pictures, extraction process for leading pictures can be done similar to that of for any trailing pictures.

Figure 3 Bitstream extractions after introduction of referenced and non-referenced leading pictures

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).