| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0160 |
| --- | --- |

| Title: | Sequence Adaptive Field/Frame metadata for field indication SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, Paul Haskell, Osnat Bar-Nir Harmonic Inc. 4300 N. First Street San Jose, CA 95134 USA | Tel: Email: | [+1 408 542 2500] [Chad.Fogg@gmail.com] [paul.haskell@harmonicinc.com] [Osnat.Bar-Nir@Harmonicinc.com] |
|  | Aaron Wells, Didier Le Gall Ambarella 2975 San Ysidro Way Santa Clara, CA 95051 USA | Email: | [awells@ambarella.com] [dlegall@ambarella.com] |
| Source: | Harmonic Inc. and Ambarella |  |  |

_____________________________

# Abstract

Additions to four separate aspects of Sequence Adaptive Field/Frame (SAFF) metadata are proposed that would provide a more complete and explicit output process for mixed progressive/interlace content. This proposal would neither modify the syntax nor behaviour of the HEVC Video Coding Layer (VCL) and HRD model in Annex C. Aspect (1): frame cadence, is proposed for frame sequences to assist display output continuity when a bitstream transitions between frame and field sequences as per SAFF. Aspect (2): temporal display output pattern, provides a limited number of traditional output durations of each coded frame within a frame sequence according to an implicit display model separate (but non-contradictory over several frame periods) from the non-normative HRD model in Annex C. Aspect (3): field pair association, explicitly instructs the display process which progressive field pictures should be woven together to re-form a progressive source frame for progressive display output. Finally, aspect (4): sequence display target type informs decoders and displays whether the bitstream was constructed with the intent of being primarlily

# Introduction

This proposal focuses on features desired, while suggesting possible solutions for JCT and the specification editors to consider. Therefore, rather than first consider syntax, it is useful to state what features are desired, and then offer two solutions (two of which are suggested in the combination of this document and JCTVC-K0165).

Those readers who are more familiar with MPEG-2 | H.262 [1] and AVC [2] display metadata conventions may find the comparison tables in the accompanying PDF slides to this document more informative to study first. The slides also serve as an update to the tutorial provided in JCTVC-I0393 [5]

## Cadence

As mentioned in JCTVC-K0165, in some applications, mixed field/frame content in SAFF encoding practice requires explicit cadence and output durations.  The accompanying slides provide more background.

## Output pattern

As mentioned in JCTVC-K0165, frame sequences currently lack the ability to output variable display duration for coded frames when mixed frame rate content encoded in a bitstream targeting a fixed display output rate, as per DVB and SCTE recommended practice.

## Progressive field pairs

Field sequences already include top/bottom cadence through the field_pic_flag element in field_indication() SEI, but it is insufficient without counting from the beginning of an uncorrupted bitstream to know absolutely which progressive fields (progressive_souce_flag=1 && field_pic_flag=1) should be paired and interleaved for progressive display output.  Unlike AVC, no two pictures can have the same POC in HEVC.

## display target type

A fixed frame rate frame sequence less than or equal to 30 frames/sec is likely to be targeted to an interlaced display, regardless of the fact that the sequence is comprised of strictly frames coded with progressive tools. Meanwhile, frame sequence with frame rate indicating > 30 Hz, such as 50 or 60 frames/sec, is likely constructed for progressive displays, such as in 720p60 broadcasts.  To distinguish a 1080p30 bitstream aimed at an interlaced display (as in SCTE or DVB) from a 1080p30 bitstream intended for example, a progressive display (as in a Blu-Ray 24 fps movie or 30 fps documentary), a bitstream display target indicator is proposed to be inserted into the VUI in proximity to field_seq_flag.. This instructs display processors to treat a sequence as either mixed content aimed at interlaced displays or pure-progressive content aimed at progressive displays, much as the progressive_sequence flag served in MPEG-2 | H.262 [1].

# Proposal

In addition to the suggested pic_struct solution in JCTVC-K0165, the following modification to field_indication SEI in the HEVC draft [3] is provided. It is also suggested to change the name of the element field_indication to display_output_indication due to the ulitility in both frame and field sequences:

Current syntax in section D.1.19 of the current HEVC draft [3]:

Proposed syntax:

Note that field_display_pattern  absorbs both bottom_field_flag and top_field_first_flag.

Interpretation of field_display_pattern

| Value | Field display pattern |
| --- | --- |
| 0 | Top field associated with previous bottom field in output order |
| 1 | Bottom field associated with previous top field in output order |
| 2 | Top field associated with next bottom field in output order |
| 3 | Bottom field associated with next top field in output order |
| 4 | Top non-associated field |
| 5 | Bottom non-associated field |
| 6 | Reserved |
| 7 | Reserved |

Interpretation of frame_display_pattern

| Value | Frame display pattern | Notes |
| --- | --- | --- |
| 0 | Progressive frame (no frame repetition) | progressive_source_flag shall be 1 |
| 1 | Top field, then bottom field |  |
| 2 | Bottom field, then top field |  |
| 3 | Top field, then bottom field, then repeat top field | progressive_source_flag shall be 1 |
| 4 | Bottom field, then top field, then repeat bottom field | progressive_source_flag shall be 1 |
| 5 | Frame doubling | progressive_source_flag shall be 1 |
| 6 | Frame tripling | progressive_source_flag shall be 1 |
| 7 | Reserved |  |

vui_parameters addition:

| [..] |  |
| --- | --- |
| neutral_chroma_indication_flag | u(1) |
| field_seq_flag | u(1) |
| display_target_type | u(1) |
| hrd_parameters_present_flag | u(1) |
| [..] |  |

display_target_type = 0 indicates that the bitstream is intended for progressive displays, while display_target_type =1 indicates that the bitstream is intended for interlaced displays.

For non-fixed frame rate applications (fixed_pic_rate_flag=0), an alternative to signalling explicit output cadence with each coded picture would be to indicate the parity of the first field output to display (top or bottom) in the sequence, and utilize dpb_output_delay to indicate whether the frame output duration is 1, 1.5, 2, or 3 picture tick periods.  The parity and cadence for all subsequent pictures in the sequence would be derived from PicOrderCntVal and display_target_type. Thus, in addition to display_target_type=0 and 1 above:

[..]

| display_target_type | u(2) |
| --- | --- |

[..]

display_target_type =2 indicates that the bitstream is intended for interlaced displays, with the first output field in the sequence having top parity.

display_target_type =3 indicates that the bitstream is intended for interlaced displays, with the first output field in the sequence having bottom parity.

# References

[1] ISO/IEC 13818-2 MPEG-2 Video | ITU-T H.262

[2] ITU-T Rec. H.264 | ISO/IEC 14496-10

[3] B. Bross et. al, “High efficiency video coding (HEVC) text specification draft 8”, JCTVC-J1003_d7, 10th meeting, Stockholm, July 2012.

[4] K. Chono, C. Fogg, D. Flynn, S. Sekiguchi, “BoG report on interlace indicators”, JCTVC-H0720

[5] C. Fogg, A. Wells, “How to use the field sequence SEI”, in JCTVC-I0393.

# Patent rights declaration(s)

Harmonic Inc. and Ambarella Inc. do not have any current or pending patent rights relating to the technology described in this contribution.