| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0162 |
| --- | --- |

| Title: | Corner case conformance bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, Aaron Wells Ambarella 2975 San Ysidro Way Santa Clara, CA 95051 USA | : Email: | [Chad.Fogg@gmail.com] [awells@ambarella.com] |
| Source: | Ambarella |  |  |

_____________________________

# Abstract

With the aim of improving the chances of interoperability, a list of conformance bitstreams is proposed that test corner cases the authors feel are likely to lead to behaviour mismatch among decoders.

# List of special corner case conformance test bitstreams

The following table contains corner cases that decoder designers should be made especially aware

| No. | Conformance bitstream description |
| --- | --- |
| 1 | Maximum number of slices permitted for Profile & Level combination; Each slice has unique short_term_ref_pic_set(), ref_pic_list_modification() and pred_weight_table() with maximum num_long_term_pics |
| 2 | CTU size is 64x64 with no cu splits, and only one coded transform unit in the lower right 4x4 pixel corner of the CTU with delaQp signalled (as raised by P. Kaspenberg on the JCT reflector 12-Sept-2012) |
| 3 | Maximum number of tiles in a random, non-uniform arrangement that changes from picture to picture, with random chosen value for loop_filter_across_tiles_enabled_flag, deblocking_filter_override_flag, slice_disable_deblocking_filter_flag, slice_loop_filter_across_slices_enabled_flag |
| 4 | A wide range of values for pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, pic_crop_bottom_offset |

# References

# Patent rights declaration(s)

Ambarella Inc. does not have any current or pending patent rights relating to the technology described in this contribution.