| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0164 |
| --- | --- |

| Title: | AHG 9: On non-referenced TSA and STSA NAL units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the 10th JCTVC meeting, it was agreed to adopt STSA NAL units and also to differentiate TSA and STSA NAL unit types into referenced and non-referenced NAL units (i.e., TSA_R, TSA_N, STSA_R, and STSA_N). Since both TSA and STSA NAL units can be referenced (i.e., TSA_R and STSA_R) and non-referenced (i.e., TSA_N and STSA_N), there is possibility that for the TSA_N and / or STSA_N are removed from the bitstream by system / network that work simply by checking whether or not a NAL unit is needed for reference. This contribution identifies two possible issues

- The removal of the TSA and / or STSA NAL unit makes bitstream may not have marker anymore for possibility for recovering to higher layer when further removal based on temporal layer is applied.

- System / network may remove too many NAL units it necessary has to do, particularly when STSA_N is used.

To deal with the issues, the following items are proposed:

- To add a constraint such that TSA and STSA NAL units shall not be discarded during bitstream extraction.

Further, following the proposed constraint, we observed that non-referenced TSA and STSA NAL units may not be needed anymore so that they can be removed. With this removal, the NAL unit types for TSA and STSA can be as follows:

- TSA_NUT – Coded slice of a TSA picture

- STSA_NUT – Coded slice of a STSA picture

- To add a constraint for STSA NAL unit such that there should be no temporal layer gap between two consecutive STSA NAL units. In other words, the constraint can be described as the following:

- The first STSA NAL unit after a RAP NAL unit (i.e., either IDR or BLA or CRA) can appear at any temporal layer except the base temporal layer (i.e., temporal ID 0)

- The next STSA NAL unit shall have one higher temporal layer than that of the previous STSA NAL unit.

# Introduction

In the 10th JCTVC meeting, it was agreed to adopt STSA NAL units and also to differentiate TSA and STSA NAL unit types into referenced and non-referenced NAL units (i.e., TSA_R, TSA_N, STSA_R, and STSA_N).

Both TSA and STSA may be considered like marker system / network application to indicate that NAL units that comes after TSA or STSA NAL units can be decoded without dependency to NAL units before the two types NAL units that have temporal layer equal or higher than that of the TSA or STSA NAL units. Figure 4.7 – 1 illustrates the example of the use of TSA NAL units. When there is situation where some NAL units must be removed / dropped from the bitstream (e.g., due to bad network condition), system can remove NAL units prior to TSA that has the same of higher temporal level. When the TSA NAL unit occurs, it indicates that the remaining NAL units can be decoded without any problem even when there are some removal has been done.

Figure 1 -- Example of the use of TSA

Figure 2 illustrates the example of the use of STSA NAL units. When there is situation where some NAL units must be removed / dropped from the bitstream (e.g., due to bad network condition), system can remove NAL units prior to STSA that has the same of higher temporal level. When the STSA NAL unit occurs, it indicates that the remaining NAL units that have the same temporal level as that of the STSA can be decoded without any problem even when there are some removal has been done. Consequently, the recovery to decoder higher temporal level is done step by step, unlike TSA, which allows instant recovery to the highest possible temporal level.

Figure 2 -- Example of the use of STSA

# Problem Description

Since both TSA and STSA NAL units can be referenced (i.e., TSA_R and STSA_R) and non-referenced (i.e., TSA_N and STSA_N), there is possibility that for the TSA_N and / or STSA_N are removed from the bitstream by system / network that work simply by checking whether or not a NAL unit is needed for reference.

Example case 1

Consider the following scenario in which an encoder encodes bitstream with the structure as illustrated in Figure 3. When network is congested and the resolution to such situation is by removing NAL units that are not needed for reference, all the NAL units at the highest temporal layer and the TSA_N in the figure can be removed. The removal of the TSA NAL unit makes the bitstream does not have marker anymore for possibility for recovering to higher layer when further removal based on temporal layer is applied.

Figure 3 – Example of using TSA_N

Example case 2

Consider the following scenario in which an encoder encodes bitstream with the following structure:

Figure 4 – Bitstream structure in problem description example

With the intention such that when network condition is gone bad, the removal can be done and recovery can be apply in two stage at every STSA such as illustrated in Figure 5.

Figure 5 – Recovery plan when network bandwidth fluctuates

However, consider the situation when the above input bitstream is sent via two MANEs (Multimedia Aware Network Environment) in which the MANEs have the following features:

- The first MANE is a simple MANE in which in case of network congestion, it will remove NAL units that are not needed for reference

- The second MANE is a more sophisticated MANE in which in case of network congestion; it will remove NAL units according by taking into consideration TSA and STSA NAL units.

When network congestion happens at the first MANE, the MANE removes the highest temporal layer of the bitstream. , then, when network congestion happens at the second MANE, the MANE shall remove more NAL units guided by the occurrence of STSA. However, since the STSA_N has been removed previously, the 2nd MANE is misguided and removes too many NAL units that are actually not necessary to be removed.

Figure 6 – The 2nd MANE removes too many NAL units

# Proposed Modifications

To deal with the above problems, we propose the following:

Proposal 1

We propose to add a constraint such that TSA and STSA NAL units shall not be discarded during bitstream extraction.

Further, following the proposed constraint, we observed that non-referenced TSA and STSA NAL units may not be needed anymore so that they can be removed. With this removal, the NAL unit types for TSA and STSA can be as follows:

- TSA_NUT – Coded slice of a TSA picture

- STSA_NUT – Coded slice of a STSA picture

Figure 7 illustrates the intended use of TSA / STSA without non-referenced type by reflecting the example case 2. When the 1st MANE removes NAL units, it shall not remove TSA / STSA NAL units since these NAL units are marker to indicate recovery point. Consequently, when the 2nd MANE needs to remove NAL units, then it shall not remove NAL units that are not necessary to be removed.

Figure 7 – Expected behaviour of TSA / STSA during bitstream removal

Proposal 2

We propose to add a constraint for STSA NAL unit such that there should be no temporal layer gap between two consecutive STSA NAL units. In other words, the constraint can be described as the following:

- The first STSA NAL unit after a RAP NAL unit (i.e., either IDR or BLA or CRA) can appear at any temporal layer except the base temporal layer (i.e., temporal ID 0)

- The next STSA NAL unit shall have one higher temporal layer than that of the previous STSA NAL unit.

Figure 8 illustrates the example of coded structure that follows proposal 2. As depicted in the figure, each STSA NAL has exactly one temporal layer different / higher that it’s previous one.

Figure 8 – Coded bitstream using STSA according to proposal 2

Figure 9 illustrates the intended use of TSA / STSA by reflecting the example described in section 4.7.2. When the 1st MANE removes NAL units, it shall not remove TSA / STSA NAL units since these NAL units are marker to indicate recovery point. Consequently, when the 2nd MANE needs to remove NAL units, then it shall not remove NAL units that are not necessary to be removed.

Figure 9 – Expected behaviour of STSA during bitstream removal with proposal 2

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).