| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0165r1 |
| --- | --- |

| Title: | Proposal to add pic_struct information to HEVC, with simulation results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Osnat Bar-Nir Harmonic Inc. 10 Ha’Amal st. Rosh-Ha’Ayin, 48092 Israel | Tel: Email: | [+972-3-9007785] [Osnat.Bar-Nir@Harmonicinc.com] |
|  | Chad Fogg Ambarella 2975 San Ysidro Way Santa Clara, CA 95051 USA | Email: | [Chad.Fogg@gmail.com] |
| Source: | Harmonic Inc. and Ambarella |  |  |

_____________________________

# Abstract

This contribution proposes a metadata to signal repeated output fields in HEVC frame sequences that are restricted, by application, to fixed frame rate operation (fixed_pic_rate_flag=1), often at a pre-determined frame rate (pic_duration_in_tc_minus1). It is proposed to add pic_struct information into the existing pic_timing() SEI.

The common use case that this proposal is intended for is broadcasting encoding of film-source content, which typically averages ~24 frames per second (fps), but is embedded within a ~30 Hz fixed frame rate frame-sequence with ~60 Hz cadence maintained between transitions to/from field sequences which may contain pure interlaced content.

In order to temporally pad the sequence from ~24 fps source to ~30 fps coded rate, it is necessary alternate frame intervals between ~2/60 and ~3/60 sec as per 3:2 pulldown pattern.

Without the ability to signal variable display output intervals with explicit cadence metadata under the constant frame rate target system constraints, such mixed frame-rate content must instead be encoded in a field sequence (field_seq_flag=1) in order to take advantage of coded repeated fields (marked by duplicate_flag=1) to temporally pad the sequence.

In the film content tested in this proposal, padded field sequences incurred a 22% average bitrate increase over direct 24 fps progressive frame sequence coding. Within the current HEVC toolkit, it is not possible to code duplicate fields within a frame sequence since no syntax exists to signal individual half-height pictures within a sequence, and there are no semantics in HEVC to handle such half-height pictures. To enable the same coding efficiency of direct 24 fps frame sequence coding for 24 fps-embedded-within-30-fps bitstreams, it is proposed to add pic_struct information into the existing pic_timing() SEI.

# Introduction

Many video applications require constant clock tick and fixed picture rate. This contribution proposes to adopt an AVC-like pic_struct information in pic_timing SEI message in order to enable signalling of repeated picture output when fixed_pic_rate_flag is equal to one.

Signaling of repeated output pictures, instead of coding at higher frame rate,  improves coding efficiency in cases that the coded video sequence (CVS) corresponds to a source with a picture rate lower than the intended fixed picture output rate, as in the common case of film content (24 Hz, frame) outputted as 60Hz (fields).

In this case, the frame rate is increased by duplicating some of the input fields in a process known as 3:2 pulldown. Duplicating fields of the progressive film content not only increases the frame rate as desired, but also breaks the frame structure, and leads to the practice of field coding. This, in many cases, involves significant degradation in coding efficiency. The coding efficiency loss is not only due to some additional all-skipped pictures added, but is mainly due to field-coding which is usually less efficient than progressive frame-coding.

Using SAFF (Sequence Adaptive Field/Frame) functionality signaled by field_indication() SEI, with frame rate switching between 24 Hz frames and 60 Hz fields, but without the display hints of a pic_struct-like entity, decoders cannot always cleanly handle the transition from 24 Hz to 60 Hz interlaced. For example, a sequence that ends in display order with a bottom field followed by a sequence that starts immediately with a bottom field rather than a top field as expected per proper alternating top/bottom cadence.  Decoders receiving 24 Hz input but generating 60 Hz interlaced output benefit from explicit metadata to tell them how to map the 24P input to 60I output.  Such metadata can be presented as  pic_struct information in pic_timing  SEI message.

Through the field_indication() SEI duplicate_flag, only whole pictures can be effectively repeated by coding a fairly low bit-cost duplicate picture that only references a previously decoded picture in DPB through almost all skipped Coding Units. However, in frame sequences (field_seq_flag = 0), repeated fields cannot be encoded in this duplicate manner since this would require sending a half-sized picture, which can neither be expressed in the current HEVC syntax, nor interpreted by current HEVC semantics.

# Proposal

It is proposed to adopt AVC-like pic_struct information in pic_timing SEI message [1] in order to enable signaling of repeated picture when fixed_pic_rate_flag is equal to one.

Using pic_struct will enable frame-coding of the original 24 Hz progressive content instead of the less efficient field-coding, with extra repeated fields.

In particular, types 3-5 would enable to indicate the appropriate display structure and timing of each of the 24Hz frames without physically coding duplicated fields that eliminate the progressiveness of the content. The new picture output time, NumClockTS*tc, keeps the fixed frame rate, since pic_struct defines the exact frame (or fields) that correspond to each frame slot.

Table 1 : Interpretation of pic_struct (based on AVC’s table D-1)

| Value | Indicated display of picture | Restrictions | NumClockTS |
| --- | --- | --- | --- |
| 0 | (progressive) frame | field_pic_flag shall be 0 | 1 |
| 1 | bottom field | field_pic_flag shall be 1 | 1 |
| 2 | top field | field_pic_flag shall be 1 | 1 |
| 3 | top field bottom field, in that order | field_pic_flag shall be 0 | 2 |
| 4 | bottom field, top field, in that order | field_pic_flag shall be 0 | 2 |
| 5 | top field, bottom field, top field repeated, in that order | field_pic_flag shall be 0 | 3 |
| 6 | bottom field, top field, bottom field repeated, in that order | field_pic_flag shall be 0 | 3 |
| 7 | frame doubling | field_pic_flag shall be 0 | 2 |
| 8 | frame tripling | field_pic_flag shall be 0 | 3 |
| 9..15 | reserved |  |  |

Syntax

Modifying table D.1.2 of HEVC draft (JCTVC-K0030) to add the associated pic_struct element from D.1.2 in AVC

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( CpbDpbDelaysPresentFlag ) { |  |
| cpb_removal_delay | u(v) |
| dpb_output_delay | u(v) |
| if( sub_pic_cpb_params_present_flag ) { |  |
| num_decoding_units_minus1 | ue(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| common_du_cpb_removal_delay_flag | u(1) |
| if( common_du_cpb_removal_delay_flag ) |  |
| common_du_cpb_removal_delay_minus1 | u(v) |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !common_du_cpb_removal_delay_flag ) |  |
| du_cpb_removal_delay[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| if (pic_struct_present_flag ){ |  |
| pic_struct | u(4) |
| } |  |
| } |  |

pic_struct indicates whether a picture should be displayed as a frame or one or more fields, according to Table ??. Frame doubling (pic_struct equal to 7) indicates that the frame should be displayed two times consecutively, and frame tripling (pic_struct equal to 8) indicates that the frame should be displayed three times consecutively.

As per AVC, vui_parameters()  the pic_struct_present_flag would be re-introduced

| [..] | [..] |
| --- | --- |
| field_seq_flag | u(1) |
| hrd_parameters_present_flag | u(1) |
| if( hrd_parameters_present_flag ) |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| pic_struct_present_flag | u(1) |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| [..] |  |

field_indication() SEI would then be modified as follows to eliminate overlap with pic_struct:

| field_indication( payloadSize ) { | Descriptor |
| --- | --- |
| field_pic_flag | u(1) |
| progressive_source_flag | u(1) |
| duplicate_flag | u(1) |
| if( field_pic_flag ) |  |
| bottom_field_flag | u(1) |
| else if( !progressive_source_flag ) |  |
| top_field_first_flag | u(1) |
| else |  |
| reserved_zero_1bit /* equal to 0 */ | u(1) |
| reserved_zero_45bits /* equal to 0 */ | u(45) |
| } |  |

Note that progressive_source_flag above signalls the same information as AVC’s ct_type in pic_timing() SEI.

# Simulation Results

In order to assess the coding efficiency loss introduced by field duplication, the three 24Hz sequences from the common tests sequences were used. In addition a few more tests added.

The coding efficiency of the original progressive 24 Hz content was compared vs. the same content with 3:2 duplicated fields, using HM8.0.

The original 24 Hz frames were compared with the 60-fields/second versions that include the fields extracted from progressive frames plus repeated fields. Since HM configuration for field coding is not optimal, it was also compared with the 48-fields version, fields extracted from progressive frames without duplicated fields.

## Configuration

The same QPs were used for both field and frame coding. Each sequence is 10 seconds long, but contains different number of frames to encode, different frame rate and different picture height (240-24-1080, 600-60-544, 480-48-544). Intra period are set to 32, 64 and 80, so each output streams contains 8 Intra slices.

## Results (HM8.0)

|  | BD-rate (60 field vs. 24 frame) |  |  | BD-rate (48 fields vs. 24 frames) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Video Sequences | Y | U | V | Y | U | V |
| Kimono1 | 20.8% | 66.0% | 72.2% | 15.7% | 56.6% | 64.9% |
| Tennis | 1.9% | 46.0% | 40.8% | -2.4% | 38.4% | 34.0% |
| ParkScene | 85.7% | 144.2% | 140.1% | 70.8% | 120.8% | 115.8% |
| Animation | 11.4% | 60.7% | 51.2% | 11.0% | 57.7% | 53.8% |
| River & Boat | 13.2% | 75.0% | 74.0% | 11.4% | 69.2% | 70.0% |
| Dance (slow motion + static background) | 3.8% | 11.6% | 15.9% | 2.4% | 10.8% | 15.3% |
| Walking crowd | 18.4% | 44.6% | 49.8% | 13.9% | 38.9% | 43.4% |
| Overall | 22.17% | 64.01% | 63.43% | 17.54% | 56.06% | 56.74% |

# Conclusion

Simulation results indicate significant coding efficiency loss introduced by coding of repeated fields, despite the fact that the repeated fields are all-skipped.

The chroma loss is higher due to the shift in chroma samples position.

Therefore it is proposed to enable signalling of repeated picture when fixed_pic_rate_flag is equal to one, instead of physically code them, either by adopting AVC-like pic_struct information in pic_timing SEI message or by adding the required metadata in another SEI message, .

# References

[1] ITU-T Rec. H.264 | ISO/IEC 14496-10

[2] Arturo A. Rodriguez, et al., “Repeat picture output and HRD management,” in JCTVC-J0444, Stockholm, SE, Jul 2012.

# Patent rights declaration(s)

Harmonic Inc. and Ambarella Inc. do not have any current or pending patent rights relating to the technology described in this contribution.