| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0166 |
| --- | --- |

| Title: | AHG 9: On random access point NAL units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution describes two observations:

- The behaviour and treatment for IDR and BLA NAL units are actually very similar

- Since it is fixed that temporal id of RAP NAL units shall be 0, there is redundancy in the utilization of the three bits for nuh_temporal_id_plus1 syntax element for RAP NAL units.

It is proposed in this contribution that:

- IDR and BLA NAL units should be merged as one NAL unit. Required modifications as the consequences of this merge are as the following:

  - IDR NAL unit should be divided into IDR_W_TFD, IDR_W_DLP, IDR_N_LP.

  - There is no need to specify IdrPicFlag, rather, the use of RapPicFlag would be enough.

  - Some modifications are needed at slice header. They are as the following:

    - pic_order_cnt_lsb and all syntax element related to Reference Picture Set in slice header is signalled even when the slice is part of IDR picture.

    - RPS for the slices that are part of IDR picture may have RPS but the RPS will be ignored

- To remove redundancy in nuh_temporal_id_plus1 for RAP NAL units as follow:

  - For RAP NAL units, there is no need to signal temporal_id information in NAL unit header. It can be simply fixed / inferred as 0.

“If nal_unit_type is in the range of 7 to 12 (coded slice of a RAP picture), inclusive, TemporalId shall be inferred as equal to 0”

  - Instead of allocating 6 NAL unit types for RAP NAL units, they can be simplify as only one NAL unit type.

RAP_NUT – Coded slice of a Random Access Point

  - For each RAP, the three bits that are assigned for ‘nuh_temporal_id_plus1’ for other NAL unit are used to indicate types of the RAP (e.g., IDR / BLA / CRA, with/without TFD/DLP/LP).

# Introduction

According to the HEVC DIS specification outputted at the 10th JCT-VC meeting, the structure of NAL unit header is as shown in the following table.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_reserved_zero_6bits | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

Out of many of possible NAL unit types (i.e., 64 types since nal_unit_type is represented with 6 bits), the NAL unit types assigned for Random Access Point (i.e., Instantaneous Decoding Refresh – IDR, Broken-link Access – BLA, and Clean Random Access – CRA) are as the following:

- NAL unit type 7 – BLA_W_TFD – Coded slice of a BLA picture with Tagged for Discard

- NAL unit type 8 – BLA_W_DLP – Coded slice of a BLA picture with Decodable Leading Picture

- NAL unit type 9 – BLA_N_LP – Coded slice of a BLA picture without Leading Picture

- NAL unit type 10 – IDR_W_LP – Coded slice of an IDR picture with Leading Picture

- NAL unit type 11 – IDR_N_LP – Coded slice of an IDR picture without Leading Picture

- NAL unit type 12 – CRA_NUT – Coded slice of a CRA picture

# Problem description and proposed modification

## IDR and BLA NAL units

Of the three types of NAL units for RAP, IDR and BLA NAL units are very similar in behaviour. The difference between these two NAL units is that naturally an encoder would not encode a picture as BLA, rather, it would use either IDR or CRA. CRA NAL unit can be changed by system / network application (e.g., Media-Aware Network – MANE) to BLA NAL unit. Once a CRA NAL unit is changed to BLA NAL unit, it will be treated similar to IDR NAL unit. Given such situation, it is redundant to have two NAL unit types with similar behaviour. These two NAL units (i.e., IDR and BLA) can be simplified by removing one of them, preferably BLA NAL unit.

The necessary modifications to the HEVC specification following the removal of BLA NAL unit and overloading its functionalities to IDR are as the following:

- IDR NAL unit should be divided into:

  - IDR_W_TFD: IDR with Tagged for Discard NAL unit type

An IDR picture having nal_unit_type equal to IDR_W_TFD may have associated TFD pictures present in the bitstream.

  - IDR_W_DLP: IDR with Decodable Leading Piture NAL unit type

An IDR picture having nal_unit_type equal to IDR_W_DLP does not have associated TFD pictures present in the bitstream, but may have associated DLP pictures in the bitstream.

  - IDR_N_LP: IDR with no Leading Picture NAL unit type

An IDR picture having nal_unit_type equal to IDR_N_LP does not have associated leading pictures present in the bitstream

- There is no need to specify IdrPicFlag, rather, the use of RapPicFlag would be enough.

- Some modifications are needed at slice header. They are as the following:

  - Previously, when a slice is part of IDR picture, then the following syntax elements are not signalled in the slice header, they are: pic_order_cnt_lsb and all syntax element related to Reference Picture Set in slice header

  - With this modification:

    - pic_order_cnt_lsb and all syntax element related to Reference Picture Set in slice header is signalled even when the slice is part of IDR picture.

    - RPS for the slices that are part of IDR picture may have RPS but the RPS will be ignored.

Syntax table of slice header that incorporate the above proposed items is shown at the end of this document.

The following constraints may need to be added:

- It is a constraint of the bitstream that if current slice is a part of an IDR picture, the value of pic_order_cnt_lsb shall be inferred as 0 regardless what the actual value signalled in the slice header.

- It is a constraint of the bitstream that if current slice is a part of an IDR picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr regardless what the actual values signalled in reference picture set (RPS).

Figure 1 illustrates one of scenario of changing CRA to IDR_N_LP NAL unit when trick mode (i.e., random access by fast forwarding) is applied. System can simply change the NAL unit type of RAP from CRA to IDR_N_LP and remove TFD NAL units. The rest NAL units that follow the IDR_N_LP are common slice (CS) that can be decoded safely.

Figure 1 – Handling trick mode at system level

## RAP NAL units and temporal id

According to the HEVC DIS specification, it is noted that:

TemporalId = nuh_temporal_id_plus1 – 1

and

“If nal_unit_type is in the range of 7 to 12 (coded slice of a RAP picture), inclusive, TemporalId shall be equal to 0; otherwise, when nal_unit_type is in the range of 3 to 6, inclusive (coded slice of a TSA or STSA picture), TemporalId shall not be equal to 0”

From the above, it is clear that there is redundancy in the use of three bits that is currently assigned for nuh_temporal_id_plus1 since for RAP NAL units, they are all fixed to the same value, that is 1 (i.e., 001b).

For better use of the three bits currently assigned for signalling nuh_temporal_id_plus1, we propose the following concept:

- For RAP NAL units, there is no need to signal temporal_id information in NAL unit header. It can be simply fixed / inferred as 0.

“If nal_unit_type is in the range of 7 to 12 (coded slice of a RAP picture), inclusive, TemporalId shall be inferred as equal to 0”

- For RAP NAL units, the three bits that is currently assigned for nuh_temporal_id_plus1 should be used for signalling other information or otherwise be reserved.

Related to the point no. 2 above, the three bits can be utilized for simplifying NAL unit types for RAP NAL units (i.e., 6 NAL unit types now) as follow:

- Instead of allocating 6 NAL unit types for RAP NAL units, they can be simplify as only one NAL unit type.

RAP_NUT – Coded slice of a Random Access Point

- For each RAP, the three bits that are assigned for ‘nuh_temporal_id_plus1’ for other NAL unit are used to indicate types of the RAP (e.g., IDR / BLA / CRA, with/without TFD/DLP/LP).

With the simplification, the table of NAL unit header is as the following:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_reserved_zero_6bits | u(6) |
| if (nal_unit_type != RAP_NUT) |  |
| nuh_temporal_id_plus1 | u(3) |
| else |  |
| nuh_rap_type | u(3) |
| } |  |

The following table tabulates the associated RAP type for each possible value of nuh_rap_type.

| nuh_rap_type value | Descriptor |
| --- | --- |
| 0 | Forbidden / unused |
| 1 | IDR_W_TFD |
| 2 | IDR_W_DLP |
| 3 | IDR_N_LP |
| 4 | CRA |
| 5 | Reserved |
| 6 | Reserved |
| 7 | Reserved |

NOTE:

- Semantic, behaviour, and processing process of each RAP type listed in the above table shall be the same as that of specified in the HEVC DIS specification outputted at the 10th JCT-VC meeting

- Value 0 shall not be used to avoid all zero bits in the second byte of the NAL unit header.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pic_slice_level_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |