| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0167_r1 |
| --- | --- |

| Title: | AHG 9: On dependent slice |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is observed that current slice header signalling of dependent slices might have the following issue:

- Error resilience at picture level.

The missing of ‘pic_order_cnt_lsb” in header of a dependent slice cause decoder cannot derive POC of the slice correctly if its previous slice(s) is / are not available. In worse case, decoder may not aware that the unavailable slice is lost and thus treated the dependent slice as part of previous picture, which may have been reconstructed.

- Parallelism issue when used together with WPP.

According to the current specification, the reference slice of dependent slice is the preceding slice containing the coding tree block for which the coding tree block address is ctbAddrTStoRS[ ctbAddrRStoTS[ slice_address ] − 1 ]. When WPP is used, the preceding slice might not be completely decoded when decoding of the dependent slice is started.

- Parallelism issue when used together with WPP.

If the lost of preceding slice is unknown to the decoder who is handling the following dependent slice, it may refer to incorrect reference slice causing incorrect reconstruction which may propagate till the last slice in the picture.

This contribution proposes:

- To signal ‘pic_order_cnt_lsb” in the header of dependent slice.

- To signal slice address of the reference slice in the header of dependent slice so that the reference slice can be easily identified and if it is lost, it can be quickly known.

# Introduction

Dependent slices are slices that have dependency (i.e., cabac and or prediction dependency) to their previous slices. Slice header of dependent slices is shorter than ordinary slices as it carries only the following syntax elements / information whereas missing information shall be inferred / copied from previous slice:

- first_slice_in_pic_flag

- no_output_of_prior_pics_flag

- pic_parameter_set_id

- slice_address

- dependent_slice_flag

- Tiles / WPP entry points information

Signalling minimum information in the header of a dependent slice while the rest of information that are not present is derived from previous slices reduce the signalling overhead. However, we identified some shortcomings in the signalling design of dependent slice as described in the following section.

# Problem Statement

## Error resilience at picture level

We observe that the missing of ‘pic_order_cnt_lsb” in header of a dependent slice cause decoder cannot derive POC of the slice correctly if its previous slice(s) is / are not available. In worse case, decoder may not aware that the unavailable slice is lost and thus treated the dependent slice as part of previous picture.

Figure 1 illustrates a bitstream in which each pictures are coded into two slices of ordinary slice and dependent slice. Consider an example case where one of the ordinary slices is lost, for example the first slice of picture 102. Such lost will cause decoder to incorrectly interpret POC of the second slice of picture 102 because decoder simply interprets the second slice of picture 102 as a dependent slice that belongs to picture 101. Such error will consequently ruin the reconstruction of picture 101. This is a worse consequence as the lost affect only current picture but also previous correctly decoded picture.

Figure 1 – Streams with 2 slices of independent and dependent slice) per picture

## Parallelism issue when used together with WPP

According to the HEVC specification at Draft for International Standard (DIS) outputted at the 10th JCTVC meeting, it is stated that “the value of each slice header syntax element not present in dependent slice is inferred to be equal to the value of corresponding slice header syntax element in the preceding slice containing the coding tree block for which the coding tree block address is ctbAddrTStoRS[ ctbAddrRStoTS[ slice_address ] − 1 ].”

According to the above definition, the naïve way to find the reference slice of a dependent slice is as follow:

- Parse slice_address of the dependent slice

- Find from among previously received slices to find the slice whose last treeblock address is slice_address of the dependent slice – 1.

With such technique, it is difficult to use dependent slice together with wave front parallel processing because the decoding of a slice can be started before previous slice has been completely decoded.

Figure 2 illustrates the parallelism problem of dependent slice when used together with WPP. As slice S4 is a dependent slice, some syntax elements in its header shall be copied from header of slice S3. However, when decoding of slice S4 is started, slice S3 might not be known yet.

Figure 2 – Parallelism problem when dependent slice & WPP are used

## Error resiliency at slice level when used together with WPP

Figure 2 illustrates a picture coded with wave front parallel processing and each substream except the first one is coded dependent slice. Consider the scenario where the 3rd dependent slice is lost. Such lost will not be noticed by parallel decoder when the 4th slice is to be decoded. Decoder may simply assume that the slice that the 4th slice depends on is the previously received slice, which is the 2nd slice. If decoder is aware that the reference slice of 4th slice is missing, it can apply error concealment.

Figure 3 – Wavefront parallel processing coded picture with dependent slices

# Proposed solutions

To address the above problems, we propose:

- ‘pic_order_cnt_lsb” is signalled in the header of dependent slice.

- For dependent slice, slice address of the reference slice is signalled in its header so that the reference slice can be easily identified and if the reference slice is lost, it can be quickly known.

The required modifications to slice header syntax and semantics are as the following.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( dependent_slice_flag ) { |  |
| ref_slice_address | u(v) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| if( !dependent_slice_flag ) { |  |
| … |  |
| } |  |
| … |  |
| } |  |

ref_slice_address specifies the address of the slice whom the current slice depends on. The length of the slice_address syntax element is Ceil( Log2( PicSizeInCtbsY ) ) bits. The value of slice_address shall be in the range of 1 to PicSizeInCtbsY – 1, inclusive.

The variable BaseSliceAddrRS is derived as follows.

- If dependent_slice_flag is equal to 0, BaseSliceAddrRS is set equal to slice_address.

- Otherwise BaseSliceAddrRS is set equal to BaseSliceAddrRS of the preceeding slice whose slice_address equals ref_slice_address. containing the coding tree block for which the coding tree block address is ctbAddrTStoRS[ ctbAddrRStoTS[ SliceCtbAddrRS ] − 1 ].

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).