| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0168 |
| --- | --- |

| Title: | AHG 9: Short Slice Header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current HEVC specification, all syntax element of slice header shall present in even if they do not change from slice to slice within the same picture, unless the slice is a dependent slice. It is observed that in environment where error resiliency is not critical, duplicating syntax elements in slice headers do not have to be signalled. Efforts to accommodate such concept have been proposed in JCTVC-I0070 and JCTVC-J00109.

This contribution proposes short slice header that is indicated by ‘short_slice_header_flag’. When the flag is set, slice header contains only syntax elements that changes from previous regular slice header (previous slice header with the flag not set) and syntax elements that are not present can be copied from previous regular slice header. It is constrained that the first slice of a picture shall always have regular slice header.

When short_slice_header_flag is set,

the following syntax elements shall not present:

- pic_output_flag

- colour_plane_id

- RPS related syntax elements

the present of the following syntax elements shall be controlled by a new flag

- Reference picture ordering & modification

- Weighted predictions

- Slice QP information

- Deblocking

# Introduction

Approaches to reduce signalling syntax elements in slice headers whose values are unchanged were proposed in JCTVC-I0070 [1] and JCTVC-J0109 [2]. It was reported in [1] that by avoiding sending those unchanged values, a 1.5% average gain in BD-rate in low –delay B main configuration can be achieved when compared to HM6.0 when a slice size of about 36 LCUs of size 64x64 was used.

The method in [1] involves carriage of slice header in the access unit delimiter (AUD) NAL unit the method in [2] involves signalling slice header in a new parameter set so called Header Parameter Set (HPS). In both approaches, a flag mechanism is used to indicate whether a slice header is the same as the one in AUD NAL unit, for method in [1], or as the one in HPS, for method in [2]. While the concepts are promising, it was commented during discussion that:

- The AUD can only be in one place and cannot be repeated within one access unit.

- It might be rather late to consider switching to the approach that defines a new parameter set with a new header prediction scheme.

To address the above concerns, in this document we propose short slice header as described below.

# Short slice header

## Concept description

Short slice header is slice header of a coded slice (e.g., HEVC slice) in which not all syntax elements / parameters in a typical slice are signalled. A short slice header cannot be an independent slice header but instead it needs to be used together with other regular slice header. Syntax elements / parameters that are not signalled in short slice header shall be inferred (e.g., copied) from other regular slice header.

Figure 1 illustrates an example of the use of regular slice headers and short slice headers when a picture is coded with multiple slices. As illustrated, there are many possible ways of application of short slice headers. For example:

- A picture can be coded with slices where all slice headers are regular slice headers as in picture 1 (ie., slice 1A ~ 1F)

- A picture can be coded with multiple slices in which only the first slice has regular slice header while the rest slices have short slice headers

- A picture can be coded with multiple slices in which each slices with regular slice headers are followed by several slices with short slice headers as in picture 3 and 4.

Figure 1 – Examples of possible combination of using short slice header

It is a restriction of this concept that when a picture is coded with multiple slices, the first slice of the picture cannot be a short slice header.

When parsing short slice header, syntax elements / parameters that are not signalled in the slice header shall be inferred (e.g., copied) from previously regular slice header as illustrated in Figure 2.

Figure 2 – Short slice header refers to regular slice header

When a slice with regular slice header is not available (e.g., lost during transmission), short slice headers of slices that come after the missing slice with regular slice header shall simply be inferred from any available regular slice header that belongs to the same picture. Figure 3 illustrates both conditions where referencing can be done from past and from future complete slice headers. When there is at least one past (i.e., previously received) complete slice header, referencing for copying non-presenting syntax element in short headers can be from the past complete slice headers as illustrated in Figure 3 (a). However, if no past complete slice header is available, referencing can be also from the future (i.e., to be received) complete slice header as illustrated in Figure 3 (b).

, slice header C, header is not available. Slice header D, which is a short slice header that supposed to refer to slice header C, shall simply refer to slice header A, which is the regular slice header that is available.

Figure 3 – Changing reference when a referenced slice header is not available

## Signaling

The table below tabulates syntax table for signalling short slice header. The modifications added to the syntax table of slice header defined in the HEVC DIS specification outputted at the 10th JCTVC meeting are as the following:

- Add a flag to indicate whether the slice has regular slice header or short slice header

  - short_slice_header_flag

- Dependent slice is considered as subset of slice with short slice, thus, the flag to indicate dependent slice is made conditionally to short slice header flag.

- The following information are always signalled no matter the type of slice header

  - First slice of a picture flag

  - PPS ID

  - Slice address, except when the slice is the first slice

  - POC LSB information

  - Slice type, except if the slice is dependent slice

  - Temporal MVP enable flag

  - Entry point information for tiles & WPP

- When short slice header is used, the following information is not signalled:

  - Output information

  - Colour plane information

  - Reference picture set for short term and long term reference pictures

- When short slice header is used, signalling of the following information is controlled by flags.

  - Reference picture ordering and modification

  - Weighted prediction parameters

  - QP slice information

  - Deblocking parameters.

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) { |  |
| slice_address | u(v) |
| short_slice_header_flag | u(1) |
| if( dependent_slice_enabled_flag && short_slice_header_flag) |  |
| dependent_slice_flag | u(1) |
| } |  |
| if( !IdrPicFlag ) |  |
| pic_order_cnt_lsb | u(v) |
| if( !short_slice_header_flag ) { |  |
| slice_type | ue(v) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| Else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if ( !dependent_slice_flag ) { |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( short_slice_header_flag ) |  |
| slice_ ref_list_modification_present_flag | u(1) |
| if( slice_ ref_list_modification_present_flag ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) { |  |
| if( short_slice_header_flag ) |  |
| slice_weighted_pred_present_flag | u(1) |
| if( slice_weighted_pred_present_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| } |  |
| if( short_slice_header_flag ) |  |
| slice_qp_present_flag | u(1) |
| if( slice_qp_present_flag ) { |  |
| slice_qp_delta | se(v) |
| if( pic_slice_level_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( short_slice_header_flag ) |  |
| deblocking_param_present_flag | u(1) |
| if( deblocking_param_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

short_slice_header_flag equals to 0 indicates that all syntax element needed for processing the slice is present in this slice header. Short_slice_header_flag equals to 1 indicates that some syntax elements are not present in this slice header and must be copied from other slice header.

slice_ref_list_modification_present_flag equals to 0 indicates syntax elements for overriding number of active reference pictures and syntax elements for reference picture modification are not present in this slice header and shall refer to other slice header. slice_ref_list_modification_present_flag equals to 1 indicates that syntax elements for overriding number of active reference pictures and syntax elements for reference picture modification may present in this slice header. When not present, the value of slice_ref_list_modification_present_flag is inferred as 1.

slice_weighted_pred_present_flag equals to 0 indicates syntax elements for weighted prediction are not present in this slice header and shall refer to other slice header. slice_weighted_pred_present_flag equals to 1 indicates syntax elements for weighted prediction may present in this slice header. When not present, the value of slice_weighted_pred_present_flag is inferred as 1.

slice_qp_present_flag equals to 0 indicates syntax elements for slice QP delta and offsets are not present in this slice header and shall refer to other slice header. slice_qp_present_flag equals to 1 indicates syntax elements for slice QP delta and offsets may present in this slice header. When not present, the value of slice_qp_present_flag is inferred as 1.

deblocking_param_present_flag equals to 0 indicates syntax elements for deblocking parameters are not present in this slice header and shall refer to other slice header. deblocking_param_present_ flag equals to 1 indicates syntax elements for deblocking parameters may present in this slice header. When not present, the value of deblocking_param_present_ flag is inferred as 1.

Semantic of other syntax elements refer to semantic of the same syntax element in HEVC DIS specification outputted from the 10th JCTVC meeting.

## Constraints

When short slice header is used and there is more than one slice with regular slice header in a picture, the following constraints shall be applied to slice headers of the picture:

- All the regular slices in the same picture shall have the same values for the following syntax elements

  - no_output_of_prior_pics_flag

  - pic_output_flag

  - colour_plane_id

  - Reference picture set sytanx elements

  - mvd_l1_zero_flag

  - cabac_init_flag

  - five_minus_max_num_merge_cand

  - slice_sao_luma_flag

  - slice_sao_chroma_flag

  - slice_loop_filter_across_slices_enabled_flag

- The following syntax element shall have the same values in all regular slice headers; otherwise, slice_ref_list_modification_present_flag in all short slice headers shall have value 1.

  - num_ref_idx_active_override_flag

  - num_ref_idx_l0_active_minus1

  - num_ref_idx_l1_active_minus1

  - slice_temporal_mvp_enable_flag

  - collocated_from_l0_flag

  - collocated_ref_idx

  - ref_pic_list_modification( )

- The following syntax element shall have the same values in all regular slice headers; otherwise, slice_weighted_pred_present_flag in all short slice headers shall have value 1.

  - pred_weight_table( )

- The following syntax element shall have the same values in all regular slice headers; otherwise, slice_qp_present_flag in all short slice headers shall have value 1.

  - slice_qp_delta

  - slice_cb_qp_offset

  - slice_cr_qp_offset

- The following syntax element shall have the same values in all regular slice headers; otherwise, deblocking_param_present_flag in all short slice headers shall have value 1.

  - deblocking_filter_override_flag

  - slice_header_disable_deblocking_filter_flag

  - beta_offset_div2

  - tc_offset_div2

# Experiment Results

We have run two experiments to investigate the effect of the proposed short slice header to the coding performance. The experiments are designed as follow:

- Common test condition with 1500 bytes per slice mode.

- Common test condition with the following condition:

  - Every picture is divided into class D size tiles

  - Each tile is coded into one slice

In both experiments, only the first slice has of each picture has regular slice header whereas the rest of the slices have short slice header.

The performance of the first experiments is as shown in the following tables (to be completed in revision of this document).

The performance of the second experiments is as shown in the following tables (to be completed in revision of this document).

# References

- JCTVC-I0070, “AHG9: Header parameter set (HPS),” 9th JCT-VC Meeting, Geneva, CH, 11-20 July, 2012.

- JCTVC-J0109, “Nested hierarchy of tiles and slices through slice header prediction”, 10th JCT-VC Meeting, Stockholm, SE, 27 April - 07 May, 2012.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).