| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0169_r1 |
| --- | --- |

| Title: | AHG 9: On layer ID partition in VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

JCTVC-J1007 was in previous meeting and it contains two candidate approaches for straw man designs for VPS designs. Both approaches provides mapping of each value of the 6-bit layer_id (i.e., the reserved_zero_6bits in the HEVC based specification) in the NAL unit header to specific list of dimension IDs (e.g., view_order_index, depth_flag, dependency_id and quality_id).

Approach 1 uses a very generic signalling method in which each layer may freely define a set of dimension type (e.g., view_order_index, depth_flag, dependency_id, quality_id, etc). It is observed that such design may be too generic as the list of dimension types can change from layer to layer within the same bitstream and causes a burden to coding system to accommodate such possible rapid change.

Approach 2 uses a more restricted signalling method in which each layer can only belong to a particular combination of dimension types defined in a look-up table within the specification. It is observed that having a look-up table consisting possible combination of dimension type is too restricting as it is difficult to predict all the possible combination of dimension types, thus, it does not seem like a future-proof design.

This contribution proposes an alternative method for signalling layer_ID in VPS with the spirit of combining the desired and avoid shortcoming of approach 1 and 2 in JCTVC-J1007. The proposed design is summarized as follow:

- List of possible dimension type (i.e., dimension_type), and the possible number of layers in this dimension (i.e., dimension_len) are signalled outside the loop for layer_ID.

- The sum of the length of dimension_len is not restricted to 6.

- The number of bits to represent vps_layer_id [i] depends on the sum of number of bits for dimension_len.

# Problem Statement

In the 10th JCTVC meeting, JCTVC-J1007 was outputted and it contains two approaches for straw man designs for video parameter set designs. Both approaches provides mapping of each value of the 6-bit layer_id (i.e., the reserved_zero_6bits in the HEVC based specification) in the NAL unit header to specific list of dimension IDs (e.g., view_order_index, depth_flag, dependency_id and quality_id). However, the two approaches have different method of defining the list of dimension IDs.

Approach 1 uses a very generic signalling method in which each layer may freely define a set of dimension type (e.g., view_order_index, depth_flag, dependency_id, quality_id, etc). Generic feature of the signalling method is desired as it will be easier to anticipate any type of combination of layer dimensions, particularly when scalability dimensions and multi-view dimension are to be combined / used together). However, approach 1 may be too generic as the list of dimension types may change from layer to layer within the same bitstream. This may be a burden to coding system to accommodate such possible rapid change.

On the other hand, approach 2 uses a more restricted signalling method in which each layer can only belong to a particular combination of dimension types. The combination is defined like a look-up table within the specification. With such method, the list of dimension types is defined and cannot change from layer to layer throughout the bitstream, which we think is good. However, having a look-up table consisting possible combination of dimension type is too restricting as it is difficult to predict all the possible combination of dimension types. Having look-up table of possible combination of dimension does not seem like a future-proof design.

In this contribution, an alternative method for signalling layer_ID in VPS is proposed by combining the desired and avoid shortcoming of both above approaches. The proposed method might be consider as the compromised between both approaches.

# Proposed layer_ID partitioning in VPS

We propose that the layer_ID partitioning in VPS is signalled as follow:

- List of possible dimension type (i.e., dimension_type), and the possible number of layers in this dimension (i.e., dimension_len) are signalled outside the loop for layer_ID.

  - This consequently prevent list of dimension types change from layer to layer within bitstream.

- The sum of the length of dimension_len is not restricted to 6.

  - Each dimension_len [i] can have value up to 8 since three bits are assigned for signalling this.

  - Having 8 bits from signalling dimension_len would be sufficient as it supports up to 256 layers.

- The number of bits to represent vps_layer_id [i] depends on the sum of number of bits for dimension_len.

The logic of not limiting the length of dimension_len to 6 is that there is no guarantee that each dimension type would have number of layer up to its potential given by its bits allocation. For example, although we may give 3 bits for spatial scalability, which means it can be up to 8 layers, there is no guarantee that the bitstreams will contains 8 spatial scalability layers. When number of layers for a particular dimension is not used up to its potential, it will affect the number of possible layers assigned for other dimension. Thus, it is safer to remove the limit of length of dimension_len.

Consider the following hypothetical use case:

- There are 3 views in the bitstream (which is a common test conditions in JCT-3V)

- There are both texture views and depth views (again, a common test condition in JCT-3V)

- Texture view components have two spatial enhancement layers (just hypothetical assumption that multiview and scalable coding would be combined)

- Each spatial layer has three quality layers

- There are no spatial or quality enhancement for the depth views (their bitrate is typically low in any case, so scalable coding might not make that much sense)

Thus, there are

- For texture: 3 views x 3 spatial scalability layers x 3 quality layers = 27 layer representations

- For depth: 3 views

- Altogether: 30 layer representations

In the bit partitioning approach, the number of bits needed would be:

- View_id: 2 bits

- Depth_flag: 1 bit

- Dependency_id: 2 bits

- Quality_id: 2 bits

- Altogether: 7 bits.

The syntax table and semantic of the proposed signalling are as the following:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| // layer specific information |  |
| num_dimension_minus1 | ue(v) |
| for (i = 0; i <= num_dimension_minus1; i++) { |  |
| dimension_type [i] | u(4) |
| dimension_len [i] | u(3) |
| } |  |
| for (i = 1; i <= vps_max_layers_minus1; i++) { |  |
| vps_layer_id [i] | u(n) |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

vps_extension_byte_alignment_reserved_one_bit shall be equal to 1.

num_dimensions_minus1 [ i ] plus 1 specifies the number of dimension types and IDs signalled for each layer.

dimension_type[ i ] specifies the j-th scalability dimension type of the i-th layer, which has layer_id equal to i, as specified in the following table:

| dimension_type[ i ][ j ] | dimension_id[ i ][ j ] |
| --- | --- |
| 0 | view order idx |
| 1 | depth flag |
| 2 | dependency ID |
| 3 | quality ID |
| 4..15 | Reserved |

dimension_len[ i ] specifies the length, in bits, of the i-th layer dimension ID. The value of dimension_len[ i ] shall be in the range 1 to 8.

vps_layer_id[ i ] specifies the value of layer_id of the i-th layer to which the following layer dependency information applies. The length of the vps_layer_id [i] syntax element is the sum of dimension_len [i].

Each bit in vps_layer_id[ i ] is structured as follow:

dimension_len [0] + dimension_len [1] + … + dimension_len [num_dimension_minus1]

num_direct_ref_layers[ i ] specifies the number of layers the i-th layer directly depends on.

ref_layer_id[ i ][ j ] identifies the j-th layer the i-th layer directly depends on.

The following example describes how the above elements are used to signal layer_ID partition. Suppose that a bitstream supports spatial and quality scalabilities. For spatial scalability, it supports up to the second enhancement whereas for quality scalability, it supports up to first enhancement scalability. The signalling can be arranged as the following:

vps_max_layers_minus1 = 5

num_dimension_minus1 = 1

dimension_type [0] = 2

dimension_length [0] = 2

dimension_type [1] = 3

dimension_length [1] = 1

vps_layer_id [1] = 1  (001)

num_direct_ref_layers[1] = 1

ref_layer_id[ 1 ][ 0 ] = 0

vps_layer_id [2] = 2  (010)

num_direct_ref_layers[2 ] = 1

ref_layer_id[ 2 ][ 0 ] = 0

vps_layer_id [3] = 3  (011)

num_direct_ref_layers[3] = 1

ref_layer_id[ 3 ][ 0 ] = 2

vps_layer_id [4] = 4  (100)

num_direct_ref_layers[4] = 1

ref_layer_id[ 4 ][ 0 ] = 2

vps_layer_id [5] = 5  (101)

num_direct_ref_layers[5] = 1

ref_layer_id[ 5 ][ 0 ] = 4

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).