| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0170 |
| --- | --- |

| Title: | AHG 9: On restricted reference picture list |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Yongjoon Jeon Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com yongjoon.jeon@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In current HEVC specification, a flag called ‘restricted_ref_pic_lists_flag’ is signalled in SPS to indicate indicate whether or not all P and B slices (if present) that belong to the same picture have an identical reference picture list 0, and that all B slices (if present) that belong to the same picture have an identical reference picture list 1. This flag provides a good feature to helps decoder to determine whether reference picture construction process can be done once per picture or must be done once per slice.

It is observed that due to scene fluctuation complexity, reference picture lists can be identical for all slices within a coded picture for some period of time and change from slice to slice within a coded picture in another period of time. In such situation, the ‘restricted_ref_pic_lists_flag’ must be set to 0. Consequently, decoder cannot take advantage of possibility to apply single reference picture list construction per picture for the period where reference picture list within a picture is identical.

This contribution proposes to move ‘restriction_ref_pic_lists_flag’ from SPS to PPS so that it is easier to change its value within same sequence. In addition, to avoid parsing problem, it is also proposed to move ‘lists_modification_present_flag’ from SPS to PPS.

# Problem Statement

For HEVC coding scheme, a flag called ‘restricted_ref_pic_lists_flag’ is signalled in SPS to indicate whether or not all P and B slices (if present) that belong to the same picture have an identical reference picture list 0, and that all B slices (if present) that belong to the same picture have an identical reference picture list 1. When the flag equals to 1, it is an indication that reference picture construction process can be done once for every picture, rather than once for every slice, which is a good feature since it can reduce processing complexity.

Further, when ‘restricted_ref_pic_lists_flag’ equals to 1, another flag called ‘lists_modification_present_flag’ must be signalled to indicate whether more information for reference picture list modification presents. On the other hand, when ‘restricted_ref_pic_lists_flag’ equals to 0, the flag ‘lists_modification_present_flag’ is not signalled since it is obvious that it must be 1.

It is observed that signalling ‘restricted_ref_pic_lists_flag’ in SPS is not flexible since it mandates that the rule, that is whether or not reference picture construction process can be done once for every picture, rather than once for every slice, is applied throughout the sequence. Such rule is too restricting because in practice due to scene complexity fluctuation, some parts of the sequence have less complicated scene so that the order of reference picture in reference picture list does not have to be changed from slice to slice but on the other hand, in some parts of the sequence have complicated scene so that the order of reference picture in reference picture list should be changed from slice to slice, in order to achieve better compression efficiency.

Figure 1 illustrates the situation in which reference picture list within a picture can be identical in all slices for a certain period of time and not identical (i.e., change from slice to slice) for a certain period of time. In such situation, current ‘restricted_ref_pic_lists_flag’ must be set to 0. Consequently, decoder cannot take advantage of possibility to apply single reference picture list construction per picture for the period where reference picture list within a picture is identical.

Figure 1 – Example of dynamically change reference picture list within a coded picture

# Proposed modification

To deal with the above problem, we propose to assign different types of NAL units for leading pictures that are needed for reference and leading pictures that are not needed for reference.

To improve current restricted reference picture list feature, we propose the following:

- Move the flag ‘restricted_ref_pic_lists_flag from SPS to PPS so that the value can change from time to time within coded bitstream.

- Move the flag ‘lists_modification_present_flag’ from SPS to PPS to avoid parsing dependency problem

With the above two points, the syntax table for PPS should be modified as the following

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| … |  |
| } |  |

With the proposed modification, the problem illustrated in section 1 can easily be solved by inserting new PPS more often such as illustrated in Figure 2. The value of ‘restricted_ref_pic_lists_flag’ shall be 1 in PPS 0 and 2 and shall be 0 for PPS 1 and PPS 3.

Figure 2 – Solution for dynamically change reference picture list within a coded picture

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).