| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0171 |
| --- | --- |

| Title: | AHG7: Transforms for extended chroma formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents an approach to transforms for the extended chroma formats that avoids the introduction of 32-point transforms for both the 4:2:2 and 4:4:4 chroma format (in either horizontal or vertical directions) by splitting the region in chroma into multiple 16x16 regions, thus reusing existing 16-point transform logic.  It is asserted that this is beneficial for implementations having separate luma and chroma processing paths, as 32-point transform logic does not need to be introduced into the chroma processing path.

# Problem statement

At the 10th JCT-VC meeting in Stockholm, two contributions presented HM software modified to introduce support for extended chroma formats [1] [2].  Each of these contributions proposed introducing non-square transforms into the chroma channel to handle cases arising from the 4:2:2 chroma format.

The available transforms for the HM-8.0-dev-ahg7 branch for extended chroma formats are listed in Table 1.

Table 1. Transform sizes for HM-8.0 extended chroma formats.

| Luma | Chroma |  |  |
| --- | --- | --- | --- |
|  | 4:2:0 | 4:2:2 | 4:4:4 |
| 4x4 | 4x4 | 4x8 | 4x4 |
| 8x8 | 4x4 | 4x8 | 8x8 |
| 16x16 | 8x8 | 8x16 | 16x16 |
| 32x32 | 16x16 | 16x32 | 32x32 |

It is asserted that implementations commonly have decoupling between the luma processing path and the chroma processing path and thus require separate transform logic for each processing path.  Introducing support for the extended chroma formats according to Table 1 will therefore require introducing 32-point transforms into the chroma processing path. Especially for the 4:4:4 chroma format, it is asserted that the required 32x32 transform occupies significant area for a hardware design.

# Description of the proposed change

For the 32x32 transform unit, for 4:2:2, the resulting 16x32 transform for each chroma channel is split into two 16x16 transforms and for 4:4:4, the resulting 32x32 transform for each chroma channel is split into four 16x16 transforms.  This operation eliminates the usage of any 32-point transform logic in the chroma processing path.

A separate coded block flag is required for each of the 16x16 transforms.

As each of the smaller transforms may have all zero coefficients and the residual coding syntax cannot express a block of all zero valued residual coefficients, one coded block flag and a set of residual syntax is required for each of the smaller transform per chroma channel.

# Experiment Results

Results are presented in Table 2 and Table 3 below. Simulations were performed under the common conditions [3]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Context initialization values were not updated in this simulation.

Table 2. Results for 4:2:2 chroma format in HM-8.0.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono | 2.7% | 4.1% | 4.5% |
| Parkscene | 0.4% | 1.2% | 1.2% |
| Birds-in-cage | 3.8% | 5.5% | 10.5% |
| Ducks-and-legs | 0.6% | 1.3% | 1.7% |
| Traffic | 0.5% | 1.1% | 1.3% |
| Crowd-run | 0.3% | 0.4% | 0.5% |
| Old-town | 0.4% | 1.5% | 1.9% |
| Seeking | 0.6% | 1.0% | 1.1% |
| Overall | 1.2% | 2.0% | 2.8% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 112% |  |  |

Table 3. Results for 4:4:4 chroma format in HM-8.0.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono | 2.9% | 3.8% | 4.7% |
| Parkscene | 0.4% | 1.1% | 1.1% |
| Birds-in-cage | 3.6% | 4.7% | 8.7% |
| Ducks-and-legs | 0.6% | 1.3% | 1.3% |
| Traffic | 0.5% | 0.9% | 1.0% |
| Crowd-run | 0.3% | 0.8% | 0.6% |
| Old-town | 0.6% | 2.5% | 1.5% |
| Seeking | 0.6% | 1.2% | 1.2% |
| Overall | 1.2% | 2.0% | 2.5% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 112% |  |  |

# Working Draft changes

See file included in archive.

# Conclusion

This contribution presents an approach to the chroma transforms for extended chroma formats that avoids introducing a requirement for a 32-point 1D transform either horizontally or vertically for processing the chroma channels.

# References

- P. Silcock, K. Sharman, N. Saunders, J. Gamei, “Extension of HM7 to Support Additional Chroma Formats”, Joint Collaborative Team on Video Coding, JCTVC-J0191, July 2012.

- K. Kawamura, T. Yoshino, S. Naito, “AHG12: 4:2:2/4:4:4 chroma format extension for HEVC Version 2”, Joint Collaborative Team on Video Coding, JCTVC-J0357, July 2012.

- D. Flynn, “BoG report: Extended chroma formats,” Joint Collaborative Team on Video Coding, JCTVC-J0581, Stockholm, July 2012.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).