| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0172 |
| --- | --- |

| Title: | Modification of zero-delay non-local means filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Seishi Takamura Hiroshi Fujii Atsushi Shimizu 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

In this contribution, non-local means (NLM) filter based on LCU-based framework with zero-delay decodability is utilized to one of the in-loop filters in HM8.0, and an adjusting method of NLM filter strength for each 2x2 sub-block according to activities of surrounding samples is proposed.

In the case of LCU-based RDO except RandomAccess, the average BD-rate for luma component and chroma component improved −0.2-0.6% and 0.9-3.5%, respectively. In the case of picture-based RDO (only RandomAccess), the average BD-rate for luma component and chroma component improved 0.4-0.6% and 0.5-1.2%, respectively. The decoding time increased 110-112% on average. Especially, chroma components of HE10 cases showed better improvements than main profile. The maximum gain evaluated by BD-rate of YUV ((6.0Y+U+V) / 8.0) was 3.8% in Low Delay P (HE10) for the sequence “Kimono”.

# Introduction

In-loop filter that supports to reduce the distortion of a decoded picture is one of important coding tools for improving coding efficiency and visual quality. Reference software of HEVC HM8.0 [1] employs two kinds of in-loop filter deblocking filter and SAO. Here, one of the important factors for in-loop filter of video codec is low-delay capability for encoding and decoding. To achieve this low-delay capability, SAO employs virtual boundary approach [2] avoiding pixel reference to un-reconstructed LCUs.

In this contribution, NLM filter based on LCU-based framework with zero-delay decodability is utilized to one of the in-loop filters in HM8.0, and an adjusting method of NLM filter strength for each 2x2 sub-block according to activities of surrounding samples is proposed. In previous meeting, zero-delay NLM filter [3] which provides LCU-based framework with zero-delay decodability is proposed. However, the framework provided only slice-based enable/disable control for the picture-based optimization. In this contribution, additional two kinds of optimization mode are available for RandomAccess (described in Table 1, and illustrated in Figure 1). Furthermore, to allocate strong filter to noisy region, the adjusting method of NLM filter strength for each 2x2 sub-block is utilized according to the activities of surrounding samples. The decoder can calculate the filter strength with the linear coupling of the integer operation by calculating the LUT table corresponding to each QP beforehand. The detail of this allocation is described in subclause 2.5 (JCTVC-K0172(Decoder description).doc).

| Table 1: Properties of optimization modes |  |  |  |
| --- | --- | --- | --- |
|  | LCU-based quad-tree | Maximum number of filter pattern in a picture | Available GOPs |
| LCU-based optimization | enable | picWidthInCtbsY * picHeightInCtbsY | AI, LD-B, LD-P, RA |
| Picture-based optimization with non-unified single filter | enable | 1 | RA |
| Picture-based optimization with unified single filter | disable | 1 | RA |

Figure 1: Sample illustration of each optimization mode

# Experimental results

In this section, objective evaluations are described.

## Objective evaluation

The source code was compiled by Microsoft Visual Studio 2010 Professional edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7, Intel Core i5-2500K (4.2GHz) and 16GB RAM. Experimental conditions are subject to the common test conditions [4]. The experimental results described in Table 2 and Table 3 present coding efficiencies calculated by BD-rate [9] and computational complexities compared to HM8.0 anchors. In this experiment, RandomAccess is optimized by picture-based optimization with additional two modes (with non-unified single filter and unified single filter), and the other GOPs are optimized by only LCU-based optimization.

| Table 2: Experimental results (mandatory) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main |  |  | All Intra HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -1.4% | -1.5% | 0.0% | -1.5% | -1.5% |
| Class B | 0.1% | -1.1% | -1.7% | 0.1% | -1.5% | -2.2% |
| Class C | 0.0% | -1.1% | -2.0% | -0.2% | -2.0% | -2.9% |
| Class D | 0.0% | -1.1% | -1.8% | 0.0% | -1.8% | -2.9% |
| Class E | 0.0% | -3.8% | -3.8% | -0.2% | -4.1% | -4.1% |
| Overall | 0.0% | -1.6% | -2.0% | -0.1% | -2.0% | -2.6% |
|  | 0.0% | -1.6% | -2.0% | 0.0% | -2.0% | -2.6% |
| Class F | -0.5% | -1.1% | -1.3% | -1.1% | -2.0% | -2.1% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 112% |  |  | 112% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -0.8% | -0.6% | -1.1% | -0.9% | -1.1% |
| Class B | -0.3% | -0.8% | -0.8% | -0.4% | -1.4% | -0.9% |
| Class C | -0.4% | -0.3% | -0.9% | -0.5% | -1.4% | -1.9% |
| Class D | -0.2% | -0.1% | -0.4% | -0.2% | -0.7% | -1.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -0.5% | -0.7% | -0.6% | -1.1% | -1.2% |
|  | -0.4% | -0.5% | -0.7% | -0.5% | -1.1% | -1.2% |
| Class F | -0.8% | -0.8% | -0.9% | -1.5% | -1.9% | -1.8% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 113% |  |  | 113% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -1.2% | -2.6% | 0.0% | -3.4% | -3.8% |
| Class C | 0.0% | -0.6% | -1.3% | -0.2% | -2.7% | -3.4% |
| Class D | 0.0% | 0.0% | -1.1% | 0.0% | -1.8% | -2.8% |
| Class E | 0.9% | -2.1% | -2.2% | 0.6% | -4.1% | -4.0% |
| Overall | 0.2% | -0.9% | -1.8% | 0.0% | -3.0% | -3.5% |
|  | 0.2% | -0.9% | -1.8% | 0.0% | -2.9% | -3.5% |
| Class F | -0.1% | -0.2% | -0.4% | -1.1% | -1.4% | -2.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 110% |  |  | 114% |  |  |

| Table 3: Experimental results (full) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main |  |  | All Intra HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -1.4% | -1.5% | 0.0% | -1.5% | -1.5% |
| Class B | 0.1% | -1.1% | -1.7% | 0.1% | -1.5% | -2.2% |
| Class C | 0.0% | -1.1% | -2.0% | -0.2% | -2.0% | -2.9% |
| Class D | 0.0% | -1.1% | -1.8% | 0.0% | -1.8% | -2.9% |
| Class E | 0.0% | -3.8% | -3.8% | -0.2% | -4.1% | -4.1% |
| Class F | -0.5% | -1.1% | -1.3% | -1.1% | -2.0% | -2.1% |
| Overall | -0.1% | -1.5% | -1.9% | -0.1% | -2.0% | -2.6% |
|  | -0.1% | -1.5% | -1.9% | 0.0% | -2.0% | -2.6% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 113% |  |  | 114% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -0.8% | -0.6% | -1.1% | -0.9% | -1.1% |
| Class B | -0.3% | -0.8% | -0.8% | -0.4% | -1.4% | -0.9% |
| Class C | -0.4% | -0.3% | -0.9% | -0.5% | -1.4% | -1.9% |
| Class D | -0.2% | -0.1% | -0.4% | -0.2% | -0.7% | -1.1% |
| Class E |  |  |  |  |  |  |
| Class F | -0.8% | -0.8% | -0.9% | -1.5% | -1.9% | -1.8% |
| Overall | -0.4% | -0.5% | -0.7% | -0.6% | -1.1% | -1.2% |
|  | -0.4% | -0.5% | -0.7% | -0.5% | -1.1% | -1.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 113% |  |  | 114% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -1.2% | -2.6% | 0.0% | -3.4% | -3.8% |
| Class C | 0.0% | -0.6% | -1.3% | -0.2% | -2.7% | -3.4% |
| Class D | 0.0% | 0.0% | -1.1% | 0.0% | -1.8% | -2.8% |
| Class E | 0.9% | -2.1% | -2.2% | 0.6% | -4.1% | -4.0% |
| Class F | -0.1% | -0.2% | -0.4% | -1.1% | -1.4% | -2.2% |
| Overall | 0.1% | -0.8% | -1.6% | -0.2% | -2.6% | -3.3% |
|  | 0.1% | -0.8% | -1.5% | -0.2% | -2.6% | -3.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 116% |  |  | 115% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.1% | -2.4% | -3.0% | -1.4% | -5.9% | -6.3% |
| Class C | -0.6% | -1.2% | -1.7% | -0.7% | -4.0% | -5.0% |
| Class D | -0.3% | -0.4% | -1.4% | -0.4% | -2.8% | -3.3% |
| Class E | -0.4% | -2.8% | -3.9% | -0.6% | -7.9% | -6.4% |
| Class F | -0.5% | -0.4% | -0.3% | -1.2% | -1.7% | -2.4% |
| Overall | -0.6% | -1.4% | -2.0% | -0.9% | -4.4% | -4.7% |
|  | -0.6% | -1.4% | -2.1% | -0.9% | -4.4% | -4.6% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 119% |  |  | 118% |  |  |

# Conclusion

In this contribution, NLM filter based on LCU-based framework with zero-delay decodability was utilized to one of the in-loop filters in HM8.0, and an adjusting method of NLM filter strength for each 2x2 sub-block according to activities of surrounding samples was proposed.

As the result, In the case of LCU-based RDO except RandomAccess, the average BD-rate for luma component and chroma component improved −0.2-0.6% and 0.9-3.5%, respectively. In the case of picture-based RDO (only RandomAccess), the average BD-rate for luma component and chroma component improved 0.4-0.6% and 0.5-1.2%, respectively. Especially, chroma components of HE10 cases showed better improvements than main profile. The maximum gain evaluated by BD-rate of YUV ((6.0Y+U+V) / 8.0) was 3.8% in Low Delay P (HE10) for the sequence “Kimono”. The decoding time increased 10-12% on average. It is recommended to incorporate the proposal into in-loop filters of HEVC ver.2 with high-coding efficiency profile.

Thanks to KDDI for the cross-verification of the coding results [6].

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” JCTVC-J1003_d7, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012.

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, and S. Lei, “Sample Adaptive Offset with Zero Pixel Line Buffers for LCU-based Decoding,” JCTVC-F055, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- M. Matsumura, S. Takamura, A. Shimizu, “LCU-based framework with zero pixel line buffers for non-local means filter,”JCTVC-J0165, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012.

- F. Bossen, “HM 8 Common Test Conditions and Software Reference Configurations,” JCTVC-J1100, 10th JCT-VC meeting, Stockholm, SE, 11-20 July, 2012.

- G. Bjøntegaard, “Improvements of the BD-PSNR model,” VCEG-AI11, ITU-T SG16 Q.6, 35th VCEG meeting, Berlin, Germany, July 2008.

- T. Yoshino, K. Kawamura and S. Naito, “xxx” JCTVC-Kxxxx, 11th JCT-VC meeting, Shanghai, CN, 10-19 October, 2012.

# Patent rights declaration(s)

NTT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).