| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0172 |
| --- | --- |

| Title: | Modification of zero-delay non-local means filter (Decoder description) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Seishi Takamura Hiroshi Fujii Atsushi Shimizu 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

In this contribution, syntax, semantics and decoder description proposed in JCTVC-K0172 is described

# Syntax and semantics

In this section, syntax and semantics for proposed NLM filter are described. The modified syntax and additional syntax are specified in Table 1 to 5. In Table 1 to 3, the difference between syntax for HM8.0 and proposal set highlighted in yellow.

non_local_means_filter_enable_flag equal to 1 specifies the non-local means filter process is applied to the reconstructed picture after the sample adaptive offset process.

| Table 1: Sequence parameter set RBSP syntax |  |
| --- | --- |
| seq_parameter_set_rbsp( ) { | Descriptor |
| ... |  |
| sample_adaptive_offset_enabled_flag | u(1) |
| non_local_means_filter_enable_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| ... |  |
| } |  |

slice_nlm_filter_enable_flag[cIdx] equal to 1 specifies that NLM filter is enabled for the colour component cIdx in the current slice. slice_nlm_filter_enable_flag[ cIdx ] equal to 0 specifies that NLM filter is disabled for the colour component cIdx in the current slice.

slice_nlm_single_filter_flag[cIdx] equal to 1 specifies that the filter pattern index of all coding tree blocks is assigned as slice_nlm_filter_pattern_index described below.

slice_nlm_unified_filter_flag[cIdx] equal to 1 specifies whether NLM filter is enabled to all coding tree blocks.

slice_nlm_filter_pattern_index[cIdx] specifies the filter pattern index for NLM filter. The value of slice_nlm_filter_pattern_indx[cIdx] shall be in the range of 0 to 3, inclusive.

| Table 2: Slice header syntax |  |
| --- | --- |
| slice_header( ) { | Descriptor |
| if( !dependent_slice_flag ) { |  |
| ... |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if(non_local_means_filter_enable_flag) |  |
| for( i = 0; i < 3; i++ ){ |  |
| slice_nlm_filter_enable_flag[ i ] | u(1) |
| if(slice_nlm_filter_enable_flag[ i ]){ |  |
| slice_nlm_single_filter_flag[ i ] | u(1) |
| if(slice_nlm_single_filter_flag[ i ]){ |  |
| slice_nlm_unified_filter_flag[ i ] | u(1) |
| slice_nlm_filter_pattern_index[ i ] | u(2) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

| Table 3: Coding tree unit syntax |  |
| --- | --- |
| coding_tree_unit( xCtb, yCtb ) { | Descriptor |
| NumPCMBlock = 0 |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| CtbAddrInSlice = CtbAddrRS − slice_address |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> Log2CtbSizeY, yCtb >> Log2CtbSizeY ) |  |
| if(non_local_means_filter_enable_flag) |  |
| for( i = 0; i < 3; i++ ) |  |
| if(slice_non_local_means_filter_flag[ i ]) |  |
| nlm(xCtb, yCtb, i) |  |
| coding_quadtree( xCtb, yCtb, Log2CtbSizeY, 0 ) |  |
| } |  |

nlm_ctb_filter_enable_flag[cIdx][xCtbIdx][yCtbIdx] equal to 1 specifies that NLM filter is enabled for the colour component cIdx in the current slice. slice_non_local_means_filter_flag[cIdx] equal to 0 specifies that NLM filter is disabled for the colour component cIdx in the current slice.

nlm_ctb_merge_left_flag equal to 1 specifies that the value of nlm_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] described below is derived from the corresponding value of the left coding tree block (nlm_filter_pattern_index[cIdx][xCtbIdx – 1][yCtbIdx]); equal to 0 specifies that the value of nlm_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] is not derived from the corresponding value of the left coding tree block.

nlm_ctb_merge_up_flag equal to 1 specifies that the value of nlm_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] described below is derived from the corresponding value of the above coding tree block (nlm_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx – 1]); equal to 0 specifies that the value of nlm_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] is not derived from the corresponding value of the above coding tree block.

nlm_ctb_filter_mode specifies filter mode of the coding tree block. The value of nlm_crb_merge_mode 0 to 3 specify disabling NLM filter, deriving from the corresponding value of the left coding tree block, deriving from the corresponding value of the above coding tree block, and setting to a new filter pattern, respectively.

nlm_ctb_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] specifies filter pattern. The detail is described in subclause 2.6.

| Table 4: non-local means filter syntax |  |
| --- | --- |
| nlm ( xCtb, yCtb, cIdx) { | Descriptor |
| xCtbIdx = xCtb / CtbSize |  |
| yCtbIdx = yCtb / CtbSize |  |
| nlm_ctb_filter_enagle_flag[cIdx][xCtbIdx][yCtbIdx] = 0 |  |
| nlm_ctb_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] = 0 |  |
| if(slice_nlm_unified_filter_flag[cIdx]){ |  |
| nlm_ctb_filter_enagle_flag[cIdx][xCtbIdx][yCtbIdx] = 1 |  |
| nlm_ctb_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] = slice_nlm_filter_pattern_index[ cIdx ] |  |
| } |  |
| else{ |  |
| if(slice_nlm_single_filter_flag[cIdx]){ |  |
| nlm_ctb_filter_enable_flag[cIdx][xCtbIdx][yCtbIdx] | ae(v) |
| nlm_ctb_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] = slice_nlm_filter_pattern_index[ cIdx ] |  |
| } |  |
| else{ |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[CtbAddrTS] == TileId[CtbAddrRStoTS[CtbAddrRS – 1]] |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS – PicWidthInCtbsY]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[CtbAddrTS] == TileId[CtbAddrRStoTS[CtbAddrRS – PicWidthInCtbsY]] |  |
| AvailableLeft = (leftCtbInSlice && leftCtbInTile) ? 1 : 0 |  |
| AvailableUp = (upCtbInSlice && upCtbInTile) ? 1 : 0 |  |
| nlm_ctb_merge_left_flag = 0 |  |
| nlm_ctb_merge_up_flag = 0 |  |
| if(AvailableLeft && NlmEnagle[cIdx][xCtbIdx – 1][yCtbIdx] == 0) |  |
| AvailableLeft = 0 |  |
| if(AvailableUp && NlmEnagle[cIdx][xCtbIdx][yCtbIdx – 1] == 0) |  |
| AvailableUp = 0 |  |
| if(!AvailableLeft ¦¦ !AvailableUp){ |  |
| nlm_ctb_filter_enable_flag[cIdx][xCtbIdx][yCtbIdx] | ae(v) |
| if(nlm_ctb_filter_enable_flag[cIdx][xCtbIdx][yCtbIdx]){ |  |
| if(AvailableLeft) |  |
| nlm_ctb_merge_left_flag | ae(v) |
| If(AvailableUp) |  |
| nlm_ctb_merge_up_flag | ae(v) |
| } |  |
| } |  |
| else{ |  |
| nlm_ctb_merge_mode | ae(v) |
| if(nlm_ctb_merge_mode) |  |
| nlm_ctb_filter_enable_flag[cIdx][xCtbIdx][yCtbIdx] = 1 |  |
| if(nlm_ctb_merge_mode == 1) |  |
| nlm_ctb_merge_left_flag = 1 |  |
| else if(nlm_ctb_merge_mode == 2) |  |
| nlm_ctb_merge_up_flag = 1 |  |
| } |  |
| if(nlm_ctb_filter_enable_flag[cIdx][xCtbIdx][yCtbIdx]){ |  |
| if(nlm_ctb_merge_left_flag) |  |
| nlm_ctb_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] = nlm_ctb_filter_pattern_index[cIdx][xCtbIdx – 1][yCtbIdx] |  |
| else if(nlm_ctb_merge_up_flag) |  |
| nlm_ctb_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] = nlm_ctb_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx – 1] |  |
| else |  |
| nlm_ctb_filter_pattern_index[cIdx][xCtbIdx][yCtbIdx] | ae(v) |
| } |  |
| } |  |
| if(cIdx == 0) |  |
| nlm_tree(xB, xB + CtbSize, yB, yB + CtbSize, 0) |  |
| } |  |
| } |  |

nlm_split_flag specifies whether a block is split into coding units with half horizontal and vertical size.

nlm_filter_enable_flag equal to 1 specifies that NLM filter is enabled for the block; equal to 0 specifies that NLM filter is disabled for the block.

NlmMinimumBlock specifies the minimum size of the quad-tree block. The value is set to 32.

| Table 5: non-local means filter quad-tree syntax |  |
| --- | --- |
| nlm_tree(xB, xE, yB, yE, Depth){ | Descriptor |
| if(xB < pic_width_in_luma_samples && yB < pic_height_in_luma_samples){ |  |
| nlm_split_flag = 0 |  |
| if(xE – xB > NlmMinimumBlock{ |  |
| nlm_split_flag |  |
| if(nlm_split_flag){ |  |
| xH = xE – xB >> 1 |  |
| yH = yE – yB >> 1 |  |
| nlm_tree(xB, xB + xH, yB, yB + yH, Depth + 1) |  |
| nlm_tree(xB, xB + xH, yB, yB + yH, Depth + 1) |  |
| nlm_tree(xB + xH, xE, yB + yH, yE, Depth + 1) |  |
| nlm_tree(xB + xH, xE, yB + yH, yE, Depth + 1) |  |
| } |  |
| } |  |
| if(!nlm_split_flag) |  |
| if(Depth != 0) |  |
| nlm_filter_enable_flag |  |
| } |  |
| } |  |

# Decoder description

In this section, the decoding process of proposed NLM filter is described.

Inputs of this process are the reconstructed picture buffers prior to non-local means filter recPictureL, recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture buffers after non-local means filter nlmPictureL, nlmPictureCb and nlmPictureCr.

For the derivation of nlmPictureL, nlmPictureCb and nlmPictureCr, following ordered steps apply:

- All samples of nlmPictureL, nlmPictureCb and nlmPictureCr are set equal to recPictureL, recPictureCb and recPictureCr, respectively.

- The setting process for boundaries as specified in subclause 2.1 is invoked with boundary information of LCUs, slices and tiles as the inputs and nlmBoundaryMapY and nlmBoundaryMapC as the outputs.

- For every coding tree blocks with the location (xCtb, yCtb) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture., the following applies:

- When slice_nlm_filter_enable_flag[0] of the current slice is equal to 1, following ordered steps apply:

- The setting process for reference picture as specified in 2.2 is invoked with colour component index cIdx set equal to 0, the location (xL, yL) set equal to (xCtb, yCtb), coding tree block size nS set equal to CtbSize, sampleWidth set equal to pic_width_in_luma_samples, sampleHeight set equal to pic_height_in_luma_samples, and reconstructed picture buffers recPicture set equal to recPictureL as the inputs and padded picture buffers recPicturePadded set equal to recPicturePaddedL and half-shrunk picture buffers recPictureHalf set equal to recPictureHalfL as the outputs.

- If nlm_ctb_filter_enable_flag[0][xCtb / CtbSize][yCtb / CtbSize] is equal to 1, the filtering process for luma samples as specified in subclause 2.3.1 is invoked with colour component index cIdx set equal to 0, the location (xL, yL) set equal to (xCtb, yCtb), coding tree block size nS set equal to CtbSize, sampleWidth set equal to pic_width_in_luma_samples, sampleHeight set equal to pic_height_in_luma_samples, reconstructed reconstructed padded picture buffers recPicturePadded set equal to recPicturePaddedL, half-shrunk picture buffers recPictureHalf set equal to recPictureHalfL, and boundary map nlmBoundaryMapL as the inputs and filterd picture buffers nlmPicture set equal to nlmPictureL as the output.

- When slice_nlm_filter_enable_flag[1] of the current slice is equal to 1, following ordered steps apply:

- The setting process for reference picture as specified in 2.2 is invoked with colour component index cIdx set equal to 1, the location (xL, yL) set equal to (xCtb >> 1, yCtb >> 1), coding tree block size nS set equal to CtbSize >> 1, sampleWidth set equal to pic_width_in_luma_samples >> 1, sampleHeight set equal to pic_height_in_luma_samples >> 1, and reconstructed picture buffers recPicture set equal to recPictureCb as the inputs and padded picture buffers recPicturePadded set equal to recPicturePaddedCb and half-shrunk picture buffers recPictureHalf set equal to recPictureHalfCb as the outputs.

- If nlm_ctb_filter_enable_flag[1][xCtb / CtbSize][yCtb / CtbSize] is equal to 1, the filtering process for luma samples as specified in subclause 2.3.2 is invoked with colour component index cIdx set equal to 1, the location (xL, yL) set equal to (xCtb >> 1, yCtb >> 1), coding tree block size nS set equal to CtbSize >> 1, sampleWidth set equal to pic_width_in_luma_samples >> 1, sampleHeight set equal to pic_height_in_luma_samples >> 1, reconstructed reconstructed padded picture buffers recPicturePadded set equal to recPicturePaddedCb, half-shrunk picture buffers recPictureHalf set equal to recPictureHalfCb, and boundary map nlmBoundaryMapCb as the inputs and filterd picture buffers nlmPicture set equal to nlmPictureCb as the output.

- When slice_nlm_filter_enable_flag[2] of the current slice is equal to 1, following ordered steps apply:

- The setting process for reference picture as specified in 2.2 is invoked with colour component index cIdx set equal to 2, the location (xL, yL) set equal to (xCtb >> 1, yCtb >> 1), coding tree block size nS set equal to CtbSize >> 1, sampleWidth set equal to pic_width_in_luma_samples >> 1, sampleHeight set equal to pic_height_in_luma_samples >> 1, and reconstructed picture buffers recPicture set equal to recPictureCb as the inputs and padded picture buffers recPicturePadded set equal to recPicturePaddedCb and half-shrunk picture buffers recPictureHalf set equal to recPictureHalfCb as the outputs.

- If nlm_ctb_filter_enable_flag[2][xCtb / CtbSize][yCtb / CtbSize] is equal to 1, the filtering process for luma samples as specified in subclause 2.3.2 is invoked with colour component index cIdx set equal to 1, the location (xL, yL) set equal to (xCtb >> 1, yCtb >> 1), coding tree block size nS set equal to CtbSize >> 1, sampleWidth set equal to pic_width_in_luma_samples >> 1, sampleHeight set equal to pic_height_in_luma_samples >> 1, reconstructed reconstructed padded picture buffers recPicturePadded set equal to recPicturePaddedCr, half-shrunk picture buffers recPictureHalf set equal to recPictureHalfCr, and boundary map nlmBoundaryMapCr as the inputs and filterd picture buffers nlmPicture set equal to nlmPictureCr as the output.

[Ed. (WJ): recPicturePaddedL, recPicturePaddedCb and recPicturePaddedCr are now a picture-size array, but actually CU size + appropriate margin is enough]

## Setting process for boundaries

Inputs to this process are boundary information of LCUs, slices and tiles.

Output of this process is boundary information nlmBoundaryMapY and nlmBoundaryMapC.

For the derivation of nlmBoundaryMaps (nlmBoundaryMapY and nlmBoundaryMapC), following ordered steps apply:

- All the location of are nlmBoundaryMaps set equal to 0.

- The location ( x, y ) of nlmBoundaryMapY belonging to top 2-line or left 2-line from each LCU boundary is set equal to 3.

- The location ( x, y ) of nlmBoundaryMaps belonging to bottom 2-line or right 2-line from each LCU boundary is set equal to 2; and the location of nlmBoundaryMaps belonging to the bottom 3 to 4-line or right 3 to 4-line from the LCU boundary is set equal to 1.

- For each LCU, when the LCU is located at the boundary of slice or tile, the location ( x, y ) of nlmBoundaryMaps belonging to the LCU that is located at 2-line from slice or tile boundary is set equal to 2; and the location of nlmBoundaryMaps belonging to 3 to 4-line from slice or tile boundary is set equal to 1.

Figure 1 and Figure 2 illustrate the boundary settings for nlmBoundaryMapY and nlmBoundaryMapC, respectively.

Figure 1: Boundary setting for luma component.

Figure 2: Boundary setting for chroma component.

## Setting process for reference picture

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables (xL, yL) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx.

- picture buffers recPicture for the colour component cIdx.

Outputs of this process are:

- padded picture buffers recPicturePadded for the colour component cIdx,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx.

When slice_nlm_filter_enable_flag[ cIdx ] of the current slice is equal to 1, following ordered steps apply:

- The generation process for padded reference picture as specified in subclause 2.2.1 is invoked with cIdx, ( xL, yL ), nS, sampleWidth, sampleHeight, and recPicture as the inputs and recPicturePadded as the output.

- The generation process for half-shrunk reference picture as specified in subclause 2.2.2 is invoked with cIdx, ( xL, yL ), nS, sampleWidth, sampleHeight, and recPicturePadded as the inputs and recPictureHalf as the output.

### Generation process for padded reference picture

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xL, yL ) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx.

- picture buffer recPicture for the colour component cIdx,

Output of this process is padded reference picture recPicturePadded for the colour component cIdx.

The padded reference picture recPicturePadded is specified in the follows:

- All samples of the location ( x, y ) recPicturePadded[ x ][ y ] with x = xL..xE and y = yL..yE are set equal to recPicture [ x ][ y ]; where ex and ey are set to:

xE = xL + nS < sampleWidth  ? xL + nS − 1 : sampleWidth  − 1,				(2-1)
yE = yL + nS < sampleHeight ? yL + nS − 1 : sampleHeight − 1. 			(2-2)

- If xL is equal to 0, for all samples of the vertical location y = yL..yE, recPicturePadded[ −1.. −4 ][ y ] are set equal to recPicture [ 0 ][ y ].

- If xL + nS is equal to or larger than sampleWidth, for all samples of the vertical location y = yL..yE, recPicturePadded[ sampleWidth..sampleWidth+3 ][ y ] are set equal to recPicture[ sampleWidth−1 ][ y ].

- If yL is equal to 0, for all samples of the horizontal location x = xL’..xE’, recPicturePadded[ x ][ −1..−4 ] are set equal to recPicture[ x ][ 0 ]; where xL’ and yE’ are set to:

xL’ = xL == 0 ? −4 : 0, 									(2-3)
xE’ = xL + nS <= sampleWidth ? xL + nS − 1 : sampleWidth + 3. 			(2-4)

- If yL + nS is equal to or larger than sampleHeight, for all samples of the horizontal location x = xL’..xE’, recPicturePadded[ x ][ sampleHeight..sampleHeight+3 ] are set equal to recPicture[ x ][ sampleHeight−1 ].

Figure 3 illustrates the setting of padded area corresponding to LCU location.

Figure 3: The setting of padded area corresponding to LCU location.

### Generation process for half-shrunk padded reference picture

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xL, yL ) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx,

- padded picture buffer recPicturePadded for the colour component cIdx,

Output of this process is half-shrunk padded reference picture buffer recPictureHalf for the colour component cIdx.

The half-shrunk padded reference picture buffer recPictureHalf[ x2 ][ y2 ] with x2 = bx2..ex2 and y2 = by2..ey2 are derived by:

bx2 = xL == 0 ? −1 : xL >> 1, 								(2-5)
by2 = yL == 0 ? −1 : yL >> 1, 								(2-6)
ex2 = xL + nS <= sampleWidth  ? ( ( xL + nS ) >> 1 ) − 1 : sampleWidth  >> 1, 		(2-7)
ey2 = yL + nS <= sampleHeight ? ( ( yL + nS ) >> 1 ) − 1 : sampleHeight >> 1, 		(2-8)

x = x2 << 1 												(2-9)
y = y2 << 1 												(2-10)
recPictureHalf [ x2 ][ y2 ] = recPicture[ x ][ y   ] + recPicture[ x+1 ][ y   ]

recPicture[ x ][ y+1 ] + recPicture[ x+1 ][ y+1 ] 		(2-11)

## Filtering process

### Filtering process for luma samples

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables (xL, yL) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx,

- padded picture buffer recPicturePadded for the colour component cIdx,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx

- boundary information nlmBoundaryMapY.

Output of this process is filtered picture buffer nlmPicture for the colour component cIdx.

The filtered picture buffer nlmPicture is derived in the following ordered steps:

- All the samples of the location (x, y) nlmPicture[x][y] with x = xL..xE and y = yL..yE are set equal to recPicturePadded[x][y]; where xE and yE are set to:

xE = xL + nS < sampleWidth  ? xL + nS − 1 : sampleWidth  − 1, 			(2-12)
yE = yL + nS < sampleHeight ? yL + nS − 1 : sampleHeight − 1. 			(2-13)

- The filter block derived by nlm_split_flag and nlm_filter_enable_flag is split into 2x2 sub-blocks. For each 2x2 sub-block with the location (xS, yS) specifying the top-left the sample the following steps apply:

- The derivation process for the index of activity and gradient as specified in 2.4.1 is invoked with recPictureHalf, (xS, yS), and nlmBoundaryMapY as the inputs and the activities activityX, activityL and directionIdx as the outputs.

- If nlmBoundaryMapY[xS][yS] is equal to 3, directionIdx is set to 10.

- The derivation process for filter strength of luma samples as specified in 2.5.1 is invoked with QP specifying quantization parameter, activities activityX and activityL, and incrementalBitDepth  set equal to internalBitDepth − 8 as the inputs and the filter strength filterStrengthPre as the output.

- The swap process for filter strength of luma samples as specified in 2.6.1 invoked with the filter strength filterStrengthPre and filter pattern index PatternIdx set equal to nlm_ctb_filter_pattern_index[cIdx][xL / nS][ yL / nS] as the inputs and the filter strength filterStrength as the output.

- If activityX is greater than or equal to 96 << (internalBitDepth − 8) and nlmBoundaryMapY[xS][yS] is not equal to 1, for each location of the sample (xP, yP) in 2x2 sub-block, the following steps apply:

- The derivation process for SSD table of cross-shaped template as specified in subclause 2.7.1 and the filter strength set equal to filterStrength, and internalBitDepth set equal to BitDepthY as the inputs and ssdTableP as the output.

- Initially, sumWeights is assigned to 255, and sumSamples is assigned to 255 * recPicturePadded[xP][yP].

- For each element of tap (the relative location of element of tap (xP2, yP2) and the location (xP, yP) corresponding to directionIdx are illustrated in Figure 4), sumWeights and sumSamples are derived as follows.

ibdiMax = ( 1 << internalBitDepth ) −1,						(2-14)
tableIdx0 = recPicturePadded[xP2 − 1][yP2 − 1]

− recPicturePadded[xS − 1][yS − 1] + ibdiMax ), 			(2-15)

tableIdx1 = recPicturePadded[xP2 + 1][yP2 − 1]

− recPicturePadded[xS + 1][yS − 1] + ibdiMax ), 			(2-16)

tableIdx2 = recPicturePadded[xP2][yP2]

− recPicturePadded[xS][ yS] + ibdiMax ), 				(2-17)

tableIdx3 = recPicturePadded[xP2 − 1][yP2 + 1]

− recPicturePadded[xS − 1][yS + 1] + ibdiMax ), 			(2-18)

tableIdx4 = recPicturePadded[xP2 + 1][yP2 + 1]

− recPicturePadded[xS + 1][yS + 1] + ibdiMax ), 			(2-19)

weight = expTable[ Min((ssdTable[tableIdx0] + ssdTable[tableIdx1]

+ ssdTable[tableIdx2] + ssdTable[tableIdx3]

+ ssdTable[tableIdx4]) >> 6, 1023 )], 			(2-20)

sumWeights += weight,								(2-21)
sumSamples += weight * recPicturePadded[xP2][yP2]. 				(2-22)

- nlmPicture[xP][yP] is derived by nlmPicture[xP][yP] = sumSamples / sumWeights.

- Otherwise, if activityX is greater than or equal to 8 << (internalBitDepth − 8), for each location of the sample (xP, yP) in 2x2 sub-block, the following steps apply:

- The derivation process for SSD table of pixel template as specified in subclause 2.7.2 and the filter strength set equal to filterStrength, and internalBitDepth set equal to BitDepthY as the inputs and expTablePel as the output.

- Initially, sumWeights is assigned to 255, and sumSamples is assigned to 255 * recPicturePadded[xP][yP].

- For each element of tap (the relative location of element of tap (xP2, yP2) and the location (xP, yP) corresponding to directionIdx are illustrated in Figure X), sumWeights and sumSamples are derived as follows.

ibdiMax = (1 << internalBitDepth ) − 1,						(2-23)
tableIdx = recPicturePadded[xP2][yP2]

− recPicturePadded[xS][yS] + ibdiMax,					(2-24)

weight = expTablePel[expIdx],							(2-25)
sumWeights += weight,								(2-26)
sumSamples += weight * recPicturePadded[xP2][yP2]. 				(2-27)

- nlmPicture[xP][yP] is derived by nlmPicture[xP][yP] = sumSamples / sumWeights.

NOTE –expTable[i] ( i = 0..1023 ) is assigned by

expTable[ i ]= 0;i=1023Floor( 255*exp( −i÷198.0 )+0.5 );otherwise

Figure 4: Relative locations (xP2, yP2) and the location (xP, yP) corresponding to GradientIdx.

### Filtering process for chroma samples

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables (xL, yL) specifying the coding tree block location,

- a coding tree block size nS,

- a width sampleWidth specifying the width for the colour component cIdx,

- a height sampleHeight specifying the height for the colour component cIdx,

- padded picture buffer recPicturePadded for the colour component cIdx,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx

- boundary information nlmBoundaryMapC.

Output of this process is filtered picture buffer nlmPicture for the colour component cIdx.

The filtered picture buffer nlmPicture is derived in the following ordered steps.

- All the samples of the location (x, y) nlmPicture[x][y] with x = xL..xE and y = yL..yE are set equal to recPicturePadded[x][y]; where xE and yE are set to:

xE = xL + nS < sampleWidth  ? xL + nS − 1 : sampleWidth  − 1, 			(2-28)
yE = yL + nS < sampleHeight ? yL + nS − 1 : sampleHeight − 1. 			(2-29)

- The filter block derived by nlm_split_flag and nlm_filter_enable_flag is split into 2x2 sub-blocks. For each 2x2 sub-block with the location (xS, yS) specifying the top-left sample, the following steps apply:

- The derivation process for activity index of chroma samples as specified in 2.4.2 is invoked with recPictureHalf, (xS, yS), and nlmBoundaryMapC as the inputs and the activities activityX and activityL as the output.

- The derivation process for filter strength of chroma samples as specified in 2.5.2 is invoked with QP specifying quantization parameter, activities activityX and activityL, and incrementalBitDepth  set equal to internalBitDepth − 8 as the inputs and the filter strength filterStrengthPre as the output.

- The swap process for filter strength of luma samples as specified in 2.6.2 invoked with the filter strength filterStrengthPre and filter pattern index PatternIdx set equal to nlm_ctb_filter_pattern_index[cIdx][xL / nS][ yL / nS] as the inputs and the filter strength filterStrength as the output.

- If activityX is greater than or equal to 32 << (internalBitDepth − 8) and nlmBoundaryMapC[xS][yS] is not equal to 1, directionIdx is set to 10; and for each location of the sample (xP, yP) in 2x2 sub-block, the following steps apply:

- The derivation process for SSD table of cross-shaped template as specified in subclause 2.7.1 and the filter strength set equal to filterStrength, and internalBitDepth set equal to BitDepthC as the inputs and ssdTableP as the output.

- Initially, sumWeights is assigned to 255, and sumSamples is assigned to 255 * recPicturePadded[xP][yP].

- For each element of tap (the relative location of element of tap (xP2, yP2) and the location (xP, yP) corresponding to directionIdx are illustrated in Figure 4), sumWeights and sumSamples are derived as follows.

ibdiMax = (1 << internalBitDepth) − 1,						(2-30)
tableIdx0 = recPicturePadded[xP2 − 1][yP2 − 1]

− recPicturePadded[xS − 1][yS − 1] + ibdiMax ), 			(2-31)

tableIdx1 = recPicturePadded[xP2 + 1][yP2 − 1]

− recPicturePadded[xS + 1][yS − 1] + ibdiMax ), 			(2-32)

tableIdx2 = recPicturePadded[xP2][yP2]

− recPicturePadded[xS][ yS] + ibdiMax ), 				(2-33)

tableIdx3 = recPicturePadded[xP2 − 1][yP2 + 1]

− recPicturePadded[xS − 1][yS + 1] + ibdiMax ), 			(2-34)

tableIdx4 = recPicturePadded[xP2 + 1][yP2 + 1]

− recPicturePadded[xS + 1][yS + 1] + ibdiMax ), 			(2-35)

weight = expTable[ Min((ssdTable[tableIdx0] + ssdTable[tableIdx1]

+ ssdTable[tableIdx2] + ssdTable[tableIdx3]

+ ssdTable[tableIdx4]) >> 6, 1023 )], 			(2-36)

sumWeights += weight,								(2-37)
sumSamples += weight * recPicturePadded[xP2][yP2]. 				(2-38)

- nlmPicture[xP][yP] is derived by nlmPicture[xP][yP] = sumSamples / sumWeights.

- Otherwise, activityX is greater than or equal to 6 << (internalBitDepth − 8), directionIdx is set to 10; and for each location of the sample (xP, yP) in 2x2 sub-block, the following steps apply:

- The derivation process for SSD table of pixel template as specified in subclause 2.7.2 and the filter strength set equal to filterStrength, and internalBitDepth set equal to BitDepthC as the inputs and expTablePel as the output.

- Initially, sumWeights is assigned to 255, and sumSamples is assigned to 255 * recPicturePadded[xP][yP].

- For each element of tap (the relative location of element of tap (xP2, yP2) and the location (xP, yP) are illustrated in Figure 4), sumWeights and sumSamples are derived as follows.

bitDepthMax = (1 << internalBitDepth ) −1, 					(2-39)
tableIdx = recPicturePadded[xP2][yP2]

− recPicturePadded[xS][yS] + ibdiMax), 				(2-40)

weight = expTablePel[tableIdx], 						(2-41)
sumWeights += weight, 							(2-42)
sumSamples += weight * recPicturePadded[xP2][yP2]. 				(2-43)

- nlmPicture[xP][yP] is derived by nlmPicture[xP][yP] = sumSamples / sumWeights.

## Derivation process for activity

### Derivation process for activities and direction index of luma samples

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables ( xS, yS ) specifying the 2x2 sub-block location,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx

- boundary information nlmBoundaryMapY.

Outputs of this process are:

- a variable activities activityX and activityL,

- a variable direction index directionIdx.

If nlmBoundaryMapY[xS][yS] is equal to 2, activityX, activityL and directionIdx are assigned to 0. Otherwise, activityX, activityL and directionIdx are assigned to:

x2 = xS >> 1, 											(2-44)
y2 = yS >> 1, 											(2-45)
db = recPictureHalf[cIdx][x2+1][y2+1] − recPictureHalf[cIdx][x2−1][y2−1], 		(2-46)
ds = recPictureHalf[cIdx][x2+1][y2−1] − recPictureHalf[cIdx][x2−1][y2+1], 		(2-47)
activityX = (Abs(db) + Abs(ds)),				 				(2-48)
activityL = Abs((recPictureHalf[cIdx][x2][y2] << 3)
             − recPictureHalf[cIdx][x2−1][y2−1]
             − recPictureHalf[cIdx][x2][y2−1]
             − recPictureHalf[cIdx][x2+1][y2]
             − recPictureHalf[cIdx][x2−1][y2]
             − recPictureHalf[cIdx][x2+1][y2]
             − recPictureHalf[cIdx][x2−1][y2+1]
             − recPictureHalf[cIdx][x2][y2+1]
             − recPictureHalf[cIdx][x2+1][y2+1] 		 			(2-49)

dir = (ds << 10) / db,									(2-50)
directionIdx= 77654321098;;;;;;;;;;;db==0dir<−3072dir<−1536dir<−683dir<−341dir<0dir<341dir<638dir<1536dir<3072otherwise.								(2-51)

### Derivation process for activities of chroma samples

Inputs to this process are:

- a variable cIdx specifying colour component index,

- a pair of variables (xS, yS) specifying the 2x2 sub-block location,

- half-shrunk reference picture buffer recPictureHalf for the colour component cIdx

- boundary information nlmBoundaryMapC.

Output of this process are the activities activityX and activityL for the colour component cIdx.

If nlmBoundaryMapC[xS][yS] is equal to 2, activityX and activityL are assigned to 0. Otherwise, activityX and activityL are assigned to:

x2 = xS >> 1, 										(2-52)
y2 = yS >> 1, 										(2-53)
db = recPictureHalf[cIdx][x2+1][y2+1] − recPictureHalf[cIdx][x2−1][y2−1], 		(2-54)
ds = recPictureHalf[cIdx][x2+1][y2−1] − recPictureHalf[cIdx][x2−1][y2+1], 		(2-55)
activityX = (Abs(db) + Abs(ds)),					 			(2-56)
activityL = Abs((recPictureHalf[cIdx][x2][y2] << 3)
             − recPictureHalf[cIdx][x2−1][y2−1]
             − recPictureHalf[cIdx][x2][y2−1]
             − recPictureHalf[cIdx][x2+1][y2]
             − recPictureHalf[cIdx][x2−1][y2]
             − recPictureHalf[cIdx][x2+1][y2]
             − recPictureHalf[cIdx][x2−1][y2+1]
             − recPictureHalf[cIdx][x2][y2+1]
             − recPictureHalf[cIdx][x2+1][y2+1] 		 			(2-57)

## Derivation process for filter strength

Pre-calculation replaces the derivation processes described in this subclaus into LUT tables.

### Derivation process for filter strength of luma samples

Inputs to this process are:

- a variable QP specifying quantization parameter,

- two variables activityX and activityL,

- a variable incrementalBitDepth.

Outputs of this process are filterStrengthPre.

filterStrengthPre is assigned to:

cs = − 13531.18992 * QP + 627214.2705, 							(2-58)
cd = − 0.020466    * QP + 1.115546, 							(2-59)
cx = − 0.05951     * QP − 4.385607, 							(2-60)
cy = 17.776044    * QP + 36.203986, 							(2-61)
ls = − 136.232239  * QP + 6976.477682, 							(2-62)
ld = 0.00701      * QP + 0.171788, 							(2-63)
lx = 0.439485     * QP − 16.68934, 							(2-64)
ly = 7.388733     * QP − 261.925012, 							(2-65)
os = 726.407119   * QP + 15862.77864, 							(2-66)
od = 0.015954     * QP − 0.104058, 							(2-67)
ox = 0.174787     * QP − 2.505108, 							(2-68)
oy = 300.388549   * QP − 18046.16876, 							(2-69)
actX = activityX >> incrementalBitDepth, 							(2-70)
actL = activityL  >> incrementalBitDepth, 							(2-71)
idx = Min(actX, 511),									(2-72)
cross    = Round(cs * (1.0 − 1.0 / (1.0 + exp(− cd * (idx / 16.0 − cx)))) + cy), 			(2-73)
laplacian = Round(ls  * (1.0 − 1.0 / (1.0 + exp(− ld * (idx / 16.0 − lx)))) + ly), 			(2-74)
offset    = Round(os * (     1.0 / (1.0 + exp(− od * (idx / 16.0 − ox)))) + oy) << 4, 		(2-75)
filterStrengthPre = Min((offset + cross * actX + laplacian * actL + 32768) >> 16, 31). 		(2-76)

### Derivation process for filter strength of chroma samples

Inputs to this process are:

- a variable QP specifying quantization parameter,

- two variables activityX and activityL,

- a variable incrementalBitDepth.

Outputs of this process are filterStrengthPre.

filterStrengthPre is assigned to:

cs = − 2970.11948  * QP + 147904.4678, 							(2-77)
cd = − 0.013339    * QP + 1.111553, 							(2-78)
cx = 0.107615     * QP − 5.355132, 							(2-79)
cy = 30.657903    * QP + 237.111728, 							(2-80)
ls = − 491.330652  * QP + 34434.29553, 							(2-81)
ld = − 0.00402     * QP + 0.434687, 							(2-82)
lx = 0.142307     * QP − 13.049523, 							(2-83)
ly = 11.594785    * QP − 409.644945, 							(2-84)
os = 782.593244   * QP + 21642.21447, 							(2-85)
od = 0.012704     * QP − 0.049073, 							(2-86)
ox = 0.131648     * QP − 3.709989, 							(2-87)
oy = − 209.706738  * QP − 17512.02092, 							(2-88)
actX = activityX >> incrementalBitDepth, 							(2-89)
actL = activityL  >> incrementalBitDepth, 							(2-90)
idx = Min(ActX, 511),									(2-91)
cross    = Round(cs * (1.0 − 1.0 / (1.0 + exp(− cd * (idx / 16.0 − cx)))) + cy), 			(2-92)
laplacian = Round(ls  * (1.0 − 1.0 / (1.0 + exp(− ld * (idx / 16.0 − lx)))) + ly), 			(2-93)
offset    = Round(os  * (     1.0 / (1.0 + exp(− od * (idx / 16.0 − ox)))) + oy) << 4, 		(2-94)
filterStrengthPre = Min((offset + cross * actX + laplacian * actL + 32768) >> 16, 31). 		(2-95)

## Swap process for filter strength

### Swap process for filter strength of luma samples

Inputs to this process are:

- a filter strength filterStrengthPre,

- filter pattern index PatternIdx.

Outputs of this process is filterStrength specifying filter strength of NLM filter.

filterStrength is derived by swap table described in Table 6.

| Table 6: Swap table for filter strength of luma samples |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Input | filterStrengthPre | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Output | PatternIdx == 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
|  | PatternIdx == 1 | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 6 | 7 | 8 | 8 | 9 | 10 | 11 | 11 | 12 | 13 | 14 | 14 | 15 | 16 | 17 | 17 | 18 | 19 | 20 | 21 | 21 | 22 | 23 | 27 |
|  | PatternIdx == 2 | 0 | 1 | 2 | 3 | 3 | 4 | 5 | 6 | 6 | 7 | 8 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 14 | 15 | 15 | 15 | 16 | 16 | 16 | 17 |
|  | PatternIdx == 3 | 1 | 3 | 4 | 5 | 6 | 6 | 7 | 7 | 7 | 8 | 8 | 8 | 9 | 9 | 9 | 9 | 9 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 |

### Swap process for filter strength of chroma samples

Inputs to this process are:

- a filter strength filterStrengthPre,

- filter pattern index PatternIdx.

Outputs of this process is filterStrength specifying filter strength of NLM filter.

filterStrength is derived by swap table described in Table 6.

| Table 7: Swap table for filter strength of luma samples |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Input | filterStrengthPre | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Output | PatternIdx == 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
|  | PatternIdx == 1 | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 6 | 7 | 8 | 8 | 9 | 10 | 11 | 11 | 12 | 13 | 14 | 14 | 15 | 16 | 17 | 17 | 18 | 19 | 20 | 21 | 21 | 22 | 23 | 27 |
|  | PatternIdx == 2 | 0 | 1 | 2 | 3 | 3 | 4 | 5 | 6 | 6 | 7 | 8 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 14 | 15 | 15 | 15 | 16 | 16 | 16 | 17 |
|  | PatternIdx == 3 | 0 | 2 | 4 | 5 | 6 | 7 | 8 | 9 | 9 | 10 | 11 | 11 | 11 | 12 | 12 | 13 | 13 | 16 | 14 | 14 | 14 | 14 | 15 | 15 | 15 | 15 | 15 | 16 | 16 | 16 | 16 | 16 |

## Derivation process for table

Pre-calculation replaces the derivation processes described in this subclaus into LUT tables.

### Derivation process for SSD table of cross-shaped template

Inputs to this process are:

- a variable filterStrength specifying the filter strength of the LCU,

- a variable internalBitDepth specifying the additional bit-depth by internal bit depth increase.

Output of this process is ssdTable specifying the SSD lookup table.

For each element of ssdTable[ n ] with n = 0..( 1 << ( internalBitDepth + 1 ) ) − 1 is set to

div = 5.0 * (filterStrength << (internalBitDepth − 8) ÷ 4.0)2,					(2-96)
ssdTable[n] = Min(Round(((n − (1 << internalBitDepth) − 1 )2 << 6) ÷ div), 65535).		(2-97)

### Derivation process for Exp table of pixel template

Inputs to this process are:

- a variable filterStrength specifying the filter strength of the LCU,

- a variable internalBitDepth specifying the additional bit-depth by internal bit depth increase.

Output of this process is expTablePel specifying the SSD lookup table.

For each element of expTablePel[n] with n = 0..( 1 << ( internalBitDepth + 1 ) ) − 1 is set to

div = (filterStrength << (internalBitDepth − 8) ÷ 4.0)2,					(2-98)
expTablePel[n] = expTable[Min(Round(((n − (1 << internalBitDepth) − 1 )2 << 6) ÷ div), 1023)].														(2-99)

# Patent rights declaration(s)

NTT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).