| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0173r1 |
| --- | --- |

| Title: | AHG9: On VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

Both VPS (Video Parameter Set) and SPS (Sequence Parameter Set) contain syntax elements “max_temporal_layers_minus1” and “temporal_id_nesting_flag”. If the value of max_temporal_layers_minus1 is 0, it means that there is only one temporal layer, and in this case temporal_id_nesting_flag becomes meaningless. JCTVC-0183 proposed to remove this redundancy, and adopted as change in semantics.

However, in the current VPS specification temporal_id_nesting_flag is located before max_sub_layers_minus1, so it is not possible to know immediately whether it is meaningful or not during the parsing process. In SPS this problem does not exist.

This contribution claims to change the place of temporal_id_nesting_flag after max_sub_layers_minus1 in VPS.

# Introduction / Problem Statement / …

Both VPS (Video Parameter Set) and SPS (Sequence Parameter Set) contain syntax elements “max_temporal_layers_minus1” and “temporal_id_nesting_flag”. If the value of max_temporal_layers_minus1 is 0, it means that there is only one temporal layer, and in this case temporal_id_nesting_flag becomes meaningless. JCTVC-0183 proposed to remove this redundancy, and adopted as change in semantics as follow:

vps_temporal_id_nesting_flag, when vps_max_sub_layers_minus1 is greater than 0,  specifies whether inter prediction is additionally restricted for for coded video sequences referring to the video parameter set. When vps_max_sub_layers_minus1 is equal to 0, vps_temporal_id_nesting_flag has no meaning and may have any value.

sps_temporal_id_nesting_flag, when sps_max_sub_layers_minus1 is greater than 0, specifies whether inter prediction is additionally restricted for coded video sequences referring to the sequence parameter set. When vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be equal to 1. When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag has no meaning, and when sps_max_sub_layers_minus1 is equal to 0 and vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag may have any value.

VPS and SPS syntax are shown in Table 1.1 and 1.2.

In SPS case, temporal_id_nesting_flag is located after max_sub_layers_minus1, so it is possible to know immediately whether it is meaningful or not when it is parsed. However, in VPS case, it is located before max_sub_layers_minus1, so it is necessary to wait max_sub_layers_minus1 be parsed to know whether it is meaningful or not.

Table 1.1

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_zero_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_and_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 1.2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| profile_and_level( 1, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| for( i = 0; i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposed Solution

To solve the problem stated above, it is proposed to move the location of temporal_id_nesting_flag after max_sub_layers_minus1 as shown in Table 2.1.

Table 2.1

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_zero_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| profile_and_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Conclusion

If the value of max_temporal_layers_minus1 is 0, it means that there is only one temporal layer, and in this case temporal_id_nesting_flag becomes meaningless. JCTVC-0183 proposed to remove this redundancy, and adopted as change in semantics.

However, in the current VPS specification temporal_id_nesting_flag is located before max_sub_layers_minus1, so it is not possible to know immediately whether it is meaningful or not during the parsing process. In SPS this problem does not exist.

It is proposed to change the place of temporal_id_nesting_flag after max_sub_layers_minus1 in VPS.

It is recommended that this modification be integrated into HEVC specification text.

# References

[1] “High efficiency video coding (HEVC) text specification draft 8”, JCTVC-J1003, July 2012

[2] “Syntax Issues”, JCTVC-J0183, July 2012

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).