| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_zero_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| profile_and_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_temporal_id_nesting_flag, when vps_max_sub_layers_minus1 is greater than 0,  specifies whether inter prediction is additionally restricted for for coded video sequences referring to the video parameter set. When vps_max_sub_layers_minus1 is equal to 0, vps_temporal_id_nesting_flag has no meaning and vps_temporal_id_nesting_flag shall be 1 may have any value.

sps_temporal_id_nesting_flag, when sps_max_sub_layers_minus1 is greater than 0, specifies whether inter prediction is additionally restricted for coded video sequences referring to the sequence parameter set. When vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be equal to 1. When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag has no meaning, and when sps_max_sub_layers_minus1 is equal to 0 and vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be 1 may have any value.