| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0175r1 |
| --- | --- |

| Title: | On inter-layer prediction enabling/disabling for HEVC scalable extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document contains two topics on inter-layer prediction for HEVC scalable extension as follows:

- Inter-layer Prediction and Temporal-layer Depth

In HEVC scalable extension inter-layer prediction enabling/disabling will be associated with trade-off between coding efficiency and complexity: it will improve coding efficiency but will cause increase in complexity.

With lower temporal layers distance in time domain between the current and reference frames are rather long and performance of inter-frame prediction is not so high. In this case more CUs are coded as intra, and with scalable extension, inter-layer prediction will improve coding efficiency.

On the other hand, with higher temporal layers distance in time domain between the current and reference frames are rather short and inter-frame prediction do works. In this case inter-layer prediction is not so helpful for coding efficiency improvement.

Therefore this contribution proposes to introduce inter-layer prediction on/off mechanism in association with temporal layer depth.

- Consideration on Hybrid-codec Scalability and Inter-layer Prediction

In the CfP document of HEVC scalable extensions, experiment of codec standard scalability (Base-layer: AVC, Enhancement-layer: HEVC) is included as an option. In addition, it is proposed by the proposal m25749 that MPEG2-HEVC scalability be supported with BLR (Base-Layer Reconstructed Pixel). It might be possible that proprietary codec be also included in hybrid-codec scalable bitstreams.

It is assumed that HEVC version 2 will have flexibility that any layer can be any codec_type, but the problem is that scalability like MPEG2-AVC does not exist in the past codec standard.

Therefore this contribution proposes to introduce restriction on the combination of hybrid-codec scalability for inter-layer prediction.

# Inter-layer Prediction and Temporal-layer Depth

## Problem Statement

In HEVC scalable extension inter-layer prediction enabling/disabling will be associated with trade-off between coding efficiency and complexity: it will improve coding efficiency but will cause increase in complexity.

In Fig 1.1.1, both base-layer and enhancement-layer contains 3 temporal layers. With sublayer0 and 1, distance in time domain between the current and reference frames are rather long. In this case, inter-frame prediction does not work so much then more CUs are coded with intra. In this case inter-layer prediction is helpful for improving coding efficiency with scalable extension.

On the other hand, with sublayer2, distance in time domain between the current and reference frames are short. In this case inter-frame prediction do works and inter-layer prediction in scalable extension is not so helpful for coding efficiency improvement.

In this way how helpful inter-layer prediction for coding efficiency is will depend on the depth of the temporal layer. Therefore it would be useful if inter-layer prediction enabling/disabling be specified in association with the temporal layer depth.

Fig 1.1.1

## Proposed Solution

To provide the mechanism that is described in the previous section it is proposed that syntax element called “max_sub_ref_layer” be introduced in vps_extension() as shown in Table 1.2.1 (Approach 1) or Table 1.2.2 (Approach 2).

Table 1.2.1

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| num_dimensions_minus1[ i ] | u(4) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) { |  |
| dimension_type[ i ][ j ] | u(4) |
| dimension_id[ i ][ j ] | u(8) |
| } |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ){ |  |
| ref_layer_id[ i ][ j ] | u(6) |
| max_sub_ref_layer[ i ][ j ] | u(6) |
| } |  |
| } |  |
| } |  |

Table 1.2.2

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| // scalability type and layer_id partitioning method |  |
| scalability_type | u(4) |
| for( i = 0; i < MaxDim( scalability_type ); i++ ) |  |
| layer_id_dim_len[ i ] | u(3) |
| // layer specific information |  |
| for( i = 0; i <= max_num_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) { |  |
| ref_layer_id[ i ][ j ] | u(6) |
| max_sub_ref_layer[ i ][ j ] | u(6) |
| } |  |
| } |  |
| } |  |

The following is the semantics of “max_sub_ref_layer[i][j]”:

max_sub_ref_layer[i][j] specifies max temporal layer where inter-layer prediction be applied for scalable extensions. Its value shall be in the range of 0 to vps_max_sub_layers_minus1+1 inclusive.

Another way for syntax modification is to add a new syntax element “max_sub_ref_layer” in nal_unit, as shown in Table 1.2.3.

Table 1.2.3

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| nal_unit_header( ) |  |
| if(nal_unit_type == SPS_NAL_UNIT) |  |
| max_sub_ref_layer | u(6) |
| NumBytesInRBSP = 0 |  |
| for( i = 2; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

The following is the semantics of “max_sub_ref_layer”

max_sub_ref_layer specifies max temporal layer where inter-layer prediction be applied for scalable extensions. Its value shall be in the range of 0 to vps_max_sub_layers_minus1+1 inclusive. If a sequence is a base-layer stream, its value should be 0.

# Consideration on Hybrid-codec Scalability and Inter-layer Prediction

## Problem Statement

In CfP for HEVC scalability document, experiment of codec standard scalability (Base-layer: AVC, Enhancement-layer: HEVC) is included as an option. In addition, it is proposed by m25749 that MPEG2-HEVC scalability with BLR (Base-Layer Reconstructed pixel only) be supported. It might be possible that proprietary codec be included in multi-codec scalable bitstreams.

It is assumed that HEVC version 2 will have flexibility that any layer can be any codec_type.

Ex.1 and Ex.2 in Fig 2.1.1 are the examples of hybrid-codec scalability. In this example, Layer L#n is the reference for inter-layer prediction of L#(n+1).

MPEG2-MPEG2 and AVC-AVC scalability have been already standardized. MPEG2-HEVC and AVC-HEVC scalability will be standardized within JCT framework, which might be with BLR scheme. However, MPEG2-AVC and AVC-MPEG2 scalability have not been standardized so it is impossible to apply inter-layer prediction.  Changing the existing standards so that MPEG2-AVC and AVC-MPEG2 scalability be included would not be feasible.

Fig 2.1.1

## Proposed Solution

To solve the problem stated above, it is proposed to introduce either of the following restrictions be satisfied for hybrid-codec scalability with inter-layer prediction:

1.	Codec types for the lower-layer and the upper-layer are same, or

2.	Codec type for the upper-layer is HEVC.

If proprietary codec that supports scalability be included, either condition 1 or 2 shall be satisfied. If the one that does not support scalability be included, condition 2 shall be satisfied.

# Conclusion

This document contains two topics on inter-layer prediction for HEVC scalable extension as follow:

- Inter-layer Prediction and Temporal-layer Depth

In HEVC scalable extension inter-layer prediction enabling/disabling will be associated with trade-off between coding efficiency and complexity: it will improve coding efficiency but will cause increase in complexity.

With lower temporal layers distance in time domain between the current and reference frames are rather long and performance of inter-frame prediction is not so high. In this case more CUs are coded as intra, and with scalable extension, inter-layer prediction will improve coding efficiency.

On the other hand, with higher temporal layers distance in time domain between the current and reference frames are rather short and inter-frame prediction do works. In this case inter-layer prediction is not so helpful for coding efficiency improvement.

Therefore this contribution proposes to introduce syntax element “max_sub_ref_layer[i][j]” in VPS, which specifies max temporal layer where inter-layer prediction be applied for scalable extensions.

- Consideration on Hybrid-codec Scalability and Inter-layer Prediction

To support hybrid-codec scalability it is proposed that either of the following conditions be satisfied between lower- and upper- layers for inter-layer prediction:

1.	Codec types for the lower-layer and the upper-layer are same, or

2.	Codec type for the upper-layer is HEVC.

If proprietary codec that supports scalability be included, either condition 1 or 2 shall be satisfied. If the one that does not support scalability be included, condition 2 shall be satisfied.

It is recommended that these topics be studied in HEVC scalable extension standardization activities.

# References

[1] “High efficiency video coding (HEVC) text specification draft 8”, JCTVC-J1003, July 2012

[2] “Solutions considered for NAL unit header and video parameter set for HEVC extensions”, JCTVC-J1007, July 2012

[3] “Joint Preliminary Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC)”, ISO/IEC JTC1/SC29/WG11 M25345, July 2012

[4] “Proposals for HEVC scalability Extension”, ISO/IEC JTC1/SC29/WG11 MPEG2012/m25749, July 2012

[5] “On support of mixed video sequences in High Level Syntaxes”, JCTVC-K0232, October 2012

[6] “VPS extension Updates in Strawman Design Approach 1 using a modified version of Scalability Map Table”, JCTVC-K0233, October 2012

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).