| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0176-r2 |
| --- | --- |

| Title: | AHG9: On LCU bit size limit |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal/Discussion |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 44 431 7588 chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution investigates the impact of the current LCU bit size limit on peak data transfer rates between DRAM and decoder. Simulation results using a synthesized test sequence show that, for QP=4, the LCU bit size limit of 1.33x expansion provides enough margins for encoder to conform while reducing the overall DRAM-decoder peak data transfer rates by 7%. It is suggested that the LCU bit size limit of 1.33x expansion or 1.60x is considered for the FDIS text adoption.

# Background

In the last JCTVC meeting, limit to 2x expansion in bits at LCU level was integrated into the DIS text, and further study was encouraged to investigate whether this should be lowered. This contribution investigates the LCU bit size limit from the viewpoint of peak data transfer rates between DRAM and decoder.

# Data transfer between DRAM and decoder

Figure 1 shows a data traffic model between DRAM and decoder: A decoder reads a chunk of bitstream from DRAM and stores them in a local buffer; the decoder also reads MC blocks of decoded pictures from DRAM for pixel reconstruction process; the reconstructed picture (decoded picture) is written to DRAM for subsequent picture decoding process and display process.

Figure 1: Data traffic between DRAM and Decoder.

In order to decode an LCU in any case, what the decoder needs to do are as follows:

- Read bits with the worst case LCU size after compression, that is, read 2x data of the original data size;

- Read MC blocks associated with worst case (8x8 bi-pred), that is, read 6.7x data of the original data size.

According to the above, Table 1 summarizes DRAM-decoder peak data transfer rates of different-resolution video decoding. (Note that motion information, etc. are excluded.) Notice that bitstream read rates take about 20% of overall DRAM-decoder peak data transfer rates and the portion is not insignificant.

Table 1: Peak data transfer rates of different-resolution (4:2:0 8-bit) video decoding with LCU bit size limit of 2x expansion.

It is reminded here that DRAM data transfer rates can not be gradually increased since they are configured by clock rates x bit width. For DDR2 SDRAM, their typical clock rates are 333MHz, 400MHz, and 533 MHz, while bit width is power of 2. Therefore if peak data transfer rates of video decoding process exceed the limit of DRAM data transfer, for example, only by 1%, a system needs to upgrade DRAM configuration either by increasing the clock rates or by doubling the bit width. The upgrade of clock rates results in higher power consumption; the upgrade of bit width increases the number of pins. Therefore it is desirable to reduce DRAM-decoder peak data transfer rates as much as possible.

# Discussion

The current DIS text specifies the LCU bit size limit of 2x expansion. If the limit is lowered with consideration of enough margins for encoder to conform, the DRAM-decoder peak data transfer rates are significantly reduced since their bitstream read rates take about 20% of overall DRAM-decoder peak data transfer rates. We investigate such a lower limit by using a synthesized test sequence depicted in Figure 2. As a typical example, Table 2 shows the average of 64x64 LCU bit size in encoding the synthesized test sequence by using low QPs with the RA settings. In Table 2, "Data expansion ratio" column shows the ratio of the number of LCU bit size to the original one. (Note that the LCU bit size is on average but their tight distribution was confirmed. Details of simulation setting is described in Appendix A. )

As shown in Table 2, for QP=4, the LCU bit size limit of 1.33x expansion proposed in JCTVC-J0082 provides some margins for encoder to conform. This limit reduces the bitstream read rates and the overall DRAM-decoder peak data transfer rates by 67% and 7%, respectively, as shown in Table 3.

If the limit of 1.33x expansion is tight for encoder to conform, the LCU bit size limit of 1.60x can be alternative one. This limit is translated into 4% reduction in the overall DRAM-decoder peak data transfer rates, as shown in Table 4.

Table 2: Summary results for 64x64 LCU bit size.

| QP | RA-MAIN | QP | RA-HE10 |
| --- | --- | --- | --- |
|  | Data expansion ratio |  | Data expansion ratio |
| 0 | 1.42 | -12 | 1.73 |
| 4 | 1.26 | -8 | 1.58 |
| 8 | 1.17 | -4 | 1.41 |
| 12 | 1.02 | 0 | 1.15 |

Figure 2: Synthesized test sequence, Sandstorms, (640x320 30Hz 30frames)

Table 3: Peak data transfer rates of different-resolution (4:2:0 8-bit) video decoding with LCU bit size limit of 1.33x expansion.

Table 4: Peak data transfer rates of different-resolution (4:2:0 8-bit) video decoding with LCU bit size limit of 1.60x expansion.

# Conclusions

This contribution has investigated the impact of the LCU bit size limit on peak data transfer rates between DRAM and decoder. Simulation results using a synthesized test sequence have shown that, for QP=4, the LCU bit size limit of 1.33x expansion provides enough margins for encoder to conform while reducing the overall DRAM-decoder peak data transfer rates by 7%. It is suggested that two typos, “coding_tree()” and “coding tree block”, are revised and that the LCU bit size limit of 1.33x expansion or 1.60x expansion is considered for the FDIS text adoption. The text change to JCTVC-K0030_v3 is shown as follows:

- The number of times read_bits( 1 ) is called in subclauses 9.3.3.2.2 and 9.3.3.2.3 when parsing coding_quadtree( ) data for any coding tree uni shall not be greater than 512 768 * ( bit_depth_luma_minus8 + 8 ) * ( 1 << ( Log2CtbSizeY − 4 ) ) * ( 1 << ( Log2CtbSizeY − 4 ) ).

# Patent rights declaration(s)

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Simulation settings

The synthesized test sequence was generated by using “JCTVC-J0059-Sandstorm-Software.zip” attached in JCTVC-J0059.

Modified versions of the RA-Main and RA-HE10 settings in JCTVC-J1100 are used: QP=0,4,8,and 12 are used for the RA-Main setting, and QP=-12,-8,-4 and 0 are used for the RA-HE10 setting; LCU size is varied from 16x16 to 64x64; QPoffset values are set to 0 in order to see the behavior in a given QP value; ("JCTVC-K0176-cfg.zip" and "JCTVC-K0176-bat.zip" contain the configuration files and MS-DOS batch files, respectively. )

The numbers of bits and bins of LCUs are gathered by the attached patch "JCTVC-K0176-HM-8.0-decoder.patch."